(Comp.sys.handhelds)

Item: 1360 by metcalf at akala.ifa.hawaii.edu

Author: [Tom Metcalf]

Subj: HP48 Celestial Navigation

Date: Thu Dec 06 1990 09:41

I posted the following sight reduction program the other day with

a slight bug.  Here is the corrected version.  Sorry to waste bandwidth!

---------------------------------------------------------

Following is an HP-48SX program I wrote to do celestial navigation.

It computes a position fix from observations of any number of celestial

bodies using a least squares fit to the altitude of the bodies as a function

of time.  The routine does all the standard corrections for dip, refraction,

parallax etc. as well as correcting for motion of the observer between sights.

It does not compute the GHA/declination of celestial bodies, so a copy of

the nautical almanac is required to use these routines.  It will, however,

interpolate the GHA and declination from the hourly entries on the daily pages

of the nautical almanac.

A detailed description of the mathematical basis of the algorithm is

available upon request.

----------------------------------------------------------

Instructions

There are several steps to go through to get a fix from a set of observations.

When prompted for input, key in the requested data, and press "ENTER".

All angular inputs must be in degrees and in the "hms" format: dddd.mmss,

where dddd is degrees, mm is minutes and ss is seconds of arc.  All times

must also be input in the "hms" format.

For example, 16.3427 is 16 degrees 34 minutes 27 seconds if an angle is

being input and 14.0153 is 14:01:53 if a time is being input.

The output is the optimum latitude and longitude, both in "hms" format.

North latitude and west longitude are positive numbers, while south latitude

and east longitude are negative numbers.  For example,

157 deg 49 min 58 sec W, 21 deg 17 min 30 sec N would be output as

LON:  157.4958

LAT:   21.1730

1. If you want to start a new set of observations purge the variable

"obs".  This variable stores all the observations, so, to start over, this

variable must be removed.  You may want to rename it rather than remove it

if it will be useful at a later time.

2. Run the program "setup".  This sets up the appropriate corrections

and the GHA-declination interpolation

for the observed body.  This program must be run whenever a new body

is observed or whenever the observations have extended beyond the

times given for the GHA/declination interpolation (TIM1,TIM2)

since the interpolation will become inaccurate.

If you are observing more than one body, input all the

observations for each before proceeding to the next (temporal order

does not matter).  The "setup" program asks for the following input:

a) BODY: S is for the Sun, M is for the Moon, VM

is for Venus or Mars, and anything else assumes a star or other planet.

Note that alpha mode is automatically initiated.

b) INDEX:  The index correction (degrees, in hms format) which is to be

*added* to the observed sextant altitude,  e.g. 1' should be

input as 0.0100.

c) SEMI-D: Semi-diameter in degrees, "hms" format: e.g. 16.2 min

should be input as 0.1612 since 0.2 min = 12" (Sun only).

d) HP: Parallax in degrees, "hms" format (Moon,Venus,Mars only).

e) LIMB: For Upper limb enter "1", for lower limb enter "-1", and

for disk center enter "0" (Sun/Moon only).

f) HEIGHT: Height above water (in meters) at which the observations

were taken (for the dip correction).

g) GHA1 DEC1 TIM1: The Greenwich Hour Angle and declination at time TIM1.

The actual values for the observations will be interpolated linearly

from this value and the next.  TIM1 should be a whole hour near the

observation times.  All three numbers should be input on the appropriate

line *before* pressing "ENTER".  To move to the next item, use the

down-arrow key.  All three entries must be in "hms" format.

h) GHA2 DEC2 TIM2: The second set of values for the linear interpolation.

TIM2 should be a whole hour after TIM1 and in "hms" format

(generally, TIM1 and TIM2 should be consecutive hours).  All observations

must be between TIM1 and TIM2.  If this is not the case, the observations

should be input in several groups, running "setup " between groups.

If the GHA passes through zero between GHA1 and GHA2, 360 degrees should

be added to GHA2.

i) SPEED: Speed of vessel during the observations (knots). If the

speed is zero, the program will terminate at this point.

j) COURSE: True course of vessel during the observations (hms format).

k) DR LAT LON:  Dead reckoning latitude and longitude to use in the

correction of the observations for course and speed (hms format).

Negative values indicate East longitude or South latitude.

l) TIME OF FIX: The time to which the course and speed corrections

are made (hms format).  This will be the time at which the fix is valid.

3.  Enter the observations:

a) Enter the time of the observation (hms format) and then the

uncorrected altitude (hms format) onto the stack.

b) Run the "correct" program to correct the observations for index,

dip, refraction (for standard conditions), parallax, and course/speed.

c) Run the "addob" program to add the observation to the "obs" variable.

d) Repeat until all observations are input.

e) Accurate dead reckoning information can be included by editing

the "obs" variable and inputing an additional row into  the matrix with

the first element equal to the DR longitude (West positive), the second

element equal to the DR latitude (North positive), and the final element

equal to 90.00.  The MatrixWriter application is particularly useful

for editing the "obs" variable.

4.  Get the fix by running the "solve" program.  This program can be run at

any time when there are at least 3 observations (including dead reckoning)

in "obs".  It does not affect "obs", so more observations can be input

after running "solve".  If "convergence error" appears or if the position

estimate is far from your dead reckoning position, there is probably an

error in the input data and it should be reentered (if the data is correct

and "convergence error" appears,  the position fix should not be trusted).

Remember: the fix is, at best, only as good as the data you supply, and

you should examine the results critically!

5.  If all observations are for a *single celestial body*, run the "error"

program to get an estimate of the position error (miles).  This program

assumes an error on the observations of one arc minute, and should be

multiplied by the actual sextant error in arc minutes if it is other than

one.

Important note:  If observations of more than one body are input, "setup"

must be run before starting the input for each body.

Disclaimer:

This software is provided "as is" and is subject to change without

notice.  No warranty of any kind is made with regard to this software,

including, but not limited to, the implied warranties of merchantability

and fitness for a particular purpose.  The author shall not be liable for

any errors or for incidental or consequential damages in connection with

the furnishing, performance, or use of this software.

Copyright 1990 by Thomas R. Metcalf.  Permission is granted to any

individual or institution to use, copy, or redistribute this software

so long as it is not sold for profit and provided that this copyright

notice and the above disclaimer are retained.