Article: 4032 of comp.sys.handhelds

Path: jpl-devvax!elroy.jpl.nasa.gov!usc!orion.oac.uci.edu!!nelson

From: (Matt Nelson)

Subject: Airfoil Plotter for HP48sx

Summary: draws airfoils of arbitrary size

Date: 29 Oct 90

Reply-To: nelson@psroot.ps.uci.edu

Organization: University of California, Irvine

This is for all of the R/C model airplane/sailplane enthusiasts out there,

especially if you are into designing and building.

Here is the latest version of my airfoil section plotter.  I needed to plot

some ribs for a glider project, and realized that I had sold my trusty HP-28s.

I quickly re-wrote my original program to run on the HP-48sx, and came up with

something MUCH prettier, and MUCH MUCH faster.  This thing is really quite

painless to use now (the 28s version was slooooow).

The speedup came from creating a PICT that is big enough to contain the

entire airfoil section.  The airfoil data is plotted only once, and then sent

to the printer, one screen at a time, through successive PVIEW and PRLCD

commands.  BE WARNED...  although this is a very fast way to print out large

objects, it is also a memory-hungry way to do it; for example, a 10" SD7003

required a PICT that was 131 by 832.  When I saved this PICT to the stack and

BYTESed it, it was 14,154 bytes big.  On top of that, just the data for the

SD7003 (as listed below) is 988.5 bytes long.  All I am saying is that this

program DOES use a lot of memory, but is very handy, and serves my purposes

very well; if you don't like it, feel free to use something else.

To use this program, simply get into the AIRFOIL directory, and hit

[AIRFOI].  It will ask for an airfoil.  Just hit the menu button corresponding

to the airfoil data you wish to use (you can hit [SD700] to try it out), then

[ENTER].  It will then ask for a chord.  Give it a number corresponding to the

desired chord, in centimeters, then [ENTER].  The program will handle other

units, and in fact presents you with the UNITS LENG menu, so you can give it a

chord size in any unit you wish.  Just be sure to give it a valid unit object

(you have to supply the underscore manually, since the calculator is in program

entry mode at this point).  You should be greeted by a "working..." message.

Point the calculator at your 82240A/B printer, and watch the airfoil section

print out.

The airfoil data is assumed to be in a complex vector, where each element

corresponds to a point (x,y) on the airfoil.  The points should be arranged so

that if you were to draw lines sequentially from one point to the next, you

would trace out a closed outline of the airfoil section.  The sample airfoil in

this posting, the SD7003, should help illustrate this.  For those who are

interested, the SD7003 is an airfoil designed specifically for use at very low

Reynolds numbers, such as those encountered by model sailplanes.

There are two parameters which control the absolute size of the printed

output.  These are the global variables ycmps and xcmps.  These represent the

number of centimeters per screen OF PRINTED OUTPUT for the y and x directions,

respectfully.  One arrives at these values by printing a screenfull of

anything, and measuring the size of the printed output in centimeters.  The

numbers supplied below are for the 82240A printer; I have never used a 'B'

version of the printer, and don't know whether these numbers will have to be

changed in order to work properly with that printer.  The only reason I left

these numbers as global variables is so that the user could have some control

over the aspect ratio of the output, without diving into the program itself.

Please direct any comments/suggestions/questions etc.  to:

Matt Nelson           internet:  nelson@psroot.ps.uci.edu

Physics Dept.         bitnet:    mnelson@uci

University of California   phone:     (714) 725-2629

Irvine, CA 92717