(Comp.sys.handhelds)

Item: 2002 by bill at flutter.tv.tek.com

Author: [William K. McFadden]

Subj: Speaker Design Equations

Date: Fri Feb 08 1991

I have put together a library of equations for designing ported and

closed-box speaker enclosures.  The basic design equations were taken from

a couple of hobbyist-oriented speaker design books.  The power rating

equations were taken from papers by Richard Small (see references below).

The equations are intended to be used with the multiple equation solver in

the equation library ROM card.  I welcome any comments or refinements.

The main directory is called SPKR and consists of two subdirectories:

CB      Closed Box Design

PORTED  Ported Box Design

Running the multiple equation solver from either subdirectory will produce

a menu of variables:

Vas     Volume of air having same acoustic complaince as driver suspension

Qts     Total driver Q at Fs

Fs      Resonant frequency of driver

SPL     Efficiency of driver in dB SPL at 1W/1m

DIA     Diameter of driver

xmax    Peak displacement limit of driver diaphragm (1/2 of "throw")

Vb      Inside volume of enclosure

Fb      Resonance frequency of enclosure

F3dB    Half-power (-3 dB) frequency of loudspeaker system response

dBPEAK  Maximum peak or dip of loudspeaker system response

Par     Acoustic power rating

PeakSPL Equivalent sound pressure level (at 1m) of acoustic power rating

Per     Electrical power rating (worst case)

\Gno    Percent driver efficiency (\Gn is greek character eta)

Sd      Effective projected surface area of driver diaphragm (approximated)

Vd      Peak displacement volume of driver diaphragm

In addition, the following variables are defined for the closed box case:

Qb      Total Q of system at Fb

AMAX    Maximum amplitude of loudspeaker system response: 10^(dBPEAK/20)

Vr      Ratio of Vas to Vb

Qr      Ratio of Qb to Qts and Fb to Fs

For the ported box case, the following apply:

1. Fb is the tuning frequency for the vent.

2. Most of the results are approximate.

To use, run MSOLVR in either directory.  Enter the speaker parameters into

the variables Vas, Qts, Fs, SPL, DIA, xmax.  (If you don't have all the

parameters available, purge the ones you don't know, so they'll be

undefined and the solver won't attempt to use them.)  For the closed-box

case, define one of Vb or Qb and solve for the other (or make it a

calculated value with MCALC).  Pressing <- ALL will solve for all the

unknowns for which a solution exists (indicated by a small box in the

menu).  This takes about 2.5 minutes for the closed box and about half as

long for the ported box.

To find the optimum box size for the closed box system, set Qb=0.707

(=1/sqrt(2)) and solve for Vb.  Solving for Vb for the ported box always

finds the optimum box size.  The optimum box size is defined as the size

which produces no peak or dip in the frequency response (e.g., dBPEAK=0).

(A B2 response is used for the optimum closed box, and B4 for the ported

box.)

To solve for given box size, for the closed box system, enter a value for

Vb, type 'Qb' MCALC, and solve for any or all unknowns.  For the ported

box, enter a value for Vb and solve for the unknowns.  To return to the

optimum enclosure, for the closed box, set Qb = 0.707 and type 'Vb' MCALC.

For the ported box, type 'Vb' MCALC.

To run a frequency response plot, press -> PLOT.  The X axis is frequency,

and the Y axis is the magnitude of the response in dB. Change the ranges,

if desired, and press ERASE and DRAW.  It takes about a minute for the

closed box, and four minutes for the ported box.

You can also use the built-in solver to locate points of interest in the

frequency response by pressing -> SOLVE.

If you get curious, the design equations are in a list called DESIGN.EQ,

and the frequency response equation is in a variable called RESPONSE.

There is a subdirectory in CB called EQUALIZER that will find the component

values for an active equalizer that can extend F3dB of any closed box

system to any desired lower limit (at the expense of efficiency and power

handling--watch out!)  See pp. 142 of the March 1990 AES Journal for theory

and circuit details.

First, use the multiple equation solver in the CB directory to solve for

the system as shown above.  Next, enter the EQUALIZER subdirectory.  Enter

the new desired cutoff frequency into F3dB, and press CIRCUIT.  The

component values will appear in the display.  The values of R, C, N are

chosen by the user to make the remaining component values realistic (see

article).

You can run a response plot of the equalizer with -> PLOT.  It's pretty

interesting, but takes FOREVER (like 20 min.).  The reason is I copied the

equations right out of the article without any optimization for speed.  (If

anybody wants to tackle this, be my guest.)  Wherever possible, I left out

the units so it would run faster.  You can also solve for points of

interest with -> SOLVE.  The point where maximum boost occurs is at F3dB.

If you put this in for f and solve for dB, you will see how much boost is

needed without having to wait all day. (Don't enter values for Fb and Qb;

they are defined in the parent directory, and entering values will redefine

them locally.  If you do this by mistake, purge Fb and Qb.)  Efficiency and

power handling of the system at this frequency will be degraded by this

amount if the equalizer is used.  This gives a pretty good worst case

scenario.

Don't be surprised if more than 20 dB of boost is needed to get down to 20

Hz, even for large drivers.  "There ain't no such thing as a free lunch."

If you don't need the equalizer program, just PGDIR the EQUALIZER

subdirectory.  Doing so will save about 1.6K.

By the way, the default speaker parameters when you first download the file

are for the Eminence 18029 18" driver.

The following is a small tutorial on speaker enclosures.

An optimum enclosure is defined as one that has no peak or droop in the

passband.

The power rating of each driver is given in watts RMS.  This is the

continuous thermal power rating of the speaker.  Most speakers can handle

two to four times as much power for brief periods without overheating.

The efficiency of the speaker is given in decibels of sound pressure level

(SPL).  0 dB SPL is defined as 2.0E-10 bar (2.0E-5 N/m^2), which is the

lowest level of 1 KHz tone the ear can detect.  A 10 dB increase in SPL

results in an apparent doubling of the loudness and requires 10 times as

much acoustic power.  Accordingly, a 10 dB decrease halves the loudness and

reduces the acoustic power by a factor of 10.

Most driver manufacturers specify the SPL of the driver with a one watt

input measured at a distance one meter away.  To calculate the SPL at other

power levels, add the following number to the SPL rating: 10*log(POWER),

where POWER is in watts, and the log is base 10.  This equation is derived

from the fact that a doubling of electrical power produces an doubling of

acoustic power.  To calculate the SPL at other distances, subtract the

following number from the SPL rating: 20*log(DISTANCE), where distance is

in meters.  This equation is derived from the inverse square law of wave

propagation.

One watt of acoustic power is equal to about 112 dB SPL at one meter. To

calculate the efficiency of the speaker in percent, use the following:

%EFFICIENCY = 100*(10^((RATING - 112)/10)), where RATING is the driver's

SPL rating in dB, at one watt, measured at one meter.  For example, a

driver with a 92 dB SPL rating @ 1W/1m is 1% efficient.

For the sealed box enclusure, the optimum volume in cubic feet can be

determined.  Many designers like to use a 0.62:1:1.62 ratio for the cabinet

dimensions.  This is known as the golden ratio.  A box designed to this

ratio will be less peaky than one whose dimensions are equal. Another ratio

sometimes used is 0.8:1:1.25.  You can determine the middle dimension by

taking the cube root of the enclosure volume. (Keep in mind this is the

inside volume and doesn't take into account the volume taken up by bracing

materials and the driver itself.)  The box will have a resonant frequency

and a Q.  For an optimum sealed box, the resonant frequency is equal to the

-3dB point, and the Q is 0.707. The frequency (in Hz) at which the

speaker's response is 3 dB down can be found.  This is also known as the

half-power point, because it is the frequency at which the acoustic output

power drops by half.  Below this frequency, the response will have a second

order roll off, e.g., the output decreases 12 dB for every halving of the

frequency below the -3 dB point.

The ported enclosure is a little more complicated.  As with the sealed box,

the ported enclosure has an optimum volume (stated in cubic feet) and -3 dB

point (stated in Hz).  The speaker also has a tuning frequency, called Fb.

This is the resonant frequency of the enclosure's duct.  The tuning

frequency is determined by the cross sectional area and length of the duct.

You may consult a book on speaker design to determine the proper duct size.

Ported enclosures have a steeper roll off than sealed boxes.  The roll off

is fourth order, or 24dB for every halving of the frequency below the -3dB

point.  At very low frequencies, the driver will be undamped, hence the

speaker could be damaged by excessive cone movement.  It is therefore wise

to roll off the signal below the -3dB frequency to avoid damage. This

constraint does not apply to sealed boxes, which damp cone movement at all

frequencies.

REFERENCES:

[1] Hobbyist speaker building books, such as the one sold at Radio Shack.

[2] L.L. Beranek, Acoustics (McGraw-Hill, New York, 1954).

[3] J.F. Novak, "Performance of Enclosures for Low-Resonance

High-Compliance Loudspeakers," J. Audio Eng. Soc., vol. 7, p 29 (Jan.

1959).

[4] A.N. Thiele, "Loudspeakers in Vented Boxes, Parts I and II," J. Audio

Eng. Soc., vol. 19, pp. 382-392 (1971 May); pp. 471-483 (1971 June).

[5] R.H. Small, "Direct-Radiator Loudspeaker System Analysis," J. Audio

Eng. Soc., vol. 20, p. 383 (June 1972).

[6] R.H. Small, "Closed-Box Loudspeaker Systems," J. Audio Eng.  Soc., vol.

20, p. 798 (Dec. 1972), and vol. 21, p. 11 (Jan/Feb 1973).

[7] R.H. Small, "Vented-Box Loudspeaker Systems," J. Audio Eng.  Soc., vol.

21, (four parts, starting in the June 1973 issue).

[8] W.M. Leach, Jr., "A Generalized Active Equalizer for Closed-Box

Loudspeaker Systems," J. Audio Eng. Soc., Vol. 38, pp. 142-145 (March

1990).

[1] is useful as an introduction and has a lot of construction tips.

[2] is a standard reference text that seems to be the industry bible.

[3] is historically significant, and is the foundation for [4].

[4] and [6] are the landmark works on loudspeaker systems (you can't

consider youself knowledgeable without having read them).

[5] is background for [6], and [7].

[7] updates the original work of [4].

[8] is a recent paper that shows how to equalize closed-box systems to any

desired low-frequency cutoff.  [3], [4], [5], [6], and [7] are reprinted in

the AES two-part "Loudspeakers" anthology.

--

Bill McFadden    Tektronix, Inc.  P.O. Box 500  MS 58-639  Beaverton, OR

97077

bill@videovax.tv.tek.com,

{hplabs,uw-beaver,decvax}!tektronix!videovax!bill

Phone: (503) 627-6920             "SCUD: Shoots Crooked, Usually Destroyed"