1.  Getting Started

SASM.EXE is an assembler for the Saturn processor family.

As a historical note, the Saturn processor first appeared in

the HP 71B, and has since been used in various fashions for

other HP calculator models, including the HP 28 and the

HP 48.

The file SASM.EXE should be installed in a directory such as

BIN, with the PATH variable set to include that directory.

An environment variable SASM_LIB should be set to point to

the file SASM.OPC.  For instance, if the opcode file is in

\LIB, the command "SET SASM_LIB=C:\LIB" should be added to

the autoexec.bat file.

A file naming convention extends throughout the Saturn

development environment.  Sasm accepts input from files with

a ".a" extension, and produces listing files with a ".l"

extension and code files with a ".o" extension.

Several options are available:

Option     Description

---------  ----------------------------------------------------------

A          Write listing to stdout

a lstfile  Write listing to "lstfile"

c column   Fields starting after "column" are considered comments

D sym=val  Defines symbol "sym" to have value "val" (default=1)

d dbgfile  Write debug information to "dbgfile"

E          Write C-like error messages to stderr

e          Write error messages to stderr

f flglist  Sets the flags indicated by "flglist" (comma separated)

H          Write object file as raw code (no object header or symbols)

h          Write object file as hexadecimal characters (no object

header or symbols)

N          Suppress listing entirely

o objfile  Write object file to "objfile"

P plevel   Sets the processor level to "plevel" (0, 1, 2, or 3)

p patelen  Do a page break each "pagelen" lines

t opcfile  Read opcodes from file "opcfile"

w width    Set output page width to "width" columns (default 80)

Page 1

Here is an example to illustrate Saturn coding and show what

a typical Saturn source file looks like.  The name of the

source file is "example.a".  The Saturn instructions are

explained later in this manual.

TITLE   Example Saturn Assembly file

*

* External entry point: =ENTRY1

*

* This routine shifts the C register right 6 digits

* without altering the sticky bit value

*

=ENTRY1 P=      5

C=0     WP

ENTRY2  CSR     W

P=P-1

GONC    ENTRY2

P=      0

RTNCC

END

Assembling this file with the command "sasm example.a"

generates a Saturn object file (example.o), and a list file

(example.l) which is shown on the next page.

Page 2

Saturn Assembler  Example Saturn Assembly file  Tue Jul 21 16:35:38 1987

Ver. 1.40, 7/21/87                              example.a      Page    1

1                     TITLE   Example Saturn Assembly file

2             *

3             * External entry point: =ENTRY1

4             *

5             * This routine shifts the C register right 6 digits

6             * without altering the sticky bit value

7             *

8 00000 25    =ENTRY1 P=      5

9 00002 A92           C=0     WP

10 00005 BF6   ENTRY2  CSR     W

11 00008 0D            P=P-1

12 0000A 5AF           GONC    ENTRY2

13 0000D 20            P=      0

14 0000F 03            RTNCC

15 00011               END

[page break]

Saturn Assembler  Example Saturn Assembly file  Tue Jul 21 16:35:38 1987

Ver. 1.40, 7/21/87  Symbol Table                example.a      Page    2

=ENTRY1        Rel       0 #00000000 -     8

ENTRY2        Rel       5 #00000005 -    10    12

[page break]

Saturn Assembler  Example Saturn Assembly file  Tue Jul 21 16:35:38 1987

Ver. 1.40, 7/21/87  Statistics                  example.a      Page    3

Input Parameters

Source file name is example.a

Listing file name is example.l

Object file name is example.o

Flags set on command line

None

Errors

None

Page 3

2.  Saturn CPU Overview

The Saturn CPU is a proprietary CPU optimized for high-

accuracy BCD math and low power consumption.  The data path

is 4 bits wide.  Memory is accessed in 4-bit quantities

called ``nibbles'' or ``nibs''.  Addresses are 20 bits,

yielding a physical address space of 512K bytes.

2.1  Registers

There are four working 64-bit registers, five scratch 64-bit

registers, two 20-bit data pointer registers, one 4-bit

pointer register, a 20-bit program counter, a 16-bit input

register, and a 12-bit output register.  Return addresses

are stored on an eight-level hardware return stack that

accepts 20-bit addresses.  In addition, there are 4 Hardware

Status bits, a Carry bit, and 16 Program Status bits.  The

lower 12 Program Status bits can be manipulated as a 12-bit

register.

2.2  Working and Scratch Registers

The working registers A, B, C, and D are used for data

manipulation.  Working registers A and C are also used for

memory access.  The scratch registers R0, R1, R2, R3, and R4

are used to temporarily hold the contents of working

registers.

Page 4

2.3  Field Selection

Subfields of the working registers A, B, C, and D may be

accessed by the use of field selection.  The possible field

selections range from the entire register to any single

nibble of the register.  Certain subfields are designed for

use in BCD calculations.  Others are used for data access or

general data manipulation.

Field Selection Description

P Nibble indicated by P register

WP Nibbles from nibble P through nibble 0, inclusive

XS Nibble 2;- Exponent sign

X Nibbles 2-0;- Exponent including exponent sign

S Nibble 15;- Mantissa sign

M Nibbles 14-3;- Mantissa

B Nibbles 1-0;- Byte field

A Nibbles 4-0;- Address field (20 bits)

W Nibbles 15-0;- Word (entire 64-bit register)

Register Nibbles

+-----------------------------------------------+

|15|14|13|12|11|10| 9| 8| 7| 6| 5| 4| 3| 2| 1| 0|

+-----------------------------------------------+

| S|                                   |XS|  B  |

|                 <- M ->           | <-X->  |

|   <- A ->    |

|                    <- W ->                    |

2.4  Pointer Registers

The 20 bit Data Pointer registers D0 and D1 are used to

contain addresses during memory access, and are used in

conjunction with the working registers.

The 4 bit Pointer register P is used in Field Selection

operations with the working registers.

2.5  Input, Output, and Program Counter Registers

The input/output registers are used to communicate with the

system bus.  The program counter points to the next

instruction to be executed by the CPU.  The input register

IN is 16 bits, the output register OUT is 12 bits, and the

PC register is 20 bits.

Page 5

2.6  Carry, Program Status, and Hardware Status Bits

The Carry bit is adjusted when a calculation or logical test

is performed.  During a calculation, such as incrementing or

decrementing a register, it is set if the calculation

overflows or borrows; otherwise it is cleared.  During a

logical test, such as comparing two registers for equality,

it is set if the test is true; otherwise it is cleared.

The upper 4 Program Status bits are typically used to

indicate the state of the operating system.  The remaining

12 Program Status bits are generally available to

applications software, and may be manipulated collectively

as the ST register.

The four Hardware Status bits are set (but not cleared) by

hardware-related events, and must therefore be cleared

beforehand in order to detect a particular occurrence.  They

are individually accessible by name.  The Module Pulled bit

(MP) is set whenever the *NINTX CPU input is pulled low

(regardless of whether an interrupt is actually executed).

The Sticky Bit (SB) is set when a non-zero bit shifts off

the right end of a working register as the result of a shift

instruction.  The Service Request (SR) bit is set as a

result of the SREQ?  instruction if any hardware service

request is pending.  The external Module Missing bit is set

by execution of a ``00'' opcode (RTNSXM instruction).

Hardware Status:  4 bits

Bit  Symbol  Name

---  ------  ------------------------------------

3     MP    Module Pulled (*NINTX pulled low)

2     SR    Service Request

1     SB    Sticky Bit

0     XM    External Module Missing

Page 6

2.7  Arithmetic Mode

The arithmetic mode is set by the SETHEX and SETDEC

instructions.  When SETHEX is executed, the arithmetic mode

is set so that all register arithmetic is performed in

hexadecimal mode.  When SETDEC is executed, the arithmetic

mode is set so that most register arithmetic is performed in

decimal mode.  The following instructions are *always*

performed in hexadecimal mode, regardless of the arithmetic

mode setting:

P=P+1

P=P-1

C+P+1

D0=D0+     n   D0=D0-     n

D1=D1+     n   D1=D1-     n

A=A+CON fs,n   A=A-CON fs,n

B=B+CON fs,n   B=B-CON fs,n

C=C+CON fs,n   C=C-CON fs,n

D=D+CON fs,n   D=D-CON fs,n

The arithmetic mode is not ``readable'', but can be inferred

by doing an appropriate operation, followed by a test.  For

example:

LCHEX   9

C=C+1   P

sets the carry if, and only if, the arithmetic mode is

decimal.

2.8  Loading Data from Memory

When data is read from memory into a register, the CPU

places the lowest addressed nibble in the least significant

nibble of the register.

For example, if the data shown below in memory is read into

the C register using the C=DAT1  4 instruction, the data in

the register will be arranged as shown.

Memory

Location  Value              C Register

--------  -----      +------------------------+

1000      6        |  | ... | 9 | 8 | 7 | 6 |

1001      7        +------------------------+

1002      8         15  ...   3   2   1   0

1003      9

This principle applies also to loading constants into a CPU

register such as C, D0, or D1, since the CPU must read the

constant from the instruction opcode in memory.  For

example, the instruction LCHEX 9876 produces the opcode

Page 7

336789 and the C register is loaded as shown above (assuming

P= 0).

2.9  Storing Data in Memory

When data is written from a register to memory, the CPU

places the least significant nibble of the register in the

lowest nibble of the addressed memory location.  For

example, if the data shown above in the C register is

written to memory using the DAT1=C  4 instruction, the data

will be written to memory as shown.

2.10  Interrupt System

All Saturn CPU interrupts cause a subroutine jump to address

#0000F.  Determining the cause of the interrupt is up to the

interrupt service routine at that address.

Page 8

3.  Conditional Assembly

The assembler supports conditional assembly tests to allow

different code to be generated based on various conditions.

Conditions which can be tested include assembly flags set on

the command line or modified with the SETFLAG or CLRFLAG

instructions, the value of an assemble-time expression

compared to zero, the presence or absence of a symbol

definition, the (guaranteed) carry state (for example, after

GOYES the carry is clear), the current assembler pass

(useful for messages), and the presence of an opcode or

macro definition.

The structure of a conditional assembly block is:

label   IF

<<code if condition is true>>

label   ELSE

<<code if condition is false>>

label   ENDIF

The label and ELSE sections are optional.

Conditional assembly blocks can be nested up to a maximum of

20 levels if a unique label is present for each nesting.  If

the label is omitted or not unique, the code assembled may

not nest as expected for the false block.  For example:

1               IF      0

2               IF      1

3       * Flags 0 and 1 are set

4               ELSE

5       * Flag 0 is set, flag 1 is clear

6               ENDIF

7               ELSE

8               IF      1

9       * Flag 0 is clear, flag 1 is set

10               ELSE

11       * Flags 0 and 1 are clear

12               ENDIF

13               ENDIF

If flag 0 is clear, the ELSE on line 4 is found as the ELSE

matching the IF      0.  The ELSE on line 7 and the ENDIF on

line 13 are flagged as errors (ELSE without matching IF,

ENDIF without matching IF).  Lines 5 and 11 are both

assembled.  If the nested IF statements had unique labels,

they would work as expected.

For a list of the conditional assembly opcodes, see

``Conditional Assembly'' in the ``Pseudo-Op Instructions''

section of the ``Saturn Assembler Mnemonics" appendix.

Page 9

4.  Using Macros

This chapter explains how to create and use macros in your

source files.  The macro directives are MACRO, EXITM, and

ENDM.

A macro is a named block of source statements.  When a macro

name is used as a statement, it is automatically replaced by

the block of source statements it represents.

Macro definitions cannot be nested; the MACRO statement is

illegal within a macro call.

Macro lines are not listed by default; to enable listing of

the macro expansion, use either the LISTM or SETLIST MACRO

statement.

4.1  Defining a Macro

A macro definition consists of a MACRO statement, followed

by the source statements to make up the macro, followed by

an ENDM statement.  The label on the ENDM statement, if any,

must match the name of the macro (the label on the MACRO

statement).  Text which follows the MACRO statement on the

same line is ignored (comment only).  It is suggested that

the comments indicate the parameters which are expected when

the macro is called.  Up to nine parameters can be passed to

a macro when it is called.  The statements between the MACRO

statement and the ENDM statement are not assembled until the

macro is called.  Any assembler pseudo-ops within a macro

definition are executed when the macro is called, not when

it is defined.  An exclamation mark (!) in the first column

is removed from the line when the macro is called.  The rest

of the line is included in the expansion.

Blank lines and comment lines are not normally included in

the macro expansion.  To include a blank line or a comment

line in a macro expansion, add an exclamation mark in the

first column.

The dollar sign ($) is used in macros as a text substitution

character.  The character which follows the dollar sign

indicates the replacement:

Sequence  Replacement Text

--------  ----------------------------------------------------

$$     $ (one dollar sign)

$0     Line number on which the macro was called

$<     Current source file name

$n     Parameter reference (1_n_9)

$(n)    Indirect parameter reference (1_n_9)

$(nf)   Formatted indirect parameter reference (1_n_9)

Parameter references are replaced by the corresponding

parameter text.

Page 10

Indirect parameter references are replaced by the value of

the corresponding parameter when interpreted as an

expression.  Formatted indirect parameter references include

a format string f which follows the parameter number.  The

format string has the form [ : ] [ length ] [ format char ].

Length indicates the minimum number of characters to use.

The value is zero-filled if it requires fewer than length

digits.  The default length is one.  Format char controls

the radix and case of the value.  The table shows valid

format char values:

Character    Description       Digit Characters

---------  ----------------  ---------------------

H,X      Hexadecimal       0123456789ABCDEF

h,x      Hexadecimal       0123456789abcdef

D,d      Signed decimal    0123456789  (default)

U,u      Unsigned decimal  0123456789

O,o      Octal             01234567

The maximum combined nesting depth for macros and include

files is 20 levels.

4.2  Calling a Macro

To call a macro, specify the name of the macro as an

instruction.  The macro body will be included at this point

in the source file.  Parameters to be passed to the macro

follow the macro name on the line.  Up to nine parameters

may be passed to a macro.

Page 11

4.3  Parameter Assignment Rules

The parameter text on the macro call line is assigned to

parameters in the following way:

1.  Skip all leading blanks and tabs and set the current

parameter number to one.

2.  If the first character is a <, all text up to a > is

assigned to the current parameter number.  If the

first character is not a < and there is a comma (,) in

the remaining text, all text up to the comma is

assigned to the current parameter number.

If the first character is a <, but there is not a > in

the remaining text, the < is considered to be a normal

text character.

3.  If the first character is not a < and there is no

comma in the remaining text, all text up to the first

blank or tab is assigned to the current parameter

number.

4.  Increment the parameter number.  If the parameter

number is less than nine, go back to step 2.

NOTE: The comment column value is ignored for macro call

lines; only characters which follow the first blank or tab

following the last parameter are ignored.

4.4  Macro Example

The (simple) example below is a macro that increments the A

register by the amount passed as a parameter.

ADDTOA  MACRO

LC(5)   $1

A=A+C   A

ADDTOA  ENDM

...

DAT0=A  M

ADDTOA  312

D0=D0+  12

Page 12

5.  File Access Statements

There are three statements which access data in other files.

RDSYMB reads the symbols from a Saturn object file, INCLUDE

reads Saturn source statements from a file, and CHARMAP

simplifies the problem of working with a non-ASCII character

set often found in calculators.

The file name for each of the statements can be specified

several ways.  The name can be specified by itself or

surrounded by quotes ("filename"), apostrophes ('filename'),

or brackets (<filename>).  If the file name contains blanks

or tabs, it must be quoted by one of these methods.

Each statement uses an environment variable to determine

where to search for the file (the default is the current

directory and a system directory).  See the "Environment

Variables" appendix for more information about environment

variable names and defaults.

5.1  RDSYMB Statement

The RDSYMB statement reads the symbol table from a Saturn

object file.  All external symbols which are defined in the

object file and are not relocatable are available for use in

the file being assembled.  Symbols defined by a RDSYMB

statement are not included in the symbol table listing

unless they are actually used in the assembly.

5.2  INCLUDE Statement

The INCLUDE statement tells the assembler to read source

statements from the specified file.  The assembler reads

from the file until an END statement is processed or the end

of the file is reached.

The lines read from include files are not normally listed.

Only pseudo-op statements and lines containing errors are

listed by default.  To enable full include file listing, use

either the SETLIST INCLUDE statement or the LISTM statement.

Included lines have a - after the line number in the

listing.  The line number shown in the listing is the line

within the include file.

5.3  CHARMAP Statement

The LAASC, LCASC, and NIBASC statements are of limited

usefulness when the character set used in a product is not

ASCII.  The CHARMAP statement allows the ASCII characters

specified in those statements to be automatically converted

Page 13

to a different character set.  For example, if a particular

product is only capable of displaying letters and digits,

the character set mapping might be A ... Z = #00 ... #19,

a ... z = #1A ... #35, and 0 ... 9 = #36 ... #40.  Using

CHARMAP with a file which contains all these pairings allows

the assembler to automatically convert all references to

ASCII characters to the corresponding character set

character.  This permits source files to be independent of

the actual character mapping by including a CHARMAP

statement at the start of the file.

The mapping becomes effective when the CHARMAP statement is

executed; it does not affect ASCII characters preceding the

CHARMAP statement.  The effect of multiple CHARMAP

statements is cumulative.

5.3.1  Charmap_File_Format   The file indicated by the

CHARMAP statement contains a list of pairings, one pair per

line.  The first character position is the ASCII character

which is used in the source file, and the second character

position is the value which is used for the generated code.

Each character position can be either an ASCII character or

an escape sequence similar to those found in the C language.

The following table summarizes the escape sequences

recognized in a CHARMAP file:

Sequence    ASCII code    Description

----------  ----------    ------------------------------------

ASCII char     Same       The specified ASCII character

\a            7        BEL (alert character)

\b            8        BS (backspace)

\t            9        HT (tab)

\n           10        LF (linefeed)

\v           11        VT (vertical tab)

\f           12        FF (formfeed)

\r           13        CR (carriage return)

\\           92        Backslash

\xhh         Hex hh    Character with hex value hh

(\xh ok if not ambiguous)

\ddd      Octal ddd    Character with oct value ddd

(\dd or \d ok if not ambiguous)

Page 14

6.  Saturn Assembler Format and Mnemonics

This chapter describes the Saturn assembler instruction set.

The Saturn CPU has three variations used in several

products.  The 1LF2 was used in the first versions of the

HP-71B.  The 1LK7 is a variation of the 1LF2 used in later

versions of the HP-71B, the HP-18C, and the HP-28C.  The

1LR2 is an integrated CPU/ROM/RAM/Display Driver IC.  Each

new version of the Saturn CPU added new instructions to the

Saturn instruction set.  Instructions available in all

Saturn CPUs are referred to as "level 0" instructions.

Instructions available in the 1LK7 and 1LR2 but not the 1LF2

are referred to as "level 1" instructions.  Instructions

available only in the 1LR2 are referred to as "level 2"

instructions.  In this section, "level 1" instructions are

marked with an asterisk (*), "level 2" instructions are

marked with two asterisks (**).  Instructions with no mark

are "level 0" instructions.

6.1  Instruction Syntax

The assembler is "free format" and a space or tab is

required to delimit the different fields.  A label, if

present, must start in column one or two.  The format below

is a recommended column alignment:

1       9       17              33       ...   80

-------------------------------------------------

Label   Opcode  Modifier        Comments ...

-------------------------------------------------

6.1.1  Comments

A comment line begins with an asterisk (*) in column one,

and may occur anywhere in the file.  An in-line comment may

begin with any non-blank character and must follow the

modifier field of an instruction (or the opcode if no

modifier is required).

6.1.2  Symbols_and_Labels

A symbol is a name for a numeric value.  A symbol acquires

its value by appearing in the label field of certain

statements.  The word "symbol" is a general term for a

label, and the two are used interchangeably.

Symbols consist of one to twelve alphanumeric characters

with the following restrictions:  the characters comma (,),

space ( ), and right parenthesis are prohibited, and special

care must be used if the first character is an equal sign

(=), colon (:), sharp (#), left parenthesis, or a digit (0

through 9).

Page 15

A symbol may be immediately preceded by an equal sign (=)

which declares the symbol to be an external symbol.  An

external symbol defined in one module may be referenced as

an external symbol by another module.  Such references are

resolved when the modules are linked together.  Certain

Saturn assemblers, such as the HP-71 FORTH/Assembler ROM,

have no associated linker and therefore do not support

external symbols.  In this case, any leading equal sign is

ignored.

A symbol may instead be immediately preceded by a colon (:)

which simply declares what follows to be a (local) symbol.

Either an equal sign or a colon must be used with any symbol

whose first character is in the special care category (=, :,

#, (, 0-9).

When a symbol is used as part of an expression, parentheses

are required to delineate it.  That is, AD1-10 is a symbol

but (AD1)-10 is a computed expression.

6.1.3  Expressions

Wherever an expression may appear in the modifier field of

an instruction, it is represented by the symbol "expr" in

the instruction descriptions below.  Expressions are

evaluated using 32-bit signed integer math.  If a value does

not fit within 32 bits, the most significant bits are lost

(only the low 32 bits are saved).

Expression Components

Component             Examples

--------------------  -----------------------------------

decimal constant      23434

hexadecimal constant  #1FF0  (less than #100000000)

ASCII constant        \AB\, 'AB' (4 or less characters)

operator              +  addition

-  subtraction

synonym for * 256 +

*  multiplication

/  integer division

%  modulo (remainder)

^  integer exponentiation

&  bitwise AND

!  bitwise OR

*                    Current assembly program counter

symbol                Symbol defined within this file

(expression)          Parenthesized expression

Page 16

Two classes of instructions require a modifier field which

contains a constant of a specific type that does not conform

to the above rules.  These are:

1. Instructions with a string constant which can exceed

4 characters:

LAASC   \ASCII\       ( 8 characters maximum) **

LCASC   \ASCII\       ( 8 characters maximum)

NIBASC  \ASCII...\    (40 characters maximum)

STRING  \ASCII...\    (40 characters maximum)

2. Instructions with a required hexadecimal constant:

LAHEX   0123456789ABCDEF    (16 digits maximum) **

LCHEX   048C3               (16 digits maximum)

NIBHEX  0123456789ABCDEF    (80 digits maximum)

6.2  Explanation of Symbols

In the descriptions of the Saturn assembler mnemonics, these

symbols are defined as follows:

a      The hex nibble used to encode the field selection in

the assembled opcode of an instruction.  See the

Field Select Table in the next section for details.

b      The hex nibble used to encode the field selection in

the assembled opcode of an instruction.  See the

Field Select Table in the next section for details.

d      The number of nibbles represented by a field

selection field.  Used in calculating the execution

cycle time of some instructions.  See the Field

Select Table in the next section for details.  When

used in an extended field selection fsd, represents

an expression which indicates the number of nibbles

of the register that will be affected by the

instruction, proceeding from the low-order nibble to

higher-order nibbles.

expr   An expression that evaluates to an absolute or

relocatable value, usually less than or equal to 5

nibbles in length.

fs     Field selection symbol.  See the Field Select Table

in the next section for details.

fsd    Extended field selection symbol.  Represents either a

normal field selection symbol fs, or an expression

that gives the number of nibbles d of the register

that will be affected by the instruction, proceeding

from the low-order nibble to higher-order nibbles.

Page 17

hh     Two-digit hex constant, such as 08 or F2.  Within an

opcode represents the hex digits used to store the

value of the expression in the opcode in reverse

order (see "Loading Data From Memory").

hhhh   Four-digit hex constant, such as 38FE.  Within an

opcode, represents the hex digits used to store the

value of the expression in the opcode in reverse

order (see "Loading Data From Memory").

hhhhh  Five-digit hex constant, such as 308FE.  Within an

opcode, represents the hex digits used to store the

value of the expression in the opcode in reverse

order (see "Loading Data From Memory").

label  A symbol defined in the label field of an

instruction.

m      A one-digit decimal integer constant.

n      Represents an expression that evaluates to a 1-nibble

value, unless specified otherwise.  Within an opcode,

represents the hex digit used to store the assembled

value of the expression in the opcode.

nn     Represents an expression that evaluates to a 2-nibble

value, unless specified otherwise.  Within an opcode,

represents the hex digits used to store the assembled

value of the expression in the opcode.

nnnn   Represents an expression that evaluates to a 4-nibble

value, unless specified otherwise.  Within an opcode,

represents the hex digits used to store the assembled

value of the expression in the opcode.

nnnnn  Represents an expression that evaluates to a 5-nibble

value, unless specified otherwise.  Within an opcode,

represents the hex digits used to store the assembled

value of the expression in the opcode.

r,s    Represents a register (r alone) or a register pair (r

and s used together).  The valid combinations are

(r,s) = { (A,B), (B,C), (C,A), (D,C) }.

ss     Represents a scratch register name (R0, R1, R2, R3,

or R4).

dp     Represents a data pointer name (D0 or D1).

Page 18

6.3  Field Select Table

The following symbols are used in the instruction

descriptions to denote field selections.

There are two ways in which field selection is encoded in

the opcode of an instruction.  These two patterns are shown

in the table below, and are designated by the letters a and

b.

Field Select Table

------------------

Opcode      Number

Representation  of Nibs

Field  Name and Description          (a) (b)         (d)

-----  ----------------------      --------------  -------

P    Pointer Field. Nibble          0   8           1

specified by P pointer

register.

WP    Word-Through-Pointer Field.    1   9          P+1

Nibbles P through 0.

XS    Exponent Sign Field. Nib 2.    2   A           1

X    Exponent Field.  Nibs 2-0.     3   B           3

S    Sign Field.  Nibble 15.        4   C           1

M    Mantissa Field.  Nibs 14-3.    5   D          12

B    Byte Field.  Nibs 1-0.         6   E           2

W    Word Field.  Nibs 15-0.        7   F          16

A    Address Field.  Nibs 4-0.      F   -           5

Some instructions have an entirely different opcode

representation for the A field.

Page 19

6.4  Instruction Set Overview

This is a summary of the Saturn instruction set, grouped by

functional category.

Fields of a register are indicated using the convention that

a register name followed by a field in parentheses means

that field of the register.  For example, C(A) means the A

field of register C, and A(3:0) means nibbles 3 through 0 of

register A.

6.5  Jump Instructions

GOTO    label   Unconditional relative jump;

range -2047, +2048 nibs.

GOC     label   Relative jump if Carry is set;

range -127, +128 nibs.

GONC    label   Relative jump if Carry is clear;

range -127, +128 nibs.

GOSHORT label   Generate a short jump to label.

If the carry state cannot be

determined at assembly time, a

GOTO is generated.  If the carry

is known to be set, a GOC is

generated.  If the carry is known

to be clear, a GONC is generated.

JUMP    label   Alias for GOSHORT.

GOLONG  label   Unconditional long relative jump;

range -32766, +32769 nibs.

GOVLNG  label   Absolute jump; range unrestricted.

PC=(A)          *  Indirect jump; A(A) is the

address of the destination address.

PC=(C)          ** Indirect jump; C(A) is the address

of the destination address.

PC=A            ** Direct jump; A(A) is the

destination address.

PC=C            ** Direct jump; C(A) is the

destination address.

APCEX           ** Direct jump and save PC in A(A);

A(A) is the destination address.

CPCEX           ** Direct jump and save PC in C(A);

C(A) is the destination address.

GOYES  label    Relative jump if test is true (second

half of test instruction);

range -125, +130 nibs from test.

Page 20

6.6  Subroutine Call Instructions

GOSUB   label   Relative jump to subroutine;

range -2044, +2051 nibs.

GOSUBL  label   Long relative jump to subroutine;

range -32762, +32773 nibs.

GOSBVL  label   Absolute jump to subroutine.

6.7  Subroutine Return Instructions

RTN             Return from subroutine.

RTNSC           Return from subroutine and set Carry.

RTNCC           Return from subroutine and clear Carry.

RTNSXM          Return from subroutine and set

hardware status bit XM.

RTI             Return from subroutine and enable

interrupt handling.

RTNC            Return from subroutine if Carry is set.

RTNNC           Return from subroutine if Carry is

clear.

RTNYES          Return from subroutine if test is true

(second half of test instruction).

Page 21

6.8  Test Instructions

All test instructions must be followed with a GOYES or a

RTNYES instruction.  The test instruction and the following

GOYES or RTNYES instruction together form a single 5-nibble

opcode.  The Carry is set when the test is true and cleared

when the test is false.  All register comparisons are

unsigned (#FFFFF is greater than #7FFFF).  The test is

performed only on the selected field.

6.8.1  Register_Tests

?r=s    fs      True if r(fs) and s(fs) are equal.

?r#s    fs      True if r(fs) and s(fs) are not equal.

?r=0    fs      True if r(fs) is zero.

?r#0    fs      True if r(fs) is non-zero.

?r>s    fs      True if r(fs) is greater than s(fs).

?s>r    fs      True if s(fs) is greater than r(fs).

?r<s    fs      True if r(fs) is less than s(fs).

?s<r    fs      True if s(fs) is less than r(fs).

?r>=s   fs      True if r(fs) is greater than or equal

to s(fs).

?s>=r   fs      True if s(fs) is greater than or equal

to r(fs).

?r<=s   fs      True if r(fs) is less than or equal

to s(fs).

?s<=r   fs      True if s(fs) is less than or equal

to r(fs).

6.8.2  Pointer_Tests

?P=     n       True if P is equal to n.

?P#     n       True if P is not equal to n.

6.8.3  Program_Status_Bit_Tests

?ST=0   n       True if status bit n is clear.

?ST=1   n       True if status bit n is set.

?ST#1   n       Alias for ?ST=0   n.

?ST#0   n       Alias for ?ST=1   n.

Page 22

6.8.4  Hardware_Status_Bit_Tests

?XM=0           True if XM bit (external module missing)

is clear.

?SB=0           True if SB bit (sticky bit) is clear.

?SR=0           True if SR bit (service request) is

clear.

?MP=0           True if MP bit (module pulled) is clear.

?HS=0   n       True if all bits corresponding to n are

clear.

6.8.5  Register_Bit_Tests

?ABIT=0 n       ** True if bit n of register A is clear.

?ABIT=1 n       ** True if bit n of register A is set.

?CBIT=0 n       ** True if bit n of register C is clear.

?CBIT=1 n       ** True if bit n of register C is set.

?ABIT#1 n       ** Alias for ?ABIT=0 n.

?CBIT#1 n       ** Alias for ?CBIT=0 n.

?ABIT#0 n       ** Alias for ?ABIT=1 n.

?CBIT#0 n       ** Alias for ?CBIT=1 n.

6.9  Pointer Instructions

All arithmetic calculations on the pointer are performed in

HEX mode.

P=      n       Set register P to n.

P=P+1           Increment P register; affects Carry.

P=P-1           Decrement P register; affects Carry.

C+P+1           Add P plus one to A field of C; affects

Carry.

C=C+P+1         Alias for C+P+1.

CPEX    n       Exchange P register and nibble n of C

register.

P=C     n       Copy nibble n of C register to P register.

C=P     n       Copy P register to nibble n of C register.

6.10  Bit Manipulation Instructions

ABIT=0  n       ** Clear bit n of register A.

ABIT=1  n       ** Set bit n of register A.

CBIT=0  n       ** Clear bit n of register C.

CBIT=1  n       ** Set bit n of register C.

Page 23

6.11  Status Instructions

6.11.1  Program_Status

ST=0    n       Set status bit n.

ST=1    n       Clear status bit n.

CSTEX           Exchange status bits 11-0 with C(X).

C=ST            Copy status bits 11-0 to C(X).

ST=C            Copy C(X) to status bits 11-0.

CLRST           Clear status bits 11-0.

6.11.2  Hardware_Status

XM=0            Clear XM bit (external module missing).

SB=0            Clear SB bit (sticky bit).

SR=0            Clear SR bit (service request).

MP=0            Clear MP bit (module pulled).

HS=0    n       Clear all bits corresponding to n.

CLRHST          Clear all Hardware Status bits (XM, SB,

SR, and MP).

6.11.3  System_State_Instructions

SETHEX          Set arithmetic mode to hexadecimal.

SETDEC          Set arithmetic mode to decimal.

SREQ?           Set C(0) to service request response

from bus.  Set SR bit if service is

requested.

C=RSTK          Pop subroutine return stack into C(A).

RSTK=C          Push C(A) onto subroutine return stack.

A=PC            ** Copy current PC into A(A).

C=PC            ** Copy current PC into C(A).

CONFIG          Configure a device to the address

in C(A).

UNCNFG          Unconfigure a device at the address

in C(A).

RESET           Send Reset command to system bus.

BUSCB           ** Issue bus command B on the system

bus.

BUSCC           Issue bus command C on the system bus.

BUSCD           ** Issue bus command D on the system

bus.

SHUTDN          Stop CPU here, stay in low-power state

until wake-up requested.

C=ID            Copy chip ID from system bus to C(A).

INTOFF          Disable maskable interrupts.

INTON           Enable maskable interrupts.

RSI             * Reset interrupt detect circuitry.

Page 24

6.11.4  Keyscan_Instructions

OUT=C           Copy C(X) to OUT register.

OUT=CS          Copy C(0) to low 4 bits of OUT register.

A=IN            Copy IN register to A(3:0).

C=IN            Copy IN register to C(3:0).

6.11.5  Scratch_Register_Instructions

A=ss            Copy ss to A.

C=ss            Copy ss to C.

ss=A            Copy A to ss.

ss=C            Copy C to ss.

AssEX           Exchange A with ss.

CssEX           Exchange C with ss.

A=ss.F  fs      ** Copy ss(fs) to A(fs).

C=ss.F  fs      ** Copy ss(fs) to C(fs).

ss=A.F  fs      ** Copy A(fs) to ss(fs).

ss=C.F  fs      ** Copy C(fs) to ss(fs).

AssEX.F fs      ** Exchange A(fs) with ss(fs).

CssEX.F fs      ** Exchange C(fs) with ss(fs).

6.11.6  Data_Pointer_Instructions

dp=A            Copy A(A) to dp.

dp=C            Copy C(A) to dp.

AdpEX           Exchange A(A) with dp.

CdpEX           Exchange C(A) with dp.

dp=AS           Copy A(3:0) to (dp3:0).

dp=CS           Copy C(3:0) to (dp3:0).

AdpXS           Exchange A(3:0) with dp(3:0).

CdpXS           Exchange C(3:0) with dp(3:0).

dp=dp+  n       Increment register dp by n; alters

Carry.

dp=dp-  n       Decrement register dp by n; alters

Carry.

dp=HEX  hh      Load hh into dp(1:0).

dp=HEX  hhhh    Load hhhh into dp(3:0).

dp=HEX  hhhhh   Load hhhhh into dp.

dp=(2)  expr    Load expr into dp(1:0); use low 2

nibbles of expr if too big.

dp=(4)  expr    Load expr into dp(3:0); use low 4

nibbles of expr if too big.

dp=(5)  expr    Load expr into dp; use low 5 nibbles

of expr if too big.

6.11.7  Data_Transfer_Instructions

If fsd is an expression, the value of the expression is the

number of nibbles to transfer.  For example, if fsd is an

expression whose value is 7, nibbles 6 through 0 will be

transferred.

Page 25

A=DAT0  fsd     Read data pointed to by D0 into A(fsd).

A=DAT1  fsd     Read data pointed to by D1 into A(fsd).

C=DAT0  fsd     Read data pointed to by D0 into C(fsd).

C=DAT1  fsd     Read data pointed to by D1 into C(fsd).

DAT0=A  fsd     Write A(fsd) to location indicated by D0.

DAT1=A  fsd     Write A(fsd) to location indicated by D1.

DAT0=C  fsd     Write C(fsd) to location indicated by D0.

DAT1=C  fsd     Write C(fsd) to location indicated by D1.

6.11.8  Load_Constant_Instructions

All constants are loaded into the target register least

significant nibble first, with the least significant nibble

loaded into r(P) and subsequent nibbles loaded at r(P+1),

r(P+2), etc. until all nibbles have been loaded.  A constant

can wrap around from r(15) to r(0).

LAHEX   hhhhhhhh    ** Load hex constant hhhhhhhh into A.

LCHEX   hhhhhhhh       Load hex constant hhhhhhhh into C.

LAASC   \ASCII\     ** Load ASCII constant ASCII into A.

LCASC   \ASCII\        Load ASCII constant ASCII into C.

LA(m)   expr        ** Load an m-nibble constant into A;

use low m nibbles of expression if

too big.

LC(m)   expr           Load an m-nibble constant into C;

use low m nibbles of expression if

too big.

LA(N)   expr        ** Start an expr-nibble Load Constant

into register A.  This is useful

for Load Constants which are too

large for LA(m) or which involve

multiple external references.

LC(N)   expr           Start an expr-nibble Load Constant

into register C.  This is useful

for Load Constants which are too

large for LC(m) or which involve

multiple external references.

6.11.9  Shift_Instructions

The term circular shift means that the nibble shifted out

gets shifted in at the other end of the selected field.

NOTE: Right shift instructions set the Sticky Bit if any

non-zero bits are shifted out.

rSRB               Shift reg r right one bit.

rSRB.F  fs      ** Shift reg r(fs) right one bit.

rSLC               Shift reg r left circular one nibble.

rSRC               Shift reg r right circular one nibble.

rSL     fs         Shift reg r(fs) left one nibble.

rSR     fs         Shift reg r(fs) right one nibble.

Page 26

6.11.10  Arithmetic_Instructions

NOTE: There is no s=r-s   fs instruction.  This means these

instructions are not available on the Saturn CPU: B=A-B,

C=B-C, A=C-A, and C=D-C.

r=0     fs         Set r(fs) to zero.

r=r-1   fs         Decrement r(fs); alters Carry.

r=r+1   fs         Increment r(fs); alters Carry.

r=s     fs         Copy s(fs) to r(fs).

s=r     fs         Copy r(fs) to s(fs).

rsEX    fs         Exchange r(fs) and s(fs).

srEX    fs         Alias for rsEX    fs.

r=r+CON fs,d    ** Add d to r(fs); alters Carry.

r=r+r   fs         Add r(fs) to itself; alters Carry.

r=r+s   fs         Add s(fs) to r(fs); alters Carry.

r=s+r   fs         Alias for r=r+s   fs.

s=r+s   fs         Add r(fs) to s(fs); alters Carry.

s=s+r   fs         Alias for s=r+s   fs.

r=r-CON fs,d    ** Subtract d to r(fs); alters Carry.

r=r-s   fs         Subtract s(fs) from r(fs); alters Carry.

s=s-r   fs         Subtract r(fs) from s(fs); alters Carry.

r=s-r   fs         Subtract r(fs) from s(fs), put result in

r(fs); alters Carry.

r=-r    fs         2's or 10's complement of r(fs); clear

Carry if r(fs) was zero, else set Carry.

r=-r-1  fs         1's or 9's complement of r(fs);

unconditionally clear Carry.

6.11.11  Logical_Operation_Instructions

r=r&s   fs         Bit-wise AND of register r(fs) with

register s(fs).

r=r!s   fs         Bit-wise OR of register r(fs) with

register s(fs).

6.11.12  No-Operation_Instructions

NOP3               Three nibble No-op.

NOP4               Four nibble No-op.

NOP5               Five nibble No-op.

6.12  Pseudo-Op Instructions

NOTE: The label field is ignored by some of the pseudo-op

instructions.  These instructions ignore the label field:

CHARMAP, CLRCARRY, CLRFLAG, CLRLIST, EJECT, INCLUDE,

LISTALL, LISTM, LIST, MESSAGE, NOTREACHED, RDSYMB, SETCARRY,

SETFLAG, SETLIST, STITLE, TITLE, and UNLIST.

Page 27

6.12.1  Data_Storage_Allocation

BSS     nnnnn      Allocate nnnnn zero nibbles here.

CON(m)  expr       Generate m-nibble constant.  The

constant is stored with the least

significant nibble at the lowest

address.  [1 _ m _ 8]

REL(m)  label      Generate m-nibble relative offset.

The offset is stored with the least

significant nibble at the lowest

address.  [1 _ m _ 8]

NIBASC  \ASCII\    Generate ASCII characters.  Each

character is stored with the least

significant nibble at the lowest

address.  The first character is

placed at the lowest address.

[40 characters maximum]

STRING  \ASCII\    Generate ASCII characters, set the

high bit on the last character. Each

character is stored with the least

significant nibble at the lowest

address.  The first character is

placed at the lowest address.

[40 characters maximum]

NIBHEX  hhhhhhh    Generate hexadecimal nibbles.  The

first nibble is placed at the lowest

address.  [80 nibbles max]

NIBFS   fs         Generate the field selection nibble

for field fs. The opcode representation

used is from column a in the Field

Select Table.

LINK    label      Generate five nibble relative offset

to the next LINK reference to label.

The value of the offset is filled in

by the linker.

SLINK   label      Generate five nibble relative offset

to the first LINK reference to label.

The value of the offset is filled in

by the linker.

INC(m)  label      Generate an m-nibble reference to

label which is passed to the linker.

The label must be an external symbol.

The linker fills in the position of

the INC(m) reference to label.  For

example, if a file contains three

INC(3)  =label references, the first

INC(3) will be filled in as 000, the

second INC(3) will be filled in as

100, and the third INC(3) will be

filled in as 200 (least significant

nibble of the position at the lowest

address).

Page 28

6.13  Conditional Assembly

Conditional assembly pseudo-ops allow alternate versions of

assembly code to be assembled dependent on some specific

conditions.  An optional label on the conditional assembly

statements allows nesting.

Conditions which can be tested include assembly flags set on

invocation, the value of an expression compared to zero, the

relationship between two strings, the presence or absence of

a symbol, the current carry state (for example, after GOYES

or RTNYES, the carry is clear), and whether a specific

mnemonic is available in this assembly (dependent on the

processor level selected).

label   IF      expr    Assemble code only if flag expr is set.

label   IFEQ    expr    Assemble code only if expr is zero.

label   IFNE    expr    Assemble code only if expr is

non-zero.

label   IFLT    expr    Assemble code only if expr is less

than zero.

label   IFLE    expr    Assemble code only if expr is less

than or equal to zero.

label   IFGT    expr    Assemble code only if expr is greater

than zero.

label   IFGE    expr    Assemble code only if expr is greater

than or equal to zero.

label   IFZER   expr    Alias for IFEQ.

label   IFNZ    expr    Alias for IFNE.

label   IFNEG   expr    Alias for IFLT.

label   IFPOS   expr    Alias for IFGT.

label   IFDEF   symbol  Assemble code only if symbol is

defined now.

label   IFNDEF  symbol  Assemble code only if symbol is

not defined now.

label   IFOPC   symbol  Assemble code only if symbol is

a valid opcode mnemonic.

label   IFNOPC  symbol  Assemble code only if symbol is

not a valid opcode mnemonic.

label   IFPASS1         Assemble code only if this is the

first pass of the assembler. This

is most useful in conjunction with

the MESSAGE pseudo-op.

label   IFPASS2         Assemble code only if this is the

second pass of the assembler.  This

is most useful in conjunction with

the MESSAGE pseudo-op.

label   IFANYCARRY      Assemble code only if the carry can't

be determined at assembly time.

label   IFCARRYCLR      Assemble code only if the carry can

be determined at assembly time and

the carry is clear.

Page 29

label   IFCARRYSET      Assemble code only if the carry can

be determined at assembly time and

the carry is set.

label   IFREACHED       Assemble code only if the current

statement can be reached.

label   ELSE            Reverse the sense of the IF test with

label "label".

label   ENDIF           End conditional assembly started by

IF with label "label".

6.14  Listing Control

When no listing file is being generated (-N option), these

pseudo-ops have no effect on the assembly.

TITLE   text    Set title to text (at most one TITLE

instruction is permitted per file).

STITLE  text    Set subtitle to text and force a new

page in the assembly listing.

EJECT           Force a new page in the assembly listing.

UNLIST          Turn off assembly listing except for

some pseudo-ops.

LIST            Turn on assembly listing.

LISTM           Turn on assembly listing for macro

expansion and include files.

LISTALL expr    Unconditionally list the next expr lines.

LISTALL is independent of LIST and

UNLIST.  If expr is less than or equal

to zero or is not a legal expression,

disable LISTALL mode.

CLRLIST type    Turn off assembly listing of type, where

type is one or more of { CODE, MACRO,

INCLUDE, PSEUDO, ALL }. If the type

includes NOLIST, the CLRLIST line is

not listed.

SETLIST type    Turn on assembly listing of type, where

type is one or more of { CODE, MACRO,

INCLUDE, PSEUDO, ALL }.  If the type

includes NOLIST, the SETLIST line is

not listed.

LIST is an alias for SETLIST CODE.  LISTM is an alias for

SETLIST MACRO,INCLUDE.  UNLIST is an alias for

CLRLIST CODE,MACRO,INCLUDE.

6.15  Symbol Definition

symbol  EQU   expr    Assigns the value expr to symbol. If

symbol is already defined, EQU

generates an error.

symbol  =     expr    Assigns the value expr to symbol.  If

symbol is already defined, it is given

Page 30

the new value expr.

6.16  Macro Definition

label   MACRO         Start definition of macro label.

label   ENDM          End definition of macro label.

EXITM         If reached while interpreting a macro,

terminate that macro interpretation

immediately.  EXITM has no effect

during the definition of a macro.

6.17  Assembly Mode

symbol  ABS   nnnnn   Specify an absolute assembly starting

at address nnnnn.

symbol  REL   nnnnn   Specify a relocatable assembly

starting at address nnnnn.

END           Terminate assembly with this line.

Any lines following the END instruction

are ignored.

6.18  File Access

These pseudo-ops allow access to other files.  This allows

commonly-used symbols, macros, etc. to be defined in a file

shared by several assembly files.

RDSYMB  file    Read the symbol table from the Saturn

file named file.  Each symbol which is

defined, external, and not relocatable

is made available for the duration of

this assembly.

INCLUDE file    Read assembly source statements from file

until either an END instruction is read

or an End-of-File condition occurs.

CHARMAP file    Read a set of character mappings from

file.  Each line in the file consists of

an ASCII character, followed by the

character which should be assembled when

the character is used in an ASCII string.

(See "Charmap File Format" ).

6.19  Assembly Flag Modification

CLRFLAG expr    Clear assembly flag expr.

SETFLAG expr    Set assembly flag expr.

Page 31

6.20  Carry State Modification

CLRCARRY        Indicate to the assembler that the carry

is always clear at this point.

SETCARRY        Indicate to the assembler that the carry

is always set at this point.

NOTREACHED      Indicate to the assembler that this point

is never reached by assembly code.

6.21  Miscellaneous

MESSAGE text    Write text to the standard error location.

The message is written once for each

assembler pass.  This is most useful when

tracking down symbols which change between

passes.

Page 32

7.  Saturn Assembly Tips

This chapter summarizes some general advice accumulated over

many many man-years of Saturn programming in Corvallis.

While no doubt incomplete, these should spare some agony.

7.1  Three Warnings

The following three "gotcha's" are lessons that have been

learned repeatedly by every Saturn programmer.  You have

been warned.

7.1.1  Return_Levels

If you have experience with standard processors, be aware

that this one has a fixed number of return stack levels (8).

As the interrupt system uses two of these whenever an

interrupt occurs (which generally can be any time), HP 48

programmers are limited to a maximum of 6 levels.  If code

you write is called, you will be further limited.  The

symptoms of violation will include a Warmstart, but may

include more severe effects.  ...So watch those levels.

7.1.2  Mode

One of the big features of the Saturn processor, is also the

programmers bane.  The processor supports both DEC and HEX

modes.  Code designed to run in HEX mode can behave very

badly if invoked from a DEC mode state and vice versa.

...So watch the mode.

7.1.3  Remember_P=0!

Many routines require P=0 as an entry condition (this

includes the RPL inner loop by the way).  Quite frequently

other routines don't care about the value of P on entry, but

use it as a resource and exit with it in various states (eg;

most of the floating point math routines).  This could be

generalized to simply "watch entry and exit conditions", but

this one seems to happen frequently.  ...So watch P=0.

Page 33

7.2  Code Packing Tips

7.2.1  A-Field_Operations

Frequently, use of the A field for register operations that

require only the P,B,X or XS fields is a code saving.  Eg;

Replace    "A=C   X" with "A=C   A"  field to save a nibble

if you don't care about nibbles 3 and 4 of A.

7.2.2  Loading_Constants

For loading small constants into a larger field, it is

frequently cheaper to clear the field and generate only the

"digits" required.  For example if  kfactor < 256, than you

will save a nibble of ROM and get the same effect with

C=0      A

LC(2)    kfactor

instead of

LC(5)    kfactor

7.2.3  The_3-Branches

There are 3 varieties of "GOTO" and "GOSUB" that require

4,6, and 7 nibbles of code.  Two of these are "relative"

branches, and the long one is "absolute".  The assembler

takes care of all the details, informing you if a branch is

out of range, so there is really no drawback to using the

shorter versions when appropriate (they also execute faster

by the way).  As a general rule, references to external

routines (routines in the HP 48) should use the long

version, and references to routines in your application

should use relative branches.  The neumonics for the three

varieties are:

GOTO           GOSUB                (4 nibs)

GOLONG         GOSUBL               (6 nibs)

GOVLNG         GOSBVL               (7 nibs)

Numerous references to an external routine may be shortened

by means of a "jump table".  Eg; Replace all "GOSBVL

=GETPTR"  by calls to the local version ("GOSUB  getptr")

below.

getptr  GOVLNG   =GETPTR

Page 34

7.2.4  GOSUB/RTN

Code that might naturally end with something like

...

...

GOSUB    dotask8

RTN

will run faster, save 2 nibs, and may make your routine take

less stack levels by replacing that combination with

...

...

GOTO     dotask8

7.2.5  Use_Expressions

Use the Saturn Assembler to evaluate certain expressions

instead of at run time.

Example

LC(5)    (=TBLADRS)+5*t2

instead of

LC(5)    =TBLADRS

A=C      A

LC(5)    5*t2

C=C+A    A

7.2.6  Count_Up

Frequently P is used as the control variable for loops that

require no more than 16 passes.  If the loop is structured

so that P is decremented until a carry test causes an exit,

the value of P on exit will be 15 (generally not a very

useful value).  Often the code which follows will reset P to

0.  Optionally you can avoid the need to reset P to 0 (save

2 nibs) by counting up.  In this way, when the carry occurs,

P will have become 0.

Example

P=      16-5       Compute x-5*y;  A:x, C:y

arglp   A=A-C   A

P=P+1

GONC    arglp

Page 35

7.2.7  Before_you_leap

One of the advantages of assembly language programming is

the plethora of methods available to the innovator.  Often

the first solution you think of will not be the most code

efficient, time efficient, resource efficient, reliable, or

easiest to implement.

Speaking of time efficient - this document tells you about

execution time.  Instruction execution time varies depending

on instruction type and the fields that it operates on.

In addition, when executing code out of standard 8-bit wide

devices (ala HP 48 256 KByte ROM), instruction timing will

vary depending on whether the instruction occurs on an even

or odd address.  If one measures instruction timing on some

sort of consistent scale, you find that instruction times

will vary from 2 to 33 units of time.  The most time

expensive instructions are those which access data in

memory.  Also expensive are full word (16-nibble)

operations.  The least expensive are operations on P.

Page 36

7.3  Some Common Operations

7.3.1  A_nibble_from_here_to_there

This type of operation is usually accomplished by one or

more of the following 3 types of CPU instructions.  The

"Thru P" variety can only be used in the C register.

Register Transfer:

==================

Example - Transfer nibble from C[P] to A[P]

A=C      P

Nibble Shifts:

==============

Example - Shift B field of A into nibs 1 and 2 of A

ASL.F    X

Thru P:

=======

Example - Copy Sign field of C into nib 4 of C

P=C      15

C=P      4

7.3.2  Testing_a_Bit

This type of operation may be accomplished in a variety of

ways depending on where the bit to be tested is located, the

state of the CPU, and what CPU resources may be used.

Direct Bit Test:

================

Generally this is the best choice when it's available (only

in nibbles 0-3 of A and C registers), as this is destructive

only to the CARRY and depends only on the thing being

tested.   It also has the advantage of working with symbolic

arguments (which makes it easy when the location of the bit

is changed).

Example:

bEDIT   EQU      6

...

...

?ABIT=1  bEDIT

GOYES    doEDIT

Page 37

Left (Arithmetic) bit shifts in a field:

========================================

Left bit shifts require HEX Mode, and are often used in

cases where the bit lives in a location other than A[0-3] or

C[0-3] and it is too expensive or otherwise undesirable to

copy it there.  The shift is done arithmetically via HEX

mode arithmetic, and is destructive to the field in which

the operation is performed.  Your code is also dependent on

the  bit#  being tested (not symbolic).

Example - Test bit 2 in A[S]

SETHEX

A=A+A    S         Shift it to msb

A=A+A    S         CS iff bit 2 was originally set.

GOC      bit2_on

Right bit shifts in a field:

============================

Right arithmetic shifts are accomplished by provided CPU

shift instructions.  When a non-zero bit is shifted out the

right side of a field, a bit in the CPU known as the "sticky

bit" (SB) is set.  This bit is "sticky" and must be

explicitly cleared before it is used for a bit test.  The

same example above could be done in either HEX or DEC mode

by:

ASRB.F   S         Move bit to position 1

ASRB.F   S                     position 0

SB=0               Prepare for test

ASRB.F   S         SB=1 iff bit 2 originally set.

?SB=0

GOYES    bit2_off

The placement of the "SB=0" instruction is important.  There

is no instruction to test  SB=1, hence the test sense

reversal.

Mask it out:

============

P=       15

LCHEX    4         C[S]: 0100  (Mask)

A=A&C    S         Mask out all bits of non-interest.

?A#0     S         bit2 set?

GOYES    bit2_on   Yes.

Page 38

7.3.3  Saving/Testing_a_State

Frequently it is advantage to record a condition that may

later be tested.  The 12 CPU (Local) Status Bits (S0-S11)

are frequently used for this purpose.  The (Global) Status

bits (S12-S15) are reserved for recording operating system

status.  There is a functional difference also.  The Global

Status bits cannot be swapped in and out of the C-Register

like the others.  Be careful to document your usage of the

status bits, as failure to exercise care here can result in

contention for the same status bit and a "Gotcha".  As far

as usage goes it's quite simple.  Use a symbolic name for

the status bit (The symbol should be global if the status

bit will be referenced in other files).

Example:

=sDMY   EQU      8               Day-Month-Year Date Format if Set.

...

...

?ST=1    sDMY            DD.MMYYYY Date Format?

GOYES    date10          Yes.

CDEX     B               No.  - Swap DD, MM

date10

7.3.4  Memory_Access

The Saturn Processor Reads (Writes) from (to) low order

memory nibble wise into (from) the low order nibs of either

the A or C registers.  You will stay out of trouble if you

remember low order of the register to low order memory.

Thus an ASCII "A" stored in the B-field of CPU register A

will appear as  A[B]: 41.  If this same value is written out

to address  #82000,  you will see

#82000   1

#82001   4

If memory is displayed from left to right in increasing

addresses (as it is on our development systems),  the data

will appear as

82000:14...

making it appear backwards.  But all is well.  If the 2

nibbles starting at address #82000 are read back into A[B]

you will get what you expect (A[B]: 41).  The transfers take

place by using one of the "Data Pointers" D0, D1 to specify

the address.

Example: Write ASCII "A" to address #82000:

P=       0

LC(2)     A

D0=(5)   #82000

DAT0=C   B               #82000: "A"

Page 39

7.4  Some Other Tips

7.4.1  Labels

Nothing prohibits you from using global labels on all

routines.  Don't.  Use global labels only on routines that

are referenced externally (ie; those that require them) or

which might reasonably be called externally in the future.

Use "global references" only where required.  This permits a

code reviewer to know whether or not a routine that is being

called is in the same file or not w/o referring to the

symbol table.  (ie; if I see    "GOSUBL  =PADDER", I assume

that PADDER is in another file - otherwise, why the "=" ?).

7.4.2  Status_Bits

All status bit usage should be symbolic.  That is; never

write something like  "ST=0   5".  Instead, this can be

"ST=0    sLOAN" where sLOAN has been equated to 5 elsewhere.

In addition to the obvious advantage of self documenting

status bit usage, this permits relativelty safe changes to

the actual status bit in use at a later time.  The symbol

table identifies all references to sLOAN for the purposes of

making changes, avoiding status bit clash, etc.

7.4.3  Entry_Points

I find it is best to not have more than one Global entry in

a given routine.  The main reason is that such a routine is

frequently difficult to maintain.  Invariably the entry

points will have different entry conditions (else why the

separate entry).  This tenet may be violated in the interest

of code conservation, but always balance it against the

maintenence cost.  If you do it, and it's not completely

obvious from a casual glance what the entry conditions need

to be, insert a "mini-header" consisting of a few comment

lines explaining what the entry conditions are.  Something

like

*****************************************************************

** Here:   A: Last Arg Count                                    *

**         C: Loop Counter                                      *

**         P: 14                                                *

*****************************************************************

=ARG50

Page 40

7.4.4  Exits

Good "style" suggests that routines should not have more

than one normal (non-error) exit.  This is something that

gets violated with impunity in the interest of code saving.

It is not unusual to find cases where use of a common exit

actually saves code.  It's best to let code cost be the

judge.  When it is a "push", use a common exit.

7.5  Documentation

7.5.1  Comments_on_Comments

Any line beginning with an "*" is a comment line.  Comments

are also inserted on the same line as code (no "*"

required). Comments can be grouped into one of 3 types:

Module level:

=============

These comments generally describe the nature of the code in

the file, perhaps providing a list of the major routines,

general conventions and notation that will be used in

subsequent documentation, etc. This should appear at the

front of the file (before code).

Depending on your style, you may choose to place symbolic

equates (eg; status bits, various constants, etc;) at the

front of a file (These generate no code, but effect the code

that references the symbols).  This makes it easy for a

reviewer to find their values.

Routine level:

==============

These comments describe the general nature of a routine,

it's inputs and outputs, and CPU and RAM resources altered

by invocation.  These should appear in a header at the front

of the routine.  Begin the coding process by inserting a

standard blank header.  Fill in certain fields immediately

(eg; name, abstract, implementation date) and others as soon

as the code is reasonably stable.

Line level:

===========

Line by line comments will help the reviewer (which often

will be you) wade thru the code later.  These should appear

on the same line as the code or on separate lines near the

code being documented.

Page 41

Guidelines:

===========

1) How Much?  Generally, the more the better.  However not

every line needs a comment, and some short, simple, local

routines don't require a header.  If a routine has an

external entry, it deserves a header.

2) Avoid no content comments  (A=C  A    Copy C[A] to A[A])

3) Avoid comments that document unimportant register

contents.  This seems to help my focus.  For example,

reviewers usually prefer #1 over #2:

#1                    #2

C=D     A                              C[A]: Last Arg Ptr

A=C     A         A[A]: Last Arg Ptr   A[A]: Last Arg Ptr

P=      0

LC(5)   PROC5     C[A]: Exec Adrs      C[A]: Exec Adrs

4) Put content into your comments.  They should be easily

decipherable, but grammar is unimportant. Don't permit

comment lines to be longer than what will be printed on

the list (.l) files.

5) The Saturn Assembler is free format allowing you to be

quite flexible in your placement of code.  DON'T  OVERUSE

THIS  FEATURE.  Employ standard fields.  The header on the

next page has reasonable fields marked (1,9,17,28). The

important thing is consistency.

Page 42

7.5.2  A_Standard_Assembly_Language_Header

Some version of this header is in use by all of the software

people generating Saturn Assembly Language Code.  There is

no law saying you have to use this one, or use one at all.

This one works, and has passed the test of time.

EJECT

*****************************************************************

*****************************************************************

** Name: XXXXXXXXXXXX - .......

**

** Category: .....

**

** Abstract: .....

**

** Entry:    .....

**

** Exit:     .....

**

** Error Exits: .....

**

** Alters: CPU - .....

**         RAM - .....

**

** Calls:    .....

**

** Stack Levels: .....

**

** Notes: ......

**

**     Date     Prog             Modification

**   --------   ----   ------------------------------------------

**   04/../91   XX     Implemented.

**===============================================================

=XXXXXX XXXXX   XXXXX      CCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCCC

12345678901234567890123456789012345678901234567890123456789012345

Page 43

Here is how we use the various fields:

NAME: Routine Name and a 1-Line description of the routine.

CATEGORY: General grouping  (eg; MATH, CLKUTL, MEM, etc;)

ABSTRACT: Purpose of the routine, with perhaps minimal

details regarding how it does it.

ENTRY:    Exactly what CPU entry conditions must be satisfied.

Sometimes worthwhile mentioning other required

conditions as well (eg; RAM values, Timer State,

interrupts disabled, etc.).

EXIT:     What conditions can be depended upon for a normal

(non error) exit (Eg; where are the results?  Does

the routine always exit CC? Is there a particular

value in P?  What about HEX/DEC Mode?)

ERROR EXITS: Same as EXIT, but for the error exits.

ALTERS:   What does the routine change (WORST CASE!) for

a non-error exit.  If no RAM is altered, eliminate

that line and just list all CPU registers altered.

This includes CARRY, P, MODE, SB, etc.  The caller

generally doesn't care what resources you use,

just what you may have altered.  So, do not list

resources used, but always restored to their entry

state before exit.

CALLS:    Names of the routines called before exit back to

the caller.  It is often useful to indicate the

stack levels of the called routine here as well.

This assists in filling out the Stack Levels field.

STACK LEVELS: The number of stack levels used by this

routine.  If this routine has no GOSUB's, and does

not employ C=RSTK or RSTK=C, then this will be 0.

NOTES:    The place for additional documentation, perhaps

an algorithm description, or certain CAVEATS.

HISTORY:  Indicate date, programmers initials, and

reason for changes.  Don't start really using

this until you feel the code is stable.

Page 44

7.5.3  Some_Header_Examples

*****************************************************************

*****************************************************************

** Name(S): PKDATE   - Pack Date Components  (YYYYMMDD order)

**

** Category: DATEUTL

**

** Purpose:  Facilitates storage of date components in a single

**    CPU register.  Also useful for comparing two dates to

**    determine which occurrs first in time.  Entry conditions

**    are designed so that this routine may easily be called

**    after CKDATE.

**

** Entry:  A[A]: 0YYYY

**         B[B]: MM

**         D[B]: DD

**

** Exit:   C: 00000000YYYYMMDD;  CC;  P=7

**

** Alters:   C;  P;  CARRY

**

** Calls:    None

**

** Stack Levels: 0

**

** Notes:

**

**     Date     Prog             Modification

**   --------   ----   ------------------------------------------

**   08/23/85   SB     Implemented.

*****************************************************************

*****************************************************************

=PKDATE C=0    W              Initialize Result

C=A    A              Copy YYYY and shift left twice

CSL    A

P=     7

CSL    WP

C=B    B              Copy MM and shift left twice

CSL    WP

CSL    WP

C=D    B              C: 00000000YYYYMMDD

RTNCC

*****************************************************************

Page 45

EJECT

*****************************************************************

*****************************************************************

** Name(S): LEAPYR?  - Determine if specified year is leap yr.

**

** Category: DATEUTL

**

** Purpose:  Determine if specified year is a leap year.

**

** Entry:    A[A]: YYYY;   DEC Mode.

**

** Exit:     CS - YYYY is a Leap Year

**           CC - YYYY is not a Leap Year

**

** Alters:   C[A];  SB;  Carry

**

** Calls:    None

**

** Stack Levels: 1

**

** Notes:  Y is a leap year iff both of the following:

**                   1)  Y MOD 4 = 0

**          -AND-    2)  (Y MOD 100 # 0) or (Y MOD 400 =0)

**

**     Date     Prog             Modification

**   --------   ----   ------------------------------------------

**   07/12/85   SB     Implemented.

*****************************************************************

*****************************************************************

=LEAPYR?

C=A    A              Init X=Y

RSTK=C                Save Y

?A#0   B            Y divisible by 100 ?

GOYES  LEAP10       No.  Leap yr iff X div by 4

ASR    A            Yes. Leap yr iff Y div by 400

ASR    A

C=A    A            C[A]=A[A]: X=Y/100

*  Reduced to testing whether X divisible by 4.

LEAP10    SB=0                Init SB=0

C=C+C  A

C=C+C  A

C=C+A  A            5*X

CSR    A            C[A]: X/2;   Sets SB if X odd.

CSRB                Set SB if X/2 odd

C=RSTK

A=C    A              Restore Y

?SB=0                 Leap Year?

RTNYES                Yes.  CS

RTN                   No.   CC

*****************************************************************

Page 46

EJECT

*****************************************************************

*****************************************************************

**

*R Name: Ticks>DOW - Make Day of Week from Time in Ticks

**

** Category:   TIMESYS

**

** Abstract:   From Time in Ticks (since 0), return a real number

**             integer (1-7) that identifies the day of week.

**

** Stack:  hxs  --> % (Day of Week)

**

** Error Exits:    Insufficient Memory

**

**     Date     Prog             Modification

**   --------   ----   ------------------------------------------

**   03/18/88   SB     Implemented.

**===============================================================

=Ticks>DOW

CON(5) (*)+5

* Pop Time(ticks) from stack

A=DAT1 A

AD1EX                Save D1* in A[A];  D1:->hxs

D1=D1+ 10            Skip over prologue and length

C=0    W

C=DAT1 13

R0=C                 R0: Time (Ticks)

D1=A                 Restore D1

D1=D1+ 5

D=D+1  A             Pop hxs and save new pointers

GOSBVL =SAVPTR

* Convert to Day of Week and put in float form

C=R0

GOSBVL =dowutil      A: Day of Week Index (1-7)

C=A    A

P=C    0

C=0    W

C=P    14

A=C    W             A: % (DOW: 1-7; 1=SUN)

*  Push % on stack and loop.

GOTO   push%lp

*****************************************************************

Page 47

8.  Mnemonic Dictionary

This section contains a description of each Saturn assembler

instruction or pseudo-op.  The description shows the binary

opcode generated by the mnemonic, if any, as well as the

execution cycle time required if the mnemonic is an

executable instruction.

?A#0   fs  -  Test for A not equal to 0

---------

fs = A                       opcode:  8ACyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9aCyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is not equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?A#B   fs  -  Test for A not equal to B

---------

fs = A                       opcode:  8A4yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a4yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is not equal to the fs field

of B. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 48

?A#C   fs  -  Test for A not equal to C

---------

fs = A                       opcode:  8A6yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a6yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is not equal to the fs field

of C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?A<=B  fs  -  Test for A less than or equal to B

---------

fs = A                       opcode:  8BCyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9bCyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is less than or equal to the

fs field of B.  Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

?A<B   fs  -  Test for A less than B

---------

fs = A                       opcode:  8B4yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b4yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is less than the fs field of

B.  Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 49

?A=0   fs  -  Test for A equal to 0

---------

fs = A                       opcode:  8A8yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a8yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?A=B   fs  -  Test for A equal to B

---------

fs = A                       opcode:  8A0yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a0yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is equal to the fs field of

B. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?A=C   fs  -  Test for A equal to C

---------

fs = A                       opcode:  8A2yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a2yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is equal to the fs field of

C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 50

?A>=B  fs  -  Test for A greater than or equal to B

---------

fs = A                       opcode:  8B8yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b8yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is greater than or equal to

the fs field of B. Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

?A>B   fs  -  Test for A greater than B

---------

fs = A                       opcode:  8B0yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b0yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of A is greater than the fs field

of B. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?B#0   fs  -  Test for B not equal to 0

---------

fs = A                       opcode:  8ADyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9aDyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is not equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

Page 51

?B#A   fs  -  Test for B not equal to A

---------

fs = A                       opcode:  8A4yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a4yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is not equal to the fs field

of A. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?B#C   fs  -  Test for B not equal to C

---------

fs = A                       opcode:  8A5yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a5yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is not equal to the fs field

of C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 52

?B<=C  fs  -  Test for B less than or equal to C

---------

fs = A                       opcode:  8BDyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9bDyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is less than or equal to the

fs field of C.  Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

?B<C   fs  -  Test for B less than C

---------

fs = A                       opcode:  8B5yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b5yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is less than the fs field of

C.  Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?B=0   fs  -  Test for B equal to 0

---------

fs = A                       opcode:  8A9yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a9yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

Page 53

?B=A   fs  -  Test for B equal to A

---------

fs = A                       opcode:  8A0yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a0yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is equal to the fs field of

A. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?B=C   fs  -  Test for B equal to C

---------

fs = A                       opcode:  8A1yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a1yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is equal to the fs field of

C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?B>=C  fs  -  Test for B greater than or equal to C

---------

fs = A                       opcode:  8B9yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b9yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is greater than or equal to

the fs field of C. Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

Page 54

?B>C   fs  -  Test for B greater than C

---------

fs = A                       opcode:  8B1yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b1yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of B is greater than the fs field

of C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?C#0   fs  -  Test for C not equal to 0

---------

fs = A                       opcode:  8AEyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9aEyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is not equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?C#A   fs  -  Test for C not equal to A

---------

fs = A                       opcode:  8A6yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a6yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is not equal to the fs field

of A. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 55

?C#B   fs  -  Test for C not equal to B

---------

fs = A                       opcode:  8A5yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a5yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is not equal to the fs field

of B. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?C#D   fs  -  Test for C not equal to D

---------

fs = A                       opcode:  8A7yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a7yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is not equal to the fs field

of D. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?C<=A  fs  -  Test for C less than or equal to A

---------

fs = A                       opcode:  8BEyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9bEyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is less than or equal to the

fs field of A.  Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

Page 56

?C<A   fs  -  Test for C less than A

---------

fs = A                       opcode:  8B6yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b6yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is less than the fs field of

A.  Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?C=0   fs  -  Test for C equal to 0

---------

fs = A                       opcode:  8AAyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9aAyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?C=A   fs  -  Test for C equal to A

---------

fs = A                       opcode:  8A2yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a2yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is equal to the fs field of

A. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 57

?C=B   fs  -  Test for C equal to B

---------

fs = A                       opcode:  8A1yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a1yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is equal to the fs field of

B. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?C=D   fs  -  Test for C equal to D

---------

fs = A                       opcode:  8A3yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a3yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is equal to the fs field of

D. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?C>=A  fs  -  Test for C greater than or equal to A

---------

fs = A                       opcode:  8BAyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9bAyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is greater than or equal to

the fs field of A. Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

Page 58

?C>A   fs  -  Test for C greater than A

---------

fs = A                       opcode:  8B2yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b2yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of C is greater than the fs field

of A. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?D#0   fs  -  Test for D not equal to 0

---------

fs = A                       opcode:  8AFyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9aFyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is not equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?D#C   fs  -  Test for D not equal to C

---------

fs = A                       opcode:  8A7yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a7yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is not equal to the fs field

of C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 59

?D<=C  fs  -  Test for D less than or equal to C

---------

fs = A                       opcode:  8BFyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9bFyy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is less than or equal to the

fs field of C.  Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

?D<C   fs  -  Test for D less than to C

---------

fs = A                       opcode:  8B7yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b7yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is less than the fs field of

C.  Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?D=0   fs  -  Test for D equal to 0

---------

fs = A                       opcode:  8AByy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9aByy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is equal to 0. Must be

followed by a GOYES or RTNYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

Page 60

?D=C   fs  -  Test for D equal to C

---------

fs = A                       opcode:  8A3yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9a3yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is equal to the fs field of

C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

?D>=C  fs  -  Test for D greater than or equal to C

---------

fs = A                       opcode:  8BByy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9bByy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is greater than or equal to

the fs field of C. Must be followed by a GOYES or RTNYES

mnemonic. yy is determined by the following RTNYES or GOYES.

Adjusts Carry.

?D>C   fs  -  Test for D greater than C

---------

fs = A                       opcode:  8B3yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     opcode:  9b3yy

cycles:   13 + d (GO/RTNYES)

6 + d (NO)

Test whether the fs field of D is greater than the fs field

of C. Must be followed by a GOYES or RTNYES mnemonic. yy is

determined by the following RTNYES or GOYES.  Adjusts Carry.

Page 61

?MP=0    - Test Module Pulled bit (MP)

---------

opcode:  838yy

cycles:   13 (GO/RTNYES)

6 (NO)

Test whether the Module Pulled bit (MP) is zero.  This

hardware status bit is set whenever a module-pulled

interrupt occurs (the *INT line of the CPU is pulled high),

and must be explictly cleared by the MP=0 mnemonic. See the

"HP-71 Hardware Specification" for more information.  Must

be followed by a RTNYES or GOYES mnemonic. yy is determined

by the following RTNYES or GOYES.  Adjusts Carry.

?P#   n    - Test if P pointer not equal to n

---------

opcode:  88nyy

cycles:   13 (GO/RTNYES)

6 (NO)

Test whether the P pointer is not equal to n. Must be

followed by a RTNYES or GOYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?P=    n    - Test if P pointer is equal to n

---------

opcode:  89nyy

cycles:   13 (GO/RTNYES)

6 (NO)

Test whether the P pointer is equal to n. Must be followed

by a RTNYES or GOYES mnemonic. yy is determined by the

following RTNYES or GOYES.  Adjusts Carry.

?SB=0    - Test Sticky Bit (SB)

---------

opcode:  832yy

cycles:   13 (GO/RTNYES)

6 (NO)

Test whether the Sticky Bit (SB) is zero. This hardware

status bit is set on right shifts when a non-zero nibble or

bit is shifted off the end of the field.  The Sticky Bit

must be cleared explicitly. Must be followed by a RTNYES or

GOYES mnemonic. yy is determined by the following RTNYES or

GOYES.  Adjusts Carry.

Page 62

?SR=0    - Test Service Request bit (SR) for zero

---------

opcode:  834yy

cycles:   13 (GO/RTNYES)

6 (NO)

Test whether the Service Request bit (SR) is zero.  This

hardware status bit is set by the SREQ? mnemonic, and must

be cleared explicitly by the SR=0 instruciton.  Must be

followed by a RTNYES or GOYES mnemonic. yy is determined by

the following RTNYES or GOYES.  Adjusts Carry.

?ST#0   n   - Test status bit n not equal to 0

---------

opcode:  87nyy

cycles:   14 (GO/RTNYES)

7 (NO)

Test whether Program Status bit n is set. Must be followed

by a RTNYES or GOYES mnemonic. yy is determined by the

following RTNYES or GOYES.  Adjusts Carry.

?ST#1   n   - Test status bit n not equal to 1

---------

opcode:  86nyy

cycles:   14 (GO/RTNYES)

7 (NO)

Test whether Program Status bit n is clear. Must be followed

by a RTNYES or GOYES mnemonic. yy is determined by the

following RTNYES or GOYES.  Adjusts Carry.

?ST=0  n   - Test status bit n equal to 0

---------

opcode:  86nyy

cycles:   14 (GO/RTNYES)

7 (NO)

Test whether Program Status bit n is clear. Must be followed

by a RTNYES or GOYES mnemonic. yy is determined by the

following RTNYES or GOYES.  Adjusts Carry.

Page 63

?ST=1  n   - Test status bit n equal to 1

---------

opcode:  87nyy

cycles:   14 (GO/RTNYES)

7 (NO)

Test whether Program Status bit n is set. Must be followed

by a RTNYES or GOYES mnemonic. yy is determined by the

following RTNYES or GOYES.  Adjusts Carry.

?XM=0    - Test External Module Missing bit (XM)

---------

opcode:  831yy

cycles:   13 (GO/RTNYES)

6 (NO)

Test the whether the External Module Missing bit (XM) is

zero.  This hardware status bit is set by the RTNSXM

mnemonic, and must be explicitly cleared by the XM=0

mnemonic.  Must be followed by a RTNYES or GOYES mnemonic.

yy is determined by the following RTNYES or GOYES.  Adjusts

Carry.

A=-A   fs  -  Two's complement of A into A

---------

fs = A                       opcode:  F8

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb8

cycles:    3 + d

Complement the specified fs field of A. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

A=-A-1 fs  -  One's complement of A into A

---------

fs = A                       opcode:  FC

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BbC

cycles:    3 + d

Perform a one's complement on the specified fs field of A.

Carry is always cleared.

Page 64

A=0    fs  -  Set A equal to 0

---------

fs = A                       opcode:  D0

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab0

cycles:    3 + d

Set the specified fs field of A to zero.  Carry is not

affected.

A=A!B  fs  -  A OR B into A

---------

fs = A                       opcode:  0EF8

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea8

cycles:    4 + d

Set the fs field of register A to its logical OR with the

corresponding field of register B.  Carry is not affected.

A=A!C  fs  -  A OR C into A

---------

fs = A                       opcode:  0EFE

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0EaE

cycles:    4 + d

Set the fs field of register A to its logical OR with the

corresponding field of register C.  Carry is not affected.

A=A&B  fs  -  A AND B into A

---------

fs = A                       opcode:  0EF0

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea0

cycles:    4 + d

Set the fs field of register A to its logical AND with the

corresponding field of register B.  Carry is not affected.

Page 65

A=A&C  fs  -  A AND C into A

---------

fs = A                       opcode:  0EF6

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea6

cycles:    4 + d

Set the fs field of register A to its logical AND with the

corresponding field of register C.  Carry is not affected.

A=A+1  fs  -  Increment A

---------

fs = A                       opcode:  E4

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba4

cycles:    3 + d

Increment the specified fs field of register A by one.

Adjusts Carry.

A=A+A  fs  -  Sum of A and A into A

---------

fs = A                       opcode:  C4

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa4

cycles:    3 + d

Double the specified fs field of register A. Adjusts Carry.

A=A+B  fs  -  Sum of A and B into A

---------

fs = A                       opcode:  C0

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa0

cycles:    3 + d

Set the specified fs field of register A to the sum of

itself and the corresponding field of register B. Adjusts

Carry.

Page 66

A=A+C  fs  -  Sum of A and C into A

---------

fs = A                       opcode:  CA

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AaA

cycles:    3 + d

Set the specified fs field of register A to the sum of

itself and the corresponding field of register C. Adjusts

Carry.

A=A-1  fs  -  Decrement A

---------

fs = A                       opcode:  CC

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AaC

cycles:    3 + d

Decrement the specified fs field of register A by one.

Adjusts Carry.

A=A-B  fs  -  A minus B into A

---------

fs = A                       opcode:  E0

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba0

cycles:    3 + d

Set the specified fs field of register A to the difference

between itself and the corresponding field of register B.

Adjusts Carry.

A=A-C  fs  -  A minus C into A

---------

fs = A                       opcode:  EA

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BaA

cycles:    3 + d

Set the specified fs field of register A to the difference

between itself and the corresponding field of register C.

Adjusts Carry.

Page 67

A=B    fs  -  Copy B to A

---------

fs = A                       opcode:  D4

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab4

cycles:    3 + d

Copy the fs field of register B into the corresponding field

of register A. Carry is not affected.

A=B-A  fs  -  B minus A into A

---------

fs = A                       opcode:  EC

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BaC

cycles:    3 + d

Set the specified fs field of register A to the inverse

difference between itself and the corresponding field of

register B. Adjusts Carry.

A=C    fs  -  Copy C to A

---------

fs = A                       opcode:  DA

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbA

cycles:    3 + d

Copy the fs field of register C into the corresponding field

of register A. Carry is not affected.

Page 68

A=DAT0 fsd -  Load A from memory

---------

fs = A                       opcode:  142

cycles:   18

fs = B                       opcode:  14A

cycles:   15

fs = (P,WP,XS,X,S,M,W)       opcode:  152a

cycles:   17 + d

fs = d                       opcode:  15Ax (x=d-1)

cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D0 into

the specified field of register A.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

A=DAT1 fsd -  Load A from memory

---------

fs = A                       opcode:  143

cycles:   18

fs = B                       opcode:  14B

cycles:   15

fs = (P,WP,XS,X,S,M,W)       opcode:  153a

cycles:   17 + d

fs = d                       opcode:  15Bx (x=d-1)

cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D1 into

the specified field of register A.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

Page 69

A=IN      - Load A with IN

---------

opcode:  802

cycles:    7

Load the low-order 4 nibbles of the A register with the

contents of the Input register.

A=R0      - Copy R0 to A

---------

opcode:  110

cycles:   19

The contents of the scratch register R0 is copied to the

working register A.

A=R1      - Copy R1 to A

---------

opcode:  111

cycles:   19

The contents of the scratch register R1 is copied to the

working register A.

A=R2      - Copy R2 to A

---------

opcode:  112

cycles:   19

The contents of the scratch register R2 is copied to the

working register A.

A=R3      - Copy R3 to A

---------

opcode:  113

cycles:   19

The contents of the scratch register R3 is copied to the

working register A.

Page 70

A=R4      - Copy R4 to A

---------

opcode:  114

cycles:   19

The contents of the scratch register R4 is copied to the

working register A.

ABEX   fs  -  Exchange Registers A and B

---------

fs = A                       opcode:  DC

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbC

cycles:    3 + d

Exchange the fs fields of registers of A and B. Carry is not

affected.

ACEX   fs  -  Exchange Registers A and C

---------

fs = A                       opcode:  DE

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbE

cycles:    3 + d

Exchange the fs fields of registers of A and C. Carry is not

affected.

AD0EX      - Exchange A and D0 (nibs 0-4)

---------

opcode:  132

cycles:    8

Exchange the A field of register A with Data pointer D0.

Carry is not affected.

Page 71

AD0XS      - Exchange A and D0 short (nibs 0-3)

---------

opcode:  13A

cycles:    7

Exchange the lower 4 nibbles of A with the lower 4 nibbles

of Data pointer D0.  Carry is not affected.

AD1EX      - Exchange A and D1 (nibs 0-4)

---------

opcode:  133

cycles:    8

Exchange the A field of register A with Data pointer D1.

Carry is not affected.

AD1XS      - Exchange A and D1 short (nibs 0-3)

---------

opcode:  13B

cycles:    7

Exchange the lower 4 nibbles of A with the lower 4 nibbles

of Data pointer D1.  Carry is not affected.

AR0EX      - Exchange A and R0

---------

opcode:  120

cycles:   19

Exchange the contents of the working register A and the

scratch register R0.

AR1EX     - Exchange A and R1

---------

opcode:  121

cycles:   19

Exchange the contents of the working register A and the

scratch register R1.

Page 72

AR2EX     - Exchange A and R2

---------

opcode:  122

cycles:   19

Exchange the contents of the working register A and the

scratch register R2.

AR3EX     - Exchange A and R3

---------

opcode:  123

cycles:   19

Exchange the contents of the working register A and the

scratch register R3.

AR4EX     - Exchange A and R4

---------

opcode:  124

cycles:   19

Exchange the contents of the working register A and the

scratch register R4.

ASL    fs  -  A Shift Left

---------

fs = A                       opcode:  F0

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb0

cycles:    3 + d

Shift the contents of the specified fs field of register A

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

Page 73

ASLC      - A Shift Left Circular

---------

opcode:  810

cycles:   21

Circular shift register A left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

ASR    fs  -  A Shift Right

---------

fs = A                       opcode:  F4

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb4

cycles:    3 + d

Shift the contents of the specified fs field of register A

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

ASRB      - A Shift Right Bit

---------

opcode:  81C

cycles:   20

Shift register A right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

ASRC      - A Shift Right Circular

---------

opcode:  814

cycles:   21

Circular shift register A right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

Page 74

B=-B   fs  -  Two's complement of B into B

---------

fs = A                       opcode:  F9

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb9

cycles:    3 + d

Complement the specified fs field of B. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

B=-B-1 fs  -  One's complement of B into B

---------

fs = A                       opcode:  FD

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BbD

cycles:    3 + d

Perform a one's complement on the specified fs field of B.

Carry is always cleared.

B=0    fs  -  Set B equal to 0

---------

fs = A                       opcode:  D1

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab1

cycles:    3 + d

Set the specified fs field of B to zero.  Carry is not

affected.

B=A    fs  -  Copy A to B

---------

fs = A                       opcode:  D8

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab8

cycles:    3 + d

Copy the fs field of register A into the corresponding field

of register B. Carry is not affected.

Page 75

B=B!A  fs  -  B OR A into B

---------

fs = A                       opcode:  0EFC

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0EaC

cycles:    4 + d

Set the fs field of register B to its logical OR with the

corresponding field of register A.  Carry is not affected.

B=B!C  fs  -  B OR C into B

---------

fs = A                       opcode:  0EF9

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea9

cycles:    4 + d

Set the fs field of register B to its logical OR with the

corresponding field of register C.  Carry is not affected.

B=B&A  fs  -  B AND A into B

---------

fs = A                       opcode:  0EF4

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea4

cycles:    4 + d

Set the fs field of register B to its logical AND with the

corresponding field of register A.  Carry is not affected.

B=B&C  fs  -  B AND C into B

---------

fs = A                       opcode:  0EF1

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea1

cycles:    4 + d

Set the fs field of register B to its logical AND with the

corresponding field of register C.  Carry is not affected.

Page 76

B=B+1  fs  -  Increment B

---------

fs = A                       opcode:  E5

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba5

cycles:    3 + d

Increment the specified fs field of register B by one.

Adjusts Carry.

B=B+A  fs  -  Sum of B and A into B

---------

fs = A                       opcode:  C8

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa8

cycles:    3 + d

Set the specified fs field of register B to the sum of

itself and the corresponding field of register A. Adjusts

Carry.

B=B+B  fs  -  Sum of B and B into B

---------

fs = A                       opcode:  C5

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa5

cycles:    3 + d

Double the specified fs field of register B. Adjusts Carry.

B=B+C  fs  -  Sum of B and C into B

---------

fs = A                       opcode:  C1

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa1

cycles:    3 + d

Set the specified fs field of register B to the sum of

itself and the corresponding field of register C. Adjusts

Carry.

Page 77

B=B-1  fs  -  Decrement B

---------

fs = A                       opcode:  CD

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AaD

cycles:    3 + d

Decrement the specified fs field of register B by one.

Adjusts Carry.

B=B-A  fs  -  B minus A into B

---------

fs = A                       opcode:  E8

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba8

cycles:    3 + d

Set the specified fs field of register B to the difference

between itself and the corresponding field of register A.

Adjusts Carry.

B=B-C  fs  -  B minus C into B

---------

fs = A                       opcode:  E1

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba1

cycles:    3 + d

Set the specified fs field of register B to the difference

between itself and the corresponding field of register C.

Adjusts Carry.

B=C    fs  -  Copy C to B

---------

fs = A                       opcode:  D5

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab5

cycles:    3 + d

Copy the fs field of register C into the corresponding field

of register B. Carry is not affected.

Page 78

B=C-B  fs  -  C minus B into B

---------

fs = A                       opcode:  ED

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BaD

cycles:    3 + d

Set the specified fs field of register B to the inverse

difference between itself and the corresponding field of

register C. Adjusts Carry.

BAEX   fs  -  Exchange Registers B and A

---------

fs = A                       opcode:  DC

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbC

cycles:    3 + d

Exchange the fs fields of registers of B and A. Carry is not

affected.

BCEX   fs  -  Exchange Registers B and C

---------

fs = A                       opcode:  DD

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbD

cycles:    3 + d

Exchange the fs fields of registers of B and C. Carry is not

affected.

BSL    fs  -  B Shift Left

---------

fs = A                       opcode:  F1

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb1

cycles:    3 + d

Shift the contents of the specified fs field of register B

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

Page 79

BSLC      - B Shift Left Circular

---------

opcode:  811

cycles:   21

Circular shift register B left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

BSR    fs  -  B Shift Right

---------

fs = A                       opcode:  F5

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb5

cycles:    3 + d

Shift the contents of the specified fs field of register B

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

BSRB      - B Shift Right Bit

---------

opcode:  81D

cycles:   20

Shift register B right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

BSRC      - B Shift Right Circular

---------

opcode:  815

cycles:   21

Circular shift register B right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

Page 80

BUSCC     - Bus Command "C"

---------

opcode:  80B

cycles:    6

Enters the Saturn bus command "C" onto the system bus (this

command is reserved for later use).  No other operation is

performed.  See the "HP-71 Hardware Specification" for more

information.

C+P+1     - Increment C by One Plus P Pointer

---------

opcode:  809

cycles:    8

The A field of the C register is incremented by one plus the

value of the P pointer.  This instruction is always executed

in HEX mode.  Adjusts Carry.

C=-C   fs  -  Two's complement of C into C

---------

fs = A                       opcode:  FA

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BbA

cycles:    3 + d

Complement the specified fs field of C. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

C=-C-1 fs  -  One's complement of C into C

---------

fs = A                       opcode:  FE

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BbE

cycles:    3 + d

Perform a one's complement on the specified fs field of C.

Carry is always cleared.

Page 81

C=0    fs  -  Set C equal to 0

---------

fs = A                       opcode:  D2

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab2

cycles:    3 + d

Set the specified fs field of C to zero.  Carry is not

affected.

C=A    fs  -  Copy A to C

---------

fs = A                       opcode:  D6

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab6

cycles:    3 + d

Copy the fs field of register A into the corresponding field

of register C. Carry is not affected.

C=A-C  fs  -  A minus C into C

---------

fs = A                       opcode:  EE

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BaE

cycles:    3 + d

Set the specified fs field of register C to the inverse

difference between itself and the corresponding field of

register A. Adjusts Carry.

C=B    fs  -  Copy B to C

---------

fs = A                       opcode:  D9

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab9

cycles:    3 + d

Copy the fs field of register B into the corresponding field

of register C. Carry is not affected.

Page 82

C=C!A  fs  -  C OR A into C

---------

fs = A                       opcode:  0EFA

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0EaA

cycles:    4 + d

Set the fs field of register C to its logical OR with the

corresponding field of register A.  Carry is not affected.

C=C!B  fs  -  C OR B into C

---------

fs = A                       opcode:  0EFD

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0EaD

cycles:    4 + d

Set the fs field of register C to its logical OR with the

corresponding field of register B.  Carry is not affected.

C=C!D  fs  -  C OR D into C

---------

fs = A                       opcode:  0EFF

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0EaF

cycles:    4 + d

Set the fs field of register C to its logical OR with the

corresponding field of register D.  Carry is not affected.

C=C&A  fs  -  C AND A into A

---------

fs = A                       opcode:  0EF2

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea2

cycles:    4 + d

Set the fs field of register C to its logical AND with the

corresponding field of register A.  Carry is not affected.

Page 83

C=C&B  fs  -  C AND B into C

---------

fs = A                       opcode:  0EF5

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea5

cycles:    4 + d

Set the fs field of register C to its logical AND with the

corresponding field of register B.  Carry is not affected.

C=C&D  fs  -  C AND D into C

---------

fs = A                       opcode:  0EF7

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea7

cycles:    4 + d

Set the fs field of register C to its logical AND with the

corresponding field of register D.  Carry is not affected.

C=C+1  fs  -  Increment C

---------

fs = A                       opcode:  E6

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba6

cycles:    3 + d

Increment the specified fs field of register C by one.

Adjusts Carry.

C=C+A  fs   - Sum of C and A into C

---------

fs = A                       opcode:  C2

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa2

cycles:    3 + d

Set the specified fs field of register C to the sum of

itself and the corresponding field of register A. Adjusts

Carry.

Page 84

C=C+B  fs  -  Sum of C and B into C

---------

fs = A                       opcode:  C9

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa9

cycles:    3 + d

Set the specified fs field of register C to the sum of

itself and the corresponding field of register B. Adjusts

Carry.

C=C+C  fs   - Sum of C and C into C

---------

fs = A                       opcode:  C6

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa6

cycles:    3 + d

Double the specified fs field of register C. Adjusts Carry.

C=C+D  fs  -  Sum of C and D into C

---------

fs = A                       opcode:  CB

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AaB

cycles:    3 + d

Set the specified fs field of register C to the sum of

itself and the corresponding field of register D. Adjusts

Carry.

C=C-1  fs  -  Decrement C

---------

fs = A                       opcode:  CE

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AaE

cycles:    3 + d

Decrement the specified fs field of register C by one.

Adjusts Carry.

Page 85

C=C-A  fs  -  C minus A into C

---------

fs = A                       opcode:  E2

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba2

cycles:    3 + d

Set the specified fs field of register C to the difference

between itself and the corresponding field of register A.

Adjusts Carry.

C=C-B  fs  -  C minus B into C

---------

fs = A                       opcode:  E9

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba9

cycles:    3 + d

Set the specified fs field of register C to the difference

between itself and the corresponding field of register B.

Adjusts Carry.

C=C-D  fs  -  C minus D into C

---------

fs = A                       opcode:  EB

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BaB

cycles:    3 + d

Set the specified fs field of register C to the difference

between itself and the corresponding field of register D.

Adjusts Carry.

C=D    fs  -  Copy D to C

---------

fs = A                       opcode:  DB

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbB

cycles:    3 + d

Copy the fs field of register D into the corresponding field

of register C. Carry is not affected.

Page 86

C=DAT0 fsd -  Load C from memory

---------

fs = A                       opcode:  146

cycles:   18

fs = B                       opcode:  14E

cycles:   15

fs = (P,WP,XS,X,S,M,W)       opcode:  156a

cycles:   17 + d

fs = d                       opcode:  15Ex (x=d-1)

cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D0 into

the specified field of register C.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

C=DAT1 fsd -  Load C from memory

---------

fs = A                       opcode:  147

cycles:   18

fs = B                       opcode:  14F

cycles:   15

fs = (P,WP,XS,X,S,M,W)       opcode:  157a

cycles:   17 + d

fs = d                       opcode:  15Fx (x=d-1)

cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D1 into

the specified field of register C.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

Page 87

C=ID      - Request chip ID

---------

opcode:  806

cycles:   11

The chip which has its DAISY-IN line high and its

configuration flag low will send its 5 nibble ID register to

the system bus which will be loaded into the low-order 5

nibbles (A field) of the C register.

C=IN      - Load C with IN

---------

opcode:  803

cycles:    7

Load the low-order 4 nibbles of the C register with the

contents of the Input register.

C=P    n    - Copy P Pointer into Nibble n of C

---------

opcode:  80Cn

cycles:    6

Copy P pointer into C register at digit position specified

by n.

C=R0      - Copy R0 to C

---------

opcode:  118

cycles:   19

The contents of the scratch register R0 is copied to the

working register C.

C=R1      - Copy R1 to C

---------

opcode:  119

cycles:   19

The contents of the scratch register R1 is copied to the

working register C.

Page 88

C=R2      - Copy R2 to C

---------

opcode:  11A

cycles:   19

The contents of the scratch register R2 is copied to the

working register C.

C=R3      - Copy R3 to C

---------

opcode:  11B

cycles:   19

The contents of the scratch register R3 is copied to the

working register C.

C=R4      - Copy R4 to C

---------

opcode:  11C

cycles:   19

The contents of the scratch register R4 is copied to the

working register C.

C=RSTK    - Pop stack to C

---------

opcode:  07

cycles:    8

Pop the top-most address off of the hardware return stack,

placing the address in the lower 5 nibbles (A field) of

register C. The high-order nibbles of C are unchanged.  As

the address is popped from the return stack, a zero address

is inserted at the bottom of the stack.  Compare with the

RTN mnemonic.

Page 89

C=ST      - Status to C

---------

opcode:  09

cycles:    6

Copy the low-order 12 bits of the status register into the

low-order 12 bits (X field) of the C register.

CAEX   fs  -  Exchange Registers C and A

---------

fs = A                       opcode:  DE

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbE

cycles:    3 + d

Exchange the fs fields of registers of C and A. Carry is not

affected.

CBEX   fs  -  Exchange Registers C and B

---------

fs = A                       opcode:  DD

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbD

cycles:    3 + d

Exchange the fs fields of registers of C and B. Carry is not

affected.

CD0EX     - Exchange C and D0 (nibs 0-4)

---------

opcode:  136

cycles:    8

Exchange the A field of register C with Data pointer D0.

Carry is not affected.

Page 90

CD0XS     - Exchange C and D0 short (nibs 0-3)

---------

opcode:  13E

cycles:    7

Exchange the lower 4 nibbles of C with the lower 4 nibbles

of Data pointer D0.  Carry is not affected.

CD1EX     - Exchange C and D1 (nibs 0-4)

---------

opcode:  137

cycles:    8

Exchange the A field of register C with Data pointer D1.

Carry is not affected.

CD1XS     - Exchange C and D1 short (nibs 0-3)

---------

opcode:  13F

cycles:    7

Exchange the lower 4 nibbles of C with the lower 4 nibbles

of Data pointer D1.  Carry is not affected.

CDEX   fs  -  Exchange Registers C and D

---------

fs = A                       opcode:  DF

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbF

cycles:    3 + d

Exchange the fs fields of registers of C and D. Carry is not

affected.

Page 91

CLRHST    - Clear Hardware Status bits

---------

opcode:  82F

cycles:    3

Clears the 4 Hardware Status bits XM, SB, SR and MP.  Note

that the opcode is actually 82x, where x is merely a mask

for which Hardware Status bits to clear, as follows:

bit 0 -  External Module Missing bit (see XM=0 mnemonic)

bit 1 -  Sticky Bit                  (see SB=0 mnemonic)

bit 2 -  Service Request bit         (see SR=0 mnemonic)

bit 3 -  Module Pulled bit           (see MP=0 mnemonic)

For example opcode 829 clears XM and MP.  Although there is

no mnemonic for this, the opcode can be inserted into the

code by using, for example, NIBHEX  829.

CLRST     - Clear Program Status

---------

opcode:  08

cycles:    6

Clear the low-order 12 bits (S0 through S11) of the Program

Status register ST.

CONFIG    - Configure

---------

opcode:  805

cycles:   11

Copy the low-order 5 nibbles (A field) of the C register

into the Configuration register of the chip which has its

DAISY-IN line high and its configuration flag low. See the

"HP-71 Hardware Specification" for information.

CPEX   n    - Exchange Nibble n of C With P Pointer

---------

opcode:  80Fn

cycles:    6

Exchange the P pointer with digit n of the C register.

Page 92

CR0EX     - Exchange C and R0

---------

opcode:  128

cycles:   19

Exchange the contents of the working register C and the

scratch register R0.

CR1EX     - Exchange C and R1

---------

opcode:  129

cycles:   19

Exchange the contents of the working register C and the

scratch register R1.

CR2EX     - Exchange C and R2

---------

opcode:  12A

cycles:   19

Exchange the contents of the working register C and the

scratch register R2.

CR3EX     - Exchange C and R3

---------

opcode:  12B

cycles:   19

Exchange the contents of the working register C and the

scratch register R3.

Page 93

CR4EX     - Exchange C and R4

---------

opcode:  12C

cycles:   19

Exchange the contents of the working register C and the

scratch register R4.

CSL    fs  -  C Shift Left

---------

fs = A                       opcode:  F2

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb2

cycles:    3 + d

Shift the contents of the specified fs field of register C

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

CSLC      - C Shift Left Circular

---------

opcode:  812

cycles:   21

Circular shift register C left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

Page 94

CSR    fs  -  C Shift Right

---------

fs = A                       opcode:  F6

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb6

cycles:    3 + d

Shift the contents of the specified fs field of register C

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

CSRB      - C Shift Right Bit

---------

opcode:  81E

cycles:   20

Shift register C right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

CSRC      - C Shift Right Circular

---------

opcode:  816

cycles:   21

Circular shift register C right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

CSTEX     - Exchange Status

---------

opcode:  0B

cycles:    6

Exchange the low-order 12 bits (S0 through S11) of the

Program Status register ST with the low-order 12 bits of the

C register.

Page 95

D0=(2)  nn    - Load 2 Nibbles Into D0

------------

opcode:  19nn

cycles:    4

Load the low-order two nibbles of D0 with nn. The upper

nibbles of D0 remain unchanged. Any overflow is ignored by

the assembler. The assembled digits of nn are stored in the

opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D0=(4)  nnnn  - Load 4 Nibbles Into D0

------------

opcode:  1Annnn

cycles:    6

Load the low-order four nibbles of D0 with nnnn.  The upper

nibble of D0 remains unchanged. Any overflow is ignored by

the assembler. The assembled digits of nnnn are stored in

the opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D0=(5)  nnnnn - Load 5 Nibbles Into D0

------------

opcode:  1Bnnnnn

cycles:    7

Load all five nibbles of D0 with nnnnn. Any overflow is

ignored by the assembler. The assembled digits of nnnnn are

stored in the opcode in reverse order so that when the

instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

Page 96

D0=A      - Copy A to D0 (nibs 0-4)

---------

opcode:  130

cycles:    8

The A field of register A is copied into Data pointer

register D0.  Carry is not affected.

D0=AS      - Copy A to D0 short (nibs 0-3)

---------

opcode:  138

cycles:    7

The lower 4 nibbles of A are copied into the lower 4 nibbles

of Data pointer register D0.  Carry is not affected.

D0=C      - Copy C to D0 (nibs 0-4)

---------

opcode:  134

cycles:    8

The A field of register C is copied into Data pointer

register D0.  Carry is not affected.

D0=CS      - Copy C to D0 short (nibs 0-3)

---------

opcode:  13C

cycles:    7

The lower 4 nibbles of C are copied into the lower 4 nibbles

of Data pointer register D0.  Carry is not affected.

Page 97

D0=D0+  n   - Add n to D0 (1<=n<=16)

---------

opcode:  16x (x=n-1)

cycles:    7

Increment D0 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D0=D0-  n   - Subtract n from D0 (1<=n<=16) ---------

opcode:  18x (x=n-1)

cycles:    7

Decrement D0 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D0=HEX hh  - Load D0 with hex constant hh

---------

opcode:  19hh

cycles:     4

Load the low-order two nibbles of D0 with the hex constant

hh.  The upper nibbles of D0 remain unchanged. The digits of

hh are stored in the opcode in reverse order so that when

the instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

D0=HEX hhhh  - Load D0 with hex constant hhhh

-----------

opcode:  1Ahhhh

cycles:     6

Load the low-order four nibbles of D0 with the hex constant

hhhh. The upper nibble of D0 remains unchanged. The digits

of hhhh are stored in the opcode in reverse order so that

when the instruction is executed the data will be loaded

into the register with the intended orientation.  See the

section on "Loading Data From Memory".

Page 98

D0=HEX hhhhh  - Load D0 with hex constant hhhhh

------------

opcode:  1Bhhhhh

cycles:    7

Load all five nibbles of D0 with the hex constant hhhhh.

The digits of hhhhh are stored in the opcode in reverse

order so that when the instruction is executed the data will

be loaded into the register with the intended orientation.

See the section on "Loading Data From Memory".

D1=(2)  nn    - Load 2 Nibbles Into D1

---------

opcode:  1Dnn

cycles:    4

Load the low-order two nibbles of D1 with nn. The upper

nibbles of D1 remain unchanged. Any overflow is ignored by

the assembler. The assembled digits of nn are stored in the

opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D1=(4)  nnnn  - Load 4 Nibbles Into D1

------------

opcode:  1Ennnn

cycles:    6

Load the low-order four nibbles of D1 with nnnn. The upper

nibble of D1 remains unchanged.  Any overflow is ignored by

the assembler. The assembled digits of nnnn are stored in

the opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

Page 99

D1=(5)  nnnnn - Load 5 Nibbles Into D1

------------

opcode:  1Fnnnnn

cycles:    7

Load all five nibbles of D1 with nnnnn. Any overflow is

ignored by the assembler. The assembled digits of nnnnn are

stored in the opcode in reverse order so that when the

instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

D1=A      - Copy A to D1 (nibs 0-4)

---------

opcode:  131

cycles:    8

The A field of register A is copied into Data pointer

register D1.  Carry is not affected.

D1=AS     - Copy A to D1 short (nibs 0-3)

---------

opcode:  139

cycles:    7

The lower 4 nibbles of A are copied into the lower 4 nibbles

of Data pointer register D1.  Carry is not affected.

D1=C       - Copy C to D1 (nibs 0-4)

---------

opcode:  135

cycles:    8

The A field of register C is copied into Data pointer

register D1.  Carry is not affected.

Page 100

D1=CS     - Copy C to D1 short (nibs 0-3)

---------

opcode:  13D

cycles:    7

The lower 4 nibbles of C are copied into the lower 4 nibbles

of Data pointer register D1.  Carry is not affected.

D1=D1+  n  - Add n to D1 (1<=n<=16)

---------

opcode:  17x (x=n-1)

cycles:    7

Increment D1 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D1=D1-  n  - Subtract n from D1 (1<=n<=16)

---------

opcode:  1Cx (x=n-1)

cycles:    7

Decrement D1 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D1=HEX hh  - Load D1 with hex constant hh

---------

opcode:  1Dhh

cycles:     4

Load the low-order two nibbles of D1 with the hex constant

hh.  The upper nibbles of D1 remain unchanged. The digits of

hh are stored in the opcode in reverse order so that when

the instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

Page 101

D1=HEX hhhh  - Load D1 with hex constant hhhh

-----------

opcode:  1Ehhhh

cycles:     6

Load the low-order four nibbles of D1 with the hex constant

hhhh. The upper nibble of D1 remains unchanged. The digits

of hhhh are stored in the opcode in reverse order so that

when the instruction is executed the data will be loaded

into the register with the intended orientation.  See the

section on "Loading Data From Memory".

D1=HEX hhhhh  - Load D1 with hex constant hhhhh

------------

opcode:  1Fhhhhh

cycles:      7

Load all five nibbles of D1 with the hex constant hhhhh.

The digits of hhhhh are stored in the opcode in reverse

order so that when the instruction is executed the data will

be loaded into the register with the intended orientation.

See the section on "Loading Data From Memory".

D=-D   fs  -  Two's complement of D into D

---------

fs = A                       opcode:  FB

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BbB

cycles:    3 + d

Complement the specified fs field of D. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

Page 102

D=-D-1 fs  -  One's complement of D into D

---------

fs = A                       opcode:  FF

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BbF

cycles:    3 + d

Perform a one's complement on the specified fs field of D.

Carry is always cleared.

D=0    fs  -  Set D equal to 0

---------

fs = A                       opcode:  D3

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab3

cycles:    3 + d

Set the specified fs field of D to zero.  Carry is not

affected.

D=C    fs  -  Copy C to D

---------

fs = A                       opcode:  D7

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ab7

cycles:    3 + d

Copy the fs field of register C into the corresponding field

of register D. Carry is not affected.

Page 103

D=C-D  fs  -  C minus D into D

---------

fs = A                       opcode:  ED

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  BaD

cycles:    3 + d

Set the specified fs field of register D to the inverse

difference between itself and the corresponding field of

register C. Adjusts Carry.

D=D!C  fs  -  D OR C into D

---------

fs = A                       opcode:  0EFB

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0EaB

cycles:    4 + d

Set the fs field of register D to its logical OR with the

corresponding field of register C.  Carry is not affected.

D=D&C  fs  -  D AND C into D

---------

fs = A                       opcode:  0EF3

cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     opcode:  0Ea3

cycles:    4 + d

Set the fs field of register D to its logical AND with the

corresponding field of register C.  Carry is not affected.

Page 104

D=D+1  fs  -  Increment D

---------

fs = A                       opcode:  E7

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba7

cycles:    3 + d

Increment the specified fs field of register D by one.

Adjusts Carry.

D=D+C  fs  -  Sum of D and C into D

---------

fs = A                       opcode:  C3

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa3

cycles:    3 + d

Set the specified fs field of register D to the sum of

itself and the corresponding field of register C. Adjusts

Carry.

D=D+D  fs  -  Sum of D and D into D

---------

fs = A                       opcode:  C7

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Aa7

cycles:    3 + d

Double the specified fs field of register D. Adjusts Carry.

Page 105

D=D-1  fs  -  Decrement D

---------

fs = A                       opcode:  CF

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AaF

cycles:    3 + d

Decrement the specified fs field of register D by one.

Adjusts Carry.

D=D-C  fs  -  D minus C into D

---------

fs = A                       opcode:  E3

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Ba3

cycles:    3 + d

Set the specified fs field of register D to the difference

between itself and the corresponding field of register C.

Adjusts Carry.

DAT0=A fsd -  Load memory from A

---------

fs = A                       opcode:  140

cycles:   17

fs = B                       opcode:  148

cycles:   14

fs = (P,WP,XS,X,S,M,W)       opcode:  150a

cycles:   16 + d

fs = d                       opcode:  158x (x=d-1)

cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D0 from the

specified field of register A.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

Page 106

DAT0=C fsd  -  Store into memory from C

---------

fs = A                       opcode:  144

cycles:   17

fs = B                       opcode:  14C

cycles:   14

fs = (P,WP,XS,X,S,M,W)       opcode:  154a

cycles:   16 + d

fs = d                       opcode:  15Cx (x=d-1)

cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D0 from the

specified field of register C.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

DAT1=A fs  -  Store into memory from A

---------

fs = A                       opcode:  141

cycles:   17

fs = B                       opcode:  149

cycles:   14

fs = (P,WP,XS,X,S,M,W)       opcode:  151a

cycles:   16 + d

fs = d                       opcode:  159x (x=d-1)

cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D1 from the

specified field of register A.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

Page 107

DAT1=C fsd -  Store into memory from C

---------

fs = A                       opcode:  145

cycles:   17

fs = B                       opcode:  14D

cycles:   14

fs = (P,WP,XS,X,S,M,W)       opcode:  155a

cycles:   16 + d

fs = d                       opcode:  15Dx (x=d-1)

cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D1 from the

specified field of register C.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

DCEX   fs  -  Exchange Registers D and C

---------

fs = A                       opcode:  DF

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  AbF

cycles:    3 + d

Exchange the fs fields of registers of D and C. Carry is not

affected.

DSL    fs  -  D Shift Left

---------

fs = A                       opcode:  F3

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb3

cycles:    3 + d

Shift the contents of the specified fs field of register D

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

Page 108

DSLC      - D Shift Left Circular

---------

opcode:  813

cycles:   21

Circular shift register D left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

DSR    fs  -  D Shift Right

---------

fs = A                       opcode:  F7

cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     opcode:  Bb7

cycles:    3 + d

Shift the contents of the specified fs field of register D

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

DSRB      - D Shift Right Bit

---------

opcode:  81F

cycles:   20

Shift register D right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

DSRC      - D Shift Right Circular

---------

opcode:  817

cycles:   21

Circular shift register D right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

Page 109

GOC    label - Go relative on carry

----------

opcode:  4aa (Carry=0)

cycles:   10 (GO)

3 (NO)

Short relative jump to label if Carry is set. label must be

in the range:

addr - 128  <=  label  <=  addr + 127

where addr is the address of the second nibble of the

opcode. The address offset aa is in two's complement form

and is relative to addr.

GOLONG label  - Go Long

-------------

opcode:  8Caaaa

cycles:   14

Long relative jump to label unconditionally.  label must be

in the range:

addr - 32768  <=  label  <=  addr + 32767

where addr is the address of the third nibble of the opcode.

The address offset aaaa is in two's complement form and is

relative to addr.

GONC   label - Go relative on no carry

----------

opcode:  5aa (Carry=1)

cycles:   10 (GO)

3 (NO)

Short relative jump to label if Carry is clear. label must

be in the range:

addr - 128  <=  label  <=  addr + 127

where addr is the address of the second nibble of the

opcode. The address offset aa is in two's complement form

and is relative to addr.

Page 110

GOSBVL label   - Gosub very long to label

-------------

opcode:  8Faaaaa

cycles:   15

Absolute subroutine jump to aaaaa, which is the absolute

address of label.  See the GOSUB mnemonic.

GOSUB  label - Gosub to label

------------

opcode:  7aaa

cycles:   12

Relative subroutine jump to label.  label must be in the

range:

addr - 2048 <= label <= addr + 2047

where addr is the starting address of the next instruction.

The address offset aaa is in two's complement form and is

relative to addr.

As with all subroutine jumps, the address (addr) of the

instruction following the gosub opcode is pushed onto the

hardware return stack, so that when a corresponding return

is executed, control resumes with the instruction at address

addr.

As the return address is pushed onto the return stack, the

bottom-most address on the stack is discarded.  Therefore,

the return stack always contains 8 addresses, and if pushes

exceed pops by 8 levels, the bottom-most return addresses

are lost. Since the interrupt system requires one level to

process interrupts, only 7 levels of the return stack can be

used by code which must execute when interrupts are enabled.

See the RTN mnemonic for further information.

GOSUBL label   - Gosub long to label

-------------

opcode:  8Eaaaa

cycles:   15

Long relative subroutine jump to label.  label must be in

the range:

addr - 32768  <= label <= addr + 32767

where addr is the starting address of the next instruction.

The address offset aaaa is in two's complement form and is

relative to addr. See the GOSUB mnemonic.

Page 111

GOTO   label - Jump relative

-----------

opcode:  6aaa

cycles:   11

Relative jump to label unconditionally. label must be in the

range:

addr - 2048  <=  label  <=  addr + 2047

where addr is the address of the second nibble of the

opcode. The address offset aaa is in two's complement form

and is relative to addr.

GOVLNG label  - Jump very long

-------------

opcode:  8Daaaaa

cycles:   14

Unconditional jump to aaaaa, which is the absolute address

of label.

GOYES  label  - Jump if Test is True

-----

opcode:  yy

cycles:   included in the accompaning

Test mnemonic cycle time.

GOYES is a mnemonic to specify part of a CPU test opcode.

GOYES must always follow a test mnemonic.  If the condition

of the test is met, a jump is performed to label with Carry

set.  label must be in the range

addr - 128 <= label <= addr + 127

where addr is the starting address of the jump offset yy. If

the test condition is not met, Carry is cleared and control

passes to the next instruction. Compare with RTNYES.

Page 112

INTOFF     - Interrupt Off

----------

opcode:  808F

cycles:    5

Disable the keyboard interrupt system.

INTON    - Interrupt On

--------

opcode:  8080

cycles:    5

Enable the keyboard interrupt system. See the "Hp-71

Hardware Specification" for more information.

LC(m)  n..n - Load C with constant (1<=m<=6)

-----------

opcode:  3xn..n (x=m-1)

cycles:    3+m

Load m digits of the expression n..n to the C register

beginning at the P pointer position, and proceeding toward

higher-order nibbles, with the ability to wrap around the

register.  See the section on "Loading Data From Memory".

LCASC  A..A  - Load C with ASCII constant

-------------

opcode:  3mc..c

(m    = 2*(# of chars)-1;

c..c = ASCII codes)

cycles:    3+2*(# of chars)

Load up to 8 ASCII characters to the C register beginning at

the P pointer position, and proceeding toward higher-order

nibbles, with the ability to wrap around the register.  Each

A represents an ASCII character.  The ASCII characters are

stored in the opcode in reverse order so that when the

instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

Page 113

LCHEX  h..h - Load C with hex constant

-----------

opcode:  3nh..h (n=# of digits-1)

cycles:    4+n

Load up to 16 hex digits into the C register beginning at

the P pointer position, and proceeding toward higher-order

nibbles, with the ability to wrap around the register.  The

hex digits are stored in the opcode in reverse order so that

when the instruction is executed the data will be loaded

into the register with the intended orientation.  See the

section on "Loading Data From Memory".

MP=0      - Clear Module Pulled bit (MP)

---------

opcode:  828

cycles:    3

Clears the Module Pulled bit (MP) and pulls the Module

Pulled Interrupt line low. See CLRHST mnemonic.

NOP3       - Three nibble No-op

----

opcode:  420

cycles:  10 (GO/RTNYES)

3 (NO)

This mnemonic generates a GOC to the next instruction,

effectively skiping three nibbles.

Page 114

NOP4       - Four nibble No-op

----

opcode:  6300

cycles:   11

This mnemonic generates a GOTO to the next instruction,

efectively skiping four nibbles.

NOP5       - Five nibble No-op

----

opcode:  64000

cycles:    11

This mnemonic generates a relative GOTO to +4 nibbles. The

fifth nibble in the opcode is a place holder and is jumped

over.  The mnemonic effectively skips five nibbles.

OUT=C      - Load 3 nibbles of OR -----

opcode:  801

cycles:    6

All nibbles of the Output register are loaded with the low-

order three nibbles of C (X field).

OUT=CS  - Load 1 nibble of OR

------

opcode:  800

cycles:    4

The least significant nibble of the Output register is

loaded with the least significant nibble of the C register.

Page 115

PC=(A)      - Jump (Set PC) indirectly thru A field of A register

---------

opcode:  808C

cycles:    23

This instruction causes the CPU to jump to the address

pointed to by memory at the address specified by the A field

of the A register.  In symbolic form, the operation is

PC=mem(A[A]).  This opcode is not available on the 1LF2

version of the Saturn CPU.

P=C    n    - Copy P pointer into C at Nibble n

---------

opcode:  80Dn

cycles:    6

Copy nibble n of register C into the P pointer.

P=P+1     - Increment P Pointer

---------

opcode:  0C

cycles:    3

Increment the P pointer.  If P is incremented past F it will

automatically wrap around to 0. This instruction is always

executed in HEX mode.  Adjusts carry.

P=P-1     - Decrement P Pointer

---------

opcode:  0D

cycles:    3

Decrement the P pointer. If P is decremented past 0 it

automatically wraps around to F. This instruction is always

executed in HEX mode.  Adjusts Carry.

Page 116

P=     n    - Set P Pointer to n

---------

opcode:  2n

cycles:    2

Set the P pointer to n.

R0=A      - Copy A to register R0

---------

opcode:  100

cycles:   19

The contents of the working register A is copied to the

scratch register R0.

R0=C      - Copy C to register R0

---------

opcode:  108

cycles:   19

The contents of the working register C is copied to the

scratch register R0.

R1=A      - Copy A to register R1

---------

opcode:  101

cycles:   19

The contents of the working register A is copied to the

scratch register R1.

Page 117

R1=C      - Copy C to register R1

---------

opcode:  109

cycles:   19

The contents of the working register C is copied to the

scratch register R1.

R2=A      - Copy A to register R2

---------

opcode:  102

cycles:   19

The contents of the working register A is copied to the

scratch register R2.

R2=C      - Copy C to register R2

---------

opcode:  10A

cycles:   19

The contents of the working register C is copied to the

scratch register R2.

R3=A      - Copy A to register R3

---------

opcode:  103

cycles:   19

The contents of the working register A is copied to the

scratch register R3.

Page 118

R3=C      - Copy C to register R3

---------

opcode:  10B

cycles:   19

The contents of the working register C is copied to the

scratch register R3.

R4=A      - Copy A to register R4

---------

opcode:  104

cycles:   19

The contents of the working register A is copied to the

scratch register R4.

R4=C      - Copy C to register R4

---------

opcode:  10C

cycles:   19

The contents of the working register C is copied to the

scratch register R4.

RESET     - System reset

---------

opcode:  80A

cycles:    6

The System Reset Bus Command is issued with all chips

performing a local reset.  The reset function will vary

according to the chip type.

Page 119

RSI       - Reset Interrupt System

---------

opcode:  80810

cycles:    6

This instruction causes CPU to consider any input line (ie

input register bits) presently high as a new interrupt.  If

the CPU is presently in the interrupt routine it will wait

for an RTI before vectoring, otherwise the CPU will vector

immediately following the RSI instruction.  For a complete

discussion on the interrupt system see the CPU Hardware

Specification (A-1LK7-9005-1).  This instruction is not

available on the 1LF2 version of the Saturn CPU.

RSTK=C    - Push C to Return Stack

---------

opcode:  06

cycles:    8

Push the low-order 5 nibbles (A field) of the C register

onto the Return Stack. See the GOSUB mnemonic.

RTI       - Return from interrupt

---------

opcode:  0F

cycles:    9

Return and re-enable the interrupt system.  See the RTN

mnemonic.

RTN        - Return

---------

opcode:  01

cycles:    9

Return control to the top address on the hardware return

stack. The top address on the hardware return stack is

popped off and placed in the program counter PC.  As the

address is popped off the stack, a zero address is inserted

at the bottom of the stack.

Page 120

Therefore the the hardware return stack always contains 8

addresses, and if more pops (returns) than pushes (gosubs)

are performed, zeros will be read off the stack.  Such an

attempt to "return" to address 0 results in a memory reset,

since the memory reset code of the operating system resides

at address 0.

RTNC      -Return on carry

---------

opcode:  400

cycles:   10 (RTN)

3 (NO)

Return if Carry is set. See RTN mnemonic.

RTNCC     - Return, clear carry

---------

opcode:  03

cycles:    9

Return and set Carry. See RTN mnemonic.

RTNNC   - Return on no carry

-------

opcode:  500 (Carry=1)

cycles:   10 (RTN)

3 (NO)

Return if Carry is not set.  See RTN mnemonic.

Page 121

RTNSC     - Return, set carry

---------

opcode:  02

cycles:    9

Return and set Carry.  See RTN mnemonic.

RTNSXM    - Return, set External Module Missing bit (XM)

---------

opcode:  00

cycles:    9

Return and set the External Module Missing bit (XM).  Since

the opcode is zero, this mnemonic is executed on a jump to a

non-existent memory device.  See the "HP-71 Hardware

Specification" for more information.  See also the RTN

mnemonic.

RTNYES  - Return if Test is True

---------

opcode:  00

cycles:  included in the accompaning

mnemonic cycle time.

RTNYES is a mnemonic to specify part of a CPU test opcode.

RTNYES must always follow a test mnemonic.  If the test

condition is met, Carry is set and a return is executed. If

the test condition is not met, control passes to the

instruction following the RTNYES.  Compare with the RTN and

GOYES mnemonics.

Page 122

SB=0      - Clear Sticky Bit (SB)

---------

opcode:  822

cycles:    3

Clear the Sticky Bit (SB).  See CLRHST mnemonic.

SETDEC    - Set decimal

---------

opcode:  05

cycles:    3

Set CPU arithmetic mode to decimal.

SETHEX    - Set hexadecimal mode

---------

opcode:  04

cycles:    3

Set CPU arithmetic mode to hexadecimal.

SHUTDN    - System Shutdown

---------

opcode:  807

cycles:    5

When this mnemonic is executed the CPU sends out the

Shutdown Bus Command and stops its clock.  Issuing the

SHUTDN command with the output register=000 (see OUT=C or

OUT=CS) will cause the PC to be set to zero, causing an

automatic cold-start in some systems, or a system halt

in the HP 48.

Page 123

SR=0      - Clear Service Request bit (SR)

---------

opcode:  824

cycles:    3

Clear the Service Request bit (SR).  See the CLRHST

mnemonic.

SREQ?     - Service Request

---------

opcode:  80E

cycles:    7

This mnemonic sets the Service Request bit (SR) if any chip

on the system bus requests service.  When it is executed, a

Service Request Bus Command is issued on the system bus to

poll all chips for a Service Request.  If any chip requests

service, a bus line will be pulled high during the next

strobe following the Service Request Bus Command.  This

value of the bus will be latched into the least significant

nibble of the C register. The bus line pulled high

determines the device type according to the following table.

Bit           Device

---   ------------------------

3     Unused

2     Card Reader

1     HP-IL Mailbox

0     Display Driver (timer)

If any bus line is high, the Service Request bit (SR) will

be set. See the "HP-71 Hardware Specification" for more

information. See also the ?SREQ and SR=0 mnemonics.

Page 124

ST=0   n  - Clear Program Status bit n

---------

opcode:  84n

cycles:    4

Clear the Program Status bit selected by n.

ST=1   n  - Set Program Status bit n

---------

opcode:  85n

cycles:    4

Set the Program Status bit selected by n.

ST=C      - C to Status

---------

opcode:  0A

cycles:    6

Copy the low-order 12 bits of the Status register (X field)

into the low-order 12 bits  of the C register.

UNCNFG    - Unconfigure

---------

opcode:  804

cycles:   12

Load the low-order 5 nibbles (A field) of the C register

into each Data pointer with the device addressed by the Data

pointer unconfiguring.  See the "HP-71 Hardware

Specification" for more information.

XM=0      - Clear External Module Missing bit (XM)

---------

opcode:  821

cycles:    3

Clear the External Module Missing bit (XM).  This hardware

status bit is set by the RTNSXM mnemonic.  See the CLRHST

mnemonic.

Page 125

9.  Alphabetic Mnemonic List

This chapter contains all the code-generating instructions

which are recognized by SASM.EXE.  Instructions found in the

HP 28 and HP 48 but not the HP 71 are marked with **.

Symbols used in modifier field:

fs      Field Select character.

rfs     Restricted Field Selection (S,P,WP, and XS not allowed).

d       Single-nibble field.

expr    Expression.

hh      Two-digit hexadecimal value.

hhhh    Four-digit hexadecimal value.

hhhhh   Five-digit hexadecimal value.

label   Label destination.

ASCII   ASCII character string.

Symbols used in code field:

tt      Code included in test instruction.

a       Field select in the range 0-7.

b       Field select in the range 8-F.

c       Single-nibble length field (Load Constant).

f       Field select including A field.

h...h   Hexadecimal value.

n       Nibble whose value is d.

n...n   Nibbles whose value is expr.

m       Nibble whose value is d - 1.

x...x   Nibbles corresponding to ASCII characters.

y       Reference to a symbol (relative or absolute).

Instruction     Code    Comments

?A#0    A       8ACyy   Requires GOYES or RTNYES; Affects Carry

?A#0    fs      9aCyy   Requires GOYES or RTNYES; Affects Carry

?A#B    A       8A4yy   Requires GOYES or RTNYES; Affects Carry

?A#B    fs      9a4yy   Requires GOYES or RTNYES; Affects Carry

?A#C    A       8A6yy   Requires GOYES or RTNYES; Affects Carry

?A#C    fs      9a6yy   Requires GOYES or RTNYES; Affects Carry

?A<=B   A       8BCyy   Requires GOYES or RTNYES; Affects Carry

?A<=B   fs      9bCyy   Requires GOYES or RTNYES; Affects Carry

?A<=C   A       8BAyy   Requires GOYES or RTNYES; Affects Carry

?A<=C   fs      9bAyy   Requires GOYES or RTNYES; Affects Carry

?A<B    A       8B4yy   Requires GOYES or RTNYES; Affects Carry

?A<B    fs      9b4yy   Requires GOYES or RTNYES; Affects Carry

?A<C    A       8B2yy   Requires GOYES or RTNYES; Affects Carry

?A<C    fs      9b2yy   Requires GOYES or RTNYES; Affects Carry

?A=0    A       8A8yy   Requires GOYES or RTNYES; Affects Carry

?A=0    fs      9a8yy   Requires GOYES or RTNYES; Affects Carry

?A=B    A       8A0yy   Requires GOYES or RTNYES; Affects Carry

?A=B    fs      9a0yy   Requires GOYES or RTNYES; Affects Carry

?A=C    A       8A2yy   Requires GOYES or RTNYES; Affects Carry

?A=C    fs      9a2yy   Requires GOYES or RTNYES; Affects Carry

?A>=B   A       8B8yy   Requires GOYES or RTNYES; Affects Carry

?A>=B   fs      9b8yy   Requires GOYES or RTNYES; Affects Carry

Page 126

?A>=C   A       8BEyy   Requires GOYES or RTNYES; Affects Carry

?A>=C   fs      9bEyy   Requires GOYES or RTNYES; Affects Carry

?A>B    A       8B0yy   Requires GOYES or RTNYES; Affects Carry

?A>B    fs      9b0yy   Requires GOYES or RTNYES; Affects Carry

?A>C    A       8B6yy   Requires GOYES or RTNYES; Affects Carry

?A>C    fs      9b6yy   Requires GOYES or RTNYES; Affects Carry

?ABIT=0 d       8086nyy ** Requires GOYES or RTNYES; Affects Carry

?ABIT=1 d       8087nyy ** Requires GOYES or RTNYES; Affects Carry

?B#0    A       8ADyy   Requires GOYES or RTNYES; Affects Carry

?B#0    fs      9aDyy   Requires GOYES or RTNYES; Affects Carry

?B#A    A       8A4yy   Requires GOYES or RTNYES; Affects Carry

?B#A    fs      9a4yy   Requires GOYES or RTNYES; Affects Carry

?B#C    A       8A5yy   Requires GOYES or RTNYES; Affects Carry

?B#C    fs      9a5yy   Requires GOYES or RTNYES; Affects Carry

?B<=A   A       8B8yy   Requires GOYES or RTNYES; Affects Carry

?B<=A   fs      9b8yy   Requires GOYES or RTNYES; Affects Carry

?B<=C   A       8BDyy   Requires GOYES or RTNYES; Affects Carry

?B<=C   fs      9bDyy   Requires GOYES or RTNYES; Affects Carry

?B<A    A       8B0yy   Requires GOYES or RTNYES; Affects Carry

?B<A    fs      9b0yy   Requires GOYES or RTNYES; Affects Carry

?B<C    A       8B5yy   Requires GOYES or RTNYES; Affects Carry

?B<C    fs      9b5yy   Requires GOYES or RTNYES; Affects Carry

?B=0    A       8A9yy   Requires GOYES or RTNYES; Affects Carry

?B=0    fs      9a9yy   Requires GOYES or RTNYES; Affects Carry

?B=A    A       8A0yy   Requires GOYES or RTNYES; Affects Carry

?B=A    fs      9a0yy   Requires GOYES or RTNYES; Affects Carry

?B=C    A       8A1yy   Requires GOYES or RTNYES; Affects Carry

?B=C    fs      9a1yy   Requires GOYES or RTNYES; Affects Carry

?B>=A   A       8BCyy   Requires GOYES or RTNYES; Affects Carry

?B>=A   fs      9bCyy   Requires GOYES or RTNYES; Affects Carry

?B>=C   A       8B9yy   Requires GOYES or RTNYES; Affects Carry

?B>=C   fs      9b9yy   Requires GOYES or RTNYES; Affects Carry

?B>A    A       8B4yy   Requires GOYES or RTNYES; Affects Carry

?B>A    fs      9b4yy   Requires GOYES or RTNYES; Affects Carry

?B>C    A       8B1yy   Requires GOYES or RTNYES; Affects Carry

?B>C    fs      9b1yy   Requires GOYES or RTNYES; Affects Carry

?C#0    A       8AEyy   Requires GOYES or RTNYES; Affects Carry

?C#0    fs      9aEyy   Requires GOYES or RTNYES; Affects Carry

?C#A    A       8A6yy   Requires GOYES or RTNYES; Affects Carry

?C#A    fs      9a6yy   Requires GOYES or RTNYES; Affects Carry

?C#B    A       8A5yy   Requires GOYES or RTNYES; Affects Carry

?C#B    fs      9a5yy   Requires GOYES or RTNYES; Affects Carry

?C#D    A       8A7yy   Requires GOYES or RTNYES; Affects Carry

?C#D    fs      9a7yy   Requires GOYES or RTNYES; Affects Carry

?C<=A   A       8BEyy   Requires GOYES or RTNYES; Affects Carry

?C<=A   fs      9bEyy   Requires GOYES or RTNYES; Affects Carry

?C<=B   A       8B9yy   Requires GOYES or RTNYES; Affects Carry

?C<=B   fs      9b9yy   Requires GOYES or RTNYES; Affects Carry

?C<=D   A       8BByy   Requires GOYES or RTNYES; Affects Carry

?C<=D   fs      9bByy   Requires GOYES or RTNYES; Affects Carry

?C<A    A       8B6yy   Requires GOYES or RTNYES; Affects Carry

?C<A    fs      9b6yy   Requires GOYES or RTNYES; Affects Carry

?C<B    A       8B1yy   Requires GOYES or RTNYES; Affects Carry

?C<B    fs      9b1yy   Requires GOYES or RTNYES; Affects Carry

?C<D    A       8B3yy   Requires GOYES or RTNYES; Affects Carry

?C<D    fs      9b3yy   Requires GOYES or RTNYES; Affects Carry

Page 127

?C=0    A       8AAyy   Requires GOYES or RTNYES; Affects Carry

?C=0    fs      9aAyy   Requires GOYES or RTNYES; Affects Carry

?C=A    A       8A2yy   Requires GOYES or RTNYES; Affects Carry

?C=A    fs      9a2yy   Requires GOYES or RTNYES; Affects Carry

?C=B    A       8A1yy   Requires GOYES or RTNYES; Affects Carry

?C=B    fs      9a1yy   Requires GOYES or RTNYES; Affects Carry

?C=D    A       8A3yy   Requires GOYES or RTNYES; Affects Carry

?C=D    fs      9a3yy   Requires GOYES or RTNYES; Affects Carry

?C>=A   A       8BAyy   Requires GOYES or RTNYES; Affects Carry

?C>=A   fs      9bAyy   Requires GOYES or RTNYES; Affects Carry

?C>=B   A       8BDyy   Requires GOYES or RTNYES; Affects Carry

?C>=B   fs      9bDyy   Requires GOYES or RTNYES; Affects Carry

?C>=D   A       8BFyy   Requires GOYES or RTNYES; Affects Carry

?C>=D   fs      9bFyy   Requires GOYES or RTNYES; Affects Carry

?C>A    A       8B2yy   Requires GOYES or RTNYES; Affects Carry

?C>A    fs      9b2yy   Requires GOYES or RTNYES; Affects Carry

?C>B    A       8B5yy   Requires GOYES or RTNYES; Affects Carry

?C>B    fs      9b5yy   Requires GOYES or RTNYES; Affects Carry

?C>D    A       8B7yy   Requires GOYES or RTNYES; Affects Carry

?C>D    fs      9b7yy   Requires GOYES or RTNYES; Affects Carry

?CBIT=0 d       808Anyy ** Requires GOYES or RTNYES; Affects Carry

?CBIT=1 d       808Bnyy ** Requires GOYES or RTNYES; Affects Carry

?D#0    A       8AFyy   Requires GOYES or RTNYES; Affects Carry

?D#0    fs      9aFyy   Requires GOYES or RTNYES; Affects Carry

?D#C    A       8A7yy   Requires GOYES or RTNYES; Affects Carry

?D#C    fs      9a7yy   Requires GOYES or RTNYES; Affects Carry

?D<=C   A       8BFyy   Requires GOYES or RTNYES; Affects Carry

?D<=C   fs      9bFyy   Requires GOYES or RTNYES; Affects Carry

?D<C    A       8B7yy   Requires GOYES or RTNYES; Affects Carry

?D<C    fs      9b7yy   Requires GOYES or RTNYES; Affects Carry

?D=0    A       8AByy   Requires GOYES or RTNYES; Affects Carry

?D=0    fs      9aByy   Requires GOYES or RTNYES; Affects Carry

?D=C    A       8A3yy   Requires GOYES or RTNYES; Affects Carry

?D=C    fs      9a3yy   Requires GOYES or RTNYES; Affects Carry

?D>=C   A       8BByy   Requires GOYES or RTNYES; Affects Carry

?D>=C   fs      9bByy   Requires GOYES or RTNYES; Affects Carry

?D>C    A       8B3yy   Requires GOYES or RTNYES; Affects Carry

?D>C    fs      9b3yy   Requires GOYES or RTNYES; Affects Carry

?HS=0   d       83nyy   Requires GOYES or RTNYES; Affects Carry

?MP=0           838yy   Requires GOYES or RTNYES; Affects Carry

?P#     d       88nyy   Requires GOYES or RTNYES; Affects Carry

?P=     d       89nyy   Requires GOYES or RTNYES; Affects Carry

?SB=0           832yy   Requires GOYES or RTNYES; Affects Carry

?SR=0           834yy   Requires GOYES or RTNYES; Affects Carry

?ST=0   d       86nyy   Requires GOYES or RTNYES; Affects Carry

?ST=1   d       87nyy   Requires GOYES or RTNYES; Affects Carry

?XM=0           831yy   Requires GOYES or RTNYES; Affects Carry

A=-A    A       F8      Affects Carry

A=-A    fs      Bb8     Affects Carry

A=-A-1  A       FC      Clears Carry

A=-A-1  fs      BbC     Clears Carry

A=0     A       D0

A=0     fs      Ab0

A=A!B   fs      0Ef8

A=A!C   fs      0EfE

A=A&B   fs      0Ef0

A=A&C   fs      0Ef6

A=A+1   A       E4      Affects Carry

Page 128

A=A+1   fs      Ba4     Affects Carry

A=A+A   A       C4      Affects Carry

A=A+A   fs      Aa4     Affects Carry

A=A+B   A       C0      Affects Carry

A=A+B   fs      Aa0     Affects Carry

A=A+C   A       CA      Affects Carry

A=A+C   fs      AaA     Affects Carry

A=A+CON rfs,d   818f0m  ** Affects Carry

A=A-1   A       CC      Affects Carry

A=A-1   fs      AaC     Affects Carry

A=A-B   A       E0      Affects Carry

A=A-B   fs      Ba0     Affects Carry

A=A-C   A       EA      Affects Carry

A=A-C   fs      BaA     Affects Carry

A=A-CON rfs,d   818f8m  ** Affects Carry

A=B     A       D4

A=B     fs      Ab4

A=B!A   fs      0Ef8

A=B&A   fs      0Ef0

A=B+A   A       C0      Affects Carry

A=B+A   fs      Aa0     Affects Carry

A=B-A   A       EC      Affects Carry

A=B-A   fs      BaC     Affects Carry

A=C     A       DA

A=C     fs      AbA

A=C!A   fs      0EfE

A=C&A   fs      0Ef6

A=C+A   A       CA      Affects Carry

A=C+A   fs      AaA     Affects Carry

A=DAT0  A       142

A=DAT0  B       14A

A=DAT0  fs      152a

A=DAT0  d       15Am

A=DAT1  A       143

A=DAT1  B       14B

A=DAT1  fs      153a

A=DAT1  d       15Bm

A=IN            802

A=PC            81B4    **

A=R0            110

A=R0.F  W       110     **

A=R0.F  fs      81Af10  **

A=R1            111

A=R1.F  W       111     **

A=R1.F  fs      81Af11  **

A=R2            112

A=R2.F  W       112     **

A=R2.F  fs      81Af12  **

A=R3            113

A=R3.F  W       113     **

A=R3.F  fs      81Af13  **

A=R4            114

A=R4.F  W       114     **

A=R4.F  fs      81Af14  **

ABEX    A       DC

ABEX    fs      AbC

ABIT=0  d       8084n   ** Clears bit d of A register

ABIT=1  d       8085n   ** Sets bit d of A register

Page 129

ACEX    A       DE

ACEX    fs      AbE

AD0EX           132

AD0XS           13A

AD1EX           133

AD1XS           13B

APCEX           81B6    ** Continues elsewhere

AR0EX           120

AR0EX.F fs      81Af20  **

AR1EX           121

AR1EX.F fs      81Af21  **

AR2EX           122

AR2EX.F fs      81Af22  **

AR3EX           123

AR3EX.F fs      81Af23  **

AR4EX           124

AR4EX.F fs      81Af24  **

ASL     A       F0

ASL     fs      Bb0

ASLC            810

ASR     A       F4

ASR     fs      Bb4

ASRB            81C

ASRB.F  fs      819f0   **

ASRC            814

B=-B    A       F9      Affects Carry

B=-B    fs      Bb9     Affects Carry

B=-B-1  A       FD      Clears Carry

B=-B-1  fs      BbD     Clears Carry

B=0     A       D1

B=0     fs      Ab1

B=A     A       D8

B=A     fs      Ab8

B=A!B   fs      0EfC

B=A&B   fs      0Ef4

B=A+B   A       C8      Affects Carry

B=A+B   fs      Aa8     Affects Carry

B=B!A   fs      0EfC

B=B!C   fs      0Ef9

B=B&A   fs      0Ef4

B=B&C   fs      0Ef1

B=B+1   A       E5      Affects Carry

B=B+1   fs      Ba5     Affects Carry

B=B+A   A       C8      Affects Carry

B=B+A   fs      Aa8     Affects Carry

B=B+B   A       C5      Affects Carry

B=B+B   fs      Aa5     Affects Carry

B=B+C   A       C1      Affects Carry

B=B+C   fs      Aa1     Affects Carry

B=B+CON rfs,d   818f1m  ** Affects Carry

B=B-1   A       CD      Affects Carry

B=B-1   fs      AaD     Affects Carry

B=B-A   A       E8      Affects Carry

B=B-A   fs      Ba8     Affects Carry

B=B-C   A       E1      Affects Carry

B=B-C   fs      Ba1     Affects Carry

B=B-CON rfs,d   818f9m  ** Affects Carry

B=C     A       D5

Page 130

B=C     fs      Ab5

B=C!B   fs      0Ef9

B=C&B   fs      0Ef1

B=C+B   A       C1      Affects Carry

B=C+B   fs      Aa1     Affects Carry

B=C-B   A       ED      Affects Carry

B=C-B   fs      BaD     Affects Carry

BAEX    A       DC

BAEX    fs      AbC

BCEX    A       DD

BCEX    fs      AbD

BSL     A       F1

BSL     fs      Bb1

BSLC            811

BSR     A       F5

BSR     fs      Bb5

BSRB            81D

BSRB.F  W       81D     **

BSRB.F  fs      819f1   **

BSRC            815

BUSCB           8083    **

BUSCC           80B

BUSCD           808D    **

C+P+1           809     Affects Carry

C=-C    A       FA      Affects Carry

C=-C    fs      BbA     Affects Carry

C=-C-1  A       FE      Clears Carry

C=-C-1  fs      BbE     Clears Carry

C=0     A       D2

C=0     fs      Ab2

C=A     A       D6

C=A     fs      Ab6

C=A!C   fs      0EfA

C=A&C   fs      0Ef2

C=A+C   A       C2      Affects Carry

C=A+C   fs      Aa2     Affects Carry

C=A-C   A       EE      Affects Carry

C=A-C   fs      BaE     Affects Carry

C=B     A       D9

C=B     fs      Ab9

C=B!C   fs      0EfD

C=B&C   fs      0Ef5

C=B+C   A       C9      Affects Carry

C=B+C   fs      Aa9     Affects Carry

C=C!A   fs      0EfA

C=C!B   fs      0EfD

C=C!D   fs      0EfF

C=C&A   fs      0Ef2

C=C&B   fs      0Ef5

C=C&D   fs      0Ef7

C=C+1   A       E6      Affects Carry

C=C+1   fs      Ba6     Affects Carry

C=C+A   A       C2      Affects Carry

C=C+A   fs      Aa2     Affects Carry

C=C+B   A       C9      Affects Carry

C=C+B   fs      Aa9     Affects Carry

C=C+C   A       C6      Affects Carry

C=C+C   fs      Aa6     Affects Carry

Page 131

C=C+CON rfs,d   818f2m  ** Affects Carry

C=C+D   A       CB      Affects Carry

C=C+D   fs      AaB     Affects Carry

C=C+P+1         809     Affects Carry

C=C-1   A       CE      Affects Carry

C=C-1   fs      AaE     Affects Carry

C=C-A   A       E2      Affects Carry

C=C-A   fs      Ba2     Affects Carry

C=C-B   A       E9      Affects Carry

C=C-B   fs      Ba9     Affects Carry

C=C-CON rfs,d   818fAm  ** Affects Carry

C=C-D   A       EB      Affects Carry

C=C-D   fs      BaB     Affects Carry

C=D     A       DB

C=D     fs      AbB

C=D!C   fs      0EfF

C=D&C   fs      0Ef7

C=D+C   A       CB      Affects Carry

C=D+C   fs      AaB     Affects Carry

C=DAT0  A       146

C=DAT0  B       14E

C=DAT0  fs      156a

C=DAT0  d       15Em

C=DAT1  A       147

C=DAT1  B       14F

C=DAT1  fs      157a

C=DAT1  d       15Fm

C=ID            806

C=IN            803

C=P     d       80Cn

C=PC            81B5    **

C=R0            118

C=R0.F  fs      81Af18  **

C=R1            119

C=R1.F  fs      81Af19  **

C=R2            11A

C=R2.F  fs      81Af1A  **

C=R3            11B

C=R3.F  fs      81Af1B  **

C=R4            11C

C=R4.F  fs      81Af1C  **

C=RSTK          07

C=ST            09

CAEX    A       DE

CAEX    fs      AbE

CBEX    A       DD

CBEX    fs      AbD

CBIT=0  d       8088n   ** Clears bit d of C register

CBIT=1  d       8089n   ** Sets bit d of C register

CD0EX           136

CD0XS           13E

CD1EX           137

CD1XS           13F

CDEX    A       DF

CDEX    fs      AbF

CLRHST          82F

CLRST           08

CON(1)  expr    n

Page 132

CON(2)  expr    nn

CON(3)  expr    nnn

CON(4)  expr    nnnn

CON(5)  expr    nnnnn

CON(6)  expr    nnnnnn

CON(7)  expr    nnnnnnn

CON(8)  expr    nnnnnnnn

CONFIG          805

CPCEX           81B7    ** Continues elsewhere

CPEX    d       80Fn

CR0EX           128

CR0EX.F fs      81Af28  **

CR1EX           129

CR1EX.F fs      81Af29  **

CR2EX           12A

CR2EX.F fs      81Af2A  **

CR3EX           12B

CR3EX.F fs      81Af2B  **

CR4EX           12C

CR4EX.F fs      81Af2C  **

CSL     A       F2

CSL     fs      Bb2

CSLC            812

CSR     A       F6

CSR     fs      Bb6

CSRB            81E

CSRB.F  fs      819f2   **

CSRC            816

CSTEX           0B

D0=(2)  expr    19nn

D0=(4)  expr    1Annnn

D0=(5)  expr    1Bnnnnn

D0=A            130

D0=AS           138

D0=C            134

D0=CS           13C

D0=D0+  d       16m     Affects Carry

D0=D0-  d       18m     Affects Carry

D0=HEX  hh      19hh

D0=HEX  hhhh    1Ahhhh

D0=HEX  hhhhh   1Bhhhhh

D1=(2)  expr    1Dnn

D1=(4)  expr    1Ennnn

D1=(5)  expr    1Fnnnnn

D1=A            131

D1=AS           139

D1=C            135

D1=CS           13D

D1=D1+  d       17m     Affects Carry

D1=D1-  d       1Cm     Affects Carry

D1=HEX  hh      1Dhh

D1=HEX  hhhh    1Ehhhh

D1=HEX  hhhhh   1Fhhhhh

D=-D    A       FB      Affects Carry

D=-D    fs      BbB     Affects Carry

D=-D-1  A       FF      Clears Carry

D=-D-1  fs      BbF     Clears Carry

D=0     A       D3

Page 133

D=0     fs      Ab3

D=C     A       D7

D=C     fs      Ab7

D=C!D   fs      0EfB

D=C&D   fs      0Ef3

D=C+D   A       C3      Affects Carry

D=C+D   fs      Aa3     Affects Carry

D=C-D   A       EF      Affects Carry

D=C-D   fs      BaF     Affects Carry

D=D!C   fs      0EfB

D=D&C   fs      0Ef3

D=D+1   A       E7      Affects Carry

D=D+1   fs      Ba7     Affects Carry

D=D+C   A       C3      Affects Carry

D=D+C   fs      Aa3     Affects Carry

D=D+CON rfs,d   818f3m  ** Affects Carry

D=D+D   A       C7      Affects Carry

D=D+D   fs      Aa7     Affects Carry

D=D-1   A       CF      Affects Carry

D=D-1   fs      AaF     Affects Carry

D=D-C   A       E3      Affects Carry

D=D-C   fs      Ba3     Affects Carry

D=D-CON rfs,d   818fBm  ** Affects Carry

DAT0=A  A       140

DAT0=A  B       148

DAT0=A  fs      150a

DAT0=A  d       158m

DAT0=C  A       144

DAT0=C  B       14C

DAT0=C  fs      154a

DAT0=C  d       15Cm

DAT1=A  A       141

DAT1=A  B       149

DAT1=A  fs      151a

DAT1=A  d       159m

DAT1=C  A       145

DAT1=C  B       14D

DAT1=C  fs      155a

DAT1=C  d       15Dm

DCEX    A       DF

DCEX    fs      AbF

DSL     A       F3

DSL     fs      Bb3

DSLC            813

DSR     A       F7

DSR     fs      Bb7

DSRB            81F

DSRB.F  fs      819f3   **

DSRC            817

GOC     label   4yy     Continues elsewhere if Carry set

GOLONG  label   8Cyyyy  Continues elsewhere

GONC    label   5yy     Continues elsewhere if Carry clear

GOSBVL  label   8Fyyyyy Affects Carry

GOSUB   label   7yyy    Affects Carry

GOSUBL  label   8Eyyyy  Affects Carry

GOTO    label   6yyy    Continues elsewhere

GOVLNG  label   8Dyyyyy Continues elsewhere

GOYES   label   tt      Must follow a test instruction

Page 134

HS=0    d       82n     Clears (MP SR SB XM) according to d

Example: 823 Clears SB and XM

INTOFF          808F

INTON           8080

LA(1)   expr    80820n          ** Load A register

LA(2)   expr    80821nn         ** Load A register

LA(3)   expr    80822nnn        ** Load A register

LA(4)   expr    80823nnnn       ** Load A register

LA(5)   expr    80824nnnnn      ** Load A register

LA(6)   expr    80825nnnnnn     ** Load A register

LA(7)   expr    80826nnnnnnn    ** Load A register

LA(8)   expr    80827nnnnnnnn   ** Load A register

LC(1)   expr    30n             Load C register

LC(2)   expr    31nn            Load C register

LC(3)   expr    32nnn           Load C register

LC(4)   expr    33nnnn          Load C register

LC(5)   expr    34nnnnn         Load C register

LC(6)   expr    35nnnnnn        Load C register

LC(7)   expr    36nnnnnnn       Load C register

LC(8)   expr    37nnnnnnnn      Load C register

MP=0            828

NIBASC  ASCII x...x

NIBFS   fs      f

NIBHEX  hhhhhhh hhhhhhh Affects Carry

NOP3            820

NOP4            6300

NOP5            64000

OUT=C           801

OUT=CS          800

P=      d       2n

P=C     d       80Dn

P=P+1           0C      Affects Carry

P=P-1           0D      Affects Carry

PC=(A)          808C    ** Continues elsewhere

PC=(C)          808E    ** Continues elsewhere

PC=A            81B2    ** Continues elsewhere

PC=C            81B3    ** Continues elsewhere

R0=A            100

R0=A.F  fs      81Af00  **

R0=C            108

R0=C.F  fs      81Af08  **

R1=A            101

R1=A.F  fs      81Af01  **

R1=C            109

R1=C.F  fs      81Af09  **

R2=A            102

R2=A.F  fs      81Af02  **

R2=C            10A

R2=C.F  fs      81Af0A  **

R3=A            103

R3=A.F  fs      81Af03  **

R3=C            10B

R3=C.F  fs      81Af0B  **

R4=A            104

R4=A.F  fs      81Af04  **

R4=C            10C

R4=C.F  fs      81Af0C  **

REL(1)  label   y

Page 135

REL(2)  label   yy

REL(3)  label   yyy

REL(4)  label   yyyy

REL(5)  label   yyyyy

REL(6)  label   yyyyyy

REL(7)  label   yyyyyyy

REL(8)  label   yyyyyyyy

RESET           80A

RSI             80810   **

RSTK=C          06

RTI             0F      Continues elsewhere

RTN             01      Continues elsewhere

RTNC            400     Continues elsewhere if Carry set

RTNCC           03      Continues elsewhere clearing Carry

RTNNC           500     Continues elsewhere if Carry clear

RTNSC           02      Continues elsewhere setting Carry

RTNSXM          00      Continues elsewhere setting XM bit

RTNYES          **      Must follow a test instruction

SB=0            822

SETDEC          05

SETHEX          04

SHUTDN          807

SLINK   label   yyyyy

SR=0            824

SREQ?           80E

ST=0    d       84n

ST=1    d       85n

ST=C            0A

STRING  ASCII x...x

UNCNFG          804

XM=0            821

Page 136

10.  Error Messages

This is a list of all errors which are generated by the

assembler and explanations of what causes that error message

to be printed.

Non-fatal error messages are lines of the form

*** ERROR: message *** which are followed by the source line

containing the error.  These messages indicate errors in the

code, but the assembly continues to subsequent lines.  For a

few error messages, the message follows the line containing

the error.  Those error messages are noted in the list.

Fatal error messages are lines of the form SASM.EXE: message

(details).  These messages indicate errors encountered

during the assembly process which are not code-related and

which terminate the assembly.  The list and object files for

the assembly, if any, may not be complete.

Command line messages are messages related to options

specified on the command line.  The messages indicate

problems with options.  If any command line messages have

been printed when all options have been processed, the

assembly stops.  No files have been altered (except the d

option, which takes effect immediately).

Some explanations include suggestions to help track down the

cause of the error.

10.1  Non-Fatal Error Messages

ERROR: ABS/REL must be at beginning

The ABS and REL statements, if present, must precede all

code-generating statements.

ERROR: Branch out of range

The distance to the target label is too far for the size of

this field.

ERROR: Can't add two external expressions

Both sides of + are external expressions.  At most one

external reference is allowed with the + operator.

Page 137

ERROR: Can't define a new macro within a macro

Executed a MACRO statement while expanding a macro.  A new

macro cannot be defined while expanding a macro.

Suggestion: A missing ENDM at the end of a macro definition

might cause this if another macro follows the one currently

being expanded.

ERROR: Can't have external reference on left side of

The left side of   is an external reference.  An external

reference is allowed only on the right side of the

operator.

ERROR: Can't have external reference on right side of -

The right side of - is an external reference.  An external

reference is allowed only on the left side of the -

operator.

ERROR: Can't redefine an existing macro

A MACRO statement was executed to define a macro which has

already been defined.

ERROR: Can't redefine an existing opcode

A MACRO statement was executed to define a macro, but the

macro name is the same as an opcode.

ERROR: Can't use !,&,*,/,%, or ^ with external reference

At least one of the operands is an external expression.  An

external reference is not allowed with these operators.

ERROR: Conditional assembly stack overflow

IF statements are nested more than 20 levels.  Simplify the

logic of the IF tests, or use the SETFLAG and CLRFLAG

statements to indicate combinations of IF tests.

ERROR: Divide by zero (result set to 0)

The expression on the right side of the / operator is zero.

ERROR: Duplicate ELSE statement

More than one ELSE statement was executed for a given test.

Page 138

ERROR: Duplicate symbol

The symbol being defined on this line has already been

defined in this file.

Suggestion: Check the cross reference to find the previous

definition; line 0 means the symbol was defined on the

command line with -D.

ERROR: ELSE without matching IF

An ELSE statement was executed without a corresponding IF

statement.

Suggestion: Check for a typing error in the label field of

the IF and ELSE statements.

ERROR: ENDIF without matching IF

An ENDIF statement was executed without a corresponding IF

statement.

Suggestion: Check for a typing error in the label field of

the IF and ENDIF statements.

ERROR: ENDM and EXITM not permitted outside of a macro

Executed an ENDM or EXITM while not in a macro.

Suggestion: Check for an error in the preceding MACRO

statement, if present.

ERROR: End of file while defining macro

The end of a file containing a macro definition was reached

while defining a macro.

Suggestion: Check for an error in an ENDM statement or a

missing ENDM statement.

ERROR: Error reading CHARMAP file (filespec)

A file system error occurred while reading the CHARMAP file.

Page 139

ERROR: Error reading RDSYMB file (filespec)

The RDSYMB file is not a Saturn object file, or a file

system error occurred while reading the header of the RDSYMB

file.

Three things must be true for the file to be considered a

valid Saturn object file:

1.  There is a 256-byte header in the file.

2.  The first six bytes of the header are ``Saturn''.

3.  The offset to the start of the symbol records

indicated in the header is contained in this file.

ERROR: Error reading from RDSYMB file (filespec)

The RDSYMB file is not a Saturn object file, or a file

system error occurred while reading the RDSYMB file.

ERROR: Expanded macro line too long (truncated)

The line resulting from macro expansion is more than 255

characters long.  Only the first 255 characters are used.

ERROR: Exponent less than zero (result set to 0)

The expression on the right side of the ^ operator is less

than zero.

ERROR: Expression can not be external

The expression for the EQU or = statement contains an

external reference.  Only absolute or relocatable symbols

are allowed for these statements.

ERROR: Expression must be absolute

The expression in the ABS, REL, BSS, or LISTALL statement

contains a relocatable or external reference.  Only absolute

symbols are allowed for these statements.

ERROR: Expression not allowed as target of branch

The modifier is not a label reference, and the -x option was

not specified on the command line.

The label is not a valid label because it contains a right

parenthesis ')'

Suggestion: If this is a REL(n) statement, an expression can

be used without specifying the -x option by subtracting (*)

from the expression and using a CON(n) statement.

Page 140

ERROR: Expression not allowed for INC(x), LINK, and SLINK

The modifier is not a label reference.  INC(n), LINK, and

SLINK require a label reference.

The label is not a valid label because it contains a right

parenthesis ')'

ERROR: Expression out of range

A digit expression is less than zero or greater than 15

after applying the statement's adjustment factor (usually 0

or -1).

ERROR: Expression stack overflow

The expression is too complicated.  There can be no more

than 20 pending data items.

Suggestion: Simplify the expression by splitting it into

several smaller expressions.  Assign each of the smaller

expressions to a symbol and combine them for the final

expression.

ERROR: Expression stack underflow

An expression was not present when expected.

Suggestion: This usually indicates a missing data item

following an operator.  This can be caused by a blank or tab

within the expression, as they indicate the end of the

field.

ERROR: Field too long

A field is longer than the maximum length allowed.  There

should be another message following this one which gives

more detailed information about the error.

ERROR: Flag value out of range

The flag expression in the IF, CLRFLAG, or SETFLAG statement

is less than zero or greater than 99.

ERROR: GOYES/RTNYES without test instruction

A GOYES or RTNYES statement was executed, but the preceding

instruction was not a test instruction.

ERROR: IF expression must be absolute

The flag expression in the IF, CLRFLAG, or SETFLAG statement

contains a relocatable or external reference.  Only absolute

symbols are allowed for these statements.

Page 141

ERROR: INCLUDE or MACRO nested too deeply

The number of nested INCLUDE and MACRO expansions is greater

than 20.

Suggestion: Check for a macro calling itself, or an include

file including itself.

ERROR: Illegal field select

The field select is not a valid field select indicator (P,

WP, XS, X, S, M, B, W, or A).

ERROR: Illegal mnemonic

The instruction was not found in the opcode table, or a

macro was used before it was defined.

Suggestion: Check that the processor command-line option (-

P) is correct, and that the label, if any, starts in either

the first or second column.

ERROR: Input line too long (extra characters ignored)

An input line is more than 255 characters long.  Only the

first 255 characters are used.

ERROR: Instruction not allowed with this CPU

A statement was executed which is not permitted by the

current value of the -P command-line option.  This message

indicates a probable assembler defect, and should be

reported as such.

ERROR: Invalid ASC constant

An ASCII constant in an expression is improperly formed.

The terminating character must match the beginning character

(\, ', or ").

This message also indicates an ASCII constant containing

more than 40 characters.

ERROR: Invalid ASC constant (too large)

More than 8 characters were specified for LAASC or LCASC, or

more than 4 characters were specified in an ASCII constant

in an expression.

ERROR: Invalid HEX constant

A character which is not a hexadecimal digit was specified

for LAHEX, LCHEX, D0=HEX, D1=HEX, or NIBHEX, or than 40

hexadecimal digits were specified for NIBHEX.

Page 142

ERROR: Invalid HEX constant (not 2, 4, or 5 digits)

The hexadecimal constant for D0=HEX and D1=HEX must consist

of two, four, or five digits.

ERROR: Invalid HEX constant (not HEX digit)

A character which is not a hexadecimal digit follows a # in

an expression.

ERROR: Invalid HEX constant (too large)

More than 16 hexadecimal digits were specified for LAHEX or

LCHEX.

ERROR: Invalid HEX constant (too many digits)

More than eight hexadecimal digits follow a # in an

expression.

ERROR: Invalid expression

The expression is not a valid expression or an expression is

not present.  There may be another message following this

one which gives more detailed information about the error.

Suggestion: Any field which starts after column 30 is

considered to be a comment unless overridden with the -c

option.

ERROR: Invalid field select/digit

The modifier is neither an expression nor a valid field

select indicator (P, WP, XS, X, S, M, B, W, or A).

Suggestion: This message occurs whenever the expression is

not valid.  There may be another message printed which gives

more detailed information about the error.

ERROR: Invalid file specifier

The file specifier for RDSYMB, CHARMAP, or INCLUDE does not

include a terminator character, or the file specifier is not

present.

Suggestion: Any field which starts after column 30 is

considered to be a comment unless overridden with the -c

option.

ERROR: Invalid flag expression

The flag expression is not a valid expression or it is not

present.  There may be another message following this one

which gives more detailed information about the error.

Suggestion: Any field which starts after column 30 is

considered to be a comment unless overridden with the -c

option.

Page 143

ERROR: Invalid format character

The indirect macro parameter reference ``$(...)'' contains

an invalid character.  The characters within the parentheses

must be a digit, colon, or one of the characters

``HhXxDdOoUu''.

ERROR: Invalid parameter indirection

The indirect macro parameter reference ``$('' was found, but

the closing right parenthesis is missing.

ERROR: Invalid symbol

No symbol name was specified with the = or EQU statement, or

an equals sign (=) in columns one or two is followed by a

blank or tab.

ERROR: Invalid symbol name

No symbol was specified for IFDEF, IFNDEF, IFOPC, or IFNOPC.

Suggestion: Any field which starts after column 30 is

considered to be a comment unless overridden with the -c

option.

ERROR: Label not allowed on GOYES/RTNYES statement

A line containing a GOYES or RTNYES statement has a label.

This is not allowed because GOYES and RTNYES are the second

part of a test operation.

ERROR: List type not CODE,MACRO,INCLUDE,PSEUDO,ALL, or

NOLIST

The modifier(s) for CLRLIST and SETLIST must start with

letters C, M, I, P, or A for Code, Macro, Include, Pseudo,

or All, respectively.

ERROR: Missing expression

An expression is required, but no expression was found.

Suggestion: Check that the expression starts before the

comment column, which defaults to 30 (use the c option to

change the column).

ERROR: Missing separator character

The string separator character for an IFSTR?? test was not

found.

Suggestion: The first non-blank character after the IFSTR??

statement is used as the separator character to delineate

the two strings.  The character must appear a total of three

times (start of first string, separator between strings, end

of second string).

Page 144

ERROR: Modulo zero (result set to 0)

The expression on the right side of the % operator is equal

to zero.

ERROR: More data than operators in expression

The expression is incomplete.  This message can occur as the

result of an incorrect operator (another error message

should be printed).

Suggestion: Check for blanks within the expression, or a

symbol which is not contained within parentheses.

ERROR: Name on ENDM doesn't match MACRO name

The label on the ENDM statement is not the same as the label

on the most recent MACRO statement.

Suggestion: A missing ENDM statement can cause this message

if there is another macro defined after the missing ENDM.

ERROR: Number of nibbles must be non-negative

The number of nibbles expression in BSS is less than zero.

ERROR: Number of nibbles too big for file

The number of nibbles requested by BSS would overflow the

address space of the Saturn CPU.

ERROR: Only one TITLE statement allowed per file

A TITLE statement was executed, but the title given does not

match the current title.

Suggestion: Check included files for TITLE statements.

ERROR: Operator left on stack

There is not enough data for the number of operators given.

This message indicates a probable assembler defect, and

should be reported as such.

ERROR: Operator stack overflow

The expression is too complicated.  There can be no more

than 20 pending operators, including right parentheses.

Suggestion: Simplify the expression by splitting it into

several smaller expressions.  Assign each of the smaller

expressions to a symbol and combine them for the final

expression.

ERROR: Operator stack underflow

This message indicates a probable assembler defect, and

should be reported as such.

Page 145

ERROR: PC changed (use old value)

The value of the label on the ABS or REL statement changed.

Suggestion: Look in the cross reference for another

reference to the label which changes its value.

ERROR: PC wrapped around to 00000

The number of nibbles in the file exceeds the address space

of the Saturn CPU.  The current PC is decremented by

#100000.

ERROR: Parameter too big (truncated)

The size of an indirect macro parameter reference field

would cause the line resulting from macro expansion to be

more than 255 characters long.  The reference size is

reduced so it fits.

ERROR: Relocatable offset not allowed here

A digit expression is relocatable.  The digit expression

must either be absolute or external.

ERROR: Single nibble field not allowed here

A field select which could evaluate to a single-nibble field

(P, XS, S, or WP) was requested for r=r+CON or r=r-CON.

Suggestion: This restriction is due to a limitation in the

1LR2 processor which causes these instructions to fail for

single-nibble fields.

ERROR: Symbol changed (use old value)

The current value of a symbol is different than the value in

pass 2.

Suggestion: Check for conditional assembly which uses the

IFPASS1 or IFPASS2 statements incorrectly.  This can also be

caused by an error in code generated before this line.

ERROR: Symbol was not defined in pass 1

A symbol which is defined in pass 2 was not defined in pass

1.

Suggestion: Check for conditional assembly which uses the

IFPASS1 or IFPASS2 statements incorrectly.

ERROR: Test instruction without GOYES/RTNYES

The previous line was a test instruction, but this line is

neither GOYES or RTNYES.

Page 146

ERROR: Too many relocatable/external references

More than one external or relocatable reference is present.

At most one external or net relocatable reference is

allowed.

ERROR: Unable to open CHARMAP file (filespec)

An error occurred opening the file indicated by the file

specifier provided.

Suggestion: Check that the file specifier is correct and the

file is readable.  Also check the value of the SASM_CHARMAP

environment variable.

ERROR: Unable to open INCLUDE file (filespec)

An error occurred opening the file indicated by the file

specifier provided.  This message follows the line

containing the error.

Suggestion: Check that the file specifier is correct and the

file is readable.  Also check the value of the SASM_INCLUDE

environment variable.

ERROR: Unable to open RDSYMB file (filespec)

An error occurred opening the file indicated by the file

specifier provided.

Suggestion: Check that the file specifier is correct and the

file is readable.  Also check the value of the SASM_RDSYMB

environment variable.

ERROR: Undefined symbol

The symbol used is not defined within this file and the

symbol is not an external symbol reference.

ERROR: Unknown PSEUDO-OP

The internal code for this instruction is not a code which

is known to the assembler.

Suggestion: If the opcode file (sasm.opc) has been modified

more recently than the assembler, it may contain new codes

which are not recognized in the older assembler.

ERROR: Unlabeled MACRO statement (*MACRO used)

The MACRO statement does not have a label.  The default name

*MACRO is used to avoid attempting to assemble the macro

instructions without macro expansion.

ERROR: Unmatched '(' in expression

More right parentheses (() than left parentheses ())

occurred in an expression.

Page 147

ERROR: Unmatched ')' in expression

More left parentheses ()) than right parentheses (()

occurred in an expression.

ERROR: Unrecognized operator

A character which is not a valid operator character was

found when an operator character was expected.

Suggestion: This error message may be caused by other errors

in the expression.  The character is ignored as if it wasn't

there, which may cause other error messages.

ERROR: Value too big

The value of the expression for ABS or REL is larger than

the maximum address for the Saturn CPU.

10.2  Fatal Error Messages

The name preceding the colon in these messages is the

current assembler name.

SASM.EXE: filename is not an opcode table file

The specified filename does not have the correct format for

an opcode table file.

SASM.EXE: can't back up objfile

The assembler attempted to write a nibble at a location in

the file which has already been used.  This message

indicates a probable assembler defect, and should be

reported as such.

SASM.EXE: corrupt opcode table entry (mnemonic not in table)

The opcode table file contains an indirect reference to

mnemonic, but mnemonic is not in the opcode table.  This

message indicates a defect in the opcode table.

Suggestion: If mnemonic is not a level 0 instruction, the

mnemonic which makes the indirect reference should have the

same level as mnemonic.

SASM.EXE: error creating a temporary file

Creation of a temporary file failed.  Temporary files are

used for input from standard input and for macros.

Page 148

SASM.EXE: error creating file description for writing

Creation of the specified file failed.

Suggestion: Verify that the directory for the file is

writable, and if the file already exists, check that it is

writable.

SASM.EXE: error getting memory for reason

A request for memory was rejected by the operating system.

Suggestion: Remove RAM-resident utilities to free up memory.

SASM.EXE: error opening filename for access

The specified filename could not be opened for access.

Suggestion: For writing, if the file already exists, check

that it is writable.

SASM.EXE: error reading from file description

A read from the file failed.  The file may be corrupt, or a

system error may have occurred.

SASM.EXE: error reading location from macro file

An attempt to read the current location failed.  The file

may be corrupt, or a system error may have occurred.

SASM.EXE: error setting location in file description

An attempt to set the current location failed.  The file may

be corrupt, or a system error may have occurred.

SASM.EXE: error writing to file description

An attempt to write to the specified file failed.

Suggestion: Check for a full disc, a write-protected file,

or other operating system limitations.

SASM.EXE: macro macro_name was not defined in pass 1

A macro definition was encountered in pass 2 which was not

found in pass 1.

Suggestion: Check for incorrect usage of the IFPASS1 or

IFPASS2 statements.

SASM.EXE: object file cannot be [stdout]

The object file name is ``-'', but neither the h nor the H

option was specified on the command line.

Page 149

SASM.EXE: operand type (value) is not valid

An invalid operand type was encountered.

Suggestion: Check that the opcode table file is not

corrupted, and that the version of the assembler matches

that of the opcode table file.

SASM.EXE: too many nibbles in output file filename

The output file already contained 1048576 nibbles when an

attempt was made to add another nibble.

Suggestion: Check for erroneous BSS statements.

SASM.EXE: unable to read current time

A request to the operating system for the current time

failed.

10.3  Command Line Messages

Code field width must be greater than 0 (nn invalid)

The n option has a width less than or equal to zero.

Code field width must be less than nn (nn invalid)

The n option has a width greater than the maximum

allowed.  The exact value of the maximum depends on the

line width field, which defaults to four, and is set to

five with the s option.

Comment column must be non-negative (nn)

The c option has a negative value.  The value must be

greater than or equal to zero.  A value of zero means

there is no comment column.

Flag number out of range (nn)

The f option argument contains a flag number which is

not between zero and 99, inclusive.

Invalid code generation level (string)

The P option argument is not between zero and three,

inclusive.

Invalid flag digit (char)

The f option argument contains a character which is

neither a digit nor a separator character.  Valid

separator characters are ',', ' ' (blank), (;), and

(:).

Page 150

List and code files both [stdout] (list disabled)

The A option is specified with the o - option.  The A

option is ignored (with the same results as specifying

the N option).

Non-numeric code field width (string)

The n option argument is not a valid integer.

Non-numeric comment column (string)

The c option argument is not a valid integer.

Non-numeric expression for -D (string)

For the D option, the expression following the equals

sign (=) is not a valid integer.

Suggestion: If the symbol to be defined contains an

equals sign and the default value is to be used, append

another equals sign to the symbol.  This forces the

default value to be used.

Non-numeric page length (string)

The p option argument is not a valid integer.

Non-numeric page width (string)

The w option argument is not a valid integer.

Options -A, -a, and -o not allowed with multiple files

Multiple file names are specified in conjunction with

the A, a, or o options.  Multiple file names are

allowed only when the default list and object file

names are used.

Suggestion: This error can also be caused by trying to

specify options after a file name.  All options must

precede the first file name on the command line.

Page length must be at least 4 (n)

The p option length is less than four.  At least four

lines must be printed per page (three for the header,

one for program data).

Page width must be at least 44 (nn)

The w option width is less than 44.  The minimum width

is big enough to allow a symbol reference and one line

number to fit on the line.

Page 151

11.  Saturn Object File Format

A Saturn object file consists of the following three

components:

+ A 256-byte header

+ Zero or more 256-byte blocks containing code

+ Zero or more 256-byte blocks containing symbols.

All two-byte and four-byte quantities in the header and

symbol records are stored with the most significant byte of

the quantity in the first byte of the field and the least

significant byte of the quantity in the last byte of the

field.

11.1  Saturn Object Header Record

Object   Size

Name    Bytes       Description [Contents]

------------------------------------------------------------------

ID         6   File identifier [``Saturn'']

Filesize   2   Number of 256-byte blocks in this file

[16-bit unsigned value]

Codesize   4   Number of nibbles of code

Symbols    2   Number of symbols [16-bit unsigned value]

Refs       2   Number of symbol references [16-bit unsigned value]

Start      4   Code start address

Absolute   1   Absolute/Relocatable [1 means ABS, 0 means REL]

Reserved   1   Reserved byte [fill to 16-bit boundary]

Date      26   Date of creation [``Day Mon DD HH:MM:SS YYYY  '']

Title     40   Title, if any; default = blanks

Reserved  20   Reserved bytes [Softkeys]

Version   26   Version of program creating file

Reserved   4   Reserved bytes [ROM ID]

Reserved 118   Reserved bytes [00]

11.2  Saturn Object Code Record

The code nibbles are stored two per byte in the code record.

The first nibble of code is in the most significant nibble

of the first code record byte, and the second nibble is in

the least significant nibble of that byte.

11.3  Saturn Object Symbol Block

Symbols are stored in alphabetical order.  The most

significant bit of the SymInfo field is the RESOLVED bit.

If the RESOLVED bit is set, the symbol is resolved, and has

value Value.  If the RESOLVED bit is not set, this is an

external symbol whose value is unknown.

Page 152

The second most significant bit of the SymInfo field is the

RELOCATABLE bit.  If the RELOCATABLE bit is set, this symbol

should be adjusted (relocated) if the starting address of

the module is changed.  If the RELOCATABLE bit is not set,

this is an absolute symbol which should not be relocated.

The remaining 14 bits of the SymInfo field indicate the

number of external references to the symbol.  Reference

records for the symbol follow the symbol record.

Object   Size

Name    Bytes       Description [Contents]

-----------------------------------------------------

SymbID     4   Symbol record identifier [``Symb'']

Record    18   Symbol record or reference record

(14 records per block)

Symbol Record

Name      12   Symbol name [blank-filled]

SymInfo    2   Symbol information and reference count

Value      4   Symbol value

Reference Record

Class      1   Fill reference class

Subclass   1   Fill reference subclass

Address    4   Fill address

Adjust     4   Adjustment to fill value

[32-bit signed value]

Fillsize   2   Size of fill reference in nibbles

[16-bit unsigned value]

Reserved   6   Reserved bytes [pad to 18 bytes]

Page 153

11.4  Fill Reference Types

Class  Subclass   Description

------------------------------------------------------

0       0       Direct reference

0       1       Direct reference **

1       0       Reference relative to start of

reference * **

1       1       Reference relative to nibble past

end of reference * **

1       2       Reference relative to start of

reference [REL(n)] *

2       0       SLINK reference

2       1       LINK reference

3       0       INC(n) reference

* Loader checks that this reference is within range

** Loader may report shortenable references for this

reference

Page 154

CONTENTS

1.  Getting Started...................................    1

2.  Saturn CPU Overview...............................    4

2.1   Registers...................................    4

2.2   Working and Scratch Registers...............    4

2.3   Field Selection.............................    5

2.4   Pointer Registers...........................    5

2.5   Input, Output, and Program Counter

Registers...................................    5

2.6   Carry, Program Status, and Hardware Status

Bits........................................    6

2.7   Arithmetic Mode.............................    7

2.8   Loading Data from Memory....................    7

2.9   Storing Data in Memory......................    8

2.10  Interrupt System............................    8

3.  Conditional Assembly..............................    9

4.  Using Macros......................................   10

4.1   Defining a Macro............................   10

4.2   Calling a Macro.............................   11

4.3   Parameter Assignment Rules..................   12

4.4   Macro Example...............................   12

5.  File Access Statements............................   13

5.1   RDSYMB Statement............................   13

5.2   INCLUDE Statement...........................   13

5.3   CHARMAP Statement...........................   13

5.3.1    Charmap File Format................   14

6.  Saturn Assembler Format and Mnemonics.............   15

6.1   Instruction Syntax..........................   15

6.1.1    Comments...........................   15

6.1.2    Symbols and Labels.................   15

6.1.3    Expressions........................   16

6.2   Explanation of Symbols......................   17

6.3   Field Select Table..........................   19

6.4   Instruction Set Overview....................   20

6.5   Jump Instructions...........................   20

6.6   Subroutine Call Instructions................   21

6.7   Subroutine Return Instructions..............   21

6.8   Test Instructions...........................   22

6.8.1    Register Tests.....................   22

6.8.2    Pointer Tests......................   22

6.8.3    Program Status Bit Tests...........   22

6.8.4    Hardware Status Bit Tests..........   23

6.8.5    Register Bit Tests.................   23

6.9   Pointer Instructions........................   23

6.10  Bit Manipulation Instructions...............   23

6.11  Status Instructions.........................   24

6.11.1   Program Status.....................   24

6.11.2   Hardware Status....................   24

6.11.3   System State Instructions..........   24

- i -

6.11.4   Keyscan Instructions...............   25

6.11.5   Scratch Register Instructions......   25

6.11.6   Data Pointer Instructions..........   25

6.11.7   Data Transfer Instructions.........   25

6.11.8   Load Constant Instructions.........   26

6.11.9   Shift Instructions.................   26

6.11.10  Arithmetic Instructions............   27

6.11.11  Logical Operation

Instructions.......................   27

6.11.12  No-Operation Instructions..........   27

6.12  Pseudo-Op Instructions......................   27

6.12.1   Data Storage Allocation............   28

6.13  Conditional Assembly........................   29

6.14  Listing Control.............................   30

6.15  Symbol Definition...........................   30

6.16  Macro Definition............................   31

6.17  Assembly Mode...............................   31

6.18  File Access.................................   31

6.19  Assembly Flag Modification..................   31

6.20  Carry State Modification....................   32

6.21  Miscellaneous...............................   32

7.  Saturn Assembly Tips..............................   33

7.1   Three Warnings..............................   33

7.1.1    Return Levels......................   33

7.1.2    Mode...............................   33

7.1.3    Remember P=0!......................   33

7.2   Code Packing Tips...........................   34

7.2.1    A-Field Operations.................   34

7.2.2    Loading Constants..................   34

7.2.3    The 3-Branches.....................   34

7.2.4    GOSUB/RTN..........................   35

7.2.5    Use Expressions....................   35

7.2.6    Count Up...........................   35

7.2.7    Before you leap....................   36

7.3   Some Common Operations......................   37

7.3.1    A nibble from here to there........   37

7.3.2    Testing a Bit......................   37

7.3.3    Saving/Testing a State.............   39

7.3.4    Memory Access......................   39

7.4   Some Other Tips.............................   40

7.4.1    Labels.............................   40

7.4.2    Status Bits........................   40

7.4.3    Entry Points.......................   40

7.4.4    Exits..............................   41

7.5   Documentation...............................   41

7.5.1    Comments on Comments...............   41

7.5.2    A Standard Assembly Language

Header.............................   43

7.5.3    Some Header Examples...............   45

8.  Mnemonic Dictionary...............................   48

9.  Alphabetic Mnemonic List..........................  126

10.  Error Messages....................................  137

10.1  Non-Fatal Error Messages....................  137

10.2  Fatal Error Messages........................  148

- ii -

10.3  Command Line Messages.......................  150

11.  Saturn Object File Format.........................  152

11.1  Saturn Object Header Record.................  152

11.2  Saturn Object Code Record...................  152

11.3  Saturn Object Symbol Block..................  152

11.4  Fill Reference Types........................  154

- iii -