(User.programs)

Item: 274 by _fbaird at hpcvbbs.UUCP

Author: [Frank Gregory Baird]

Subj: FFT plotting

Date: Fri May 03 1991

This is an FFT program taken from Oppenheim and Schafer's Digital Signal

Processing. The first argument is a complex 2^n-element vector. The

second argument is 1 for a forward FFT, and -1 for a backward (inverse)

FFT. Also included is a neat little program PLFFT which plots the

magnitudes the results of the FFT program. If the plot doesn't look

right, you may need to execute a RESET first. If this doesn't fix all the

problems, you may wish to also execute 'X' INDEP first. Alternatively you

may add this code

{ (-6.5,-3.1) (6.5,3.2) X 0 (0,0) BAR Y } 'PPAR' STO

# 83h # 40h PDIM

before

1 XCOL BARPLOT GRAPH.

Example:

Enter the 16-element vector (16x1 matrix) then a 1 for a forward FFT

[ (1,0) (1,0) (1,0) (1,0)

(0,0) (0,0) (0,0) (0,0)

(0,0) (0,0) (0,0) (0,0)

(0,0) (0,0) (0,0) (0,0) ]

1

The returned result is also a 16-element vector

[ (4,0)

(3.01366974607,-2.01366974607)

(.999999999998,-2.41421356237)

(-.248302881332,-1.24830288133)

(0,0)

(.834089318959,.165910681042)

(1,-.41421356237)

(.40054381631,-.599456183685)

(0,0)

(.40054381631,.599456183693)

(1,.41421356237)

(.834089318956,-.165910681041)

(0,0)

(-.248302881331,1.24830288133)

(.999999999999,2.41421356237)

(3.01366974607,2.01366974606) ]

Author:

Frank G. Baird

HP BBS: fbaird

Internet:UG0342@applelink.apple.com.