Important note about System Flag -34

by Tony Duell

Flag -34 is mis-documented in the manual. Here is the correct definition:

clear : send printer data to the I/R port in 12-bit HP printer format

set   : send printer data to the device specified by flag -33, in

8-bit (standard serial) format.

Thus, if both flags -33 and -34 are set, the printer data is sent to the

I/R port in a format similar to that used for the kermit transmissions

(My I/R decoder can thus send that data to a PC).

--

Tony Duell, Particle Physics, H.H.Wills Physics Labs, Bristol University,

Tyndall Avenue, Bristol, England