(Comp.sys.handhelds)

Item: 3382 by anderson at truebalt.caltech.edu

Author: [Eric R. Anderson]

Subj: HP48: Aviation/Flight Planning programs

Date: Thu Jun 06 1991

[Note: Eric describes how to "unpack" the program.  He is referring to the

source code listing, called FLY.SRC on this disk.  But you don't need to

bother "unpacking" it, because I already did.  The binary file, called

FLY on this disk, is fully "unpacked" and ready to download and run.  -jkh-]

What follows is a group of programs making up an electronic flight

computer of sorts.  The flight computer is made up of 6 functions:

Weight and Balance with 9 plane memory and CG graphing

Flight plan of up to eight legs

Winds aloft calculation

Density altitude

Distance and heading between two points

Crosswind components

Each will be described more fully later.  These functions can be accessed

either alone, or by selecting from a menu presented by running START.

There is a minimal amount of error checking in the programs, but if run

from the START menu, the stack will be cleaned up if an error does occur.

If flag 10 is set, the output of some of the program is changed to metric.

Units are allowed (even encouraged) unless noted below, and appropriate

conversions made.  Extensive use is made of PGROB and UPGROB from Erik Bryntse,

and they are included in the directory.  The last four functions could be done

much more compactly with the equation solver, but the programs provide a front

end for things.  The CST menu is also defined for conversions between fuel

volume and weight (at 6lbs/gal).

The whole thing is rather large, and to download on my 30K calculator

requires removing everthing but ASC-> and ->ASC.  ASC-> is required to unpack

the directory.  When I download it, everything goes fine until the last packet

or two, when the calculator quits responding.  Both Kermits eventually time out,

and the directory is present and whole on the calculator.  I'm not sure if this

is just my calculator, or what.  If it happens, just be patient.  Once

downloaded, press AERO to get into the directory, then SETUP to unpack it.

Press START to begin.

Notes:

FPLAN, WIND, and CWIND use vector arithmetic, so beware if you have an HP 48

with the vector bug.  The checksums are:

Before unpacking: #11040d

23664.5

After unpacking:  # 5241d

17881.5

Disclaimer:  Use at your own risk.  Feel free to redistribute in whole

or in part if it strikes your fancy.  If you want to make improvements, go

ahead - I'm graduating in a week and a half, and losing net access, so I can't

post any I make.

Program Descriptions:

The primary programs are described in the order they appear in the VAR menu.

Since the directory is rather large, I have included a list of programs

that call each program, so if you decide you don't want one of the functions,

you can delete all the programs that only it calls.  To save memory, I run

without START and HELP, which saves about 5K.  I apologise if the descriptions

are a little cryptic.  The program follows the descriptions.

START:  Called by: None

Use the arrow keys to move around, and ENTER to select choice.

Press any key to return to menu after program execution.

HELP:   Called by: START, self executable

Presents three short help screens.

FPLAN:  Called by: START, self executable

Computes magnetic heading, groundspeed, time en route, and fuel

burn for up to eight legs of a flight.  If units are entered for

distance or fuel consumption rate on the first leg, units must be

entered for each remaining leg.  Time is shown in Hours:minutes

format.

W&B:    Called by: START, self executable

Compute weight and balance for selected aircraft, and plots weight

vs moment arm if graph is defined for aircraft.  Output is metric

if flag 10 is set.  If a graph has been defined for the plane chosen,

flag 10 must be set so the output matches the units used in defining

the graph for the graph to be valid.  Choose ADD/DELETE to add a new

plane or delete an old one.  When adding a plane, select 0 for the

number of points in the graph to save memory.  If you want to define

a graph, select a number of points equal to the number of line

segments in the graph + 1.  A line will be drawn from each point to the

next point entered.  Points are entered in the form (without units)

{ moment/1000 weight }.  When asked for a station name, keep in

mind that W&B will append "weight and moment arm" after the name

you supply.

WIND:   Called by: START, self executable

Calculates winds aloft from true course, ground speed, true heading

and true airspeed.

DALT:   Called by: START, self executable

Calculates density altitude from pressure altitude and temperature.

Valid only for altitudes less than 36000 ft.

DISTANCE: Called by: START, self executable

Calculates distance and true heading between two points of

latitude and longitude entered in hhh.mmss form.  Points in

the southern or eastern hemisphere may need to be entered as

negative.  The program has only been tested for points N and W.

CWIND:  Called by: START, self executable

Given a heading and wind speed and direction, calculates headwind

and crosswind components.  Does not support units - units of

components are same as of wind speed.

PAUSE:  Called by: START

TRAP:   Called by: START

MAKEMENU: Called by: none

Utility to make a menu for use with CHOOSE.

ADDPLANE: Called by: W&B

STRIP:  Called by: START, FPLAN

PAD:    Called by: START

CHOOSE: Called by: START, W&B, ADDPLANE

Call with initial cursor position in stack level 2, and the menu grob

in level 1.  Returns cursor postion when ENTER is pressed.

ADDGRAPH: Called by: ADDPLANE

SCL:    Called by: ADDGRAPH, CGGRAPH

WBIN:   Called by: W&B

CGGRAPH: Called by: W&B

PGROB, UPGROB: Called by: HELP,CHOOSE, ADDPLANE, CGGRAPH, ADDGRAPH, FPLAN

PLANES: Called by W&B, ADDPLANE

Stores the list of planes for use by W&B. Each plane list has the

format

{ { CGgraph maxmoment minmoment maxweight minweight }

{ { station1name station1weight station1arm }

{ station2name station2weight station2arm }

.

.

.

{ stationNname stationNweight stationNarm } }

"Plane name"

}

FPLAN.GROB: Called by: FPLAN

HELP.{123}: Called by: HELP

STMENU: Called by: START

WBMENU: Called by: W&B, ADDPLANE

CURSOR: Called by: CHOOSE

UNITMENU: Called by: W&B, ADDPLANE

PLMENU: Called by: ADDPLANE

DESCMENU: Called by: ADDPLANE

Gal: Called by: W&B, FPLAN, ADDPLANE, CST    (Defines 1 Gallon = 6 lbs)

L: Called by: W&B, FPLAN, ADDPLANE, CST

CST: Called by: None