FAST - How to speed up your hp48, V1.1

From: detlef@mwhh.hanse.de (Detlef Mueller)

Newsgroups: comp.sources.hp48

Date: 7 Oct 91 02:35:18 GMT

Hi ...

Anybody left who says `you can't speed up the HP48` ??? Yes ... ? Ok,

here is how to do it, version 1.1 .

Displaying the display :-] is time consuming. The display refresh logic

steal about 11% of the CPU time from the Saturn, so switch off the LCD

and all is fine.

The following program turns the display off (that's the disadvantage of

this program, you can't see what's going on), executes the object in the

first stack level (sorry, no tagged object), then turns the display on

again.

The entry to turn the display off is not supported in ENTRIES.A (there

is a entry named 'DispOff', it switches off the display 'dirty'. I used

this entry in version 1.0 of FAST). 'CleanDispOff' is used by the OFF

routine (hard to find :-).

If your program terminates and the display is still off, switch the HP48

off (blind) then on again. This should reenable the display.

IMO you can further speed up your programs, if you activate the commented

lines in FAST.S because timer interrupts are time consuming, too.

I've not tested this!  Do it at your own risk!

To generate FAST, put the listings of FAST.S and FAST.M in the appropriate

files, then invoke the following commandlines at the MesS-DOS prompt:

rplcomp fast.s fast.a

sasm -e -N fast.a

sload -H fast.m

Bye,

8-), Detlef

+-----------------------------------+---------------------------------------+

|       `You mean this isn't        |             Detlef Mueller            |

|          the afterlife ?`         |          detlef@mwhh.hanse.de         |

| Arthur in THGTTG, part II, p. 78  |...!uunet!mcsun!unido!mcshh!mwhh!detlef|

+-----------------------------------+---------------------------------------+

[Note: Not only do programs run faster in FAST mode, but they actually run

using less power per second (since driving the display takes juice).  Using

FAST will extend the life of your batteries.  See DRAIN2.DOC in the POSTINGS

directory on this disk for further battery drain information.  -jkh-]