HP48SX

FLIGHT PLANNING/TRACKING LIBRARY

{ 1786 FLIGHT LIB }

and

TIMER LIBRARY

{ 1785 TIMER LIB }

by Steve Mauser

Copyright, 1991

Victoria Designs

377 Nature Trail Lane

Murrysville, Pa 15668

TABLE OF CONTENTS

I.  INTRODUCTION

II.  DESCRIPTION

1.  Flight Activities

a.  Flight Status

b.  Start/Ckpt Xing/Close

c.  Enroute Calculations

i.  TIMER

ii.  TAS/IAS/Alts

iii.  Timed Groundspeed

iv.  Calc. Winds

v.  External Routines

d.  Reset Last CKPT Xing

e.  CLOSE PLAN

2.  Calculate Flight Plan

3.  Enter/Edit Route Data

a.  Load Existing Route

b.  Store Current Route

c.  Extend/Add CKPTS

d.  Edit CKPT(s) Data

e.  Erase Current Data

4.  Enter/Edit Winds

a.  Add Data

b.  Edit Data

c.  Erase Data

5.  Select Plane

III.  PROCEDURES

APPENDIX A

Downloading and Linking the Library

APPENDIX B

Programming Airplane Characteristics

APPENDIX C

Warranty

APPENDIX D

Registration

HP48SX

FLIGHT PLANNING/TRACKING LIBRARY

{ 1786 FLIGHT LIB }

and

TIMER LIBRARY

{ 1785 TIMER LIB }

I.  INTRODUCTION

The Flight Planning/Tracking Library provides a menu-driven library

which facilitates flight planning calculations -

- weight/balance

- winds aloft

- route planning

- flight plan times and headings

- fuel requirements

and flight tracking and in-flight calculations -

- audible alarms for expected checkpoint crossings

- TAS/IAS conversions

- pressure and density altitudes

- groundspeed determination

- winds aloft

as well as providing an interconnection to the TIMER LIB, a large-

digit timer function for holds and approaches.

The library takes over the HP48SX while executing the various

functions, but permits normal operation of the HP48SX upon exiting

the library menu system while remembering current data upon re-

entry to the library.  Alarms will break-in on operations outside

of the library functions, but will not break in while in the flight

library execution mode.

The flight library requires 29+ kb of memory and requires that data

for the current trip be stored in user memory.   The timer library

requires 5 kb of memory.  These sizes dictate that the flight

library be stored in an external memory card, while the data for

the flight library and the TIMER LIB can be stored in the

calculator memory.

The flight library menu contains only one executable - the

PLANr.dvr function.  The timer library menu provides only the TIMER

executable.  Short programs in the HOME directory can be written to

permit rapid access to the planner/tracker and the time.  However,

the timer can also be accessed through the flight library's menu

system.

The remainder of this writeup provides a description of the

operation for the flight planning/tracking library, menu by menu,

instructions for downloading and storing the library, and a

registration form for receiving future versions of the library.

II.  DESCRIPTION

This following is a description of the use of the flight library

only.  A description of the operation of the timer library is

included under the "Enroute Activities" menu of the flight library.

The flight library provides numerous planning, tracking, and in-

flight computations for basic small airplane flying.  While most

calculations are as precise as the data provided, the library only

provides accurate flying information for altitudes up to 12,000 ft.

MSL.  For instance, in the IAS/TAS and pressure and density

altitude calculations, conversion approximations are only valid to

this 12,000 ft. MSL level.

While the flight library is stored in ROM memory, it does require

the use of user RAM for data storage and manipulation.

Specifically, the library REQUIRES that a directory named AIR be

created, and that this directory contain two subdirectories, named

RTE.FILES and PLANES.  Permanent storage of flight plans and

airplane data are located in these two subdirectories, while

current flight plan data are stored in the AIR directory.

After loading the library (see Appendix A), the library can be

executed from any directory/subdirectory, will move to and execute

in the AIR directory, and should then return to the

directory/subdirectory from which it was called.

Except for the airplane-specific data files stored in the PLANES

subdirectory of AIR, all data files are generated and stored from

within the library, so little original programing experience is

necessary to use the library.  Sample copies of one airplane's data

files which would be stored in the AIR/PLANES subdirectory is

included with the library files, and are listed in Appendix B.

The following description of the library functions follows the

menus which are the cruising elements in the library.  To move

around the menus, the arrow keys are used to move the cursor to the

selected activity, and the ENTER key is used to select the

activity.  The EXIT key in the menu is the only way to back up into

the previous menu or exit the program.

The discussions in each of the following sections correspond to

menu items and are discussed in the order in which they are

presented in the menu screens.

The SELECT ACTIVITY menu is the introductory screen and provides

entry into all the other functions provided by the library.  These

other functions are as follows.

1.  Flight Activities

The flight activities item will provide access to

functions normally associated with in-flight procedures.

These include the current flight status, confirmation of

checkpoint crossing, flight calculations, an Oops

function to reset the last checkpoint crossed, and a

CLOSE PLAN function which discontinues the flight plan

tracking function.

a.  Flight Status

The flight status selection presents a screen

which gives the current status of the flight.

This includes the previous checkpoint name and

crossing time, the current checkpoint name,

expected minutes to crossing, the estimated

crossing time (in ZULU), the estimated

nautical miles to crossing, the magnetic

course to the checkpoint, and the estimated

heading to fly to the checkpoint.  This

information is also provided for the

checkpoint immediately beyond the upcoming

checkpoint.

On pressing the ENTER key, the flight status

screen will display the total time in flight

(in hours), the estimated time remaining (in

hours), and the estimated fuel remaining in

the tanks (in hours).

Pressing ENTER again, will return to the

FLIGHT ACTIVITIES menu.

b.  Start/Ckpt Xing/Close

Selecting this menu item will start the

tracking function, indicate that the upcoming

checkpoint has been crossed, or will indicate

the crossing of the final checkpoint

(destination) and close the tracking function.

The user is asked to confirm checkpoint

crossing, and then the groundspeed along the

direct checkpoint course is calculated.

Following this, if the actual heading flown is

entered at the request, the actual winds

encountered will be calculated and summarized.

[NOTE:  The user may call up this function and

delay pressing the ENTER key until actual

checkpoint crossing is made.  The time of

crossing is taken as the instant that the

ENTER key is pressed, with only a "Y" (default

value) on the entry line.]

This routine returns to the Flight Activities

menu.  To check that the function performed

properly, select the Flight Status function

from the same menu, which should show that the

checkpoints have been incremented.

c.  Enroute Calculations

Selecting this item will call another screen

from which to select calculation functions

which are generally used during flight.  The

selections are as follows.

i.  TIMER

The timer function is a countdown

timer which, when finished counting

down, will revert to counting up

until it is stopped.  Any time for

countdown can be entered, up to 9

minutes and 59 seconds.

If the timer has been used at any

time previously, the timer will

start with the last countdown time.

The timer is started by pressing the

GO menu key, and is stopped by

pressing the STOP menu key.

If the timer is stopped (either when

it is started initially, or after a

countdown sequence is complete) and

the STOP key is pressed, the timer

will ask for the initial countdown

time to be entered.  The new time

can be entered at this point, or, if

ENTER is pressed with "0" for the

input time, the system will reset to

the Enroute Calcs menu, exiting the

timer function.  The last countdown

time is saved.

ii.  TAS/IAS/Alts

Selecting this menu item will result

in a request for the current

indicated altitude and the altimeter

setting.  Next, a request is made

for one airspeed value - either

theIndicated Air Speed or the True

Air Speed - and for the Outside Air

Temperature (degrees C).  From this

information, a screen is provided

which summarizes the estimated

altitudes (indicated, pressure, and

density), and the indicated airspeed

and true airspeed.

(These estimated values are based on

a three-point exponential fit of the

standard curves of temperature,

pressure, and density, and a 2% per

1,000 ft. increase in TAS over IAS,

corrected for density altitude)

iii.  Timed Groundspeed

This selection enables timer

functions which are used to compute

the actual groundspeed, and, if the

actual heading flown is given, will

calculate the actual winds aloft

being experienced.

The first time this option is

selected, a request for the distance

to be timed is made.  Upon entering

the distance, AND upon pressing

ENTER, the timer will commence.

Upon entering this menu option the

second time, pressing ENTER will

stop the timing run, and will result

in the presentation of the

calculated groundspeed, and then a

request for course (default is the

course for the leg being flown from

the flight plan) and heading flown.

Providing this latter data will

result in a calculation of the winds

aloft experienced.

iv.  Calc. Winds

This selection provides a summary of

the LAST actual winds aloft

calculation.  Wind aloft

calculations can be those performed

at the last checkpoint crossing, or

can be a result of requesting the

Timed Groundspeed option, also

provided in this menu.

The summary screen in this selection

gives the data for the leg which was

timed, the calculated groundspeed

and the heading flown (if entered)

for this leg, the calculated winds

experienced, and the winds which

were forcast and interpolated for

this leg and altitude.

v.  External Routines

The library permits the user to

execute additional programs which

are written and stored in the { HOME

AIR } directory.  When the Enroute

Calculations option is invoked, the

library will form a temporary menu

of all the program variables in the

{ HOME AIR } directory.  To execute

a selected program, simply press the

appropriate menu key.  After

execution of the external program,

the execution will return to the

flight library.

d.  Reset Last CKPT Xing

In the excitement of flight, the checkpoint

crossing for the next checkpoint may be

entered before the actual crossing time.  To

reset the last checkpoint which is now

incorrectly noted as being crossed so it again

becomes the next checkpoint to be crossed,

select this option.  No screen will be

provided, but the result of this operation can

be displayed by requesting the Flight Status

option in this same menu.

e.  CLOSE PLAN

If at any time you wish to discontinue

tracking of the flight plan, this option will

shut down the tracking function.  While

selecting the Start/Ckpt Xing/Close option in

this menu will close the plan upon its

activation over the last checkpoint

(destination), this selection will do the

same, except it will conclude

trackingregardless of the current location

within the plan.

This has also been considered to be a shortcut

"panic" button.

2.  Calculate Flight Plan

Activating this option, providing the checkpoint route

data has been input (see Enter/Edit Route Data below) and

winds aloft have been entered for each location cited in

the checkpoint data, will calculate the flight plan.

Selecting this option will step through each location,

giving the calculated groundspeeds for the winds aloft

data for 3, 6, 9, and 12,000 feet, and interpolating

inbetween (i.e. 4,500, 7,500, and 10,500 feet), and will

request the altitude to be flown TO this checkpoint.

Following the altitude request for the first checkpoint

and for the last checkpoint, the program asks for a time

adder which it will add to the calculated time for the

leg.  This tinme adder should be used to account for

climbing time, and/or vectoring time.

Upon calculating all of the data for the plan, a summary

is provided for the cummulative nautical miles to be

flown (between checkpoints), the total time expected to

be required for the flight, and the estimated amount of

fuel required for the trip.

3.  Enter/Edit Route Data

This function provides for entering, and storing for

future use, the route data for the flight.  A second menu

screen is brought up when this option is selected to

provide the various options.  Route files are stored in

the AIR directory, RTE.FILES subdirectory.  A description

of each of the options on the secondary menu is as

follows.

a.  Load Existing Route

This selection will drop into the RTE.FILES

subdirectory and prompts for the name of the

route file to load.  Press the appropriate

menu key to load the desired route filename.

b.  Store Current Route

After inputing a route from within the flight

planner, the route information can be stored

in the RTE.FILES subdirectory using

thisselection.  This selection drops into the

RTE.FILES subdirectory and prompts for a route

file name (suggest a format similar to AGC.CAK

for Allegheny Airport to Clarksburg).  Upon

pressing enter after providing a name, you

will be returned to the EDIT/ENTER Route Data

menu.

c.  Extend/Add CKPTS

This selection provides for starting a new

route file or for adding extra checkpoints to

an existing route file.  The route file must

be the current route file (i.e. a new route

being entered, or a route loaded from the Load

Existing Route feature).The data requested in

this option is the LOC identifier (three

alpha-newmerics), the winds aloft location

(three alpha-numerics), and the checkpoint

data (mag. variation, distance TO, and heading

TO).

Completing the data for one checkpoint will

generate a request for the next, until ENTER

is pressed when asked for the next checkpoint

LOC identifier.

d.  Edit CKPT(s) Data

This menu item permits modifying the route.

First, the checkpoints in the route are

listed, and the user is asked if any

checkpoints are to be deleted.  To delete the

checkpoint, simply enter the index number in

the table for the appropriate LOC to be

deleted.

Next, the user is prompted for changing the

checkpoint order.  To change the order, enter

the checkpoint index to move and then the

desired index for that checkpoint.

Finally, the user is given the option to

Review/Edit the data for each checkpoint.  In

this function, the data for each checkpoint is

shown and editing is permitted.

e.  Erase Current Data

Use this option to completely clear all of the

checkpoint data (prior to entering a new set

of route data).

4.  Enter/Edit Winds

This series of functions provides for entering the

current winds aloft data.  The secondary menu consists of

the following operations.

a.  Add Data

Use this option to enter new data or to extend

the existing winds aloft data.  Since the

locations for winds aloft are not connected to

each other, any location for winds aloft can

be entered.  However, only those locations

which match the locations specified in the

checkpoint data are actually used.

Winds aloft are entered as they would be

reported in written reports, as a four-digit

number.  The first two digits are the "from"

bearing of the winds, and the second two

digits are the speed.  The program recognizes

"99" entries for light and variable winds,

but, since we are dealing with light aircraft,

it is not programed to recognize coding for

winds in excess of 99 mph.

Winds can be entered for from 3,000 to 12,000

feet.  Entries must be made to bracket the

altitude to be flown (i.e. the altitude of

flight must be equal to one of the winds aloft

altitudes or lie between two winds aloft

altitude entries).  If the flight altitude

lies outside of the given wind altitudes, the

flight calculation mode will later use zero

winds aloft.

b.  Edit Data

This function provides for deleting an

individual location's winds aloft data, or for

editing individual locations' data.

To delete an individual location's data,

simply enter the location's index number from

the displayed list of winds aloft locations.

In editing the data, each location's data is

presented individually and the user is

permited to over write the existing data.  The

data is presented in vector notation, using a

TRUE from bearing.  Because of the vector

notation, bearings from 180 degrees to 359

degrees will be shown as a negative value.

c.  Erase Data

This selection provides for completely erasing

the current winds aloft data prior to

beginning entry of new data for the immediate

flight.

5.  Select Plane

Selecting this option drops the library into PLANES

subdirectory and permits the user to use the HP48SX menu

keys to enter the airplane tail number.  The tail number

represents a routine which then loads the weight and

balance calculation information for the airplane,

theweight and balance envelope for plotting the

calculated weight and balance within the envelope, and

the fuel burn rate for the airplane.

No provision has been made for entering the unloaded

airplane characteristics from within the library.

Details on setting up the routines for each airplane are

provided in Appendix B, as well as samples of the

routines which are provided with the library files.

III.  PROCEDURES

The procedures for using the library are generally in reverse order

from the initial menu screen.  The most important functions, those

performed during flight, are at the tops of the menus, while the

more casual planning functions are at the bottom of the menus.

This permits a "no-look" procedure for calling the library,

pressing ENTER twice, and immediately toing to the flight status

screen (probably the most-used function during flight).

To ready the system for a flight, the following order of procedures

is recommended.

1)  Enter the library

2)  Select an airplane

3)  Select the route menu and load or enter the route

(also store the route after entry, if desired)

4)  Enter the winds aloft data for the locations

referenced in the route

5)  Calculate the flight plan

6)  Check the flight status to verify that things are as

expected

7)  Set the cursor on the Start/Ckpt Xing/Close option of

the Flight Activities menu - DO NOT press ENTER

8)  Press ENTER just prior to the takeoff roll.

The tracker is now in progress and an alarm will sound

approximately two minutes prior to each checkpoint's crossing.

When the alarm sounds, either confirm crossing by pressing ENTER

with the "Y" on the entry line, or press N (STO) and press ENTER.

If you press "Y", the program will begin prompts for groundspeed

and winds aloft calculation.  If you do not press "Y", then the

program will go back to sleep for one minute and will resound the

alarm.  This procedure continues until you confirm checkpoint

crossing.

APPENDIX A

DOWNLOADING AND LINKING THE LIBRARY

APPENDIX A

After downloading the flight and timer library files from the

bulletin board, or from a file on a disc for your PC, download one

of the library files to the HP48SX.  To do this, go to the HOME

directory and set up for a BINARY file transfer (in the SETUP sub-

menu of the I/O menu).  Following the transfer, and depending on

which file has been transfered, there will be a file in the current

directory named FLYING or TIMER.

Store this file in a port.  For example, If you have one 32 kb card, the

flight library should be stored in the memory card port, and the timer

in port 0.

By pressing the FLIGHT menu key, a new menu will appear which reads

"PLANr.dvr".  The "r.dvr" does not appear in the menu, but

PLANR.DVR is the complete name of this flight planner entry point.

From this point, any of the planner/tracker functions can be

accessed.

To store the timer library in the calculator memory, repeat the

above procedure using port 0 (i.e. :0:Y - where Y is any number).

If you have a memory card which is larger than 32 kb, the timer

library can also be stored in the memory card.  To do this, follow

the same instructions as for storing the flight library, above.

Use of the library requires making a directory and two

subdirectories.  The directory must be named AIR.  To do this, in

the HOME directory (blue-shift ' ), enter the name AIR ( alpha,

alpha, AIR, ENTER), enter the memory mode (orange-shift VAR) and

select CRDIR.  Upon pressing the VAR key, you should now see the

AIR directory in the variable list.  To make the subdirectories,

press the AIR menu key, and create a directory named PLANES just as

in creating the AIR directory.  Follow this by also creating the

subdirectory RTE.FILES under the AIR directory.

To permit more rapid access to the library, a program can be placed

in the HOME directory.  The program consists of simply:

<< 1786 MENU PLANR.DVR KILL >>

[Note: The program above is on the disk as 'FLY'.  -jkh-]

The KILL is important to clear and return to the stack display.

Storing this program in the HOME directory and executing it will

immediately call up the first menu in the library.

Exiting the library will return to the directory from which the

library was initiated.

APPENDIX B

PROGRAMMING AIRPLANE CHARACTERISTICS

APPENDIX B

Entering individual airplane data requires a small amount of

programming.

For each make/model of airplane, two routines are required - a

routine which performs the calculations for the weight and balance

and a routine which plots the weight and balance envelope and plots

the actual weight and balance on this envelope.

For each individual airplane tail number, a single routine is

needed to place the default unloaded airplane characteristics into

the weight and balance calculation routines, and to call the

appropriate routines for its make and model.

These three routines are all placed in the PLANES subdirectory of

the AIR directory.  After all the airplanes are entered, use the

MEMORY (orange-shift VAR) ORDER functions to place all of tail

number routines at the front of the menu list.  This makes it more

convenient to load the routine from the library's Select Plane menu

item.

Each of these routines are discussed in the following sections.

1.  Plotting Routine

This routine requires the most programming patience.  The routine

should be named to start with the letter "P" and further named to

correspond with a type of airplane (i.e. P152, P172, PPIPARCH,

etc.) since this routine can be used for any airplane tail number

which corresponds to this make/model of airplane.

The purpose of the routine is to draw the weight/balance envelope

for the airplane.  The procedure used is to draw lines between

coordinate points using the HP48SX's graph LINE function.  All

coordinates are in a (X,Y) format, where X is usually in thousands

of in-lbs and Y is the weight in lbs.

The listing below is for the PPIPARCH routine which describes the

weight/balance envelope for the Piper Archer:

\<< ERASE {

(81.0000000004,1200)

(94.0000000004,2600)

X 0 {

(81.0000000004,1200)

"C.G.(aft.datum)"

"WT" } FUNCTION Y }

'PPAR' STO DRAX

LABEL

(81.9999999998,1200)

(92.9999999989,1200)

LINE

(92.9999999989,1200)

(92.9999999999,2550)

LINE

(92.9999999999,2550)

(88.5000000017,2550)

LINE

(88.5000000017,2550)

(81.9999999984,2050)

LINE

(81.9999999984,2050)

(81.9999999998,1200)

LINE MOMT WGHT /

1000 * .1 - WGHT 10

- R\->C MOMT WGHT /

1000 * .1 + WGHT 10

+ R\->C BOX { # 0h

# 0h } PVIEW

\>>

In the above listing, the first two coordinates in the first list

( {} ) are the lower left corner of the plot area and the upper

right corner of the screen (resulting in a screen a little larger

than the weight/balance envelope).  The next coordinate in this

same list is the location on the screen where the axes should cross

each other (usually, also the lower left-hand corner of the

screen).  The other elements in this first list provide the axes

labels and the type of function to be plotted.  No other changes

should be necessary for this list.

The next series of statements simply draw the outline of the

envelope.  This can consist of as many lines and breakpoints in the

envelope as you feel are necessary to model it.  Each line is

defined by the coordinate of the starting point (usually the end of

the previous line) and the coordinate of end point, followed by

invoking the HP48SX LINE function.

2.  Weight/Balance Calculation Routine

This routine uses the total front passengers' weight (fpwt), the

rear passengers' weight (rpwt), the first baggage compartment

weight (b1wt), the second baggage compartment weight (b2wt), and

the amount of fuel on board (gal), to calculate the total weight

(WGHT) and the total moment (MOMT) in the ramp/takeoff

configuration.  This routine applies to a make and model of

aircraft and not a specific tail number, so name it accordingly.

A listing of the routine for the Piper Archer follows.

\<< WGHT fpwt + rpwt

+ b1wt + b2wt + gal

6 * + 'WGHT' STO

MOMT fpwt 32.5 401

/ * + rpwt 38 325 /

* + gal 6 * 29 300

/ * + b1wt 27.5 199

/ * + b2wt 100 1 /

* + 'MOMT' STO {

b1wt b2wt rpwt fpwt

} PURGE

\>>

The only entries which need be changed to represent a different

airplane are the factors which multipy each element of weight to

obtain the corresponding moment for each compartment.  In the above

routine, rather than calculate and insert each multiplier (the

slope of each line for each compartment), the weight/balance curves

were consulted and an arbitrary point at the extreme end of one

compartment's line was selected.  The value of the selected moment

was entered, followed by the weight corresponding to this moment,

and the two divided to obtain the moment per pound in each

compartment.  This conversion factor is then multiplied by the

weight in that compartment (fpwt, rpwt, b1wt, b2wt - and recall the

RPN notation used in the HP48SX calculations).

3.  Individual Airplane Tail Number Driver Routine

The final routine which defines an airplane's characteristics is

particular to the airplane equipment on board, and therefore,

should be titled with the airplane tail number.

The routine first places on the stack the names of the routines

which plot the weight/balance envelope and which calculate the

weight/balance for this airplane's make and model (PPIPARCH and

PIPARCH in the examples here).  Then, a list which is later used in

an INPUT statement is generated.  The list contains default values

which can be edited during the weight/balance activity in the

Select Plane activity in the library.

Default inputs are specified for the gallons/hour burn rate, the

TAS, the base weight, and the base moment of unloaded airplane.

The following is a listing of the routine N26H for airplane tail

number N8226H.

\<< 'PPIPARCH'

'PIPARCH' {

":Gal Per Hr:10.0

:TAS:125

:Base Wt:1601

:Base Mom:140.806"

{ -1 0 } V }

\>>

APPENDIX C

Warranty

APPENDIX C

Absolutely no warranty, expressed or implied, applies to a user who

is not registered in writting.

For a user who is registered, the warranty excludes any liability

whatsoever from the use of this software.  Accurate calculations

for the flight of airplanes are solely the responsibility of the

pilot.  This software is intended to be an aid to those

calculations, but the accuracy of the calculations is not

guaranteed.  Use of this computational aid is not intended to

replace required competence and cross-checking on the part of the

pilot.

The warranty on the software provides for either correcting a

software error or providing instructions for a work-around.

Program updates or improvements are not guaranteed nor implied.

Should future updates or modifications to this software be made

available, they are not guaranteed to be available in a size which

will fit in a memory size of 32 kb or less (i.e. if you are going

to purchase a memory card for the HP48SX, a card larger than 32 kb

is recommended).

APPENDIX D

Registration Form

APPENDIX D

The registration fee for this program is $50.  Payment of this fee

entitles the registered user to communications regarding the use

and application of the program and handy tips received from other

users.  Additionally, should updates become available, the

registered user will be notified of the availability of the

updates.

Upon receipt of the registration fee, the user will be provided

with a description of the data files generated and used by the

library.  Due to the target of making this library available in a

size which would fit onto a 32 kb memory card, other optional

programs which would be handy to have, were not included.  Such as

a program to automatically generate a full route plan for a return

trip from the route plan entered to get to the original

destination.  This is a simple program which can be written by

knowing the format and definitions of the various data files used

by the library.  Also knowing the the data formats, a program can

be generated to summarize the entire trip, a facility which is not

presently available in the library.  Or, after calculating actual

winds aloft experienced during the flight, a program could be

written to update the remaining flight estimates based on the most

recently-calculated winds.

The data files used by the library have been packed and coded for

memory savings and their use, therefore, are not straight forward.

An explanation of each of the files, data constants, and other

tidbits will be sent to each registered user.

Use of the registration form below entitles the user to the use and

to remain informed on the use and improvements to the library.

Registration does not apply to anyone other than the registered

individual.

Club registrations are available, please contact the home office

for information.

USER REGISTRATION FORM FOR FLIGHT LIB { 1786 }

NAME _____________________________________________________________

ADDRESS __________________________________________________________

__________________________________________________________

CITY ________________________ STATE ______________ ZIP ___________

For making available future updates, where did you obtain this copy

of the library (BBS, copied from a friend, etc.)

__________________________________________________________________

__________________________________________________________________

For the purposes of providing you updates, which is the most

convienent for you (3.5" disc, 5.25" disc, DOS, MacIntosh, etc.)?

__________________________________________________________________

__________________________________________________________________

Is a 32 kb - size library a limiting size for you?  ___ YES ___ NO

What is the maximum memory card size which you have ?  ________ kb

COMPLETE AND MAIL THIS FORM TO:

VICTORIA DESIGNS

377 NATURE TRAIL LANE

MURRYSVILLE, PA 15668