Countdown Timers by Steve Mauser

[See FLYING.DOC for more complete documentation.  -jkh-]

This is a library of two countdown timer applications.  Both use

the graphics display to generate LARGE minute and second numerals

during the countdown.

[Note: TIMER.LIB seems to be an "older" version of TIMER2.LIB, but

I'm not sure.  If you figure out which library does what,

congratulations.  Sorry for the confusion.  -jkh-]

The first application, TIM1, is a dedicated one-minute timer for

initial holding pattern and standard-rate turn timing.  To start

this timer, press TIM1 in the library function menu, and the timer

will come up with 1:00 on the screen.  By pressing "GO" on the

menu, the timer will start and will not stop until reaching 0:00,

or until the "STOP" menu key is pressed.  Pressing the STOP key

prior to reaching zero will produce a beep, hold the display for

two seconds, and reset for a one-minute countdown.  Upon reaching

zero, the timer will continuously display zero and beep until the

STOP key is pressed.  After pressing STOP, the display will

continue timing upwards until the STOP key is pressed again.  At

this STOP, a beep is issued and the display will freeze for two

seconds prior to being reset to one minute for another count

cycle.

Exit from this timer is by pressing the "STOP" menu key while 1:00

is displayed.

The second application, TIMER, is a variable-time countdown timer

with a maximum time of 9 minutes and 59 seconds.  It comes up upon

activation and behaves just like the one-minute timer, except that

it provides for changing the starting time for countdown (such as

for final approach times, or second loops around holding

patterns).

To change the countdown time, press "STOP" while the large display

is showing. Instead of exiting, as with the 1:00-minute timer, you

will be prompted for a new time which you enter in "248 enter"

format (for 2 minutes and 48 seconds). The new time will be

displayed and count begun upon your pressing the "GO" menu key.

Upon completion of the countdown, the 0:00 display will freeze and

a continuous alarm will sound.  Pressing the STOP key will begin

the upcount and the timer will continue to count upward until the

the STOP key is again pressed.  Following this STOP command, the

timer will reset to the initial time and be ready to countdown

again.

To exit this second application, press "STOP" to get to the new-

time prompting menu, then press "enter" (not "STOP"), which will

enter a time of "0" for counting down.

After running either the TIM1 or the TIMER application, you are

returned to the library menu (with TIM1 and TIMER showing in the

menu).  If you run TIM1, no changes are made to the rest of the

calculator memory.  However, if you  execute the TIMER function,

the last time used in the timer as the timer starting time is

stored in the directory to which the library is attached.  The

variable, TSTART, holds the contents of the start time.  Upon the

next startup of TIMER, it will look for this variable, and if it

exists in the current directory, or above, it will use the time

stored in this variable for the length of countdown.

Consequently, you can enter the desired countdown time prior to

executing the timer application, if you wish.

If the TSTART variable does not exist, the timer will use 1:00 for

its initial starting time.

Enjoy - I am also working on other pilot applications, although I

may have to ask for a fee to cover the divorce attorneys.  If you

have an interest in weight and balance calculations with a plot of

the relative position of the loaded airplane  within the envelope,

or if you have interest in a navigation program which selects the

best altitude for optimum winds, let me know.  These are a month

or two away - And then testing is a so and so.

Steve Mauser