(Comp.sources.hp48)

Item: 138 by kcooke@milton.u.washington.edu [Ken Cooke]

Subj: Machine Code FFT

Date: 12 Jul 1992

Here is some FFT code that I wrote several months ago but never got around

to posting.  I tried to make it fast enough to be useful.  The code is based

on the standard DIT radix-2 FFT, so the input length must be a power of 2.

All intermediate math is done in 15-digit precision, which gives very accurate

results. (a 1024-point FFT matches MATLAB to 12 digits)

I used the following definition of the FFT:

N-1

X[k] = Sum( x[n] * exp(-j*2pi*k*n/N) )

n=0

N-1

x[n] = 1/N Sum( X[k] * exp(j*2pi*k*n/N) )

k=0

Quick description of the programs in this UUENCODED directory:

FFT: machine code FFT.  Input is a vector (either real or complex).

Size must be a power of two.  Output is a complex vector.  Includes

error checking.  I believe this is as fast as the HP48 can be at a complex

FFT.  Random 128-pt complex FFT: 7.5 seconds.  About 8 times faster than

system RPL, and 1.5 times faster than the FFT in the HP71 mathpack after

compensating for clock speed differences.  (I did not look at the mathpack

code)

IFFT: inverse FFT.  Could have been combined with the FFT program, but I

hate using an additional argument to tell it which one to do.

ABSV: machine code absolute value.  Input can be a vector or matrix.

Output is a vector or matrix containing the absolute values of the complex

elements.  About 5 times faster than RPL.

ANGLE: same as ABSV, but returns the angle (i.e. phase) according to the

current angle mode.  This is only 2 or 3 times as fast as RPL, because

the mcode arctan is such a bottleneck.

SPEC(trum): User code quickie that does FFT, ABSV, and then plots using the

built-in barplot function.  Note that plot is from 0..2pi as returned by FFT.

DISCLAIMER

While I believe these programs to be accurate and bug-free, I

will assume no responsibility for any Memory Lost, hardware damage, or

poor test scores that result from use of this code.

Please let me know if you find a bug or have suggestions.

Ken Cooke

N7VFE

kcooke@u.washington.edu