(Comp.sys.hp48)

Item: 438 by stevev@greylady.uoregon.edu

Author: [Steve VanDevender]

Subj: REMOTE4: IR and IR2--IR remote-control sampling programs

Date: Tue Dec 31 1991

[Note: Also see REMOTE3 on this disk for another programmer's approach

to turning the HP 48 into a universal infrared remote control.  -jkh-]

These programs embody a new way of performing infrared remote control

sampling on the HP-48.  The previously distributed remote sampling

programs sampled IR input by continually reading the IR input and

immediately writing it out to memory.  These programs use a different

approach--they continually sample the IR input while incrementing a

timing count, and only write information to memory when the IR state

changes or the counter overflows.  This approach increases the

effective sampling rate under many circumstances and greatly reduces

the amount of memory needed to store samples.  I have also paid

special attention to instruction timing to make sure that the timing

characteristics of the sampling and playback routines are as nearly

identical as possible; this has been a problem with some previous

attempts at remote sampling.

One of my motivations for writing these programs was that the other IR

samplers I tried wouldn't work at all with my VCR.  Both of these work

with my VCR, although I have obtained mixed results using these

programs with other kinds of remotes.  I am hoping that these programs

will be more effective than previous attempts, and that the techniques

I have used here can be improved upon by others.

There are two separate IR sampling systems included here.  The first,

in the IR subdirectory, are the first programs I developed that

worked.  They store 3-nibble sample units--1 nibble for the current IR

input state, and 2 nibbles for a repeat count.  The inner loop of the

sampling and playback routines runs at about 25 microseconds per loop

iteration, and take about 40 microseconds to write out the sample data

and re-enter the inner loop.  These routines will make samples of

about 90 bytes that work with my VCR.  The second, in IR2, are

2-nibble samples containing a 1-nibble repeat count.  The inner loop

of these routines runs at about 23 microseconds per iteration, but

sample string are a little over twice as long, at 224 bytes for

samples that work with my VCR. Interestingly enough, though, Lutz

Vieweg's RFP program will pack samples created by either set of

routines to about the same length of 60 bytes.  [Note: See RF on

Goodies Disk #5 in the PROGRAMR directory.  -jkh-]

The directory IR contains the 3-nibble sampling routines, and the

directory IR2 contains the 2-nibble sampling routines.  IR contains

the user RPL programs SAMPLE and PLAYBACK and the machine code objects

IRSAMP and IRPLAY.  IR2 contains SAMPLE2, PLAYBACK2, IRSAMP2, and

IRPLAY2.

The program SAMPLE (SAMPLE2) will take either a real number in level

1, which is used to generate a string of that length to receive the

sample, or a string which will be overwritten with sample data. It

calls the machine-language program IRSAMP (IRSAMP2) which performs the

sampling.  IRSAMP checks for a string object in level 1, which must

have SIZE > 1, and aborts if it doesn't find one.  It then waits up to

15 seconds for IR input, (that is, for you to place your remote

control next to the calculator and press the button for the remote

function you want to sample) and aborts if none is received in that

time. Otherwise it begins sampling and exits when the end of the

string in level 1 is reached.  I have found that holding an IR remote

too close to the calculator during sampling will often result in

ineffective samples; I get best results with the remote about 5-10 cm

from the IR input port.

The program PLAYBACK (PLAYBACK2) will take a string in level 1 and

feed it to IRPLAY (IRPLAY2), then drop the string.  You can feed

strings directly to IRPLAY if you wish, but they will be left on the

stack (although this is a useful way to test samples without having to

recover them with LASTARG).  You should only feed strings created with

IRSAMP to PLAYBACK or IRPLAY.  It is possible to create strings which

leave the HP-48 IR LED on for long periods of time, which drains the

batteries and may cause hardware damage.  In any case, IRPLAY will

turn the IR LED off before exiting.

Two of my friends who have tested these programs have reported that

they have experienced crashes while using them, although one report

did not appear to involve memory corruption or any other damage.

Therefore, I do not guarantee these programs to be bug-free--they may

cause crashes, memory corruption, or even hardware damage if used

improperly, although I can also say that I have had no problems for

over a week since I debugged the routines in the IR directory.

Neither are they guaranteed to work with any IR remote control,

although I suspect they will have greater success than previous remote

sampling programs.

I would like to thank Dave Marsh for his original IR sampling

programs, which I have copied small parts of and which were my main

source of technical information in writing these programs, and Jan

Brittenson for his excellent STAR assembler and MLDL debugger which

made writing and debugging these programs easy.

--

Steve VanDevender   stevev@greylady.uoregon.edu