(Comp.sys.hp48)

Item: 1487 by akcs.n2kyw@hpcvbbs.cv.hp.com [Paul Smith]

Subj: How to open your hp48

Date: 26 Jul 1992

[No *WAY* am I gonna try this!!  It voids the warranty and uglifies

the machine!  But you go right ahead!  #  -jkh-]

Disassembling the hp48sx (may or may not correspond to the 48s, but

probably will to some extent).

Sufficient numbers of people (more than 1) have requested information

on how to disassemble their hp48.  Despite the fact that that's pretty

scary, I will entail what I know here.  Thanks to David Holmes for

taking the first step and helping make my attempt more informed.

None of the information that follows came from hp, so if what I say

starts to sound rather official, it's not.

For the few folks out there who've actually had to send their hp48

back to hp for service, you probably ended up with an entirely new

unit.  I suspect the reason for this, aside from some labor cost

savings on the part of hp, is that they really aren't much fun to take

apart.  You can draw your own conclusions as to why it was designed

that way.

0.  Backup your memory.  You will need to remove the batteries for

awhile to disassemble the unit, and you may have them out long enough

to cause memory loss (although in my case, even 2 days didn't wipe

anything out).

1.  Remove the tin key overlay.  The overlay is attached with double-

sided tape of some sort.  A little care and patience will allow you to

remove it only slightly warped, and it is fairly easy to straighten

out later.  I took a small jewelers screwdriver and started prying

near the "ON" button, and worked upward toward the screen.  When you

get near the screen, try to preserve the shape of the overlay because

that portion is a bit more difficult to properly straighten later.

Once the overlay is removed, put it in a container or somewhere away

from dust.  The double-sided tape should stay with the overlay (mine

did, at least) and will gather dust and stuff making it less sticky if

left out in the open.  48sx owners will need to remove the little hp

logo insert above the screen as well.

2.  Defeat the 10 plastic rivets.  Believe it or not, these rivets

aren't all that critical to holding the unit together, so you can be

as careful or as reckless as you want (I prefer reckless, because I

hate plastic rivets).  A good way to carefully remove the rivet heads

is to use a flat head screwdriver that is the same width as the

recessed rivet hole (4mm?) and "drill" the head away by hand with the

screwdriver.  The head is about 2mm deep, so stop "drilling" there to

leave the keyboard material shoulder intact for easier reconnection

later.  There are 6 rivets near the number keys and four above the

screen.  These rivets above the screen perform more of a holding

function than the others, and you may want to consider using some

screws and small washers to replace them when you re-assemble.

3.  Separate the lower half from the upper half.  The upper half of

the calculator contains almost all of the components except for the

piezo "beeper" element, and there are no interconnecting wires between

halves.  The only obstacle is the snap-together "hooks" that David

Holmes refers to. These hooks are positioned near the [A], [F], [SIN],

[1/x], [ON], [+] and [.] keys (the one by the [.] key is a real

bugger!).  Refer to the cutaway diagram below for details.

Key overlay here ___

\     ____________

\   |            |

____________|    ______  |

|   _  | |  <--- Upper section.

|  | | | |

________________|  | | |_|

|  | |___

Circuit board and other    |  |     |

components (affixed to     | /      |  <--- Lower section.

upper section)             |/__     |        Has a lip which

|   |    |        engages with metal

Metal "hook" ---> | / |    |        "hook" from upper

|/  |    |        section.

|   |    |

________________|   |    |

|    |

To separate the two sections, you will need to push the lower section

out and down around the hooks.  You can't do this from the outside

because the upper section hangs over the lower, so you have to go from

the inside.  Luckily, the six slots in the keyboard (used by the

separate user-overlays that fit over the original overlay) near the

[MTH], [ENTER], [blushift], [NXT], [backspace] and [-], come in handy

for this.  These slots are not exactly lined up with the hooks, but

are close enough.  You can insert something in these holes (I used a

jewelers screwdriver, flat head) at an angle that is mostly down and

somewhat out, to a depth of about 5mm, and come in contact with the

lower section.  Push the lower section out about 2mm (this will take a

bit of force), while wedging something in the outside gap to separate

and hold the sections apart while working on the other hook positions (

a wooden matchstick works).  I recommend starting with the [A] or [F]

positions first, working down whichever side you started with, then do

the other side, and leave the [.] position for later (there is no

helpful slot there).

4.  Remove the battery cover and the batteries.  The two battery

contacts which come through the case will need to go back through the

case when the sections separate.  The upper contact is hooked on a

plastic boss, and needs to be freed from it.  Just pop it off with

your finger or a screwdriver.

5.  Very carefully pull the two sections apart at the top (above the

screen).  If you have freed all the hooks, the case should sort of

hinge at the bottom edge.  This is due to the last hook (near the [.]

key).  Some careful twisting and working of the sections should free

this hook as well. You should now be able to completely separate the

two sections.  I recommend at this point that you take a pair of

pliers and flatten that last hook so that it will not hold next time.

It is really not needed anyway, as the other six hooks hold the unit

together just fine.

Now you can poke around and explore things.  Be careful what you

touch, I'm not sure how sensitive the insides are to static discharge

from your fingers, etc.  If you're interested in adding stuff, some

open real estate can be found in the areas between the battery

compartment and the card receivers, and in the cavities in the lower

section below the tin shielding on either side of the card ports (this

is where I put some jacks).  If you own a 48s, you may find a lot more

space.

If you want to disassemble the unit further, like removing the circuit

board from the upper section, you have a considerably tougher job on

your hands, and you're on your own.  Also, if you separate the screen

from it's circuit board, you will disturb the rubber conductors (there

are two) which provide electrical connection to the lcd rows and

columns (zebra strips).  Upon reassembly of the screen, you may find

that you have lost some rows or columns in the display (not permanent,

just a zebra strip alignment problem) and will have to keep

reassembling and perhaps cleaning until it comes out okay.  I haven't

done this to my hp (yet) but I have done it to other cheap calculators

and it wasn't fun.

As to questions about what exactly is inside, I can only guess.  David

Holmes had some observations, and there are apparently a lot of people

out there in netland that know a lot more about the insides already.

Reassembly-

Putting it back together is much easier.  You may need to clean up the

remains of the rivet heads so they will easily reinsert into their

holes.  Make sure the battery connections align with their respective

holes, and snap the unit together, applying pressure where the six (or

seven, if you left that one near the [.] key alone) hooks are.  You

will probably want to hold the case together at the top with one or

two of the rivets there.  If you can find a wood screw with a flat or

thin head, you can screw it into the body of the rivet and let the

head hold the upper section surface.  You may need a small washer for

this.  A machine screw will probably work as well, but will strip the

plastic easier and not hold as well.  I have only one screw holding

mine together and it works fine.

You will probably need to re-shape your tin overlay, as it probably

took a beating during removal.  I removed the sticky tape from mine,

but it's probably better not to (unless it really won't stick

anymore). Put wax paper over the tape and put the overlay face down on

a hard cover book.  Grab a hammer with a smooth and somewhat flat

head, and pound away.  DONT pound away on the part that surrounds the

screen (beveled part).  You can probably use your fingers and a little

massaging to fix that area.  With a little care you can end up with an

overlay that looks like new.  Press the overlay in place and hope it

sticks.  If not, a little rubber cement wouldn't hurt.

Have fun!

Paul Smith

v055qmd6@ubvms.cc.buffalo.edu