V o y a g e r   R e f e r e n c e   M a n u a l

================================================================

Overview

================================================================

Voyager is a tool to interactively disassemble the memory (RAM or

ROM) in the HP 48SX.  In this context, the term disassemble is

used to refer to disassembly of both machine code (aka

unassemble) and threaded instructions (aka unthread).

Voyager consists of several images:

VOYAGER.EXE	Main program

DMPTOMEM.EXE	Converts ROM dump to Voyager MEM file

BINTOMEM.EXE	Converts binary file to Voyager MEM file

The calculator's address space in Voyager is divided into "ports"

(similar to the ports in the calculator itself).  The ports are:

Port 0: 00000-7FFFF

Port 1: 80000-BFFFF

Port 2: C0000-FFFFF

A 'module' can be loaded into one of Voyager's ports with the

LOAD command.  The HP48 module is automatically LOADed into port

0 when you invoke Voyager.

Each module consists of these files:

module.MEM	Voyager MEM file (memory dump)

module.CA	Address Comments

module.CC	Code Comments

module.CAX	Index for the CA file

module.CCX	Index for the CC file

The MEM file is required; the other files will be created, if

absent.  [NOTE! See MEM.DOC before using Voyager.]

The MEM file is an ASCII listing of the nibbles in the module to

unthread or unassemble.  It can is produced by downloading via

the Memory Scanner or KERMIT transfer and then filtered with

either the DMPTOMEM or BINTOMEM programs.

Two comment files are used by Voyager. Both have the same format.

Each line consists of the 5 nibble address being described,

followed by a space and a one-line comment describing the

address. Lines beginning with a '-' are considered deleted.

The CA file is the address comment file.  It provides comments

explaining the purpose of address references.  For example, the

DUP routine is at 1FB87.  Unthreading is done with the UNTHREAD

or UT commands.  Unthreading 1FB87 yields:

Voyager> UT 1FB87

1FB87: ; *** DUP (XLIB 2 269) ***

1FB87: 02D9D ! Program

1FB8C: 18AA5 ; Save last RPL token and verify DEPTH >= 1

1FB91: 03188 ; Internal DUP

1FB96: 0312B ! End Marker

Each comment following the threaded routine address is stored in

the CA file.  Note that comments that start with an exclamation

point are generated by Voyager; Voyager knows about object

prologs and can format in-line objects (including Programs)

without assistance.  Comments that start with a semicolon are

from the CA file.  Address comments are created and deleted with

the CA and CAD commands.

Another example: the RPL routine at 14039 is a very deep

internal routine to the code for ERRN.  It pushes the Last Error

Number as a System Binary (also called "short" or "address" by

some people).

Voyager> UT 14039

14039: ; *** Push Last Err# as System Binary ***

14039: 1403E ! Machine Code at 1403E

1403E: 34FF607    LCHEX  706FF          ; Save Last Err#

14045: 8D3B411    GOVLNG 114B3          ; Push @C.A as new System

Binary

114B3: ; *** Push @C.A as new System Binary ***

114B3: DA         A=C    A

114B5: 8FB9760    GOSBVL 0679B          ; Save D,B,D1,D0 (uses C)

114BC: 130        D0=A

114BF: 142        A=DAT0 A

114C2: 8DC7530    GOVLNG 0357C          ; Push A as new System Binary

The CC file is a code comment file.  It provides comments

explaining what the code at a particular address is doing.  When

the corresponding address is unassembled, the comment is

appended to the output line.  Code comments are created and

deleted with the CC and CCD commands.

Note that GOTOs are followed.  This can be disabled or enabled

with the SET MODE NOFOLLOW and SET MODE FOLLOW commands.

Example: the RPL routine at 03188 executes the inner-most code

for the DUP routine (i.e. after error checking and setup has

been done). It is an assembler routine with an RPL entry point,

and unthreading the entry point yields:

Voyager> UT 03188

03188: ; *** Internal DUP ***

03188: 0318D ! Machine Code at 0318D

0318D: CF         D=D-1  A              ; room on stack?

0318F: 41F        GOC    03181          ; if not, make some

03192: 143        A=DAT1 A              ; get obj @ TOS

03195: 1C4        D1=D1- 5              ; push object again

03198: 141        DAT1=A A

0319B: 142        A=DAT0 A              ; continue RPL

0319E: 164        D0=D0+ 5

031A1: 808C       PC=(A)

The CAX and CCX files are index files for the CA and CC files.

================================================================

Conventions

================================================================

UPPERCASE	Uppercase characters indicate a command or

keyword.  These can be entered as either upper

or lower case.  Most commands can be

abbreviated.

lowercase	Lowercase is used as a placeholder for

information that you would enter.

[optional]	Square brackets indicate an optional parameter

or keyword.

addr		A five digit hexadecimal address address, a

"." or a "$".

A "." means the next address or 'continue'.

A "$" means the last specified address or

'repeat'.  The last specified address can be

either explicitly specified or implicitly

specified.

If addr is optional and omitted, it defaults to

the next address (continue).

addr:addr	Indicates an address range.  You may omit either

the start or end addresses.  The start address

will default to 00000.  The end address will

default to FFFFF.  You may also use the "." and

"$" as specified above.

If you specify a single address without the

range specifier (the ":") then the range is

that single address.

For example:	Means:

:		00000 to FFFFF

:14D20		00000 to 14A20

1175:		01175 to FFFFF

3E7:400		003E7 to 00400

2B7		002B7 only

================================================================

Voyager Quick Reference

================================================================

CA addr text

CAD addr

CC addr text

CCD addr

CL addr

DUMP [addr]

DA [addr]

DH [addr]

DT [addr]

EXAMINE HASH_TABLE addr lib-no

EXAMINE LINK_TABLE addr lib-no

EXAMINE MESSAGE_TABLE addr lib-no

EXIT

FIND addr:addr partial-comment

HELP

LIST addr:addr

LOAD port module

MERGE port filename

SEARCH addr:addr hex-string

SET LIMIT number

SET MODE FOLLOW

SET MODE NOFOLLOW

SET MODE INFO

SET MODE NOINFO

SET MODE PAGE

SET MODE NOPAGE

SET MODE PROMPT

SET MODE NOPROMPT

SHOW LIMIT

SHOW MODES

SHOW PORTS

SHOW VERSION

SORT port

UNASSEMBLE [addr]

UA [addr]

UNCONTINUE [addr]

U [addr]

UNLOAD port

UNTHREAD [addr]

UT [addr]

================================================================

Voyager Command Dictionary

================================================================

----------------------------------------------------------------

CA addr text

Creates or replaces an Address Comment for the specified address.

----------------------------------------------------------------

CAD addr

Deletes the Address Comment for the specified address.

----------------------------------------------------------------

CC addr text

Creates or replaces a Code Comment for the specified address.

----------------------------------------------------------------

CCD addr

Deletes the Code Comment for the specified address.

----------------------------------------------------------------

CL addr

Lists the Address and Code comments for the specified address.

----------------------------------------------------------------

DUMP [addr]

DA [addr]

DH [addr]

DT [addr]

Displays the contents of memory at the specified address.  The

number of lines displayed can be changed with the SET LIMIT

command.  DUMP displays the information in the current dump

mode (defaults to HEX).  DA, DH and DT set the dump mode to

ALTERNATE, HEX or TEXT and then displsy the information.

----------------------------------------------------------------

EXAMINE HASH_TABLE addr lib-no

Examines the Binary Integer at the specified address as if it

were the Hash Table for the specified library.

----------------------------------------------------------------

EXAMINE LINK_TABLE addr lib-no

Examines the Binary Integer at the specified address as if it

were the Link Table for the specified library.

----------------------------------------------------------------

EXAMINE MESSAGE_TABLE addr lib-no

Examines the Array of String at the specified address as if it

were the Message Table for the specified library.

----------------------------------------------------------------

EXIT

Exit Voyager and return to DOS.

----------------------------------------------------------------

FIND addr:addr partial-comment

Searches the Address Comment files for the specified partial

comment.  The case of the comment is ignored.

For example:  To search for all occurrences of "prolog" in the

range 2000 to 3000, you would say:

Voyager> FIND 2000:3000 prolog

----------------------------------------------------------------

HELP

Provides brief help on Voyager commands.

----------------------------------------------------------------

LIST addr:addr

List the Address Comments in the specified range.

For example: list the address comment for 3ADA.

Voyager> LIST 3ADA

For example: list all the address comments from 3B234 to 3DB15.

Voyager> LIST 3B234:3DB15

----------------------------------------------------------------

LOAD port module

Loads the specified module into the specified port.  If a module

has already been loaded in that port, the old module is unloaded

first.  The comment files are created if they are missing.  The

index files are automatically recreated if they are out-of-date

or missing.  The required MEM file (module.MEM) can be created

with either the BINTOMEM or DMPTOMEM programs.

For example: load the REV module into port 1.

Voyager> LOAD 1 REV

----------------------------------------------------------------

MERGE port filename

Merges the comments in the specified file into the address

comment file for the specified port.  Only new comments are

added to the comment file.  Existing comments are not

overwritten.

The format of the comments in the file to be merged can be up to

255 characters long and must be in this format:

<hex-address><separator><comment>

Where:

<hex-address> = a 1 to 5 digit hexadecimal address.

<separator> = a single character followed by optional whitespace.

<comment> = any text, up to the new-line character.

----------------------------------------------------------------

SEARCH addr:addr hex-string

Search part of the memory.  Hex-string specifiers the data for

which to search.  For example, to search for references to the

internal DUP command (at address 03188), you would say:

Voyager> SEARCH : 03188

To search for a hex-string in Memory Scanner format (backwards),

you would prefix the hex-string with a single quote.  For

example (this is equivalent to the previous example):

Voyager> SEARCH : '88130

----------------------------------------------------------------

SET LIMIT number

Sets the number of lines displayed by the UNASSEMBLE and DUMP

commands.  The default is ten.

----------------------------------------------------------------

SET MODE FOLLOW

SET MODE NOFOLLOW

Specifieds whether or not the UNASSEMBLE command follows

absolute gotos (GOTO, GOLONG and GOVLNG).  Conditional gotos

(GOC, GONC, RTNC or RTNNC) that have a Code Comment of "BET"

(Branch Every Time) are also subject to the same logic.  The

default is FOLLOW.

----------------------------------------------------------------

SET MODE INFO

SET MODE NOINFO

Specifies whether or not information (debug) messages are

displayed.  The default is NOINFO.

----------------------------------------------------------------

SET MODE PAGE

SET MODE NOPAGE

Specifies whether or not Voyager pauses after a screen full has

been displayed.  The default is PAGE.

----------------------------------------------------------------

SET MODE PROMPT

SET MODE NOPROMPT

Specifies whether or not the Voyager prompt is displayed.  The

default is PROMPT.

----------------------------------------------------------------

SHOW LIMIT

Displays the number of lines that will be displayed by the

UNASSEMBLE and DUMP commands.

----------------------------------------------------------------

SHOW MODES

Displays the various mode settings.

----------------------------------------------------------------

SHOW PORTS

Displays which ports have been loaded.

----------------------------------------------------------------

SHOW VERSION

Displays the software version for Voyager.

----------------------------------------------------------------

SORT port

Sorts the address and code comment files for the specified port.

The files are sorted by address and the index files are

recreated.  You must fist LOAD the port before sorting it.

Sorting is not required, but it will eliminate deleted comments

and make the comment files nice and pretty for printing.

----------------------------------------------------------------

UNASSEMBLE [addr]

UA [addr]

Disassembles the memory as machine code starting at the

specified address.  The number of lines displayed is specified

with the SET LIMIT command.

----------------------------------------------------------------

UNCONTINUE [addr]

U [addr]

Continues disassembly based on the last mode (unassemble or

unthread).

----------------------------------------------------------------

UNLOAD port

Unloads the module in the specified port.

----------------------------------------------------------------

UNTHREAD [addr]

UT [addr]

Disasembles one object starting at the specified address.

================================================================

BINTOMEM input[.BIN] [output[.MEM]]

(program)

Converts a binary file into a flat memory dump (Voyager format).

================================================================

DMPTOMEM input[.DMP] [output[.MEM]]

(program)

Converts a Memory Scanner dump into a flat memory dump (Voyager

format).