---------------------------------------------------------------------------

WATR V1.0

WATER PROPERTIES

by

CONAN J. FEE

UNIVERSITY OF WAIKATO, NEW ZEALAND

This is a set of routines for calculating water properties (both liquid and

vapour) (eg:viscosity, specific volume, density, etc, etc...) as a function of

temperature. The simple user interface allows you to input the temperature

and then to determine the desired property. It's self-explanatory, but I'll

explain it anyway:

The temperatures must be within the specified range of 32_F to 440.3_F

(see "HELP" screens) and the package will not let you enter temperature

values outside this range.

Units may be specified in advance as SI or ENGL (default). Outputs have

appropriate units attached. Units are checked if you follow the correct

procedure (ie:don't go by-passing the built-in menus).

Just try it - you'll see and I hope you'll find it useful. You ought to if

you're a Chem Engg major.

The calculations are based on a set written by D.S. Dickey in two articles

in 'Chemical Engineering' Sept 1991, pp207-8 and Nov 1991, pp235-6. All

credit for the correlations should go to him. The correlations were

created from tables in Perry's Chemical Engineers' Handbook 6th Edtn.

Some variables hidden for user convenience using the HIDE application from

Educalc Goodies Disk #1, written by Morgan Lindqvist and Mattias Dahl.

I will accept absolutely NO liability for any problems, losses or damage that

might arise by any (correct or incorrect) application of the properties

calculated by this package - basically if it's THAT important to you,

check the results first!!!

The following is a list of the maximum errors in the calculated properties

as given by Dickey in the second of his two articles:

Property Correlation    Maximum Error (%)

Vapour Pressure         3.64

Boiling Point           2.66    (as a function of pressure)

Liq. Specific Vol.      0.12

Therm. Expans. Coeff.   -

Liq. Density            0.26

Vap. Specific Vol.      8.47

Vapour Density          7.81

Liq. Enthalpy           3.08

Lat. Heat Vap.          0.63

Vapour Enthalpy         0.26

Liq. Specific Heat      0.25    (0.11 for 32_F < T < 250_F)

Vap. Specific Heat      0.49

Liq. Viscosity          20.23

Vap. Viscosity          0.37

Liq. Therm. Conduc.     0.43

Vap. Therm. Conduc.     1.09

Liq. Prandtl No.        11.10   (3.14 for 53.3_F < T < 260.3_F)

Vap. Prandtl No.        0.76

To use this package, just transfer the following ->ASC listing to the

HP48SX via Ascii transfer, recall the string to the stack, run ASC->

on it and STO