LOGPlus! version 3.0

Designed for the CASUAL OPERATOR to the SERIOUS DX'ER

Setting a NEW WORLD STANDARD!

Written by:  Robert A. Winters, KD7P

Distributed by: DMW Systems

9909 121ST Ave N.E.

Lake Stevens, Washington 98258

BBS: (206) 653-9652

Voice: (206) 653-8304

LOGPlus! version 3.0 - Table of Content

Table of Content                                            i-vii

Copyright Notice                                            1

Disclaimer                                                  1

Acknowledgements                                            1,2

Introduction                                                2

Hardware/Software Requirements                              2

Modifications to your CONFIG.SYS                         2

Multi-tasking Software                                   2

DO'S and DO'NTS                                             2,3

Upgrade/Update Policy                                       3

Technical Support                                           3,4

Bulletin Board System                                       4

Configuration                                            4

User Access                                              4,5

Comments to all users                                    5

Receiving an upgrade                                     5

Conversion programs                                      5

Function Keys                                               5,6

Special Edit Keys                                        6

Logging Into LOGPlus!                                       6,7

Multiple Call signs                                         7

Creating and Shifting between logs                       7

Directory Structure                                      7,8

Required Database Files in each sub-directory            8

Getting Around the Browse/Modify                            8,9

The Main Menu                                               9

Function Keys Active at Main Menu                         9

Statistics Window (Top Window)                            9,10

Menu Window (Bottom Window)                               10

A> Logbook Management                                     10

Working with Deleted Countries                         10

How Deleted Countries are determined                   10,11

Real-Time and Off Air Logging Screen                   11

QSO Display Window                                     11,12

Data Entry Window                                      12,13

General Information Window                             13

DXCC Statistics Window                                 13

Selection Menu (Bottom of Screen)                      13,14

Function/Control Keys                                  14

F2 Quick Call Saver                                14

F3 General Information Update                      14

F4 Quick Call Checker                              14,15

F5 Browse LOGTEMP File                             15

F6 Partial Call Search                             15

F7 Display Previous QSOs (w/PacketCluster Only)    15

ALT_A Sunrise/Sunset Browse                        15

ALT_B Buffer Scroll (w/PacketCluster Only)         15

ALT_H Help!                                        15

ALT_J Jump to Spot                                 15

-i-

LOGPlus! version 3.0 - Table of Content

ALT_K Radio On/Off                                 15,16

ALT_M Temperature/Mileage Conversion               16

ALT_N Callsign Notebook                            16

ALT_Q QRP Logging                                  16

ALT_R Jump Back (Return) From Spot                 16

ALT_Y YL Logging                                   16

ALT_Z WAZ Quick Display                            16

PacketCluster ALT Keys                             16,17

ALT_F2 PacketCluster Alert Browse Routine

ALT_F4 PacketCluster Spot Announcement

ALT_F5 PacketCluster Talk

ALT_F6 PacketCluster Alert Setup

ALT_F7 PacketCluster Special Spot Alerting

ALT_F8 PacketCluster Alarm On/Off (default OFF)

ALT_F9 PacketCluster Silence Alarm

ALT_F10 PacketCluster Commands

CTRL_F2 Change Radio Frequency

CTRL_F3 Change Radio Mode

CTRL_F4 Change Radio VFO ( Kenwood only )

CTRL_F5 Up Radio Freq

CTRL_F6 DN Radio Freq

CTRL_F7 Drop to DOS Shell

E> Browse/Modify                                       17

F> Search Routines                                     17,18

G> Print Alphanumeric Logbook                          18

H> Print Chronological Logbook                         18,19

I> Band & Mode Statistics                              19

J> Update QSL Manager in Logbook                       19

K> Contest Conversions                                 19,20,21

L> Logbook Refresh                                     21,22

M> Indexing                                            22

B> Satellite Management                                   22

Data Entry Window                                      22

DXCC Statistics Window                                 22

C> QSL Card & Managers                                    23

A> QSL Card Updating                                   23

The Three QSL Database Files                        23,24,25,26

Function Keys                                       26

B> Direct - Printing/Browse                            26

A> Browse/Modify                                    26,27

B> Update QSLMgrs Field                             27

C> QSL Label Printing                               27,28

A> Standard Mailing Label w/Note                 28

B> Standard Mailing Label (op1)                  28

C> Standard Mailing Label w/QSLMgr               28

D> Standard Mailing Label (op2)                  28,29

E> 2-15/16 x 7/8 Label                           29

F> 2-15/16 x 7/8 Label w/QSLMgr                  29

G> W4MPY QSL Format Label                        29

-ii-

LOGPlus! version 3.0 - Table of Content

H> Modified - W4MPY QSL Format                   29

I> Western Washington DX Label                   29

J> 2 QSO Format w/QSLMgr                         29

K> 4 QSO Format w/Note                           29,30

L> 4 QSO Format w/Note & QSLMgr                  30

M> 5 QSO Format                                  30

N> 5 QSO Format w/QSLMgr                         30,31

O> Large Label w/Rig & QSLMgr                    31

P> Set Radio & Antenna Defaults                  31

D> Delete ALL Records                               31,32

E> Pending - Printing/Browse                           32

A> QSL Pending Update                               32

B> QSL Pending Browse/Modify                        32,33

C> QSL Pending Printing                             33

D> Delete ALL Records                               33

F> SWL - Printing/Browse                               33

A> SWL Card Entry                                   33

G> QSL Manager Database                                33,34

H> Yearly QSO/QSL Statistics                           34

I> Index QSL Card Databases                            34

J> Index QSL Manager Databases                         34

D> DX Centry Club Award                                   34

A> Auto Update (log)                                   34,35

C> Browse/Modify                                       36

D> Single Band Awards                                  36

A> Auto Update                                      36

B> Browse/Modify                                    37

C> Statistics                                       37

D> Printing                                         37

E> Indexing                                         37

E> Status Update                                       37

A> Confirmed to Submitted                           37

B> Submitted to Approved                            37

F> Band & Mode Country Statistics                      37,38

G> Band/Mode Statistics                                38

H> DX Alert                                            38

A> DX Alert DXPedition Update                       39

B> DX Alert Activity Browse/Modify                  39,40

C> Today's DX Activity Browse/Modify                40

D> Weekly DX Activity Browse/Modify                 40,41

E> Monthly DX Activity Browse/Modify                41

F> Upload DX Alert Information                      41

G> Download DX Alert Information                    41

H> Delete OUTDATED Information                      41

I> DX Alert File Indexing                           41

I> Country List Update                                 41,42

Adding a New DXCC Country                           42,43

Modifying an Existing DXCC Country                  43

Changing the Deleted Country Status                 43

-iii-

LOGPlus! version 3.0 - Table of Content

J> Callsign to Country Mapping                         44

K> Prefix to Country Mapping                           44

L> Update Beam Heading & Sunsets                       44

M> SunRise & SunSet Browse/Modify                      44

N> Clean Up DXCC/LOGBOOK Databases                     44

O> Printing                                            44

P> Indexing                                            44

E> Worked All Zones Award (WAZ)                           44

Band & Mode Tracked                                    45

Required Files                                         45

A> Auto Update (log)                                   45

B> Auto Update (sat)                                   45

C> Browse/Modify                                       45,46

D> Quick Look Statistics                               46

E> Band/Mode Statistics                                46

F> Zone/QSO Statistics                                 46,47

G> Printing                                            47

H> Indexing                                            47

F> Worked All States Award (WAS)                          47

Bands & Modes Tracked                                  47

Required Files                                         47

A> Auto Update                                         48

B> Browse/Modify                                       48

C> Quick Look Statistics                               48,49

D> Band/Mode Statistics                                49

E> State/QSO Statistics                                49

F> Printing                                            49

G> Indexing                                            49

G> Worked All Prefixes Award (WPX)                        49

Bands & Modes Tracked                                  49,50

Required Files                                         50

A> Auto Update                                         50

B> Manaul Update                                       50

C> Browse/Modify                                       50,51

D> Prefix Display                                      51

E> LookUp (all files)                                  51

F> Field Update (log)                                  51

G> Status Update                                       51,52

A> Confirmed to Submitted                           52

B> Submitted to Approved                            52

H> Statistics                                          52

I> Printing                                            52

J> Indexing                                            52,53

H> VHF/UHF Century Club (VUCC)                            53

Bands & Mode Tracked                                   53

Required Files                                         53

A> Auto Update                                         53,54

B> Browse/Modify                                       54

C> Grid Display                                        54

-iv-

LOGPlus! version 3.0 - Table of Content

D> Grid/Grid Distance Calculator                       54

E> Grid/Posn Conversion                                54

F> Posn/Grid Conversion                                54

G> Grid Statistics                                     54

H> Printing                                            54,55

A> Worked - QSL Pending                             55

B> Confirmed - QSLs Received                        55

C> Submitted to Awards Manager                      55

D> Approved by Awards Manager                       55

E> Complete Listing of Grids                        55

I> Indexing                                            55

I> Islands On The Air (IOTA)                              55,56

Bands & Modes Tracked                                  56

Required Files                                         56

A> Auto Update                                         56,57

B> Manual Update                                       57,58

C> Browse/Modify                                       58,59

D> Statistics                                          59

E> Printing                                            59

F> Indexing                                            59

J> Worked All Russian Oblasts                             59

A> Auto Update                                         60

B> Browse/Modify                                       60

C> Band/Mode Statistics                                60

D> Printing                                            60

E> Indexing                                            60

K> Worked All U.S. Counties                               60

A> Auto Update                                         60,61

The Note Field                                      61,62

The Update Process                                  62

B> Browse/Modify                                       62

C> Statistics                                          62,63

D> Printing                                            63

E> Indexing                                            63

L> Generic Award System                                   63,64,65

A> Auto Update                                         65

B> Browse/Modify                                       65,66

C> Printing                                            66

D> Indexing                                            66

M> Dumb Terminal Program (DTP)                            66

Status Line (bottom line)                              66,67

Main (center) section                                  67

PacketCluster                                          67,68

CTRL_F1 Radio Interface On/Off                      68,69

CTRL_F2 Change Radio Frequency                      69

CTRL_F3 Change Radio Mode                           69

CTRL_F4 Change Radio VFO ( Kenwood only )           69

CTRL_F5 Up Radio Freq                               69

CTRL_F6 DN Radio Freq                               69

-v-

LOGPlus! version 3.0 - Table of Content

CTRL_F7 Drop to DOS Shell                           69

ALT_F1 Turn PacketCluster On/Off                    69

ALT_F2 PacketCluster Spot Browse                    69,70

ALT_F4 PacketCluster Spot Announcement              70

ALT_F5 PacketCluster Talk                           70,71

ALT_F6 PacketCluster Alert Setup                    71

ALT_F7 PacketCluster Special Spot Alerting          71,72

ALT_F8 PacketCluster Alarm On/Off (default OFF)     72

ALT_F9 PacketCluster Silence Alarm                  72

ALT_F10 PacketCluster Commands                      72

ALT_A Display SunRise/SunSet for Cities             72

ALT_B Scroll Buffer                                 72

ALT_C Clear Screen                                  73

ALT_D DXCC FOUR Others                              73

ALT_F Data File Capture                             73,74

ALT_H Help!                                         74

ALT_J Jump to Spot                                  74

ALT_K KeyMenu - Programmable Keys (F1/F10)          74,75

ALT_L Logging Module                                75

ALT_M Temperature/Mileage                           75

ALT_O Options/Parameters                            75,76

ALT_P Printing                                      76,77

ALT_R Return From Spot                              77

ALT_S Send File                                     77

ALT_T HamBase                                       77

ALT_U SAM                                           77

ALT_V BuckMaster                                    77

ALT_W QRZ                                           77

ALT_X Exit to Log                                   77,78

ALT_Z WAZ Quick Zone Display                        78

SHFT_F1 Browse Worked All States Databases          78

SHFT_F2 Browse Worked All Russian Oblasts Databases 78

SHFT_F3 Browse Islands On The Air (IOTA) Databases  78

SHFT_F4 Browse Worked All Zones (WAZ) Databases     78

SHFT_F5 Browse County Award Database                78

SHFT_F6 Browse Worked All Prefixes (WPX) Databases  78

SHFT_F7 Browse VHF/UHF Century Club (VUCC) Database 78

SHFT_F8 Browse DX Century Club (DXCC) Database      78

SHFT_F9 Browse Logbook Database                     78,79

SHFT_F10 Browse QSL Manager Database                79

N> Program Parameter Defaults                             79

Date Format (Program)                                  79

Date Format (Labels)                                   79

GMT OffSet                                             79

Latitude                                               79

Longitude                                              80

Update Sunrise/Sunset at 0000z                         80

Grayline Width (1-45)                                  80

DXAlert! at program load                               80

-vi-

LOGPlus! version 3.0 - Table of Content

HP Laser Support                                       80

Default Log Freq (14.023)                              80

Default Log Mode (CW, SSB, RTY)                        80

Default Power Setting (1KW)                            80

Default Orbitor (FO-12, AO-13)                         81

Default Satellite SM (J, K, KT)                        81

Call sign of 1st/4th DXCC Databases                    81,82

LOGPlus! Drive/Directory                               82

HamBase/SAM/BuckMaster/QRZ/External Program 1/2/3      82,83

CT by K1EA                                             83

O> Database Backup (ZIP/ARC)                              83,84

P> Global Database Indexing                               84

Appendix

A  DXCC Listing                                          A1-A8

B  County Hunters Listing   (FILE: COUNTY.DOC)       (EXTERNAL)

C  Report Generator         (FILE: REPORTER.DOC)     (EXTERNAL)

D  Order Form                                            D1

-vii-

LOGPlus! version 3.0

COPYRIGHT NOTICE

=-=-=-=-=-=-=-=-

LOGPlus! program(s), data files, diskette(s) and documentation are

copyrighted (c) 1988-1994 as an unpublished proprietary work of

Robert A. Winters, KD7P and DMW Systems.  No duplication whatsoever

of this program(s), data files, diskette(s) or documentation is

authorized without the expressed written consent of Robert A. Winters,

KD7P and DMW Systems.  At no time will you reproduce, in any manner,

any portion of this package for use by others or resale.  Licenses

can not be transferred.  You may make (encouraged) backups of the

LOGPlus! distribution diskette(s) for your private use only.  You

do not own any part of the LOGPlus! system (program(s), data files,

diskette(s) or documentation), ownership remains with Robert A.

Winters, KD7P and DMW Systems.  The program license granted herein

is licensed to only that individual whose name and address along with

Amateur Radio Callsign is embedded in this program.  You are licensed

to use the LOGPlus! package on a single computer only.

DISCLAIMER

=-=-=-=-=-

Considerable care has been taken to insure the LOGPlus! program conforms

to the specifications herein.  I disclaim all warranties, expressed or

implied, including but not limited to implied warranties with respect to

defects in the diskette(s) and documentation.  Under no circumstances

shall I be liable for any loss or damage to media or hardware.

ACKNOWLEDGMENTS

=-=-=-=-=-=-=-=

I'd like to take this opportunity to acknowledge a group of ham's who have

taken time to BETA test LOGPlus!.  Their time and effort has made this the

most successful Amateur Radio program on the market today.

James Reisert/AD1C

Don Beeson/KY7X

A very special thank you must go out to Kaj/LA5XGA.  Over the past

couple of years, Kaj has been a driving force in Europe, assisting

fellow hams during difficult times.  A friend we should never forget.

dBase III+ is a registered trademark of Borland International Inc.

Clipper is a registered trademark of Computer Associates

MS-DOS is a registered trademark of Microsoft Inc.

O/S2 is a registered trademark of International Business Machines (IBM)

WB2DND is a registered trademark of WB2DND

G3ZCZ is a registered trademark of G3ZCZ

2nd Op is a registered trademark of N6RJ

CT is a registered trademark of K1EA

HamBase is a registered trademark of J-Com

SAM is a registered trademark of RT Systems

-1-

LOGPlus! version 3.0

If I failed to mention a particular product or company, it was not

intentional.  All products mentioned in this documentation are protected

either by copyright or programming ethics.

INTRODUCTION

=-=-=-=-=-=-

LOGPlus! version 3 is possibly the most comprehensive Amateur Radio

program on the market today.

LOGPlus! provides the greatest flexibility of any other Amateur Radio

program on the market.  Far exceeding programs like 2ndOp, DXLog, DXBase,

SWISSLOG and others, LOGPlus! makes every effort to incorporate the

latest information available.  This version of LOGPlus! represents major

revisions and improvements.  Many new and powerful features have been

added to assist you in your everyday Amateur Radio needs.

HARDWARE/SOFTWARE REQUIREMENTS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

IBM PC AT/PS2 or true IBM compatible computer

Hard drive

MS/PS-DOS 3.3 or higher

640K Memory capability

EPSON compatible printer (80 column)

NOTE: A 386 or higher system is highly recommended.

MODIFICATIONS TO YOUR CONFIG.SYS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Your computer must have sufficient resources to run.  Please

modify your CONFIG.SYS file to conform with these settings:

BUFFERS=30

FILES=50

MULTI-TASKING SOFTWARE

=-=-=-=-=-=-=-=-=-=-=-

LOGPlus! v3 has been tested in both Windows (Windows for WorkGroups)

and O/S2 environments.  An ICON file (LOGPLUS.ICO) and PIF file

(LOGPLUS.PIF) are provided for Windows users.

Programs such as Desqview have been tested to some extent, and

work well.  NEVER, load more than one copy of LOGPlus! when using

multi-tasking software such as Windows. You may end up with some very

strange results.  Even though some users have reported excellent

results running two copies of LOGPlus! ( 1 on Dumb Terminal and 1

for logging ), this practice is not recommended.

DO'S AND DO'NTS

=-=-=-=-=-=-=-=

-2-

LOGPlus! version 3.0

DO  Make copies of your distribution disk(s) and store them away in

a safe place

DO  Read the manual cover to cover before running the program.  99.9%

of all support calls or letters received, relate to something

specified in the manual.  BUFFERS and FILES settings in the

CONFIG.SYS file are the major problem many individuals face.

DO  Play with all the features of the program before loading your

final log.  Kick the tires to make sure you're knowledgeable in

what the program will do.  Once you've done this, delete everything

from the \PLUSV3 directory and re-install.

DO  Make periodic backups of your LOGBOOK and DXCC database.  It's

terrible to see hours or possibly days of hard work go down the

tubes.  I can not stress this point enough.  It would be horrible

to lose hundreds of hours of work!  The built in backup feature

provided with LOGPlus! is easy to use and painless!

DO  Exit the LOGPlus! program in an orderly fashion.

DO  PASS THE WORD!!!!

DO NOT   Distribute this program to other individuals.  This only

discourages me from writing additional updates to the program.

DO NOT   Run TSR (Memory Resident Programs) while using the LOGPlus!

program.   Even though VMM (Virtual Memory Management) is

used (see definitions), TSR's cause havoc to memory addresses

this program may need to use.

DO NOT   EVER turn off or re-boot your computer while LOGPlus! is

running.  Data will be lost or files may become corrupted.

DO NOT   Rename any file associated with the LOGPlus! program.  Naming

conventions are extremely important to the overall performance.

UPGRADE/UPDATE POLICY

=-=-=-=-=-=-=-=-=-=-=

LOGPlus! is a continually evolving program with modifications added to

improve the performance and to keep up to date with changes in the

Amateur Radio World!  The current version, which will be displayed at the

main menu, is extremely important for update purposes.  A bulletin

board system has been established to allow users the opportunity to

download updates at anytime.  If you're unable to utilize the bulletin

board system, upgrades can be obtained by sending $4.00 USD.  Remember,

these are MINOR upgrades to the program not MAJOR releases.

TECHNICAL SUPPORT

-3-

LOGPlus! version 3.0

=-=-=-=-=-=-=-=-=

I strongly feel you, the user, deserves the best technical support

possible.  However, if you fail to read this manual cover to cover, you'll

be wasting your money.  If a problem arises which is not documented, or

you just want to ask questions relating to possible improvements or

changes, you can:

1.  Write me a letter.  When asking questions via mail, it's best

to return the original disk(s) along with a copy of your CONFIG.SYS

and AUTOEXEC.BAT files.  This allows me to fully understand the

problems you may be having.

YOU MUST PROVIDE SUFFICIENT POSTAGE TO COVER THE SAFE RETURN OF

YOUR DISKETTE(S).  NO ACTION WILL BE TAKEN IF POSTAGE IS NOT

RECEIVED.

2.  Call the BBS and leave a message for the SysOp.  Responses will

be available usually within 72 hours.

BULLETIN BOARD SYSTEM

=-=-=-=-=-=-=-=-=-=-=

In an effort to provide the latest information possible and to answer

any questions, I have establishing a 24 HOUR BBS system.  The major

purpose of the BBS will be to assist with LOGPlus!.  Free minor upgrades

will be available to all registered users at no cost.  In addition, if

you have a particular print out, customizing you'd like to see, whatever

I can possibly do, I will provide them only on the BBS.  The following

files will ALWAYS be available for downloading:

DXCC.DBF

PRFX.DBF

PREFIXES.DBF

QMGR.DBF

QLABELS.DBF

If you maintain an extensive QSL MANAGER SYSTEM using LOGPlus!, please

share the information with others.  Updated QMGR.DBF and QLABELS.DBF

files will be provided.  Rememeber, you must send both QMGR.DBF and

QLABELS.DBF if you wish the information incorporated with other

information.

CONFIGURATION

=-=-=-=-=-=-=

Baud rates:  300/1200/2400/9600/14400          Data Bit:  8

Parity:  None                                  Stop:  1

USER ACCESS

=-=-=-=-=-=

Everyone will have access to the system.  DEMO programs as well

-4-

LOGPlus! version 3.0

as other Amateur Radio programs will be available for downloading.

Every registered user will be automatically upgraded to V3USER

status after the first login.  Please, don't get in the habit of

using your call sign as your password.  It's not difficult to

determine your password if you do!  Make sure you leave a note

for the SysOp if you wish your status updated.

I do not like ALIASES, "THE HACKER", "THE DXER" etc.. I think you

understand.  Those individuals who insist on using such names will

be logged out and never allowed in.

COMMENTS TO ALL USERS

=-=-=-=-=-=-=-=-=-=-=

Feel free to use the system as a mailbox to other users.  You'd be

surprised at the number of people who have shortcuts which may help

you out.

RECEIVING AN UPGRADE

=-=-=-=-=-=-=-=-=-=-

Each time you log on to the system, the program will tell you what

the latest version is along with the changes made.  If you want to

receive a copy of the latest update, you MUST request a copy.  Once

this is done, you'll be able to log back in within 72 HOURS and

download the update.  This is for MINOR UPDATES ONLY!

CONVERSION PROGRAMS

=-=-=-=-=-=-=-=-=-=

Conversion programs for many of the logging programs available

today can be provided upon request.  If I do not currently have

a conversion, one can usually be provided; however, I will need to

see the program and ALL data files you wish converted.  If you send

a copyrighted program, I will use it only to convert your data and

destroy it afterward.  No other users will have access to it.  I am

a strong believer in the rights of the programmer.  Unfortunately,

its impossible to write a conversion program if I don't have all

the tools.  To keep the cost of LOGPlus! down, I can't afford to

purchase every logging program out there.  Here is a basic list

of program currently available.  Other can/may be available upon

request:

DXLOG, DXBASE, 2NDOP, TURBOLOG, TOPLOG, HYPERLOG, LOGMASTER,

G3ZCZ, WB2DND, EASYDX, SWISSLOG, LOGIC3, GEMRADIO, EQF-LOG,

WJ2O

FUNCTION KEYS

=-=-=-=-=-=-=

Function keys are used throughout the program to activate various pop

up windows and programs.

-5-

LOGPlus! version 3.0

ALT-F1      Hold down both the ALT key and F1 key at the same time.

CTRL-F1     Hold down both the CTRL key and F1 key at the same time.

SHFT-F1     Hold down both the SHIFT key and F1 key at the same time.

SPECIAL EDIT KEYS

=-=-=-=-=-=-=-=-=

Special edit keys have been programmed to closely follow the WordStar

control sequence for navigation and entry.  These edit keys can usually

be used when entering data in most fields.  It's best to "PLAY AROUND"

to become familiar with each.

Edit Key                           Action

==-=-=-=                           =-=-=-

Left Arrow or CTRL-S               Move one character to the left

Right Arrow or CTRL-D              Move one character to the right

CTRL-Left Arrow or CTRL-A          Move one word to the left

CTRL-Right Arrow or CTRL-F         Move one word to the right

Up Arrow or CTRL-E                 Move to previous field

Down Arrow, CTRL-X, Enter, CTRL-M  Move to next field

Home                               Beginning of field

End                                End of field

CTRL-Home                          Beginning of first field

CTRL-End                           End of field

Del or CTRL-G                      Delete character at cursor position

Backspace or CTRL-H                Delete character to left of cursor

CTRL-T                             Delete to end of word

CTRL-Y                             Delete to end of field

CTRL-U                             Restore field to former value

NOTE: This will only function if you press CTRL-U before exiting

the next field

Ins or CTRL-V                      Turn insert On/Off

CTRL-W, CTRL-C, PgUp or PgDn       Save all fields and terminate read

ESC                                Terminate read, don't save field

NOTE:  The <ESC> key is used in most modules throughout this program

to exit and return to a menu.  This key is also used to terminate

the program.

LOGGING INTO LOGPLUS!

=-=-=-=-=-=-=-=-=-=-=

LOGPlus! is a protected program which requires you to use your CallSign

when logging in.  To activate LOGPlus! from DOS:

LOG KD7P       >>  Execute LOGPlus! with COLOR

LOG KD7P /N    >>  Execute LOGPlus! in monochrome

LOG KD7P -R    >>  Execute LOGPlus! without Statistics Display

Remember, use your call sign not mine!

-6-

LOGPlus! version 3.0

If you enter the wrong CallSign, the program will politely tell you the

wrong call was entered.

MULTIPLE CALLSIGNS

=-=-=-=-=-=-=-=-=-

After playing around with various other logging programs, I found one

major deficiency which restricted DX stations from using one program

to maintain numerous logs.  When LOGPlus! was written, one of the major

objectives was to provide a quick and effective way of maintaining more

than one log, and having the ability to shift between them quickly.

LOGPlus! v3 has the ability to maintain 20 different logbooks.

CREATING AND SHIFTING BETWEEN LOGS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The first thing we must do is tell LOGPlus! how many logs we'll be

tracking.  To create or shift between logs you must be at the main

menu.  By pressing the F6 key, a pop up window will display on the

right hand side of the screen.  You'll notice that menu A> has

already been assigned the call you ordered the program under.  This

can not be changed.  To add a new log, move the highlight bar down

to the next available line which says "No CallSign" and press the

ENTER key.  Enter the CallSign you wish to track then press ENTER

again.  Continue this process for each CallSign.  Once you're done,

exit to the main menu to insure the information is saved.  A file

called CALLS.MEM will be located in the \PLUSV3\ directory.  You

will also notice that each of the sub-directories required was

automatically created.  All that will be required at this point is

to placing the necessary files into those sub-directories.  From this

point on each time you press F6 from the main menu, you'll notice

the calls you entered.

DIRECTORY STRUCTURE

=-=-=-=-=-=-=-=-=-=-

When working with multiple logs it's necessary to make sure the data

is separated to reduce any conflicts.  Sub-directories are the method

we use to keep this separation.  The major directory will always be

called \PLUSV3\ and contain the log files for your master log (the

call you ordered the program under) and all the MAJOR files which are

not required in the sub-directories.  Here is an example of how my

logging system and sub-directories would look.  Except for the first

call, the order is strictly up to you.

\PLUSV3\         >>    KD7P

\PLUSV3\LOG1\    >>    KD7P/NH2

\PLUSV3\LOG2\    >>    KD7P/NH4

\PLUSV3\LOG3\    >>    KD7P/NH7

\PLUSV3\LOG4\    >>    KD7P/KC4

\PLUSV3\LOG5\    >>    KD7P/OA4

-7-

LOGPlus! version 3.0

\PLUSV3\LOG6\    >>    KD7P/LU2

REQUIRED DATABASE FILES IN EACH SUB-DIRECTORY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The following files MUST be located in each of the sub-directories

created.  If the program can not find the necessary files, it will

make an effort to use the \PLUSV3\ files.  This may cause problems

with other logs, especially the master log.  Please use caution!!!

A batch file called MULTI.BAT has been provide to make this task

easier, please read the README.1ST file for instructions on it's

use.

BANDCNT.DBF          BUREAU.DBF        COUNTY.DBF

DIRECT.DBF           DX.DBF            DXCC.DBF

GENERIC.DBF          LOGBOOK.DBF       LOGTEMP.DBF

OBLAST.DBF           OTHER.DBF         SETUP.DBF

SWL.DBF              PENDING.DBF       WAS.DBF

SATELITE.DBF         WAZ.DBF           IOTA.DBF

TEMPQSL.DBF          IOTANF.DBF

GETTING AROUND THE BROWSE/MODIFY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The Browse/Modify modules in LOGPlus! v3 are powerful features which

offer a birds-eye view of your data, extreme flexibility and easy of

operation.  Every effort has been made to standardize the keys

(arrow, function etc..), to reduce the learning curve.  This section

will describe these standards to reduce duplication.  If a particular

module uses different function keys to accomplish a task, information

will be provided under the Browse/Modify section for that particular

module describing the changes.

Arrow Keys

=-=-=-=-=-

NOTE:  If the record pointer is at the top or bottom of the

databases and the corresponding key is pressed in an attempt

to go further, a BEEP will sound, alerting you this can not

be done.

Up Arrow         -  Move record pointer up one record.

Down Arrow       -  Move record pointer down one record.

PgUp             -  Move record pointer up one full page.

The actual number of records moved

dependings on the module.

PgDN             -  Move record pointer down one full page.

The actual number of records moved

dependings on the module.

-8-

LOGPlus! version 3.0

CTRL+PgUp        -  Move record pointer to top of file.

CTRL+PgDn        -  Move record pointer to bottom of file.

ESC              -  Exit module and return to previous menu.

ENTER            -  Editor function.  This allows you to quickly

modify data for the record currently

highlighted.

THE MAIN SCREEN

=-=-=-=-=-=-=-=

With LOGPlus! v3, a new MAIN SCREEN has been introduced to provide an

wealth of information.  Broken down into two windows, the information is

easily read.

FUNCTION KEYS ACTIVE AT MAIN SCREEN

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

ALT_H    =  Help!

ALT_R    =  Refresh Statistics Window

ALT_T    =  Run HamBase

ALT_U    =  Run Sam

ALT_V    =  Run BuckMaster

ALT_W    =  Run QRZ

F6       =  Select different logbook

CTRL-F7  =  Drop to DOS Shell

CTRL-F8  =  Run External Program

CTRL-F9  =  Run External Program

CTRL-F10 =  Run External Program

SHFT-TAB =  Screen blanking >> can be used throughout the program.

User programmable function keys provide the flexibility to

run nearly ANY program of your choicing.  Simple by pressing

the SHIFT and corresponding F key, you'll quickly be teleported

to the program of of choicing.  Once you exit the external program,

you'll be placed directly back to the point you left off!

Programming these keys is simple and will be explained in the

Program Parameter Defaults (N) section later in this manual.

STATISTICS WINDOW (TOP WINDOW)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The Statistics window provides a wealth of information about

your logbook the moment you log into LOGPlus!.  At a glance,

you'll know the exact breakdown of your logbook as well as

your DXCC statistics.

LOG:     6303   SAT:       6                DXCC Countries:   385

CW:     4821    CW:       4              ACTIVE Countries:   329

SSB:     1479   SSB:       2             DELETED Countries:    56

-9-

LOGPlus! version 3.0

RTY:        0   RTY:       0        ARRL    CFMD    WRKD    NEED

FM:        3    FM:       0  CW:   0(  0)  0(  0)  2(  2)  383(327)

AM:        0    AM:       0 SSB:   0(  0)  0(  0)  0(  0)  385(329)

OTHER:      0   OTHER:     0 RTY:   0(  0)  0(  0)  0(  0)  385(329)

DELETED:   73   DELETED:   0 MIX:   0(  0)  0(  0)  2(  2)  383(327)

Most of the fields are self-explainatory.

OTHER:    Those modes not covered by those listed.

DELETED:  Those QSOs in your log associated with a deleted DXCC

Country.

383(327)  First number represents DXCC Countries including both

DELETED and ACTIVE.

Second number represents only ACTIVE DXCC Countries

NOTE:  On slower machines this window can take up to 30/60

seconds to update.  Because of this, this screen is refreshed

only during the initial start up or by pressing the ALT_R

key combination.

MENU WINDOW (BOTTOM WINDOW)

=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Menu window contains all the different menu selections offered

within LOGPlus!.  Each different menu selection will be explained

below.

A>  Logbook Management

=-=-=-=-=-=-=-=-=-=-=-

The following discreption describes menu options A>, B>, C> and

D>. The power and flexibility of LOGPlus! is no doubt centered around

the Real-Time and Off Air logging modules.  Both of these modules

provide at your finger tips, information which can be brought up

quickly and viewed. Function keys have been programmed to provide

you with flexibility and ease of operation.  The information described

below will be general in nature and cover the Real-Time, Off Air and

most of the general features of the Satellite logging modules.  The

major difference between standard logging and Satellite logging is

the data field sizes and location.  All other features remain the same.

WORKING WITH DELETED COUNTRIES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Before we start to explain the power of this module, we must first

spell out how the program manipulates DXCC data.  Unfortunately our

world is changing on a daily basis.  Countries seem to be popping up

all over or being absorbed into existing one.  Appendix A contains a

complete listing of countries which are active and those deleted.

HOW DELETED COUNTRIES ARE DETERMINED

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

We must have a way in which LOGPlus! can easily determine the current

-10-

LOGPlus! version 3.0

status of a contact.  Working with deleted countries is not easy!  We

can really screw up the whole program by not following the standard

guidelines spelled out in this manual.  The DXCC section contains a

complete breakdown on how to enter deleted countries and assign

prefixes to those countries. Please read this section carefully

before making any changes.

EXAMPLE

=-=-=-=

On June 30, 1962 at 23:50GMT you work 9U5AA on 15m.  When LOGPlus!

conducts a DXCC search, the country Ruanda-Urundi was credited with

the contact.  A short while later on July 01, 1962 at 00:53GMT you

work 9U5AA again but this time on 20m.  When LOGPlus! conducted a

DXCC search this time, the country Burundi was credited.  WHY?

The reason is simple, at 00:01 on July 01, 1962 the country of Ruanda-

Urundi ceased to exist as a DXCC country.  Independence was granted

with self-determination and rule.  LOGPlus! knew this because of the

information in the DXCC.DBF and PRFX.DBF files to automatically credit

the newly independent country.  Remember, this may not work with every

deleted country.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

Three database files (DXCC.DBF, PRFX.DBF and PREFIXES.DBF) are

critical and must be maintained to provide up to date information.

Obviously some users may feel uncomfortable with making these changes.

If a country is added or deleted from the DXCC Countries List,

you can do one of three things to update these files:

1.  Do it yourself!

2.  Call the BBS system which will be established by the end

of the year.  All registered users will receive the telephone

number once established.  You can download a new copy of each.

3.  Send a request along with sufficient postage ( $4.00 USD)

for new copies.  I'll forward them immediately.

REAL-TIME AND OFF AIR LOGGING SCREEN

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The screen is broken down into four sections.  Each are critical to

the performance of LOGPlus!

QSO DISPLAY WINDOW

=-=-=-=-=-=-=-=-=-

The QSO Display Window is the upper screen area used to display the

last 11 QSOs, previous QSOs and the PacketCluster data.  During

standard Real-Time and Off Air logging, the QSO Display Window will

-11-

LOGPlus! version 3.0

continuously display the last 11 QSOs in a basic logbook format. When

a call is entered, the log is checked for any previous QSOs.  If

any are found, the QSO Display Window will clear and display only

those QSOs which meet the call criteria. If the call is not found,

you will see something like:

ZA1AA not found in log...

After the log has been searched, the program will extract the DXCC

information from the call.  Once the prefix has been extracted the

DXCC database is checked to determine if the country is a new one.

If the DXCC database reports to the program that Albania is an ALL

TIME NEW COUNTRY, the screen will display:

Albania is an ALL TIME new DXCC Country!

However, if Albania was previously worked on 15m and you've now

logged him on 20m, the program will determine the DXCC band and mode

status and inform you.  If Albania turns out to be a new band

country, the screen will display:

Albania is a NEW CW DXCC Country!

Once the QSO has been saved to the log, the screen will restore the

last 11 QSOs on the screen and await your next entry.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

When working with the PacketCluster monitoring ON, the last 11 QSOs

are NOT displayed automatically, to display this information you

must press the F7 key.  The QSO Display Window is used to monitor

the PacketCluster system.  See the Dumb Terminal Program for more

information on how the PacketCluster monitor works.

DATA ENTRY WINDOW

=-=-=-=-=-=-=-=-=

The Data Entry Window provides the means of logging contacts.  To

change from field to field, use either the TAB or ENTER keys. LOGPlus!

has been written to accept DEFAULTS whenever possible.  When the

program first loads, the cursor will be located in the FREQ field.

In some cases, the ENTER key is not required to move from field to

field.  If you attempt to exceed the maximum number of characters

allowed, the cursor will automatically jump to the next field.  To

move back to a previously entered field, use the arrow keys.

If you're in Off Air logging one additional field can be modified,

that is the QSL field.  This field is usually updated only if you're

entering data into your log from the past year.  If you're updating a

current QSO, use the QSL Card Updating to automatically update this

-12-

LOGPlus! version 3.0

field.

GENERAL INFORMATION WINDOW

=-=-=-=-=-=-=-=-=-=-=-=-=-

The General Information Window provides critical information on the

QSO currently being entered.  Data in this block can be modified by

using the F3 General Information Update.

DXCC STATISTICS WINDOW

=-=-=-=-=-=-=-=-=-=-=-

The DXCC Statistics Window provides up to date information on the

current BAND/MODE breakdown of the currently entered DXCC country.

EXAMPLE

=-=-=-=

You enter VK9NS in the CALL field.  When the DXCC database is

checked, data relating to VK9N/Norfolk Island is displayed on the

screen. Each BAND and MODE is displayed in an easy to read format.

"C"   Confirmed.  QSL card has been received.

"W"   Worked.     This country has been worked previously. No QSL!

"A"   Approved.   This country has already received ARRL approval.

SELECTION MENU (BOTTOM OF SCREEN)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Once all the fields of the Data Entry Window and General Information

Window have been updated correctly, it's time to do something with

the data.  At the bottom of the screen (after you've pressed ENTER

and left the NOTE field), the following menu selections are available:

<D>elete:      Clear all fields (except FREQ and MODE) and move

cursor to CALL field.

<E>xit:        Exit the module.  Take NO action on currently

entered data in Data Entry Window.

<F>req:        Allows you to change both FREQ and MODE fields.

<M>odify:      Modify the currently displayed QSO.  Cursor will

move to the CALL field.

<P>rev DX:     Scroll DXCC database.  CAUTION, if you change the

DXCC Country, from that point only, the call sign

will be associated with that DXCC Country.  Use

CAUTION!.

<N>ext DX:     Scroll DXCC database.  CAUTION, if you change the

DXCC Country, from that point only, the call sign

-13-

LOGPlus! version 3.0

will be associated with that DXCC Country.  Use

CAUTION!.

<Q>SL:         Save entry to selected QSL database.

<S>ave:        Save QSO to LOGTEMP database and move cursor to

CALL field.

FUNCTION/CONTROL KEYS

=-=-=-=-=-=-=-=-=-=-=

SHIFT-F1 through SHIFT-F10 will allow you to have access to various

database files.  For information on using these features you'll need

to read the section concerning the Browse/Modify feature for those

particular awards.

SHIFT-F1  Browse WAS Database  (See F> Worked All States Award (WAS))

SHIFT-F2  Browse Oblast Database (See J> Worked All Russian Oblasts)

SHIFT-F3  Browse IOTA Database (See I> Islands On The Air (IOTA))

SHIFT-F4  Browse WAZ Database (See E> Worked All Zones Award (WAZ))

SHIFT-F5  Browse County Database (See K> Worked All County Award)

SHIFT-F6  Browse WPX Databases (See G> Worked All Prefixes Award (WPX))

SHIFT-F7  WPX Prefix LookUp (See G> Worked All Prefixes Award (WPX))

SHIFT-F8  Browse DXCC Database (See D> DX Century Club Award)

SHIFT-F9  Browse VUCC Databases (See H> VHF/UHF Century Club (VUCC))

SHIFT-F10 Browse QSL Manager Database (See C> QSL Cards & QSL Managers)

SHIFT-TAB Screen blanking

F2 Quick Call Saver

=-=-=-=-=-=-=-=-=-=

The Quick Call Saver is an excellent tool for DXpeditions or those

who are working the pile ups.  Just enter the call and press F2.

The QSO will be saved to the logbook and your DXCC database will be

automatically updated.

F3 General Information Update

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

General Information is that data which is displayed in the lower

left hand corner of the screen.  This is usually used when updating

IOTA, STATE, GRID and other minor information which you'd like to

maintain for later referencing.

F4 Quick Call Checker

=-=-=-=-=-=-=-=-=-=-=

The Quick Call Checker allows you to quickly check your logbook for

previous contacts.  Let's say you're scanning the band and hear VK9WW

working the pile up.  You can't remember if you've ever worked VK9WW

or VK9W as a new country.  By entering the call VK9WW in the call

field and pressing F4, LOGPlus! will check your logbook for any

previous QSOs, search your DXCC database for VK9W and update the

screen accordingly.  This is the quick way to check on previously

-14-

LOGPlus! version 3.0

worked QSOs/DXCC.  To delete unwanted data which is in the CALL field,

press BACKSPACE or CTRL-BACKSPACE.

F5 Browse LOGTEMP File

=-=-=-=-=-=-=-=-=-=-=-

Browsing the LOGTEMP file will allow you to make modifications and

delete unwanted QSOs.  This feature take the place of 3 previous

function keys.

F6 Partial Call Search

=-=-=-=-=-=-=-=-=-=-=-

Search database and display ALL call signs which contain entered

data.  Enter any call or partial call in the call sign field then

press F6.  All calls which contain a partial match will be displayed.

F7 Display Previous QSOs (w/PacketCluster only)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display previous QSOs when PacketCluster is in use.

ALT_A Sunrise/Set Browse

=-=-=-=-=-=-=-=-=-=-=-=-

Display Sunrise and Sunset browse window.  This feature allows

you to display countries which fall with either your grayline.

You can select all, sunrise only or sunset only.

ALT_B Buffer Scroll (w/PacketCluster only)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Pop up windows allowing to scroll up to 6,000 characters of

incoming Dumb Terminal program.

ALT_H Help!

=-=-=-=-=-=

Quick look at programmed function keys.

ALT_J Jump to Spot

=-=-=-=-=-=-=-=-=-

Jump to Spot works in conjunction with the PacketCluster interface

and the radio interface.  When a DX Spot is reported on the Packet-

Cluster, the frequency is stored in a variable until used.  If you

decide you'd like to QSY the radio to that frequency to work the

DX, press ALT_J and your radio will shift.  Note the mode will not

change.

ALT_K Radio On/Off

=-=-=-=-=-=-=-=-=-

This is a toggle switch which will turn the radio's interface on or

off.  Each time this is pressed, the frequency and mode are updated.

Please note that each time the radio functions are changed ( freq,

mode or vfo ), it is automatically updated on the screen. You don't

-15-

LOGPlus! version 3.0

need to use this toggle.  Use it only to disable or enable the

interface.

ALT_M Temperature/Mileage

=-=-=-=-=-=-=-=-=-=-=-=-=

Temperature/Mileage conversion is a quick display which will allow

you to either either fahrenhiet or celcius and obtain the conversion.

Mileage, either in miles or kilometers can also be obtained.

ALT_N CallSign Notebook

=-=-=-=-=-=-=-=-=-=-=-=

The CallSign Notebook provides an excellent location to keep

extensive notes on the call entered.  Once the call is entered in

the CALL field, the program will automatically search the \NOTES

directory for any .LP (note) which matches the call.  If found, you

will see <<< Note found!! flashing on the screen.  You can bring

up that note by pressing ALT_N. I have used this field for copying

CW exchanges and keeping general comments on a particular station.

You can delete any entry made by using the F2=Delete data key.

ALT_Q QRP Logging

=-=-=-=-=-=-=-=-=

The QRP Logging feature is an easy way to keep track of QSOs you work

when operating QRP.  When turned ON, the upper left portion of the

screen will display <QRP Logging ON!>.  When this is displayed, all

log entries will have the QRP field updated.  This information is

critical to the DXCC updating.  Note, entries made will highlight

in GREEN in the display screen.

ALT_R Jump Back (Return) From Spot

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Return radio to previous frequency before ALT_J Jump to Spot.

ALT_Y YL Logging

=-=-=-=-=-=-=-=-

The YL Logging feature is an easy way to keep track of QSOs with

YL stations.  When turned ON, the upper left portion of the

screen will display <YL Logging ON!>.  When this is displayed, all

log entries will have the YL field updated.  This information is

critical to the DXCC updating.  Note, entries made will highlight

in YELLOW in the display screen.

ALT_Z WAZ Quick Display

=-=-=-=-=-=-=-=-=-=-=-=

See WAZ Section for description.

PacketCluster ALT Keys

=-=-=-=-=-=-=-=-=-=-=-

The keys listed below are explained in detail under the Dumb

-16-

LOGPlus! version 3.0

Terminal Program (DTP), PacketCluster section.

ALT_F2 PacketCluster Alert Browse Routine

ALT_F4 PacketCluster Spot Announcement

ALT_F5 PacketCluster Talk

ALT_F6 PacketCluster Alert Setup

ALT_F7 PacketCluster Special Spot Alerting

ALT_F8 PacketCluster Alarm On/Off (default OFF)

ALT_F9 PacketCluster Silence Alarm (temporary, until next spot)

ALT_F10 PacketCluster Commands

CTRL_F2 Change Radio Frequency

CTRL_F3 Change Radio Mode

CTRL_F4 Change Radio VFO ( Kenwood )

CTRL_F5 Up Radio Freq (10HZ)

CTRL_F6 Dn Radio Freq (10HZ)

CTRL_F7 Drop to DOS Shell

E>  Browse/Modify

=-=-=-=-=-=-=-=-=

See Getting Around The Browse/Modify for general information.

The following functions are provided in this module:

F2=Delete Entry   -  Permanently remove entry from logbook.

F3=Call Search    -  Search for entered call sign.  If found

screen will update and display information.

Database will remain in call sign order.

F4=Date Search    -  Search for entered date.  If found screen

will update and display information.

Database will remain in date order.

F5=DXCC Search    -  Search for DXCC prefix entered.  If found

will update and display information.

Database will remain in DXCC order.

F6=Append         -  Add new record to logbook.

F10=QSL Entry     -  Save QSO to QSL database (selectable)

Enter=Editor      -  Edit highlighted QSOs.  All standard fields

F> Search Routines

=-=-=-=-=-=-=-=-=-

Search routines have been provided on most of the important data

fields.  Select the desired search field and enter the data you

wish to find.

The upper left hand corner of the screen will depict which search

-17-

LOGPlus! version 3.0

method is currently being employed.  (i.e.  CALL, DATE, MODE etc...).

[ CALL - Search Routine ]

Search for: [ _____________ ]

Double line space print out (y/n): [ N ]

(P)rinter, (S)creen:  [ S ]

Enter starting Page Number:   [ 1 ]

Search for:  Enter the data you want to find.

Double line space print out (y/n):  Default NO.

Many users have asked for this feature.

(P)rinter, (S)creen:  Default SCREEN.

Enter starting Page Number:   Default 1.

G>  Print Alphanumeric Logbook

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Print a complete log for every contact within the logbook.

(F)ile or (P)rinter:    Default PRINTER.

Double line space print out (y/n):   Default NO.

Enter starting Page Number:  Default 1.

1) 8x11 Standard, 2) 8x11 Condensed, 3) 11x14 Standard:  Default 1.

8x11 Standard: Standard printer paper.  Not all fields are printed.

8x11 Condensed: Standard printer paper. ALL fields are printed.

11x14 Standard: Standard printer paper. ALL fields are printed.

H>  Print Chronological Logbook

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Chronological prints a partial log from date to date.

Starting date:    Default today's date.  Enter the date you wish

to start the print out at. If that date is not

found, the next closest date will be the starting

point.

Ending date:      Default today's date.  Enter the date you wish to

end the print out at.  If that date is not found,

-18-

LOGPlus! version 3.0

the next closest dates will be the ending point.

I>  Band & Mode Statistics

=-=-=-=-=-=-=-=-=-=-=-=-=-

The Band & Mode Statistics provides a complete band and mode break-

down with an accurate percentage and count.  This module can be used

to judge your activity for years!  Each time the module is updated,

information is saved to the BANDCNT file for future reference.  If

you wish to display information which you ran 3 years ago, press the

ENTER key until the information is displayed.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

CW and SSB totals will provide an accurate accounting of the number

of contacts worked.  The RTY combines RTY, PKT, ASC, FEC, GTR, PTR and

AMT contacts into one.  The total number of contacts listed on the

bottom right may not reflect the exact number of contacts if your

logbook uses anything other than the modes mentioned above.

J>  Update QSL Manager in Logbook

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

This is a quick and easy way to automatically update the QSL

Manager field in the logbook.  Information is extracted from the

QSL Manager databases and updated.

K>  Contest Conversions (K1EA/N6TR/WF1B)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

A>  CT by K1EA - Version 8 and 9

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

VERY IMPORTANT!: You must specify the PATH location for the

.BIN and B2R8.EXE or B2R9.EXE files under the program default

parameters. (i.e. C:\CT\ ).

This is menu driven conversion program which provides ease of

operation. Move the highlight bar down to the specific contest or

press the letter displayed to the left. (i.e. A B C etc..).

Once the contest has been selected, an additional window called

FILES will pop up.  Move the highlight bar down to the .BIN file

you wish to convert, then press <ENTER>.

Next the program will load either the file B2R8.EXE or B2R9.EXE

file into memory and convert the .BIN file specified.

Upon completion, you'll be prompted to enter the contest name.

The contest name is placed in the NAME FIELD of the log.  If you

don't wish this to occur, press <ENTER> without entering any data.

If you'd like to display the contest name enter the year followed

by an abbreviation. (i.e.  92-CQWW, 92-ARRL, 92-WPX etc).

-19-

LOGPlus! version 3.0

NOTE:  When logging, the program will automatically fill in the

name field if the name field was filled in during a previous QSO.

This will NOT occur if the name field starts with a numeric.

When the updating has completed, a browse window will be displayed

showing you the converted data.  If everything looks correct, press

<ESC> to save; otherwise, press SHIFT-F10 to discard.

NOTE:  This program has been extensively tested. If your data does

note come out correctly, it's possible that K1EA has again changed

the B2R conversion program.  Please advise me if this occurs.

NOTE:  As of this writing, v9.x has numerous difficulties and

may not convert properly.  This has been reported to K1EA.  The

problem lies in CT not LOGPlus!.

B>  N6TR v3.x

=-=-=-=-=-=-=

The N6TR to LOGPlus! conversion program is designed to extract

STANDARD logging information (i.e. DATE/TIME/CALL/FREQ/MODE/

SENT/RCVD).  STATE and NAME information will be extracted for

specified contests.

The following is a list of contests which will extract ONLY

standard logging information.  Menu selection "A> Standard

Conversion" MUST be used:

All Asian

CQ WPX

CQ M

Helvetia

IOTA

IARU

JA Intl DX

KCJ

Scandinavian

VK ZL

WAE

Cal QSO Party ( NON-CALIFORNIA STATIONS )

General QSO

HA DX

OK DX

SP DX

YO DX

Region One Field Day

The following contests will convert standard fields as well as the

STATE field. Menu selection " B> Standard & State" MUST be used.

-20-

LOGPlus! version 3.0

ARRL 10

ARRL 160

CQ 160

Cal QSO Party ( CALIFORNIA STATIONS ONLY )

PACC

The following contest will convert standard fields as well as

providing an option of placing the QTH field in the NOTE field.

Menu selection " C> County Hunter " MUST be used.

County Hunter

The following contests will convert standard fields as well as

placing the NAME received in the NAME field and the QTH received

in the STATE field. Menu selection " D> No. American QSO Party"

MUST be used.

No. American QSO Party

C>  WF1B v2.x

=-=-=-=-=-=-=-

As soon as this menu is selected, you'll be dumped directly into

the DOS Shell.  The following steps must be followed exactly!:

1.  Path over to the directory containing your WF1B files.

2.  Type CONVERT <ENTER>

3.  Select Logbook Format (option 6 under v2).

4.  Select the .BIN file you'd like converted.

5.  Press <Q> to Quit

6.  During the conversion a .TXT file was created. You'll need

to copy this file over to the \PLUSV3\ directory and rename

the file WF1B.TXT. (i.e. COPY 92CQWW.TXT \PLUSV2\WF1B.TXT )

7.  Type EXIT <ENTER>  This will return you to LOGPlus!

The program will follow the same steps as the CT by K1EA

program from the contest name on. Please see that section

for additional information.

L>  Logbook Refresh

=-=-=-=-=-=-=-=-=-=

Logbook Refresh was added to assist users when inputting data from

other database sources, (i.e. conversion programs, contest programs,

DBase etc..).  It's major purpose in life is to insure your DXCC

-21-

LOGPlus! version 3.0

field in the logbook contains "NO BLANKS", your band and mode

information conforms to the standards employed in LOGPlus! and

tags all your DELETED QSOs so credit can not be given to those

countries which do not meet the specific criteria for and award.

It is recommend this feature be run once every few months.

M>  Indexing

=-=-=-=-=-=-

Recreate required database index files.  This is only required if

problems are encountered locating data.

B>  Satellite Management

=-=-=-=-=-=-=-=-=-=-=-=-

Only very minor differences distinguish the Real-Time/Off Air logging

modules apart for the Satellite Logging.  Instead of duplicating

effort, only those differences will be mentioned.  It should be noted

that the Satellite and standard log files are separate and can not be

intermingled.  PacketCluster monitoring is NOT available while in the

Satellite logging module.

DATA ENTRY WINDOW

=-=-=-=-=-=-=-=-=

When the Satellite logging modules are loaded, the cursor will always

be in the DATE field first.  Because Satellite DXCC is general in

nature, band and mode DXCC checking is not required.  When entering

data, you'll notice the cursor will always go to the QSL field (Q),

after the STATE field.  Like the standard logging method, it's

recommended that this field only be used when entering OLD data.

DXCC STATISTICS WINDOW

=-=-=-=-=-=-=-=-=-=-=-

Four different fields are displayed here.

DXCC Status:      This will display the current status of the

DXCC Country being displayed.

Worked:  Country worked but no QSL card received.

Confirmed:  QSL card received.

Approved:   ARRL approved credit received.

Worked:           Worked is a combination of Pending and Confirmed.

Pending:          Those counties in which no QSL card has been

received.

Confirmed:        Those countries in which a QSL card has been

received.

-22-

LOGPlus! version 3.0

C>  QSL Card & QSL Managers

=-=-=-=-=-=-=-=-=-=-=-=-=-=

The chore of handling QSL cards can be time consuming when doing things

manually.  Computer logging provides us with a quick and effective

way to get the job done quickly and efficiently without the hassle of

paper logs.  This module is intended to assist you in your QSL chores

as well as provide an interface to the QSL Managers database file.

A>  QSL Card Updating

=-=-=-=-=-=-=-=-=-=-=

The QSL Card Updating is an intricate part of the overall success of

LOGPlus!.  The QSL field in the log must be maintained for the program

to accurately update the DXCC, WAS, WAZ, WPX and IOTA modules. If you

fail to maintain the QSL field, LOGPlus! will never provide accurate

information.

THE THREE QSL DATABASE FILES

=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Three QSL database files have been provided to allow you flexibility

in the way cards are handled.  Some of us send cards to various clubs

or individuals.  If we could provide a method of sorting out those cards

to different database files and print them when we're ready to mail

them off, our shacks would be less crowded.

DIRECT:     This file is used to hold cards which you plan to send

directly to the DX station via U.S. Mail.

BUREAU:     This file is used to hold cards which you plan to send

to the ARRL outgoing bureau for processing.

OTHER:      This file is used to hold cards which are going to a

particular QSL Manager (i.e. WF5E, W3HNK etc. ).

how it works

=-=-=-=-=-=-

We have just finished working VK9NS Jim on Norfolk Island and wish

to send him a card.  Here's how we can save the entry to one of the

database files for printing at a later date.

NOTE: The QSL and QSLDate have been left off of this display

because of space restrictions.

NOTE: All QRP entries will be highted YELLOW on BLACK.

Direct:  22

Bureau: 123

Other:   5

Enter CallSign:  VK9NS

-23-

LOGPlus! version 3.0

Record     Date  Time    Freq  Mde  Snd Rcv  St  Grid  IOTA  Name

-------------------------------------------------------------------

1023  10/10/92 00:23  14.232 SSB  53  59                   Jim

(A)ck, (B)ureau, (D)irect, (O)ther, (R)eset, (S)kip, (Q)uit:

(A)ck:    ACKNOWLEDGE.  This menu selection is only used to

mark the QSL field with an "X" if the QSL field

was previously marked with "B", "D", "O" or "S".

(B)ureau: BUREAU DATABASE SELECTED.

(D)irect: DIRECT DATABASE SELECTED.

(O)ther:  OTHER DATABASE SELECTED.

When one of the three menu selection are chose, you will

be required to answer a few additional questions to make sure

the contact is properly tagged.

Acknowledge QSL as received ('X' in QSL field) (y/n) ?

YES:   Used only if you have received the QSL card from the

DX Station.  For example, when Jim receives your QSL

card, he'll type in your call, select the appropriate

database then 9 times out of 10 answer YES to this

question.  He has received your card and is sending

one back.

NO:   This is the most common one used, at least in our

example.  The QSL field in the log will be affixed

with "B" if you selected BUREAU, "D" if you selected

DIRECT, or "O" if you selected OTHER.

Should card(s) say PSE, otherwise TNX (y/n) ?

YES:  If you want the card to tell the DX station you'd

like his card then answer YES.  The word PSE will be

affixed to most cards.

NO:   If you're returning the card, like in Jim's case, you

would answer NO.  The word TNX will be affixed to most

cards.

QSL all QSOs for this CALL (y/n) ?

This will only pop up on the screen if more than one entry

-24-

LOGPlus! version 3.0

for the call was found.  It's quite common for DX stations

to have 2 to 10 or more entries with your call, if you're

active.

YES:  If you're a DX station it's best to knock off all the

QSL cards for a particular station in one quick and

easy stroke.  Granted, you'll possibly receive letters

from some individuals who are dead set on receiving

individual cards for each band and mode, but I can't

see why that's necessary. 99.9% of the time I answer

YES to this. Also, if you have 6 QSOs with a station

and 5 have already been QSL'd, you can answer YES

but only the QSO which has not been QSL'd will be

processed.

NO:   If you don't want the station to receive one of the

contacts he had, you can answer NO here.  The next

question will pop up on the screen.

Enter record number(s) separated by '/':

This is where you can QSL only specific cards for a station.

Let's say that when we brought up VK9NS on our screen we

found 6 QSOs listed.  None have been QSL'd before.  I

want to only QSL two of the six.  On this line I would enter

the record number for each separated by a /.

1023/4322

LOGPlus! will only update those records and nothing else.

(R)eset: Reset is used when you wish to clear the QSL field and

re-send a QSL card.  Let's say I received a card from

W6TWN back on 10/10/90 for my 1989 DXPedition to Midway

and submitted a card shortly after I received his. I

have proof I sent him a card because the QSL field is

marked with "X" and the QSL Date tells me the exact

date the card was processed.  However, and I'm guilty

of this too, W6TWN sends me a card on 10/10/92 stating

that he has lost the card and needs another.  At this

point, you would enter W6TWN as the call and press

"R" from the menu.  You'll have an option to reset ALL

the QSOs or just on in particular.  The format

listed above for entering record numbers applies here

also.

A precaution.  When you select this option, ALL three

of the database files will be purged of that call.

This insures duplications are removed before printing.

-25-

LOGPlus! version 3.0

(S)kip:  This clears the screen and takes no action on the call

displayed.

(Q)uit:  Exit the module.  Take no action on the call displayed.

FUNCTION KEYS

=-=-=-=-=-=-=

Five function keys have been programmed to assist you in data

processing.

F2=Redisplay:     Redisplay data for the previously entered call

F3=DIRECT Browse: Pop up window which will allow you to quickly

view the DIRECT database.  Delete functions

are available.  This will only pop up if data

is found in the file.

F4=BUREAU Browse: Pop up window which will allow you to quickly

view the BUREAU database.  Delete functions

are available.  This will only pop up if data

is found in the file.

F5=OTHER Browse:  Pop up window which will allow you to quickly

view the OTHER database.  Delete functions

are available.  This will only pop up if data

is found in the file.

F6=Add Record:    Add record to regular log.  Record will be

appended to the end of file.  Once the record

has been added, you can update it.

B>  Direct - Printing/Browse

=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The information described here is the same for C> Bureau - Printing/

Browse and D> Other - Printing/Browse.  The only difference will be

the database file selected.

A>  Browse/Modify

=-=--=-=-=-=-=-=-

See Getting Around the Browse/Modify.

The following functions are provided in this module:

F2=Delete Entry:  This will delete the highlighted record.

F3=Call Search:   This will allow you to quickly search and

display any given call. Enter the call you

wish to locate. If found, the record will be

highlighted.  If not found, you'll be informed.

-26-

LOGPlus! version 3.0

Enter=Editor:     The editor will allow you to quickly modify

ALL data fields associated with the record

currently highlighted.

B>  Update QSLMgrs Field

=-=-=-=-=-=-=-=-=-=-=-=-

The Update QSLMgrs field provides you with a quick and easy way

to automatically update the QSL Manager field for later printing

on QSL labels.  When this option is selected, the QMgr databases

are checked for any call which matches the calls in the selected

database.  If found, the QMgr field will be updated with that

manager's call.  If nothing was found, the QMgr field will be

updated with either "DIRECT", "BUREAU" or "OTHER" depending on

the database selected.  After you've run this feature, you can

manually change any entry by using the browse/modify option.

Checking:  VK0GC

QSLMgr field set to:  VK9NS

C>  QSL Label Printing

=-=-=-=-=-=-=-=-=-=-=-

Four types of labels are used in LOGPlus!.  All of these label

formats can be purchased from local sources in large bulk if

needed.  Over the years, I have used both TAYCO and GLOBAL

Computer Supplies and found their prices to be reasonable.  If

you find additional sources, please let me know.  I'll include

the information for others to benefit from.  I do not endorse

either of the companies I mentioned above.

After selecting the label you wish to use, you will be prompted

with the following.  This is standard for each label:

Print QSL label(s) for specific DX Country (y/n) ?

Print QSL cards for a specific DX Country.  When prompted, enter

the DXCC prefix you wish to print (see Appendix A), only those

cards will be printed.  Once printing is done, you'll have the

opportunity to delete only those cards.

Sort cards in DXCC/Alphanumeric order (y/n) ?

Sort cards in DXCC order will sort the cards by country, followed

by call order.  This is an excellent way of printing cards which

are being submitted to the bureaus.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

Having been on 19 DXPeditions over the years, I receive a lot

-27-

LOGPlus! version 3.0

of QSL cards.  A couple of years back, VK9NS and I talked about

this and decided we'd add the SORT as an option.  The basic

reason for this was that when you receive HUNDREDS of cards at a

time, it's best to work the stack, FIRST IN FIRST OUT!.  By

SORTING, you must continuously search for the card and envelope

for a match up.  When you don't SORT, QSLing become MUCH MUCH

easier.

<ENTER> test label. <ESC> to exit

When ENTER is pressed, ONE test label will be printed to make sure

everything is lined up.  If so, you can answer YES to the next

prompt and start your printing.  If you answer NO, the above screen

will again display.  Realign your labels and press ENTER again.

A>  Standard Mailing Label w/Note

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Prints a standard 3x15/16 inch mailing label.  Before printing

can begin, you will have an option to enter a 30 character

comment which will be printed on the bottom of the label.

To:  AD1C

Confirming the following QSO:

Aug-21-91   1143z     20m   2XCW  599

This is a 30 character comment field

B>  Standard Mailing Label (op 1)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Prints a standard 3x15/16 inch mailing label.

To: LA5XGA

From: KD7P/NH2

Confirms the following QSO:

Date          Time       MHz      RST      2x     QSL

Oct 30 88     22:03Z     28       59       SSB    TNX

C>  Standard Mailing Label w/QSLMgr

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Prints a standard 3x15/16 inch mailing label.

To: LA5XGA

From: KD7P/NH2

QSL via:  DIRECT

Date          Time       MHz      RST      2x     QSL

Oct 30 88     22:03Z     28       59       SSB    TNX

D>  Standard Mailing Label (op 2)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Print a standard 3x15/16 inch mailing label.

-28-

LOGPlus! version 3.0

KD7P    << large print

Confirming our QSO of Jan 01 93

00:00Z 14MHz  SSB  59  << 14MHz SSB large print

PSE QSL - 73

E>  2-15/16 x 7/8 Label

=-=-=-=-=-=-=-=-=-=-=-=

Prints a standard 2-15/16 x 7/8 label.  Format is identical

to Standard Mailing Label mentioned above but fits smaller label.

F>  2-15/16 x 7/8 Label w/QSLMgr

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Prints a standard 2-15/16 x 7/8 label.  Format is identical to

Standard Mailing Label w/QSLMgr mentioned above but fits smaller

label.

G>  W4MPY QSL Format Label

=-=-=-=-=-=-=-=-=-=-=-=-=-

Prints strip labels (5 x 7/16 inch). Date format printed

depends on the format selected under Default Program Parameters.

LA3PU       1-JAN-92    10:23    21   599   2xCW

H>  Modified - W4MPY QSL Format

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Prints strip labels (5 x 7/16 inch). Date format printed

depends on the format selected under Default Program Parameters.

The difference between this and the regular format is field

placement.

I>  Western Washington DX Label

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Prints strip labels (5 x 7/16 inch).  Date format printed

depends on the format selected under Default Program Parameters.

KD7P/NH2      1-JAN-92    19:44   14    SSB   59   PSE

J>  2 QSO Format w/QSLMgr

=-=-=-=-=-=-=-=-=-=-=-=-=

Prints a standard 3x15/16 inch mailing label.

To:  KD7Z

Date       Time     Mtr    RST   2x

Jul 23 83  1833z    20     59    SSB

Jul 29 83  1533z    15     569   CW

Via: DIRECT       - 73 de KD7P -

K>  4 QSO Format w/Note

=-=-=-=-=-=-=-=-=-=-=-=

Prints large labels (3-1/2 x 1-7/16 inch).  Before printing

-29-

LOGPlus! version 3.0

can begin, you will have an option to enter a 40 character

comment which will be printed on the bottom of the label.

TO: XW8CW

KD7P/NH2 confirms following QSO(s):

MMM DD YY   GMT     MHz   2x   RST   NAME/CONTEST

FEB 24 91   06:24   14    SSB  59     HECTOR

JUN 18 91   17:23   1.8   SSB  55     HECTOR

AUG 14 91   08:24   21    SSB  59     HECTOR

AUG 18 91   18:23   10.1  SSB  55     HECTOR

This is the comment line, up to 40 char. , 73 Bob

L>  4 QSO Format w/Note & QSLMgr

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Prints large labels (3-1/2 x 1-7/16 inch).  Before printing

can begin, you will have an option to enter a 40 character

comment which will be printed on the bottom of the label.

TO: XW8CW

QSL card sent via: DIRECT

MMM DD YY   GMT     MHz   2x   RST   NAME/CONTEST

FEB 24 91   06:24   14    SSB  59     HECTOR

JUN 18 91   17:23   1.8   SSB  55     HECTOR

AUG 14 91   08:24   21    SSB  59     HECTOR

AUG 18 91   18:23   10.1  SSB  55     HECTOR

This is the comment line, up to 40 char. , 73 Bob

M>  5 QSO Format

=-=-=-=-=-=-=-=-

Prints large labels (3-1/2 x 1-7/16 inch).

TO: XW8CW

KD7P/NH2 confirms following QSO(s):

MMM DD YY   GMT     MHz   2x   RST   NAME/CONTEST

FEB 24 91   06:24   14    SSB  59     HECTOR

JUN 18 91   17:23   1.8   SSB  55     HECTOR

AUG 14 91   08:24   21    SSB  59     HECTOR

AUG 18 91   18:23   10.1  SSB  55     HECTOR

FEB 04 92   16:23   1296  SSB  59     HECTOR, 73 Bob

N>  5 QSO Format w/QSLMgr

=-=-=-=-=-=-=-=-=-=-=-=-=

Prints large labels (3-1/2 x 1-7/16 inch).

TO: XW8CW

QSL card sent via: DIRECT

MMM DD YY   GMT     MHz   2x   RST   NAME/CONTEST

FEB 24 91   06:24   14    SSB  59     HECTOR

JUN 18 91   17:23   1.8   SSB  55     HECTOR

-30-

LOGPlus! version 3.0

AUG 14 91   08:24   21    SSB  59     HECTOR

AUG 18 91   18:23   10.1  SSB  55     HECTOR

FEB 04 92   16:23   1296  SSB  59     HECTOR, 73 Bob

O>  Large Label w/Rig & QSLMgr

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Prints large labels (3-1/2 x 1-7/16 inch).  This prints only

one QSO per label in block format.

Radio: KD7Z  via: DIRECT

Date     UTC     RST    MHz    2Way

Jul 23 83  18:33    59     14     SSB

Kenwood TS930S, TL922A         QSL*TNX

4 ELE KLM KT-34A                    73

Because of the graphics, I am unable to show you the blocks.

You'll need to print this out to determine exact layout.

P>  Set Radio & Antenna Defaults

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Rig information can be printed on the <N> option label. The

following screen will allow you to enter rig/antenna information

which will be printed on the label.

HF Radio/Amplifier models: [                           ]

VHF Radio/Amplifier models: [                           ]

160m antenna: [                              ]

80m antenna: [                              ]

40m antenna: [                              ]

30m antenna: [                              ]

20m antenna: [                              ]

17m antenna: [                              ]

15m antenna: [                              ]

12m antenna: [                              ]

10m antenna: [                              ]

6m antenna: [                              ]

2m antenna: [                              ]

Add additional LINE FEED between labels (y/n)? [ N ]

As you can see, everything is self explanatory except the

additional LINE FEED.  Some labels (Japanese for example) space

the labels out a bit more than the U.S. labels.  To compensate

for this, I've added the ability to add an additional line feed.

D>  Delete ALL Records

=-=-=-=-=-=-=-=-=-=-=-

The Delete ALL Records will purge ALL records for the selected

-31-

LOGPlus! version 3.0

database file.  Use caution here!  If you delete all the data

and find you really needed it, you'll need to go back into the

QSL Card Updating, RESET all calls individually and re-QSL!

E>  Pending - Printing/Browse

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Over the years we lose track of who we've sent cards to.  Instead

of searching each entry in the log and spending countless hours,

we can quickly and easily obtain the information we need here.

A>  QSL Pending Update

=-=-=-=-=-=-=-=-=-=-=-

The QSL Pending Update will search your log for any QSO in which

the QSL field is marked with either a "B", "D", "O" or "S".

These records are copied to the PENDING.DBF file which can be

maintained or updated at anytime.  During the update process

the screen will display the number of QSOs it has copied over.

Direct:   32

Bureau:  123

Other:    5

B>  QSL Pending Browse/Modify

=-===-=-=-=-=-=-=-=-=-=-=-=-=-

See Getting Around the Browse/Modify.

The following functions are provided in this module:

F2=Delete Entry:  This will delete the highlighted record.

F3=Call Search:   This will allow you to quickly search and

display any given call. Enter the call you

wish to locate. If found, the record will be

highlighted.  If not found, you'll be informed.

F4=Add Record:    Append record to pending database.

F5=CW:            Display only those CW QSOs pending for

DXCC credit.  Hide all other QSOs.

F6=SSB:           Display only those SSB QSOs pending for

DXCC credit.  Hide all other QSOs.

F7=RTY:           Display only those RTY QSOs pending for

DXCC credit.  Hide all other QSOs.

F8=MIX:           Display only those MIX QSOs pending for

DXCC credit.  Hide all other QSOs.

F9=ALL:           Display all data.

-32-

LOGPlus! version 3.0

Enter=Editor:     The editor will allow you to quickly modify

ALL data fields associated with the record

currently highlighted.

C>  QSL Pending Printing

=-=-=-=-=-=-=-=-=-=-=-=-

The QSL Pending Printing will print out a listing of ALL QSOs

which are in the database.

D>  Delete ALL Records

=-=-=-=-=-=-=-=-=-=-=-

Delete ALL Records will remove all entries from the database. It's

recommended you use this prior to doing a QSL Pending Update.  This

will insure no duplications are entered.

F>  SWL - Printing/Browse

=-=-=-=-=-=-=-=-=-=-=-=-=

Menu options B>, C> and D> are identical to those listed under

the Direct - Printing/Browse.  Please refer to that section for

additional information.

A>  SWL Card Entry

=-=-=-=-=-=-=-=-=-

Most of us these days receive hundreds of SWL cards which in the

spirit of ham radio, need responding to.  Being a DX station

for the past 10 years, I've received more than my fair share!

Well folks here is the module to help us along.  By entering

the required data in the blocks, the SWL cards are appended to

the SWL.DBF file for printing.

how it works

=-=-=-=-=-=-

Boris/UA1-923-223-2 sends you a card for a contact you made

with KW7Y on 01/05/85 on 7MHz.  I would enter the contact as

follows:

SWL heard you work: KW7Y

Once you've entered the call and pressed ENTER, a pop up

screen will appear with all contacts with KW7Y.  Move the

highlight bar down to the appropriate QSOs, press ENTER

then ESC to exit the module.  All the fields will be filled

in automatically.  At this point, enter the SWL station

callsign and save the data.

G>  QSL Manager Database

=-=-=-=-=-=-=-=-=-=-=-=-

The QSL Manager Database is a powerful utility which will allow you

-33-

LOGPlus! version 3.0

to maintain information relating to past, current and future QSL

Managers.  The QSL Manager system  utilizes two databases.

QMGR.DBF which contains a listing of all DX Stations their managers

and the year active.  The second database called QLABELS.DBF contains

only the address of those managers you wish to maintain.

H>  Yearly QSO/QSL Statistics

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Yearly QSO/QSL Statistics provides a complete breakdown of the

number of contacts, QSLs sent and those which are pending via DIRECT

or BUREAU during the years 1960 to 1999.

QSLs         Pending

Year      Contacts    Sent      Direct/Bureau

1980

1981

1982       14323      5322          1 /  23

1983        3541      1352         14 /

1984        2102      1322          9 /  22

etc...

I>  Index QSL Card Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The Index QSL Card Databases will only reindex those databases which

effect QSLing.  Because of the amount of time required to reindex the

QSL Manager databases, I've opted to split the indexing chores between

two menu options to speed execution.

J>  Index QSL Manager Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Index QSL Manager Databases will only reindex those databases which

effect the QSL Manager files.

D>  DX Century Club Award

=-=-=-=-=-=-=-=-=-=-=-=-=

The DX Century Club Award module is possibly the most important part of

LOGPlus!.  This module encompasses a great deal of data which must be

read thoroughly to grasp the power it offers.

A>  Auto Update (log)

=-=-=-=-=-=-=-=-=-=-=

This section will also describe information relating to menu option

B>  Auto Update (sat).  The only difference between the two is the

database file selected.

The Auto Update feature provides a completely automated way of

keeping track of your DXCC award.  Information is extracted from your

log and cross checked with three database files to insure the correct

DXCC country is credited each time. Unfortunately like all good

-34-

LOGPlus! version 3.0

plans, nothing is absolute.

CALL:    The most important field! This determines the country

which will receive credit.  Make sure you use option

I>  Unique Country/Prefix Update or J>  CallSign to

Prefix Update if the call can not be cross referenced.

DXCC:    Updated by extracting information from the CALL field.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

If the DXCC field displays incorrect information, it may be

necessary to manually update the field.  For instance, let's say

UA1OIL, which in previous years was on Franz Josef Land,  recently,

changed from UA1O to 4K2 then to R1/FJ.  During the course of the

update, you notice UA1OIL was only credited as UA instead of R1/FJ.

You'll need to do one of two things:

1.   Conduct a manual update of the DXCC field for UA1OIL.

This would require you to manually update this field each time

you came across UA1OIL.

2.   Select option J> CallSign to Country Mapping and update the

information ONCE.

3.  The DXCC field in the log is very important as is the call

field.  Call signs are becoming more and more "INTERESTING" these

days which makes updating sometimes difficult.  For instance,

let's take the call group VP8.  99.9% of the time, the DX station

will never sign /O, /F, /S etc... they will usually give their

call followed by South Shetland Islands.  In all my years, I have

rarely heard the portable given.  When you encounter a call like

this (3D2, SV, VP8 etc.. are some that also fall into this

category), you need to use option J> CallSign to Country Mapping.

FREQ:    Used to determine band credit.

MODE:    Used to determine mode credit.

QSL:    Used to determine DXCC status.  This field is a major

key in the way information is processed.  Make sure

you keep this field updated.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

If Deleted Countries are encountered the program will make every

effort to assign the correct DXCC prefix to that country.  It is

extremely important you read K> Prefix to Country Mapping for

more information on how deleted countries are processed.

-35-

LOGPlus! version 3.0

C>  Browse/Modify

=-=-=-=-=-=-=-=-=

See Getting Around the Browse/Modify for general information.

The following function keys are provided:

F2=Delete Data:      This will delete ONLY the modifiable fields.

DXCC, Country Name, Lat/Long and ZONE information

will not be deleted.

F3=Prefix Search:    Enter the STANDARD DXCC prefix associated with

the country.  STANDARD in this case means those

commonly assigned to the country by the ARRL.

(i.e. KH2 not WH2, NH2 etc..)  You can scroll

through the DXCC database to get a basic

understanding of what these prefixes are.

F4=Statistics:       When selected, an additional browse/modify screen

will appear listing all the common bands for a

particular mode.  When first selected, the MIXED

mode will be displayed.  The following function

keys are provided:

Enter=Editor:        The editor will allow you to quickly modify ALL

data fields except those which are locked and can

not be changed.  Locked fields, if changed, will

cause serious problems with DXCC totals.

F8=All Countries:       Display all active and deleted countries.

F9=Active Countries:    Display only active countries.

F10=Deleted Countries:  Display only deleted countries.

D>  Single Band Awards

=-=-=-=-=-=-=-=-=-=-=-

The Single Band Awards updates and maintains individuals awards

for both band and modes.

A>  Auto Update

=-=-=-=-=-=-=-=

One of the excellent features of LOGPlus! is the ability to

maintain your DXCC records to meet your needs.  The auto

update, will automatically cross-reference your LOGBOOK and

DXCC databases and create a databases of contacts which have

been confirmed but not submitted yet to the ARRL.  Unlike

LOGPlus! v2, this update automatically creates single band

award information for all modes and bands including YL, QRP

and SAT.

-36-

LOGPlus! version 3.0

B>  Browse/Modify

=-=-=-=-=-=-=-=-=-

When first selected, only those records associated with the

General Mixed Award are displayed.  Use F5 to F9 to change

modes and bands.

C>  Statistics

=-=-=-=-=-=-=-

Display number of QSOs pending for each band and mode.  These

are QSOs pending DXCC submission.

D>  Printing

=-=-=-=-=-=-=

The printing option is menu driven for quick selection.  Each

band is broken down individually to reduce paper waste.  Once

you've selected the band or award you wish to print, you'll

be prompted to select the mode you wish to print.

E>  Indexing

=-=-=-=-=-=-

Recreate DX.NSX file.  This is only required if difficulties

occur locating data.

E>  Status Update

=-=-=-=-=-=-=-=-=

The Status Update provides a quick and easy way to update

any of the DXCC band and mode columns.

how it works

=-=-=-=-=-=-

Select the mode or band you wish to update.  Remember, this

will do a GLOBAL change in the DXCC database.  Use caution!

A>  Confirmed to Submitted

=-=-=-=-=-=-=-=-=-=-=-=-=-

Change the band/mode from 'C' to 'S' for all records in

selected file. This is usually done after the cards have

been sent to the ARRL.  These cards have not been returned.

B>  Submitted to Approved

=-=-=-=-=-=-=-=-=-=-=-=-=

Change the band/mode from 'S' to 'A' for all records in

selected file. This is usually done after the ARRL has

accepted all cards submitted. If for some reason a particular

card was not accepted, use the Browse/Modify and change the

field accordingly.

F>  Band & Mode Country Statistics

-37-

LOGPlus! version 3.0

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The Band & Mode Country Statistics is a unique display which

provides you information concerning your DXCC database.  In

addition to display Worked/Confirmed/Submitted information

for each band and mode, the program also provides you a total

band QSO total.  Numbers located within parens are DELETED

countries.

G>  Band/Mode Statistics

=-=-=-=-=-=-=-=-=-=-=-=-

The Band/Mode Statistics provides a COMPLETE breakdown for each

band and mode.  On color monitors, each status is displayed in

different colors to allow for quick look up.  The following is a

breakdown of CW 1.8MHz:

1.8

******

ARL    1

C  SUB

CFM    1

W  PND    3

WKD    5

ARL:  Band field marked with "A"

SUB:  Band field marked with "S"

CFM:  Band field marked with "C"

PND:  Band field marked with "W"

WKD:  Total of all fields.

H>  DX Alert

=-=-=-=-=-=-

The DX Alert take the hassle out of keeping track of upcoming

DXPedition.  This system is a powerful feature which will alert

you at program execution if any DXPeditions are scheduled for

that day.  Information can be easily shared by other LOGPlus!

users quickly and easily by the use of ASCII text files which

can be either uploaded or downloaded. If updated on a regular

basis, this program alone can provide up to the minute

information on upcoming DXPeditions and assist in increasing

your DXCC country count.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

As mentioned above, you can be alerted of daily DX activity on

program execution only if you turn on this feature under the

Program Default Parameters.

-38-

LOGPlus! version 3.0

A>  DX Alert DXPedition Update

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The DX Alert DXPedition Update allows you the flexibility of adding or

looking up DX information quickly.

how it works

=-=-=-=-=-=-

Enter the call sign of the station you wish to add.  If the station

was previously entered, the program will display the data and ask

you if it's correct. If this is a new entry you'll be prompted with

the following entry screen:

DXPedition beginning date:    Enter the DATE the DXPedition is

scheduled to commence.  This date is

very important to the DX Alert check

at execution.

DXPedition ending date:       Enter the DATE the DXPedition is

scheduled to end. This date along

with the beginning date will determine

if the entry will be deleted under

option H>  Delete DX Alert.

DXPedition common freq:       Enter the frequency the DXPedition

is most commonly heard to work.  With

most DXPeditions hopping from one band

to the next, this information is

basically arbitrary.

DXPedition common mode:       Enter the mode the DXPedition works

most.  Some DXPeditions may only work

RTY.

DXPedition general times:     Enter the start and stop times the

station is generally heard at.

DXPedition information:       Any comments you wish to make.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

You are NOT restricted to one entry per DXPedition.  If you know

the station operates 15 Meters from 0100-0300 then 20 Meters

from 0300-0500, make two entries.  There are no restrictions to

the number of times you can enter the same station.

B>  DX Alert Activity Browse/Modify

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

VERY IMPORTANT!!!!

-39-

LOGPlus! version 3.0

=-=-=-=-=-=-=-=-=-

The information described here will basically echo those in the

Todays, Weekly and Monthly Browse/Modify modules.  The only

difference will be in the way the F6/F7 and F8 keys are utilized.

The Browse/Modify will allow you to scroll through your data quickly,

utilizing the arrow keys.  A BLACK highlight bar will indicate the

record you're currently on.

Function keys are provided to allow quick data searches as well as

record editing.

F2=Delete:           This will delete the highlighted record.

F3=Call Search:      This will allow you to quickly search and

display any given call.  Enter the call you

wish to locate. If found, the record will be

highlighted. If not found, you'll be informed.

F4=Date Search:      This will allow you to quickly search and

display any given date.  Enter the date you

wish to locate. If found, the record will be

highlighted. If not found, you'll be informed.

F5=Append Record:    This will allow you to add a new record to

the database.

F10=Print Log:       Print the log in the order currently displayed.

If you conduct a Call Search, the file will be

in alphanumeric order.  If a Date Search was

displayed, records will be printed in date

order.

Enter=Editor:        The editor will allow you to quickly modify

ALL data fields associated with the record

currently highlighted.

C>  Todays DX Activity Browse/Modify

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

See B> above for information.

D>  Weekly DX Activity Browse/Modify

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

See B> above for information.  The following additional function keys

are available under this option.

F6=Prev Week:        Display only those records which fall under

the previous calendar week.

-40-

LOGPlus! version 3.0

F7=Next Week:        Display only those records which fall under

the next calendar week.

F8=Current Week:     Display only the current week.

E>  Monthly DX Activity Browse/Modify

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

See B> above for information.  The following additional function keys

are available under this option.

F6=Prev Month:       Display only those records which fall under

the previous calendar month.

F7=Next Month:       Display only those records which fall under

the next calendar month.

F8=Current Month:    Display only the current month.

F>  Upload DX Alert Information

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

This feature allows you to UPLOAD information from other users to

your DXINFO database.  The file DXALERT.TXT must be found in the

\PLUSV3 directory.

NOTE:  Previous version of LOGPlus! (prior to v3) CAN NOT be uploaded.

The file structure is different.  Duplicate files will require manual

deletion.

G>  Download DX Alert Information

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

This feature allows you to DOWNLOAD information from the DXINFO

database to a delimited ASCII file for transmission to other users.

The file DXALERT.TXT will contain a complete listing of ALL records

contained in the DXINFO database.

H>  Delete OUTDATED Information

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

This feature allows you to delete OUTDATED information from the

DXINFO database.  OUTDATED information are those records which the

beginning date and the ending date are EARLIER than the system date.

Once deleted, the DXINFO database file is automatically reindexed.

I>  DX Alert File Indexing

=-=-=-=-=-=-=-=-=-=-=-=-=-

At times it may become necessary to recreate index files because of

corrupt data.  If you're having difficulties with searching and

updating, it's not a bad idea to run this feature.

I>  Country List Update

-41-

LOGPlus! version 3.0

=-=-=-=-=-=-=-=-=-=-=-=

The Country List Update along with Prefix to Country Mapping and

CallSign to Country Mapping are critical areas which are extremely

delicate and require alot of thought.  These three menu selections

provide the interface necessary to add or delete DXCC Countries from

DXCC credit. Every attempt will be made to give full details on the

process needed to add or delete a country. Please read this completely

before making any attempts. First let me explain what I mean by "delete".

Actually, delete in this instance means to remove the country from any

further credit, not complete removal.

Currently the DXCC database contains 385 countries.  These are

countries which are both active and no longer creditable.  With the

ever changing climate in todays world, countries seem to be popping

up left and right and at the same time others are disappearing.

Because of this, it's necessary to develop a method in which you can

easily and effectively make the necessary changes to your DXCC database

without paying for upgrades.

ADDING A NEW DXCC COUNTRY

=-=-=-=-=-=-=-=-=-=-=-=-=

Adding a new DXCC Country involves two major steps.  Each of these

steps are critical to program performance and MUST be done with

care.

The following questions MUST be answered before any attempts are

made to update your program.  This information can be obtained from

the ARRL, local PBBS or BBS systems.  We'll first look at adding a

new country then modifying an existing one.

First we must obtain the necessary information.  Here is a basic

check list which will help:

1.  What is the ITU PREFIX GROUP assigned?

2.  What is the OFFICIAL (United Nations) name?

3.  What is the EFFECTIVE DATE the country will be/was accepted

for DXCC credit?

4.  What PREFIXES are being effected by the change?

5.  Does the country observe Daylight Savings Time?

6.  What is the GMT OffSet for the country?

Once these questions have been answered we can start to add the new

country.  Below is an example of adding Croatia/9A.

-42-

LOGPlus! version 3.0

DXCC:  9A

Country:  Croatia

Effective Date:  06/26/91

Deletion Date:

Latitude:  28.0      << Do not leave default at 0.0

Longitude:  32.0     << Do not leave default at 0.0

CQ Zone:  15   ITU Zone:  28   Continent: Eu

GMT OffSet:  2     Day Light Savings: N

GMT OffSet: Hours ahead or behind GMT.  This information can be

obtained from any good atlas.

Day Light Savings:  Enter Y only if the country observes DLS.

The only purpose for this entry is to make sure LOCAL TIME is

displayed correctly in the Realtime/OffAir modules.

Once you're satisfied your NEW COUNTRY data is correct, save the

entry to the DXCC database.

MODIFYING AN EXISTING DXCC COUNTRY

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Modifying an existing DXCC Country is very easy and straight

forward. One precaution, it's highly recommended that once you've

set the DXCC prefix and made modification to K> Prefix to Country

Mapping, use this prefix continuously or until you're confortable

with making changes.

VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-

DO NOT MAKE ANY CHANGES TO ANY OF THE CURRENT DXCC COUNTRIES

ASSIGNED. SOME MAY BE HARD CODED INTO OTHER PORTIONS OF THE

PROGRAM.

CHANGING THE DELETED COUNTRY STATUS

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

At times it may become necessary to modify the deletion date of any

existing country.  This usually happens when the country no longer

counts for DXCC credit.  This is very easy and straight forward but

a few precautions.  After you've made the changes under this option,

make sure you also make the changes under option K> Prefix to

Country Mapping.

-43-

LOGPlus! version 3.0

J>  Callsign to Country Mapping

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

During logging, you may find an occassional callsign which could not

be properly associated with a country.  This is usually the case with

3D2, VK9, 3Y, JD callsigns.  The Callsign to Country Mapping allows

you the facilities to associate the callsign with the country and

zone if needed.

K>  Prefix to Country Mapping

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Caution should be the first order of busy when using the feature.

Place the wrong information in the PRFX.DBF file can result in

major problems with your DXCC databases.  This feature should

only be used when adding a new country or itu group.

L>  Update Beam Heading & Sunsets

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Recreate Beam Headings and updates the Sunrise/Sunset information

for each DXCC country.  Make sure you've placed your latitude and

longitude in the default options before running this feature.

Otherwise, your information will be incorrect.

M>  SunRise & SunSet Browse/Modify

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

This feature allows you to add or subtract cities from the SUNCITY

as well as update.

N>  Clean Up DXCC/LOGBOOK databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Periodically you may want to delete the band information from

your DXCC and LOGBOOK and start over from scratch.  This is

particular useful when first converting from v2 to v3.  Run

this program, then do a DXCC Auto Update to bring all your

records in line with version 3.0.

O>  Printing

=-=-=-=-=-=-

Basic printing options.

P>  Indexing

=-=-=-=-=-=-

Recreate indices associated with your DXCC database and PREFIXES/

PRFX files.

E>  Worked All Zones Award (WAZ)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The Worked All Zones Award program is supported by CQ Magazine.

This module will assist in tracking the various facits of the

WAZ award and assist in recordkeeping tasks.

-44-

LOGPlus! version 3.0

Bands & Modes Tracked

=-=-=-=-=-=-=-=-=-=-=

General Mixed, CW, PHONE and RTTY

160, 80, 40, 30, 20, 17, 15, 10, 6, 2 and Satellite. Tracking

of Mixed, CW, PHONE and RTTY for each band done automatically.

Required Files

=-=-=-=--=-=-=

Two files are required to track the WAZ award program.  If

either of these files are missing, NONE of the WAZ features

described herein will function.

WAZ.DBF and WAZ.NSX (the NSX file is created automatically

during the indexing process).

A>  Auto Update (log)

=-=-=-=-=-=-=-=-=-=-=

This module is the heart of the WAZ award system.  It's major

purpose is to reduce the time and effort required to manually

update and track your WAZ award.

The auto update will:

1>  Check the ZONE field in your logbook for any blanks. If

found, the appropriate ZONE information will be updated.

2>  Cross reference LOGBOOK entries with those in the WAZ

databases and update appropriately.  During the update,

each ZONE is checked individually and the number of QSOs

per ZONE is displayed alongside the ZONE.  This module is

approximately 50% faster then earlier versions of LOGPlus!.

B>  Auto Update (sat)

=-=-=-=-=-=-=-=-=-=-=

See A>  Auto Update (log) for information relating to the

Auto Update (sat).  The only difference between the two is

the databases used, SATELLITE or LOGBOOK.

C>  Browse/Modify

=-=-=-=-=-=-=-=-=

See Getting Around the Browse/Modify.

When first selected, only those records associated with the

General Mixed Award are displayed.  Use F5 to F9 to change

modes and bands.

F3=Zone Search    -  Allows you to quickly move between zones.

Zones 1 thru 9 must be preceeded by a "0".

-45-

LOGPlus! version 3.0

F9=Change Bands   -  Allows you to select any of the other bands

including Satellite and General.

D>  Quick Look Statistics

=-=-=-=-=-=-=-=-=-=-=-=-=

This module provides you with a graphical display by mode, of

the zones you have worked, confirmed or need.  This feature can

be activiated in the logging and dumb terminal modules by pressing

SHIFT+F5.

1         2         3         4

1234567890123456789012345678901234567890

160  *****WWWWWWWW********......***...**..**.

80  ****************WW*****......***.*WWWWWW

40  ******...........*******WW.WWW..WWW*****

etc..

* = Confirmed   W = Worked    . = Needed

E>  Band/Mode Statistics

=-=-=-=-=-=-=-=-=-=-=-=-

This module provides you with a complete breakdown of your current

zone status by band and mode.  This format is standard throughout

LOGPlus! and will be used for most all statistical displays.

WKD = Worked.  This total includes all pending and confirmed

entries.

PND = Pending.  This counts those entries with "Worked" in

the status field of the WAZ database. QSL card NOT

received.

CFM = Confirmed.  This counts those entries with "Confirmed"

in the status field of the WAZ database. QSL card HAS

been received.

5 Band totals only include those entries on 80,40,20,15 and

10 meters.

MIX           CW           PHONE         RTTY

WKD/PND/CFM   WKD/PND/CFM   WKD/PND/CFM   WKD/PND/CFM

General   39/  0/ 39    40/  0/ 40    40/ 10/ 30     0/  0/  0

160m   38/  3/ 35    40/  0/ 40    26/ 10/ 16     0/  0/  0

....

....

5 Band Worked: 178  5 Band Pending:  3   5 Band Confirmed: 175

F>  Zone/QSO Statistics

=-=-=-=-=-=-=-=-=-=-=-=

This feature provides a quick breakdown of the number of QSOs

-46-

LOGPlus! version 3.0

worked in each of the 40 zones. This feature provides the same

information as the Auto Update but much quicker.

Zone  Records    Zone  Records   Zone  Records   Zone  Records

01     10        11     45       21     100      31     39

02     45        12     123      22     4        32     99

03     800       13     23       23     200      33     3

etc...

G>  Printing

=-=-=-=-=-=-

The printing option is menu driven for quick selection.  Each

band is broken down individually to reduce paper waste.  Once

you've selected the band or award you wish to print, you'll

be prompted to select the mode you wish to print.

The print out will match the standard award form which can be

either sent to CQ Magazine or checked by one of many check

points.

NOTE:  Entries marked with *W* next to the call are worked

and NOT confirmed.

H>  Indexing

=-=-=-=-==-=

Recreate the WAZ.NSX file.  This is necessary only if data

becomes corrupt.

F>  Worked All States Award (WAS)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Worked All States Award program is supported by The American

Radio Relay League. This module will assist in tracking the various

facits of the WAS award and assist in recordkeeping tasks.

Bands & Modes Tracked

=-=-=-=-=-=-=-=-=-=-=

General Mixed, CW, PHONE and RTTY

160, 80, 40, 30, 20, 17, 15, 10, 6, 2 and Satellite. Tracking

of Mixed, CW, PHONE and RTTY for each band done automatically.

Required Files

=-=-=-=--=-=-=

Two files are required to track the WAS award program.  If

either of these files are missing, NONE of the WAS features

described herein will function.

WAS.DBF and WAS.NSX (the NSX file is created automatically

during the indexing process).

-47-

LOGPlus! version 3.0

A>  Auto Update

=-=-=-=-=-=-=-=

This module is the heart of the WAS award system.  It's major

purpose is to reduce the time and effort required to manually

update and track your WAS award.

The auto update will:

1>  Cross reference LOGBOOK entries with those in the WAS

databases and update appropriately.  During the update,

each STATE is checked individually and the number of QSOs

per STATE is displayed alongside the STATE.  This module is

approximately 50% faster then earlier versions of LOGPlus!.

NOTE:  LOGPlus! v2 checked for duplicate entries by call

and assigned the state to these records.  Because of problems

encountered with this feature, it has been removed.

B>  Browse/Modify

=-=-=-=-=-=-=-=-=

See Getting Around the Browse/Modify.

When first selected, only those records associated with the

General Mixed Award are displayed.  Use F5 to F9 to change

modes and bands.

F3=State Search   -  Allows you to quickly move between states.

F9=Change Bands   -  Allows you to select any of the other bands

including Satellite and General.

NOTE:  You can use this feature in both the dumb terminal program

and logging modules.

C>  Quick Look Statistics

=-=-=-=-=-=-=-=-=-=-=-=-=

This module provides you with a graphical display by band and mode,

of the states you have worked, confirmed or need.

[ States ]

1   2   3   4   5   6   7   8   9   0

CT  NY  DE  AL  AR  CA  AZ  MI  IL  CO

MA  NJ  PA  GA  LA      ID  OH  IN  IA

ME      MD  KY  MS  HI  MT  WV  WI  KS

NH          NC  NM      NV          MN

RI          FL  TX  AK  OR          MO

VT          SC  OK      UT          NE

TN          WA          ND

VA          WY          SD

-48-

LOGPlus! version 3.0

Confirmed QSOs = BRIGHT WHITE on color and monochrome

Worked QSOs = YELLOW on color and LIGHT WHITE on monochrome

Neeeded QSOs = GRAY on color and monochrome

D>  Band/Mode Statistics

=-=-=-=-=-=-=-=-=-=-=-=-

This module provides you with a complete breakdown of your current

state status by band and mode.  This format is standard throughout

LOGPlus!, see WAZ E> Band/Mode Statistics for general information.

E>  State/QSO Statistics

=-=-=-=-=-=-=-=-=-=-=-=-

This feature provides a quick breakdown of the number of QSOs

worked in each of the 50 states. This feature provides the same

information as the Auto Update but much quicker.

State  QSOs  State  QSOs  State  QSOs  State  QSOs  State QSOs

AL     53    HI     52    MA     31    NM     11    SD    18

AK     31    ID     9     MI     53    NY     74    TN    56

etc....

F>  Printing

=-=-=-=-=-=-

The printing option is menu driven for quick selection.  Each

band is broken down individually to reduce paper waste.  Once

you've selected the band or award you wish to print, you'll

be prompted to select the mode you wish to print.

The print out will match the standard award form which can be

either sent to the ARRL or checked by one of many check points.

NOTE:  Entries marked with *W* next to the call are worked

and NOT confirmed.

G>  Indexing

=-=-=-=-==-=

Recreate the WAS.NSX file.  This is necessary only if data

becomes corrupt.

G> Worked All Prefixes Award (WPX)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

The Worked All Prefixes Award program is supported by CQ Magazine.

This module will assist in tracking the various facits of the WPX

award and assist in recordkeeping tasks.

Bands & Modes Tracked

=-=-=-=-=-=-=-=-=-=-=

General Mixed, CW, PHONE and RTTY

-49-

LOGPlus! version 3.0

160, 80, 40, 20, 15, and 10. Tracking of Mixed, CW, PHONE and

RTTY for each band done automatically.  In addition, each of

the continents are tracked simotaneously.

Required Files

=-=-=-=--=-=-=

During the updating process, 30 files are created.  These files

are both .DBF files and .NSX files required by LOGPlus!.

A>  Auto Update

=-=-=-=-=-=-=-=

LOGPlus! will automatically determine the correct prefix for

each entry in your logbook.

Call             extracted prefix

===========      ================

LX/AD1C                LX0

JT/0K1XC               JT0

NH7/KD7P               NH7

KD2YP/3                KD3

WA7ULL/M               WA7

MM3/KD7P               ......

3DA0BDC                3DA0

U4H/UA4BDS             U4H

F/DK5PR                F0

W7JVL                  W7

AD1C/LX                LX0

NOTE: For maritime mobiles (MM1/MM2/MM3) and mobile (M1/M2, etc..)

the WPX field in your log will be appended with five periods

(.....).  You can manually update these fields later if you wish.

During the update process, alot of disk activity will be noted.

Various files are created, appended and deleted to develop the

final outcome.  Overall, this is the slowest updating process

within LOGPlus!.  Please have patience, depending on the number

of QSOs in your logbook, this may take up to 15 minutes.

B>  Manual Update

=-=-=-=-=-=-=-=-=

The Manual Update allows you to search any of the WPX databases

and update or add new prefixes.  This module is usually used to

update the QSL status of a particular prefix.  In addition,

updating can be done more effectively in the Browse/Modify.

C>  Browse/Modify

=-=-=-=-=-=-=-=-=

See Getting Around the Browse/Modify.  When first selected,

select the file you wish to work with.  Use F5 to F9 to change

-50-

LOGPlus! version 3.0

modes and bands.

D>  Prefix Display

=-=-=-=-=-=-=-=-=-

The Prefix Display will allow you to view, in alphanumeric order,

all prefixes which have been worked, confirmed, submitted and

approved in a column layout.

how it works

=-=-=-=-=-=-

Select the database you wish to view.  At the bottom of the

screen, totals will display an accurate accounting of the

prefix status. On a screen, each of the different categories

will be displayed in different colors.  On monochrome or

green screens, each different category will be affixed with a

particular symbol denoting the prefix status.  To view

additional prefixes, press any key.  Press <ESC> to return to

the menu.

Status         Color          Monochrome

Worked         White          Prefix + '*'   KD2  *

Confirmed      Green          Prefix Only

Submitted      Red            Prefix  + '$'  KD2  $

Approved       Blue           Prefix  +  "@'      KD2 @

Total Worked   Blue

E>  LookUp (all files)

=-=-=-=-=-=-=-=-=-=-=-

The LookUp is a quick and dirty way of checking all your WPX files

for a particular prefix.

Prefix:  KH2

CW:  C   PH:  W   MX:  C  10:  C  12: N   15: N

17:  N   20:  N   30:  N  40:  C  80: N  160: N

Na:  N   Sa:  N   Oc:  C  Eu:  N  As: N   Af: N

C=Confirmed    N=Needed     W=Worked

The LookUp is a pop up window which can be viewed within the

dumb terminal program and logging module.

F>  Field Update (log)

=-=-=-=-=-=-=-=-=-=-=-=

The Field Update (log) will update the WPX field in the log

without having to conduct an Auto or Manual Update.  This same

feature can be done during the Auto Update process.

G>  Status Update

-51-

LOGPlus! version 3.0

=-=-=-=-=-=-=-=-=-

The Status Update automatically updates ALL award files.

A>  Confirmed to Submitted

=-=-=-=-=-=-=-=-=-=-=-=-=-

Change STATUS from 'C' to 'S' for all records in selected

file. This is usually done after the cards have been sent to

the CQ WPX Awards Manager.  These cards have not been returned.

B>  Submitted to Approved

=-=-=-=-=-=-=-=-=-=-=-=-=

Change STATUS from 'S' to 'A' for all records in selected file.

This is usually done after the CQ WPX Awards Manager has

accepted all cards submitted. If for some reason a particular

card was not accepted, use Manual Update or Browse/Modify and

change the STATUS field accordingly.

H>  Statistics

=-=-=-=-=-=-=-=

The Statistics provides an accurate total of each of the five

categories.

I>  Printing

=-=-=-=-=-=-=

Printing allows you to print out award forms. Contacts are

printed 500 per sheet in columns of 10 across and 50 per column.

These print outs follow the WPX Award sheet provided by CQ

Magazine. After selecting the file you wish to print, an

additional pop up menu will allow you to print 1 of 5 awards.

Worked:   Complete list of ALL contacts in a particular file.

Pending:   Contacts marked with 'W' in the STATUS field.

Confirmed:   Contacts marked with 'X' in the STATUS field.

Submitted:   Contacts marked with 'S' in the STATUS field.

Approved:   Contacts marked with 'A' in the STATUS field.

After selecting the category you wish to print, you'll be

prompted by another menu to select either PREFIX or CALL order.

Prefix:   Print PREFIXES only of selected award.

Call:   Print CALLS only of selected award.

J>  Indexing

=-=-=-=-=-=-=

-52-

LOGPlus! version 3.0

Recreate all files associated with the WPX award. This is

necessary only if data becomes corrupt.

H>  VHF/UHF Century Club (VUCC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The VHF/UHF Century Club (VUCC) program is supported by The

American Radio Relay League (ARRL).  This module will assist

in tracking the various facits of the VUCC award and assist

in recordkeeping tasks.  Contact the ARRL for additional

information concerning the VUCC program.

Bands & Modes Tracked

=-=-=-=-=-=-=-=-=-=-=

All bands from 6 meters to 24GHz.

Required Files

=-=-=-=--=-=-=

Twenty Two files are required to track the VUCC award program.

Two for each of the eleven VHF/UHF bands.  Index files (.NSX)

will be created automatically during the indexing process.)

A>  Auto Update

=-=-=-=-=-=-=-=

The Auto Update module will extract grid information from your

logbook and update each of the VHF/UHF files accordingly.

NOTE:  During PacketCluster monitoring in the dumb terminal

program, each spot reported for 2 and 6 meters will cause an

immediate check of the VUCC50 and VUCC144 files to determine

if you need the grid reported.  However, for this feature to

work properly, the first 4 characters of the spot comments

field MUST contain the grid information.

Example of a typical spot (shorten to fit on screen):

DX de KD7P:   50323.2   NH2G     OC26       1432Z

When the program sees this entry, it will check and see if

the check 2 meter grid switch under the dumb terminal Packet

Alerts has been turned on.  If it has, the VUCC50 database

will be checked to determine if the grid has been worked

previously.  In the upper right hand corner of the screen,

the grid and status will be displayed:

OC26[C]     >> grid has been confirmed

OC26[W]     >> grid has been worked no card

OC26[N]     >> grid not in database. It is very

possible that erronuous data may be

displayed.  The program will make every

-53-

LOGPlus! version 3.0

effort to determine the correct info

but this is not always the case.

B>  Browse/Modify

=-=-=-=-=-=-=-=-=-

See Getting Around the Browse/Modify.  When first selected,

select the file you wish to work with.

F3=Grid Search:   Enter 6 or 4 grid combination. If found, the

screen will update and highlight that record.

C>  Grid Display

=-=-=-=-=-=-=-=-=

The Grid Display allows you to view a columnized listing of all

grids in a particular file.  Select the file you wish to view

from the menu selection.  On a color monitor, data will be

displayed in various colors depicting the different grid status.

On monochrome or black and white screens, the status will be

depicted by side characters.

Confirmed:  No side character

Pending:    *

Submitted:  $

Approved:   @

D>  Grid/Grid Dist. Calc.

=-=-=-=-=-=-=-=-=-=-=-=-=

The Grid/Grid distance calculator allows you to enter to different

and display the corresponding Lat/Long, distance in both

kilometers and miles as well as the azimuth.  Locations are

to the CENTER of the grids entered.

E>  Grid/Posn Conversion

=-=-=-=-=-=-=-=-=-=-=-=-=

The Grid/Posn conversion will allow you to enter a grid and

display the corresponding Lat/Long for that grid.  Locations are

to the CENTER of the grids entered.

F>  Posn/Grid Conversion

=-=-=-=-=-=-=-=-=-=-=-=-

The Position to Grid conversion will allow you to enter a Lat/Long

position and display the corresponding grid.

G>  Grid Statistics

=-=-=-=-=-=-=-=-=-=-

The Grid Statistics provide worked, confirmed, pending, submitted

and approved record totals for each of the VHF and UHF bands.

H>  Printing

-54-

LOGPlus! version 3.0

=-=-=-=-=-=-=

Printing allows information relating to each of the VUCC award

to be printed.  Five printing styles are available.  GRID, CALL,

DATE, TIME, FREQ, MODE, SENT, RCVD, QSL and STATUS information

from the database(s) will be printed.

A>  Worked - QSL Pending

=-=-=-=-=-=-=-=-=-=-=-=-

List of contacts worked but not confirmed.  QSL has been sent,

but no card has been returned.  Cards which have a STATUS of 'W'.

B>  Confirmed - QSLs Received

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

List of cards which have been sent and a QSL has been returned.

These are cards which have a STATUS of 'C'.

C>  Submitted to Awards Manager

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

List of cards which have been sent to the awards manager for

checking.  These are cards which have a STATUS of 'S'.

D>  Approved by Awards Manager

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

List of cards which have been submitted for ARRL credit and

credit has been received.  These are cards which have a STATUS

of 'A'.

E>  Complete Listing of Grids

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

List of ALL contacts in the selected database regardless of

status.

I>  Indexing

=-=-=-=-=-=-

Recreate all VUCC###.NSX files.  This is necessary only if data

becomes corrupt.

I>  Islands On The Air (IOTA)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Islands On the Air (IOTA) module is provided as an interface

to the program offered by the Radio Society of Great Britain. The

information found in the IOTA.DBF file can be obtained from various

sources (i.e. PacketCluster's, magazines, newletters etc..). The

information here is not intended to replace the IOTA directory, but

to assist in recordkeeping.  To gain the full scope of how this

program functions, you should obtain up to date information by

writing to the below address.

Roger Balister, G3MKA

-55-

LOGPlus! version 3.0

IOTA Director and Awards Manager

LA Quinta, Mimbridge

Chobham, Woking

Surrey GU24 8AR

England

NOTE: DMW Systems or Robert A. Winters, KD7P provide the interface,

nothing more.  You will be required to maintain the IOTA.DBF file on

your own.

Bands & Modes Tracked

=-=-=-=-=-=-=-=-=-=-=

General Award only.

Required Files

=-=-=-=--=-=-=

Three files are required to track the IOTA award program.  If

any of these files are missing, NONE of the IOTA features

described herein will function.

IOTA.DBF, IOTANF.DBF and IOTA.NSX  (the NSX file is created

automatically during the indexing process).

A>  Auto Update

=-=-=-=-=-=-=-=

This module is the heart of the IOTA award system.  It's major

purpose is to reduce the time and effort required to manually

update and track your IOTA award.

The auto update will:

1>  Cross reference LOGBOOK entries with those in the IOTA

databases and update appropriately.  This module is

approximately 1000% faster then earlier versions of LOGPlus!.

Islands On The Air (IOTA)

Award Auto Updating

Updating IOTA reference number:  EU022

IOTA reference numbers updated:     32

IOTA reference numbers NOT found:      5

Updating IOTA reference number:  << Current reference number

being queried.

IOTA reference numbers updated:  << Number of reference numbers

found and cross referenced.

-56-

LOGPlus! version 3.0

IOTA reference numbers NOT found: << Number of reference numbers

which could not be found in the IOTA databases.  This indicates

your IOTA database is not up to date with current islands.  Use

the manual update module to bring your databases up to date.

If this number is greater than 0, then you'll be allowed to

print out a list of reference numbers and their associated

calls to your printer for easy updating.

B>  Manual Update

=-=-=-=-=-=-=-=-=

This modules allows you to manually view and edit data for a

particular reference number quickly and easily.  Those fields

marked with "*" can be edited in this module.  To edit other

fields, use the Browse/Modify Editor feature.

*RefNo               *Island Group

EU005                British Isles

Prefix(es)          *Island Name

G,GM,GW

Continent           *Comments

European/B

Position            *Callsign       *QSO Date      *Status

53N 02W             G3UID           09/10/78      Confirmed

1>  Enter the 5 character reference number in the RefNo block.

If the data was found, all the blocks will be updated with

information relating to the reference number.

Ref No:  5 character IOTA reference number.  Number MUST be

proceeded with either EU, SA, NA, OC, AF , AN, or AS.  If the

RefNo is EU-006, remove the '-' and use only EU006.  If you wish

to view UN-Referenced information, enter just the continent

abbreviation and press <ENTER>.  The LAST record in the database

which matches the entry will be displayed.  To view other

UN-Referenced numbers, use the F5 - Prev RefNo or F6 - Next

RefNo function keys.

Prefix:  Prefix is the standard prefix assigned to the island

group.  See IOTA Directory for additional information.

Continent:  Continent island is located in. If the continent is

followed by a /A, /B or /W, this indicates the island falls

under the IOTA area awards.  See IOTA Directory for additional

information.

QTH:  Approximate location of island. These positions may not

-57-

LOGPlus! version 3.0

match those listed in the IOTA directory.  See IOTA Directory

for additional information.

Callsign: 12 character alphanumeric field.  This field will be

automatically updated if the IOTA Auto Update is used. If not,

enter the call desired.

QSO Date: Date format.  This format must match date format used

in log.

Status:  Provides status of displayed island. The following

entries can be used.

Worked: Worked, QSL may or may not have been sent.

Confirmed: Confirmed contact.  QSL has been received.

Submitted: Card has been submitted to the IOTA awards board

for checking.

Approved: Card has been approved and accepted by the IOTA

award board.

Island Name:  Enter the Island actually worked.  This information

is extremely important when submitting for the award.

Island Group: Island groups list the major group in which the

islands are associated with.  See function keys for editing

features. See IOTA Directory for additional information.

Function Keys

=-=-=-=-=-=-=

Four function keys have been provided to assist in viewing and

updating information.

F5=Next RefNo: Scroll record down one.

F6=Prev RefNo: Scroll record up one.

F7=Chng RefNo: Allows you to update UN-Referenced data.

F8=Modify Islands: Places cursor in Islands field and allows

you to modify the data.

C>  Browse/Modify

=-=-=-=-=-=-=-=-=

See Getting Around the Browse/Modify.

When first selected, only data which has been assigned a REFERENCE

number are displayed.

-58-

LOGPlus! version 3.0

F3=RefNo Search   -  Allows you to quickly move between different

reference numbers.

F5=RefNo          -  Displays only those records with assigned

5 digit reference numbers.

F6=UnRefNo        -  Displays only those records which have not

been assigned a 5 digit reference number.

F9=Chng Continents-  Allows you to select any of the other awards.

D>  Statistics

=-=-=-=-=-=-=-

This feature provides a quick breakdown of the number of QSOs

worked in each of the various awards.

Needed  Worked  Confirmed  Submitted  Approved

Islands of the World  1159

Africa                  73      32       45

Antarctica              50

Asia                   173      10       10

etc...

E>  Printing

=-=-=-=-=-=-

The printing option is menu driven for quick selection.  Each

award is broken down individually to reduce paper waste.

1>  Select the award

2>  Select the status

The print out will match the standard award form which can be

either sent to the RSGB or checked by one of many check points.

F>  Indexing

=-=-=-=-==-=

Recreate the IOTA.NSX file.  This is necessary only if data

becomes corrupt.

J>  Worked All Russian Oblasts

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

With the breakup of the Soviet Union, the old Soviet Oblast Award

was discontinued.  A new award Worked All Russian Oblasts (W.A.R.O.)

was created.  This award became effective on 1 January 1992.  All

contacts with Russians in the call group R, UA to UI will count

towards this ward.  In addition a 5BWARO award is being offered

for contacts made on 10,15,20,40,80mtrs only.

-59-

LOGPlus! version 3.0

A>  Auto Update

=-=-=-=-=-=-=-=

Check logbook for all Russian QSOs.  Update the W.A.R.O.

database with contacts after 1 Jan 92.  All other Russian entries

are disregarded.

B>  Browse/Modify

=-=-=-=-=-=-=-=-=

When first selected, only those records associated with the

General Mixed Award are displayed.  Use F5 to F9 to change

modes and bands.

C>  Band/Mode Statistics

=-=-=-=-=-=-=-=-=-=-=-=-=

This module provides you with a complete breakdown of your current

oblast status by band and mode.  This format is standard throughout

LOGPlus!, see WAZ E> Band/Mode Statistics for general information.

D>  Printing

=-=-=-=-=-=-=

The printing option is menu driven for quick selection.  Each

band is broken down individually to reduce paper waste.  Once

you've selected the band or award you wish to print, you'll

be prompted to select the mode you wish to print.

E>  Indexing

=-=-=-=-=-=-

Recreate OBLAST.NSX file.  This is only required if difficulties

occur locating data.

K>  Worked All U.S. Counties

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Worked All U.S. Counties Award system is provided, to ASSIST users

in tracking the U.S.A. Counties Award.  This program is not intended

to replace the Record Book currently required by the Counties Award

committee, but to provide document control.  You are encouraged to

write CQ Magazine (sponsors of the award), and obtain the rules and

regulations as well as the Record Book.  This feature will track the

Mixed, Phone and CW awards.

A>   Auto Update

=-=-=-=-=-=-=-=-

Tracking this award is not easy.  Alot of time and effort can go into

maintaining the Record Book and ensuring the data is correct. LOGPlus!

has taken the hassle out of maintaining the Record Book by providing a

means in which data can be automatically extracted from your log.  The

NOTE field as mentioned previously, is extremely important to the

Counties Auto Update feature. Data is entered in the NOTE field during

-60-

LOGPlus! version 3.0

QSO entry, which can be extracted during the update processes.

The NOTE Field

=-=-=-=-=-=-=-

The County Hunters Award utilizes a County Reference Sheet (see

COUNTY.DOC for listing), in providing the necessary interface for

automatic update.  The COUNTY.DOC file contains a list of all 3076

U.S.A. Counties, their respective states and record number. The record

number is the KEY to the automatic updating.

EXAMPLE:

1                   Alabama                Greene

255                   Colorado               Larimer

W4XYZ reports to you his location is Tucker in Greene County Alabama

during your QSO.  After filling in all the standard log information

(i.e. call, date, time, rst, name etc...) you're ready to enter the

data in the note field.  The characters { and } are unique characters

which are utilized by the Counties Award program ONLY.  Do not use

either of these for the Generic Award program.

The first character { is a delimiter which specifies the starting

point of the Counties update.  When this character is encountered by

the update process, the data listed between the {} is extracted and

cross referenced with the three COUNTIES file. The last character } is

a delimiter which specifies the end of the Counties update.  To log

W4XYZ correctly, the note field would contain:

{1Tucker}

However, because the note field is used by the Generic Award program

as well as standard information, the exact location of the Counties

data is not critical.

EXAMPLES OF A TYPICAL NOTE FIELD:    !10232{1Tucker}

!10232 could be data used by the Generic Award program to keep track

of the 10x10 award.

HVY QRM{1Tucker}   or  {1Tucker}HVY QRM

HVY QRM is a reminder to you of heavy interference during the QSO and

has no bearing on any award system.

As you can see, the exact location is not critical.  Another example

would be that during mobile operations, W4XYZ reports he's in Larimer

County Colorado between different cities.  Because the exact city is

not known, the information between the {} characters would be entered

as:  {255X}  The 255 represents the record number and the X is used to

-61-

LOGPlus! version 3.0

inform the update that no city was specified. Make sure you use the X.

If you don't the record will not be updated.

The Update Process

=-=-=-=-=-=-=-=-=-

You'll be asked if you wish to update the Counties data from the

beginning of the log.  If you press "Y" the record counter is reset to

1 and update will commence from record 1 of the log. This is only

recommended if you've made a number of major changes or you're lazy

like I am and don't wish to do all the updating manually!  By press

"N" the record counter is set to the last record updated and will

commence from that record in the log.

The next screen you'll see will show the update process.

Processing:  1235

Remaining: 23232

Viewed:     2

Mix Updated:     2

Phone Updated:     1

CW Updated:     1

Processing:  Current record being checked in the log.

Remaining:   Number of log records still pending.

Viewed:      Number of records in the log found to contain the correct

information (i.e.  {1Tucker} etc..)

Updated:     Number of records actually updated in one of the COUNTY

files.  This number may be lower than the viewed number.

The viewed number takes into account ALL records viewed.

Update only counts those records that are actually

changed.

B>  Browse/Modify

=-=-=-=-=-=-=-=-=-

When first selected, only those records associated with the

mixed aAward are displayed.  Use F5 to F9 to change mode and bands.

C>  Statistics

=-=-=-=-=-=-=-=

Statistics provides a quick view of the current status of your

Counties award.  Data will be displayed in three blocks with

four columns each.  Here is an example of one block.

State  Counties  Wkd  Apvd

AL       67       10     9

AK        4        1

-62-

LOGPlus! version 3.0

State:     List the common two letter state abbreviation.

Counties:  Number of counties located with the state.

Wkd:       Number of counties worked in that state.

Apvd:      Number of counties approved by the awards committee.

D>  Printing

=-=-=-=-=-=-=

Printing features allows you to print your data by individual

states.  A pop up menu will allow you to specify the award status

to print.  Menu options are self explaintory.

E>  Indexing

=-=-=-=-=-=-

Recreate required database index files.  This is only required if

problems are encountered locating data.

L>  Generic Award System

=-=-=-=-=-=-=-=-=-=-=-=-

The Generic Award System program is simple, but extremely powerful.

This program will allow you to keep track of up to twenty (20) awards

of your choosing.  Seven data fields most commonly used to track

awards are maintained.

Call, Date, Time, Freq, Mode, Rcvd RST and Info/Note

The log NOTE field is the key field used to maintain UNIQUE data

associated with the award(s) you plan on tracking.

When you first select the Generic Award System program, 21 menu

selections are available to choose from.  However, the first 20 will

say "Generic Award Tracking System" which allows you the ability to

"ROLL YOUR OWN".  Whatever award(s) you'd like to track (within

limitations, sorry!), can be tracked using one of these menu selections.

how it works

=-=-=-=-=-=-

You wish to keep track of the Japanese JCC Award and the 10x10 Award

Programs.  If you tried to select A> thru T> the computer will beep

then shoot you right back to the main menu.  The reason for this is no

parameters have been set for these fields.  First, select option U> and

bring up the award parameter screen.  This screen will allow you to

provide the program with UNIQUE parameters for the award you wish to

track.

Like many of the function in LOGPlus!, the Generic Award System

-63-

LOGPlus! version 3.0

utilizes a Browse/Modify screen which you should be very familiar

with at this point.  Modifying data is as simple as pressing ENTER

to enter the EDITOR which will allow you to create the necessary

variables.

The first parameter is the name of the award, up to 30 characters can

be used to specify the name.  Enter:

Japanese JCC Award Program

The next parameter asks you to come up with a UNIQUE file name for the

database file.  Up to 8 characters can be specified.  Enter:

JAJCC

The next parameter asks you to provide a Special Identifier character.

This is where things could get a bit confusing.  This Special Identifier,

is used in the log NOTE field to act as a marker.

When the Auto Update is used, the log NOTE field is checks against the

Special Identifier. If a match is found, data is extracted and

placed in the database file you specified.  In this case JAJCC.DBF.

An important note, you CAN NOT specify the same character for the

different awards.  The characters { and } have been reserved for the

COUNTIES Award program and CAN NOT be used here, UNLESS you have no

intentions of tracking COUNTIES.  Enter:

#

The next parameter asks you to provide a set number of search characters

the program will extract from the log NOTE field to distinguish the data

you wish to track.  Let say you're tracking the JCC and need the JCC

number which in this case is 3 characters long.  I would specify in this

field a 3 and press <ENTER>.  NEVER specify more then you need.  The

program has been written to extract from the log NOTE field the exact

number of characters AFTER the UNIQUE search character and no more, no

less.  Enter:

3

The last parameter asks if you wish to delete the data from the log after

the update has taken place.  Okay, this is where the exception comes into

into play I mentioned earlier!  Let say you have the following entry

in your log.

Call     Date      Time   Freq  Mode    Rcvd  Name     Note

JA1ELY   11/10/91  10:45   14    CW     589   Toshi    #283&26991

If you specified YES to this parameter, during the Auto Update,

-64-

LOGPlus! version 3.0

the program will look for and see the pound sign # when it came

to this entry.  The generic data necessary to track most awards would

be copied over to the JAJCC file you specified earlier (Call, Date, Time,

Freq, Mode, Rcvd).  The next thing the program would extract is the next

3 characters (you specified this also) following the pound sign #, and

place this data in the INFO field of the JAJCC file.  Next, the pound

sign followed by the next 3 characters (#283) would be deleted from the

log NOTE field.   As you can see, &26991 would now become the first 6

characters allowing us to do an Auto Update for the 10x10 Award we also

wish to track. Remember, you must conduct the search for the first

Special Identifier, in this case #, before you could update the

10x10 award.

Call     Date      Time   Freq  Mode    Rcvd  Name     Note

JA1ELY   11/10/91  10:45   14    CW     589   Toshi    &26991

IMPORTANT NOTE:  The Auto Update ALWAYS starts it's search from the

beginning of the log.  If you DO NOT delete the data after it has been

used, each time the update is run, the records will again be copied

over to the database file causing duplication.

As you can see, by answering YES to this parameter, additional awards

can be tracked using the same entry and save duplication in entering

data manually!  If however, you answer NO to this field, the entry

would not change and use of this entry for other awards would require

manual intervention.

You'd use this process for all five awards or any number you wish to

track. Remember make sure you use UNIQUE file names and search

characters for each award.

The information described here effects ALL menu selections A>

through T>.  Once all the parameters have been set for the award(s)

you wish to track, the next thing is to input data. When the menu

is selected the above menu will pop up allowing you to do various

functions.  Remember that the generic system was not designed as a

tracking program for all awards. No doubt some awards will be

difficult to track; however, the system was designed to make life

a bit easier.

A>  Auto Update

=-=-=-=-=-=-=-=

The Auto Update provides a quick method of automatically updating

each of the 20 generic awards.  As mentioned above, the Special

Identifier is the key to the auto update.  Make sure you follow the

instructions listed in "how it works".  Once these parameters have

been set properly, everything is strictly automatic from this point.

B>  Browse/Modify

=-=-=-=-=-=-=-=-=

-65-

LOGPlus! version 3.0

See Getting Around the Browse/Modify.

When first selected, only those records associated with the

General Mixed Award are displayed.  Use F5 to F9 to change

modes and bands.

F3=Append Data:     Add new data to the databases.  This feature

is identical to the manual update in previous

version of LOGPlus!.

F9=Index Order:     This allows you to change the index order

as well as the order of the display.

Enter=Editor:       The editor will allow you to quickly modify ALL

data fields associated with the record currently

highlighted.

C>  Printing

=-=-=-=-=-=-

Printing will allow you to print the data in 7 different index

orders.  Each award printed will consist of a cover sheet which

contains basic information necessary to submit most awards.  If

this form is not satisfactory, you'll need to develop your own.

Each award will print out 50 contacts per page.  At the bottom of

each page a listing of the number of contacts per page and the

overall total of contacts is printed.

D>  Indexing

=-=-=-=-=-=-

Recreate the generic award index (.NSX) file.  This is necessary

only if data becomes corrupt.

M>  Dumb Terminal Program (DTP)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

The Dumb Terminal Program (DTP) is a powerful module offering

flexibility, power and a wealth of features.  The main objective of

the DTP is to provide a sound interface between all the popular

TNC's and their internal programming.  Please refer to your TNC

manual for command and control.  Remember, the DTP is general in

nature and is NOT limited to PacketCluster monitoring only.  Any

mode your TNC can operate on, can be accessed thru the DTP program.

When you access the DTP for the first time, you'll need to provide

the program will default parameters.  This feature can be accessed

at anytime by pressing ALT_O from within the DTP.

For program control see Help! (ALT_H) for additional information.

Status Line (bottom line)

=-=-=-=-=-=-=-=-=-=-=-=-=

-66-

LOGPlus! version 3.0

ALT_H = Help!Cap OffPrntr OfflinePCluster OffAlarm OffSpoct Inactive

The status line provides the current status of the Data Capture,

Printer, PacketCluster link, alarm and special spot status.

Main (center) section

=-=-=-=-=-=-=-=-=-=-=

The Main section is used for data processing (send/receive) as well

as pop up windows and other related information.  The size of this

window has been defaulted to line 0 column 0 to line 23 column 79.

However, the size is effected by the PacketCluster and Radio switches.

PacketCluster

=-=-=-=-=-=-=

PacketCluster by AK1A has become a major operating tool in

nearly every serious DX'er shack.  With this tool, features had

to be developed to provide as much of a seemless monitoring

operation as well as the ability to check spots for various

award information.  Version 3, has been developed to provide

a powerful interface which is user friendly and flexibly.

PacketCluster monitoring is provided in the dumb terminal

program as well as the Realtime and Offair logging modules.

During a spot, LOGPlus! will quickly determine the DXCC

country of the spot, report if the country is needed as a new

DXCC country as well as report IOTA and GRID information.

However, there are always limitations and parameters which

must be met for these features to work.  Overall, the DXCC

country section is seemless and requires no special features

to operate.  During a typical DX spot, the top portion of

the DTP screen will look something like this:

Date: 08/23/93  Time: 1414Z   DX: DL3NY         Freq:  7003.3

Country: DL - Germany                           Zone: 14/24    Cont: Eu

Modes: CW  SSB  RTY  MIX    Bands: 160 80 40 30 20 17 15 12 10  6 2 SAT

The Modes and Bands will be hightlighted if the country is needed.

In addition two additional features have been added which will

assist in the tracking of your GRIDS and IOTA awards.  These

features do require special consideration and are explained in detail.

Examples:

DX de KD7P:  50110.0   W1NG          CN99 Ken good signal.     1414Z

DX de KD7P:  14086.7   VE7IU         NA033 Rees on XYZ Isl.    1414Z

or

DX de KD7P:  14086.7   VE7IU         IOTA NA033 Rees XYZ Isl.  1414Z

Basically, the GRID/IOTA combination must be the first set of

-67-

LOGPlus! version 3.0

characters sent in the comment field.  We must also try to discourage

the use of "-" or spaces within the combination. (i.e. NA-033 or

NA 033).

If we can establish this basic format, LOGPlus! will be extremely

successful in providing up to the second information.

IOTA - To support the ever popular IOTA system, I have provided a

means in which the IOTA database (IOTA.DBF) will automatically be

checked if an IOTA REFNO (EU001) is reported in the comment field.

With the many methods in which data is reported in the comment field,

I can not provided any assurances this system will work 100% at all

times.  The following will work:

If the first 4 characters in the comment field or after the

word IOTA are either EU001, eu-001, EU-5, EU 5, EU 001 etc.

the program will extract the data and alerting provided.

( Basically, any upper/lower case combination ).

Notification will be given in the upper left hand corner next to the

date field in flashing GREEN. See below for example.

REFNO       STATUS BLOCK OF IOTA.DBF

=====================================

EU001(N)    >> Not found in database -- Needed

EU001(A)    >> Found -- Approved

EU001(S)    >> Found -- Submitted

EU001(C)    >> Found -- Confirmed

EU001(W)    >> Found -- Worked

GRIDS - Each time a 6 or 2 meter spot is reported, the first 4

characters of the comment field will be checked against the

appropriate database. (VUCC50 or VUCC144 if previously created).

Notification will be given in the upper right hand corner next to

the frequency field in flashing GREEN.  See below for example.

GRID       STATUS BLOCK IN VUCC50.DBF

=====================================

CN89(N)    >> Not found in database -- Needed

CN89(A)    >> Found -- Approved

CN89(S)    >> Found -- Submitted

CN89(C)    >> Found -- Confirmed

CN89(W)    >> Found -- Worked

Overall, the PacketCluster system will provide a wealth of information

quickly and effectively.

CTRL_F1 Radio Interface

=-=-=-=-=-=-=-=-=-=-=-=

-68-

LOGPlus! version 3.0

Toggle switch which will turn ON/OFF radio interface.  Periodically,

it may be necessary to toggle this on/off a few times to "WAKE UP"

the radio!

CTRL_F2 Change Frequency

=-=-=-=-=-=-=-=-=-=-=-=-

This feature is extremely important!  When a frequency is entered,

the program will automatically default to VFO A (Kenwood Only).

If the currently displayed VFO is B, you'll not notice any screen

changes. The following formats can be used to change frequencies:

A14.023  =  Change VFO A to 14.023

14.023  =  Change VFO A to 14.023

B14.023  =  Change VFO B to 14.023

14       =  Change VFO A to 14.000

CTRL_F3 Change Modes

=-=-=-=-=-=-=-=-=-=-

This is a toggle switch which will cycle through the available

modes on your radio.

CTRL_F4 Change  VFOs (Kenwood Only)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Toggle switch which will change between VFO A and B or vice versa.

CTRL_F5 Up Freq (10Hz)

=-=-=-=-=-=-=-=-=-=-=-

Change VFO frequency by 10Hz upwards.

CTRL_F6 Dn Freq (10Hz)

=-=-=-=-=-=-=-=-=-=-=-

Change VFO frequency by 10Hz downwards.

CTRL-F7 Drop to DOS Shell

=-=-=-=-=-=-=-=-=-=-=-=-=

Allows full DOS access.  Type EXIT and press ENTER to return to

LOGPlus!

CTRL_F8 thru CTRL_F10 External Programs

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

See ALT_N Program Default Parameters for general use.

ALT_F1 Turn PacketCluster On/Off

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Toggle switch which turn PacketCluster On/Off.  Screen size will

change depending on toggle.

ALT_F2 Packet Spot Browse

=-=-=-=-=-=-=-=-=-=-=-=-=

-69-

LOGPlus! version 3.0

Each time a SPOT is displayed, the information is automatically

saved to the PacketCluster database file (PACKET.DBF).  The major

purpose of this is to provide a FAST method of checking what was

reported without clogging the PacketCluster system with SH/DX

requested.

F2=Delete Entry:   Delete currently highlighted entry.

F3=Delete ALL:     Delete ALL entries in file.

F5=Date Order:     Rearrange spots in date-time order.

F6=DX Order:       Rearrange spots in dxcc order.

F7=Need Order:     Rearrange spots in needed order.

F8=Freq Order:     Rearrange spots in frequency order.

F9=Print:          Print complete listing of all spots in database.

F10=Reindex        Recreate the PACKET.NSX file.

ALT_F4 Packet Spot Announcement

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Once in a while you have the opportunity to report on the

PacketCluster DX which you SPOT and no one else.  Rather than

type out the DX de line and make sure you format it correctly,

LOGPlus! provides you with a quick and easy way to enter and

transmit the data.  Remember one thing, when you use this feature,

the data you transmit will also display on your screen.  This may

cause your PacketCluster Alarm to sound!  In addition, if you're

using the radio interface, the frequency from the radio will be

read into the spot frequency field.

Here is a sample screen of the PacketCluster SPOT.

[ Packet SPOT Announcement ]

DXStation: KD7P/NH7

Spot Freq: 14023.0

Comments:  Excellent signal QSX up 5

Once you press <ENTER>, the data is automatically sent.

DX de AA6TT:     14023.0  KD7P/NH7    Excellent signal QSX up 5  1242Z

ALT_F5 Packet TALK ( Realtime and OffAir Logging ONLY)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

At times you may which to TALK with another station who is logged

into the PacketCluster system while in the logging modules.

-70-

LOGPlus! version 3.0

[ Packet Talk ]

Station: JH1ORA

Comment: HELLO KAZ-SAN! DID YOU WORK THE P5?

Enter the station callsign followed by a 60 character comment line.

Each time the <ENTER> key is pressed the data will be transmitted on

the system.

ALT_F6 Packet Spot SetUp

=-=-=-=-=-=-=-=-=-=-=-=-

The Packet Spot SetUp screen will allow you to set "SWITCHES" to

report DX on particular band and modes.  This is an excellent

method of removing unwanted spots from bands you don't operate

on.  In addition, you can quickly establish frequency parameters

depending on your operating technics.

Examples:

Your a CW operator and could careless about SSB or RTTY spots.

By turning OFF all the SSB and RTTY alarms, only spots which

fall within the established frequency parameters for CW will

sound the alarm.

Warning:  DO NOT overlap frequencies.

As a standard default to alarm sounding, only those countries

which are needed will set the alarm off.  However, a switch has

been provided which will allow you to also sound the alarm if

the country is worked by not confirmed.

In addition to that, three additional switches have been provided

which will allow you to turn on or off IOTA and GRID search.

Check IOTA status?   << If you're an IOTA chaser, turn this on

Check 6m grid status?   << If you're a 6 mtr grid chaser, turn this on.

Check 2m grid status?   << If you're a 6 mtr grid chaser, turn this on.

ALT_F7 Special Spot Filter

=-=-=-=-=-=-=-=-=-=-=-=-=-

One of the new features to LOGPlus! is the ability to FLAG spots

which will automatically sound your "CHARGE" alarm.  The best way

to describe this feature is by example.

Example:  You are one of those individuals who has nearly everything.

You've worked the 3Y0/P before but when the next DXPedition comes up

you want to be ready.  The special spot filter system will allow you

to enter any call or prefix.  Whenever that call or prefix is reported

on the system, the program will sound the alarm.  This is a totally

separate system from Packet alarm system.  In addition, whenever a

special call is pending in the queue, the screen will display

-71-

LOGPlus! version 3.0

Spot Active! in the right hand corner.  No calls in the queue, will

show Spot Inactive.

ALT_F8 Packet Sound On/Off

=-=-=-=-=-=-=-=-=-=-=-=-=-

Toggle switch to turn sound On/Off.  Each time a SPOT which you

need is reported, a "CHARGE" tone will be sounded until you turn

it off by using either ALT_F8 or ALT_F9.  By using ALT_F8 the sound

will be silenced until ALT_F8 is pressed again.  If you're busy

doing something else around the shack or house, you may miss a rare

one! I use this at nights when I'm ready to turn in.  Obviously my

XYL gets mightily upset at 3AM when the computer starts YAPP'n!

ALT_F9 Packet Silencer (Current Spot Only)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

As a TEMPORARY alternative to ALT_F8, I've added this switch to turn

off sound just for the previously displayed SPOT.  This way, when

the next SPOT is reported and you need it, you'll be alerted.

NOTE:  The "CHARGE" sound will not stop until you stop it with

either ALT_F8 or ALT_F9.  When you're not home make sure you use

ALT_F8 so you're XYL or OM doesn't throw your system out the window!

ALT_F10 Packet Commands ( Realtime and OffAir Logging ONLY )

=-=-=-=-=-=-=-=-=-=-=-=

The Packet Commands line allows you to query the PacketCluster for

information like DX, WWV information or general database information

while in the logging modules.

[ Packet Commands ]

Enter Command:  SH/DX/5

Each time the <ENTER> key is pressed the data will be transmitted

on the system.  Your query if entered correctly will be promptly

answered by the PacketCluster system.

ALT_A Display SunRise/SunSet for Cities

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display a standard browse/modify screen which displays over 295

cities throughout the world.  See the DX Century Club (DXCC)

section for general information on use.

ALT_B Scroll Buffer

=-=-=-=-=-=-=-=-=-=

LOGPlus! will store the last 6,000 characters received in a

buffer for quick viewing.  Use standard keys described in

the first section of this manaul to get around.  No editing

features are provided, viewing only.

-72-

LOGPlus! version 3.0

ALT_C Clear Screen

=-=-=-=-=-=-=-=-=-

Clear the data input window.

ALT_D DXCC Four Others

=-=-=-=-=-=-=-=-=-=-=-

DXCC Four Others is a new feature added to LOGPlus! v3 to allow

LOGPlus! users the ability to share DXCC information.  This is

particularly useful for those individuals who do not have access

to a PacketCluster but a near by friend does.  Under the Program

Default Options (ALT_O - Main Menu), you have the ability to

enter up to 4 callsigns.  This information is important to the

DXCC Four Others to function.  In addition, a few very important

procedures MUST be followed for this to work.

how it works

=-=-=-=-=-=-

KW7Y/Paul and KD7Z/Jack are very interested in DX chasing but

don't have access to a PacketCluster.  They usually pass around

the PAPER DXCC listing out to friends asking them to call when

DX they need is reported.  The DXCC Four Others removes the

PAPER!

Under the Program Defaul Options, you entered:

Call sign of 1st DXCC databases:   KW7Y

Call sign of 2nd DXCC databases:   KD7Z

Call sign of 3rd DXCC databases:

Call sign of 4th DXCC databases:

Next you MUST get a copy of their DXCC.DBF and DXCC.NSX files.

They MUST be renamed to the call sign, and copied into your

\PLUSV3 directory.

Example:

Rename KW7Y's DXCC.DBF and DXCC.NSX to KW7Y.DBF and KW7Y.NSX.

Rename KD7Z's DXCC.DBF and DXCC.NSX to KD7Z.DBF and KD7Z.NSX.

From this point on, whenever you press ALT_D, the screen

will display the DXCC statistics for the SPOT reported.

NOTE:  YOU CAN NOT MODIFY THEIR FILES.  IF THEY MAKE CHANGES

YOU WILL BE REQUIRED TO OBTAIN NEW COPIES.  MAKE SURE YOU

BACKUP YOUR DXCC.DBF FILE PERIODICALLY IF YOU USE THIS

FEATURES TO REDUCE THE POSSIBILITIES OF ACCIDENTLY OVERWRITING

YOUR OWN DXCC.DBF AND DXCC.NSX FILES!!!

ALT_F  Data File Capture

=-=-=-=-=-=-=-=-=-=-=-=-

-73-

LOGPlus! version 3.0

Data File Capture allows you to save data as it comes in.  Just

enter the file name you wish the data to be saved to.  Next time

you press the ALT_F key, the file will be closed and saved to

your hard drive.

ALT_H Help!

=-=-=-=-=-=

Basic help screen listing the programmed function keys.

ALT_J Jump to Spot

=-=-=-=-=-=-=-=-=-

Jump to Spot is used inconjunction with your attached Kenwood or

ICOM radio.  Whenever a spot is reported over the PacketCluster,

the ALT_J key will cause your radio to shift to the frequency of

the spot.

ALT_K KeyMenu - Programmable Keys (F1 thru F10)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

One of the powerful features of LOGPlus! is the ability to program

your F keys to reduce typing.  Radioteletype operations (ASCII,

BAUDOT,GTOR,PTOR etc..) will benefit the most from these keys.

how it works

=-=-=-=-=-=-

Programming any of the F keys is easy.  Just type in the

information you want to send, it's that basic.  However, one

of the added features new to version 3, is the ability to

send files as well as plain text.  7 standard text files can

be created and sent.  These file can be any length, but must

be named as one of the following and be located in your \PLUSV3

directory.

CQ.DTP   RYRY.DTP   TEST.DTP   GEN.DTP   BRAG.DTP

DX.DTP   INFO.DTP

In addition, it's extremely important you TNC has HARDWARE

CONTROL capabilities and they be used.  On the PK232 units,

under the options (ALT_O), set Terminal xon/xoff to YES.

During the file transfer, you'll notice the screen periodically

halt output while the TNC buffer empties.  Also, you may

notice GARBLE on your screend during the transfer, this is

a resolve of ECHOing back to the screen from the TNC.  It

is NOT what is being transmitted.

Here is an example on programming the F1 key to send the BRAG

tape out:

F1 :  Here is my brag tape  \\BRAG  ^

-74-

LOGPlus! version 3.0

The two backwards slant bars are REQUIRED to transmit a

file.  This is a delimiter which tells LOGPlus! that the

following is a text file which should be sent.  The "^"

caret key (SHIFT 6) is a CRLF which will be transmitted.

NOTE:  After you save and exit the KeyMenu function, the

"^" will be converted by LOGPlus! to another ASCII character.

This is normal and should not be changed back.

Here is an example on programming the F2 key to send a SHOW

DX to PacketCluster node:

F2 :  SH/DX^

ALT_L Logging Module

=-=-=-=-=-=-=-=-=-=-

A popular feature in version 2, this feature provide a quick and

easy way to enter QSOs into your logging module.  QSOs are saved

to the LOGTEMP file and updated to the general log only after the

user enters the realtime logging module.

ALT_M Temperature/Mileage

=-=-=-=-=-=-=-=-=-=-=-=-=

Temperature/Mileage conversion is a quick display which will allow

you to either either fahrenhiet or celcius and obtain the conversion.

Mileage, either in miles or kilometers can also be obtained.

ALT_O Options/Parameters

=-=-=-=-=-=-=-=-=-=-=-=-

1.  Terminal comm port: - Select the COM port which your TNC is

connected to.  You'll notice that only those ports which the

program could recognized are selectable.  IRQ's can not be selected.

2.  Terminal baud rate: - Select the baud rate your TNC can operate

at. The highest baud rate (usually 9600) is recommended.  If your

TNC can accept data faster, then select that speed.

NOTE: KAM Users:  I have found at least with frameware v3.0 that the

"*" function does not work correctly.  If your KAM does not respond,

change the baud rate and try again.  The ABAUD command of the KAM

and the Terminal Baud Rate MUST match.

3. Terminal data bits: - Select the data bits best suited for your

TNC. 8 is the common data bits used by most TNCs.

4. Terminal parity: - Select the parity best suited for your TNC.

NONE is the common parity used by most TNCs.

5. Terminal stop bits: - Select the stop bits best suited for your

-75-

LOGPlus! version 3.0

TNC. 1 is the common stop bits used by most TNCs.

6. Terminal xon/xoff: - This feature is related to flow control. To

start off set xon/xoff to "N".  If you run into difficulties or data

is lost between the TNC and LOGPlus! shift this to "Y".  Your TNC

manual will explain xon/xoff for your particular unit.

7. Terminal rts/cts: - This is the REQUEST TO SEND/CLEAR TO SEND

control line.  Most TNC's will not require this feature.  This has

been added "JUST IN CASE" your TNC requires software control. The

default is "N". Your TNC manual will explain RTS/CTS for your

particular unit.

8. Terminal dtr/dsr: - This is the DATA TERMINAL READY/DATA SEND

READY control line.  Most TNC's will not require this feature.

This has been added "JUST IN CASE" your TNC requires software control.

The default is "N".  Your TNC manual will explain DTR/DSR for your

particular unit.

9. Radio interface installed: - If you wish to utilize the DTP for

controlling your radio, turn this feature ON.  Only limited control

is provided.

10.  Radio attached: - Select the type of radio attached.

11.  Radio comm port: - Specify the comm port of your radio.

If radio interface (9 above) is "YES", you must specify a comm port

different from the TNC.  If 9. above is "N", select the same com port.

12. Radio baud rate: -  This is defaulted to 4800 per the IF-232c

manual. Unless you've physical modified the circuit board of the

IF-232c, leave this at 4800.

13. Radio data bits: -  This is defaulted to 7 per the IF-232c manual.

14. Radio parity: - This is defaulted to NONE per the IF-232c manual.

15. Radio stop bits: - This is defaulted to 2 per the IF-232c manual.

16. Local Echo: - Local Echo allows you to echo back to the DTP

everything you type.  This can also be set in your TNC.  If you

receive DOUBLE CHARACTERS during data input, turn this feature OFF.

17. Add LF to CR: - Some TNCs strip the Carriage Return from incoming

data.  If this occurs, turn this feature ON.

ALT_P Printing

=-=-=-=-=-=-=-

Toggle switch to allow incoming and outgoing data to be sent to the

-76-

LOGPlus! version 3.0

printer.  This is a virtual system whereby all data inputted by the

keyboard will be sent to the printer, this means BACKSPACES also! If

this feature is turned ON and no printer is found, the program WILL

NOT crash. Status line will update accordingly.

ALT_R Return from Spot

=-=-=-=-=-=-=-=-=-=-=-

Once you've Jump to a Spot, by pressing the ALT_R keys your radio

will return to the previous frequency displayed.  However, if you

pressed the ALT_J keys more than once, this feature will not work.

ALT_S Send File

=-=-=-=-=-=-=-=

DTP allows you to transmit ASCII files to other users.  When ALT_S is

pressed, a pop up window will allow you to select the file you wish to

transmit.  If the file you wish to transmit is not in the current

directory, by pressing ALT_P you can change the PATH the program will

search.  You can also select only those extension files you want to

display by using the ALT_M and specifying a MASKING.

NOTE:  It may be necessary to use the xon/xoff and dts/dtr features of

this program depending on your TNC.  It may take a few tries before you

get a hang of it.  Your TNC manual will expand upon this. See ALT_P

Options/Parameters for additional information.

ALT_T HamBase

=-=-=-=-=-=-=

Run HamBase in a DOS environment.  See Program Default Options

ALT_U SAM

=-=-=-=-=

Run SAM in a DOS environment.  See Program Default Options

ALT_V BuckMaster

=-=-=-=-=-=-=-=-

Run BuckMaster in a DOS environment.  See Program Default Options

ALT_W QRZ

=-=-=-=-=

Run QRZ in a DOS environment.  See Program Default Options

ALT_X Exit to log

=-=-=-=-=-=-=-=-=

Exit to log closes the communications port and clears all open files.

NOTE:  Even though you are required to establish a packet link in the

DTP prior to entering the Realtime and OffAir modules, this will NOT

disconnect you from the PacketCluster.  Once you've entered the logging

module, the link between the software and TNC is reestablished. Incoming

-77-

LOGPlus! version 3.0

data maybe lost depending on the type of TNC. However, if you sent

XFLOW OFF in your TNC, data will in most cases print out to your

terminal as soon as the comm port is reopened.

ALT_Z WAZ Quick Zone Display

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

See WAZ section, Quick Zone Display for general information.

SHFT_F1 Browse Worked All States Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Display WAS Browse/Modify screen.  See Worked All States (WAS) section

for general information.

SHFT_F2 Browse Worked All Russian Oblasts Database

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display W.A.R.O. Browse/Modify screen.  See Worked All Russian

Oblasts section for general information.

SHFT_F3 Browse Islands On The Air (IOTA) Database

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display IOTA Browse/Modify screen.  See Islands On The Air (IOTA)

section for general information.

SHFT_F4 Browse Worked All Zones (WAZ) Database

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Display WAZ Browse/Modify screen.  See Worked All Zones (WAZ)

section for general information.

SHFT_F5 Browse County Award Database

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Display County Browse/Modify screen.  See Worked All Counties

section for general information.

SHFT_F6 Browse Worked All Prefixes (WPX) Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display WPX Browse/Modify screen.  See Worked All Prefixes (WPX)

section for general information.

SHFT_F7 Browse VHF/UHF Century Club (VUCC) Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Display VUCC Browse/Modify screen.  See VHF/UHF Century Club (VUCC)

section for general information.

SHFT_F8 Browse DX Century Club (DXCC) Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display DXCC Browse/Modify screen.  See DX Century Club (DXCC)

section for general information.

SHFT_F9 Browse Logbook Databases

-78-

LOGPlus! version 3.0

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Display LOGBOOK Browse/Modify screen.  See Logbook Management

section for general information.

SHFT_F10 Browse QSL Manager Databases

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Display QSL Manager Browse/Modify screen.  See QSL Managers

section for general information.

N>  Program Parameter Defaults

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Program Parameter Defaults are very critical to the way LOGPlus!

handles various functions. It's extremely important that you follow

each procedure step by step.  If you don't, you'll run into many

problems!

VERY VERY VERY IMPORTANT!!!!

=-=-=-=-=-=-=-=-=-=-=-=-=-=-

NEVER EVER REPEAT NEVER EVER PRESS <ESC> TO EXIT THIS MODULE.  USE

THE ENTER KEY AND GO THROUGH EACH FIELD.  IF YOU PRESS <ESC> IT IS

HIGHLY LIKELY THAT SOMETHING ODDBALL WILL OCCUR!!!!

Date Format (Program)

=-=-=-=-=-=-=-=-=-=-=

Move the highlight bar down to the date format you wish to use

throughout the life of this program.  You are given 7 options to

choose from.  Default is mm/dd/yy.

mm/dd/yy    dd/mm/yy    yy/mm/dd    mm-dd-yy

dd-mm-yy    dd.mm.y     yy.mm.dd

Date Format (Labels)

=-=-=-=-=-=-=-=-=-=-

Move the highlight bar down to the date format you wish to use when

printing QSL labels.  You are given 6 options.  Default FEB 28 56.

FEB 28 56      FEB-28-56      28 FEB 56

28-FEB-56      56 FEB 28      56-FEB-28

GMT OffSet

=-=-=-=-=-

This is a critical field.  If your SYSTEM clock is set to GMT time

(recommended), then enter 0.  Otherwise, enter your offset from GMT.

This is the number of hours (+/-) you use to determine GMT.

Latitude (13.5)

=-=-=-=-=-=-=-=

Enter your latitude.  If you're in the SOUTHERN latitudes, preceed

the entry with a minus sign.   13.5S = -13.5

-79-

LOGPlus! version 3.0

Longitude (-144.5)

=-=-=-=-=-=-=-=-=-

Enter your longitude.  If you're in the EASTERN longitudes, preceed

the entry with a minus sign.   144.5E = -144.5

Update Sunrise/Sunset at 0000z

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

YES:  At the new radio day, update both the DXCC and SunCity databases.

This will only happen when you return to the main menu.

NO:   No update will occur.

Grayline width (1-45)

=-=-=-=-=-=-=-=-=-=-=

Grayline width to use when calculating Sunrise and Sunset.  When

operating on the low bands, it's best to use a smaller width.

DXAlert! at program load

=-=-=-=-=-=-=-=-=-=-=-=-

YES:  Each time the program is loaded, check the DXINFO.DBF file for

any DXPeditions which occur today.

NO:   No action taken.

HP Laser Support

=-=-=-=-=-=-=-=-

If you have a Hewlett Packard laser printer attached answer YES.

This supports FORM printing only.  Future upgrades will focus more

closely on AVERY label printing.

Default Log Freq (14.023):

=-=-=-=-=-=-=-=-=-=-=-=-=-

If you wish the frequency field in the realtime and offair logging

modules to be field in automatically each time, enter the frequency

you operate on most.

Default Log Mode (CW, SSB, RTY):

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

If you wish the mode field in the realtime and offair logging

modules to be field in automatically each time, enter the mode

you operate on most.

Default Power Setting (1KW):

=-=-=-=-=-=-=-=-=-=-=-=-=-=-

If you wish the pwr field in the realtime and offair logging

modules to be field in automatically each time, enter the power

you operate with most.

-80-

LOGPlus! version 3.0

Default Orbitor (FO-12, AO-13):

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

If you wish the orbitor field in the realtime and offair logging

modules to be field in automatically each time, enter the orbitor

you operate with most.

Default SATELLITE SM (J, K, KT):

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

If you wish the satellite mode field in the realtime and offair

logging modules to be field in automatically each time, enter

the satellite mode you operate with most.

Default SATELLITE Mode (CW, SSB, RTY):

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

If you wish the mode field in the realtime and offair logging

modules to be field in automatically each time, enter the mode

you operate on most.

Call sign of 1st/4th DXCC databases:

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

This is a new feature which allows you to assist your friends

with their DXCC award.  In the dumb terminal program, this

is called DXCC Four Others.

DXCC Four Others is a new feature added to LOGPlus! v3 to allow

LOGPlus! users the ability to share DXCC information.  This is

particularly useful for those individuals who do not have access

to a PacketCluster but a near by friend does.

how it works

=-=-=-=-=-=-

KW7Y/Paul and KD7Z/Jack are very interested in DX chasing but

don't have access to a PacketCluster.  They usually pass around

the PAPER DXCC listing out to friends asking them to call when

DX they need is reported.  The DXCC Four Others removes the

PAPER!

To assist they gentlemen, I would entered:

Call sign of 1st DXCC databases:   KW7Y

Call sign of 2nd DXCC databases:   KD7Z

Call sign of 3rd DXCC databases:

Call sign of 4th DXCC databases:

Next you MUST get a copy of their DXCC.DBF and DXCC.NSX files.

They MUST be renamed to the call sign, and copied into your

\PLUSV3 directory.

Example:

-81-

LOGPlus! version 3.0

Rename KW7Y's DXCC.DBF and DXCC.NSX to KW7Y.DBF and KW7Y.NSX.

Rename KD7Z's DXCC.DBF and DXCC.NSX to KD7Z.DBF and KD7Z.NSX.

From this point on, whenever you press ALT_D is pressed in

the dumb terminal program when pack spotting is on, the screen

will display the DXCC statistics for the SPOT reported.

NOTE:  YOU CAN NOT MODIFY THEIR FILES.  IF THEY MAKE CHANGES

YOU WILL BE REQUIRED TO OBTAIN NEW COPIES.  MAKE SURE YOU

BACKUP YOUR DXCC.DBF FILE PERIODICALLY IF YOU USE THIS

FEATURES TO REDUCE THE POSSIBILITIES OF ACCIDENTLY OVERWRITING

YOUR OWN DXCC.DBF AND DXCC.NSX FILES!!!

LOGPlus! Drive/Directory

=-=-=-=-=-=-=-=-=-=-=-=-

LOGPlus! can be loaded in a sub-directory; however, this is not

recommended.  If you're determined to load this program in a

sub-directory make sure you provide the exact drive and directory

for the program.  You will also need to re-create all the \LOGx\

sub-directories OFF the sub-directory you move LOGPlus! to.  You

will also need to create \NOTES\ and  \HELP\.

DEFAULT:  \PLUSV3\

HAMBase/SAM/BuckMaster/QRZ/Ext. Program 1/2/3

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

NOTE:  HAMBase, SAM, BuckMaster and QRZ run the actual

executionable file for these awards.  it's extremely important

you following these instruction closely.  This information

also effects the three Ext. Programs which will be explained.

LOGPlus! has the ability to jump directly into DOS and run

external programs without effecting the execution of the log.

Because of the various renaming which has gone on in various

QSL Manager programs such as HamBase, SAM etc...  It became

difficult to write an effective interface which would automatically

extract data from these databases and input them into LOGPlus!.

Because of this, it was necessary to provide QUICK access only.

When entering data in the Hambase, SAM, BuckMaster and QRZ

fields, its extremely important you not only provide the

drive and directory, but also the executionable file name.

Example:  SAM is located on your D: hard drive in the \SAM\

directory.  The .COM or .EXE file to run SAM is called SAM.EXE.

In the SAM Drive/Directory field I would enter:

C:\SAM\SAM

|   |   |

-82-

LOGPlus! version 3.0

hard drive ------|   |   |------- .EXE file name.

directory -----------|

Example:  You'd like to run both GEOCLOCK and MINIPROP from

within LOGPlus!.  In the Ext. Program  fields I would enter:

E:\RADIO\GEOCLK\GEOCLK

|   |     |     |

hard drive ------|   |     |     |------- .EXE file name.

directory -----------|     |

sub-directory--------------|

E:\RADIO\MINI\MP

|   |    |   |

hard drive ------|   |    |   |------- .EXE file name.

directory -----------|    |

sub-directory ------------|

When the associated ALT key is pressed from the main menu,

logging modules or dumb terminal program, the EXE file will

be executed.

CT by K1EA Drive/Directory

=-=-=-=-=-=-=-=-=-=-=-=-=-

Enter the drive and directory LOGPlus! can find the B2R9.EXE

program in.  If you don't have the program enter the same

drive and directory as you did for LOGPlus!  See Contest

Conversions for additional information.

DEFAULT:  C:\CT\

O>  Database Backup (ZIP/ARC)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

To provide flexibility and ease of operation, a backup feature has

been written into LOGPlus!.  Any file can be archived quickly and

easy and saved to a drive of your choosing.

The file PKZIP.EXE, LHARC.EXE or ARJ.EXE (depending on the method used)

MUST be found in a directory specified in your AUTOEXEC.BAT PATH line

(PATH=C:\UTILITY; etc..), if not, no action will be taken.

The file BACKUP.BAT MUST be found in the \PLUSV3 directory.  Using the

file BACKUP.BAT, you can specify which file(s) to back up.

If you use the COPY command within the .BAT file, you MUST insure:

1> a disk has been placed in the drive specified

2> the disk is FORMATTED

3> the disk must contain sufficient space for the file, if not,

a CRASH will occur.

-83-

LOGPlus! version 3.0

Example (1):  BACKUP.BAT  << Using PKZIP.EXE

PKZIP DBFAD1C *.DBF  << Create file DBFAD1C.ZIP with DBF files.

PKZIP NTXAD1C *.NTX  << Create file NTXAD1C.ZIP with NTX files.

COPY B:DBFAD1C.ZIP   << Copy DBFAD1C.ZIP to drive B:.

Example (2):  BACKUP.BAT  << Using PKZIP.EXE

PKZIP AD1C *.*      << Create file AD1C with ALL files located

in the current directory.

P>  Global Database Indexing

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Global Indexing provides a quick and easy way to update ALL

index files (.NSX) associated with LOGPlus!.  No action is

required by the users other than pressing the appropriate

selection key.

Index files are used to rearrange data for quick access.

LOGPlus! utilizes a couple of different indexing methods

which makes these .NSX files incompatible with previous

versions.  Encryption as well as BINARY conversions are

used in an effort to reduce file size.

IMPORTANT NOTE:  If for some reason you decided to DELETE

.DBF files from your hard drive, make sure you delete the

associated .NSX files.

-84-

LOGPlus! Version 3.0 - Appendix A

Deleted Prefix      Country                           Effective    Deleted

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=--=-=-=-

NO      1A        Sov Military Order of Malta        11/15/45

YES     1B        Blenheim Reef                      05/04/67    07/01/75

YES     1G        Geyser Reef                        05/04/67    03/01/78

YES     1M        Minerva Reef                       11/15/45    07/15/72

NO      1S        Spratly Islands                    11/15/45

NO      3A        Monaco                             11/15/45

NO      3B6       Agalega & St. Brandon Islands      11/15/45

NO      3B8       Mauritius Island                   11/15/45

NO      3B9       Rodrigues Island                   11/15/45

NO      3C        Equatorial Guinea                  11/15/45

NO      3C0       Pagalu Island (Annobon)            11/15/45

NO      3D2       Fiji Islands                       11/15/45

NO      3D2/C     Conway Reef                        11/15/45

NO      3D2/R     Rotuma                             11/15/45

NO      3DA       Swaziland                          11/15/45

NO      3V        Tunisia                            11/15/45

NO      3X        Guinea                             11/15/45

NO      3Y        Bouvet Island                      11/15/45

NO      3Y/P      Peter 1st Island                   11/15/45

NO      4J        Azerbaijan                         11/15/45

NO      4L        Georgia                            11/15/45

NO      4S        Sri Lanka (Ceylon)                 11/15/45

NO      4U1/I     ITU Geneva Switzerland             11/15/45

NO      4U1/U     United Nations, Headquarters       11/15/45

YES     4W        Yemen Arab Republic                11/15/45    05/22/90

NO      4X        Israel                             11/15/45

NO      5A        Libya                              11/15/45

NO      5B        Cyprus                             11/15/45

NO      5H        Tanzania                           11/15/45

NO      5N        Nigeria                            11/15/45

NO      5R        Madagascar                         11/15/45

NO      5T        Mauritania                         06/20/60

NO      5U        Niger                              08/03/60

NO      5V        Togo                               11/15/45

NO      5W        Western Samoa                      11/15/45

NO      5X        Uganda                             11/15/45

NO      5Z        Kenya                              11/15/45

NO      6W        Senegal                            06/20/60

NO      6Y        Jamaica                            11/15/45

NO      7O        Yemen                              05/22/90

YES     7O/K      Karaman Island                     11/15/45    03/11/82

YES     7O/P      People's Dem Rep of Yemen          11/15/45    05/22/90

NO      7P        Lesotho                            11/15/45

NO      7Q        Malawi                             11/15/45

NO      7X        Algeria                            11/15/45

NO      8P        Barbados                           11/15/45

NO      8Q        Maldive Islands                    11/15/45

-A1-

LOGPlus! Version 3.0 - Appendix A

NO      8R        Guyana                             11/15/45

YES     8Z4       Saudi/Iraq Neutral Zone            11/15/45    12/26/81

YES     8Z5       Kuwait/Saudi Neutral Zone          11/15/45    12/15/69

NO      9A        Croatia                            06/26/91

NO      9G        Ghana                              03/05/57

NO      9H        Malta                              11/15/45

NO      9J        Zambia                             11/15/45

NO      9K        Kuwait                             11/15/45

NO      9L        Sierra Leone                       11/15/45

NO      9M2       West Malaysia                      09/16/63

NO      9M6       East Malaysia                      09/16/63

NO      9N        Nepal                              11/15/45

NO      9Q        Zaire                              11/15/45

YES     9S4       Saar                               11/15/45    04/01/57

NO      9U        Burundi                            07/01/62

YES     9U5       Ruanda-Urundi                      07/01/60    07/01/62

NO      9V        Singapore                          11/15/45

NO      9X        Rwanda                             07/01/62

NO      9Y        Trindade & Tobago                  11/15/45

YES     A1        Abu Ail, Jabal at Tair             11/15/45    03/31/91

NO      A2        Botswana                           11/15/45

NO      A3        Tonga                              11/15/45

NO      A4        Oman                               11/15/45

NO      A5        Bhutan                             11/15/45

NO      A6        United Arab Emirates               11/15/45

NO      A7        Qatar                              11/15/45

NO      A9        Bahrain                            11/15/45

YES     AC3       Sikkim                             11/15/45    05/01/75

YES     AC4       Tibet                              11/15/45    06/01/74

NO      AP        Pakistan                           11/15/45

NO      BV        Taiwan (Formosa)                   11/15/45

NO      BY        China                              11/15/45

NO      C2        Nauru                              11/15/45

NO      C3        Andorra                            11/15/45

NO      C5        Gambia                             11/15/45

NO      C6        Bahamas                            11/15/45

NO      C9        Mozambique                         11/15/45

YES     C9/M      Manchuria                          11/15/45    09/17/63

NO      CE        Chile                              11/15/45

NO      CE0/A     Easter Island                      11/15/45

NO      CE0/X     San Felix Island                   11/15/45

NO      CE0/Z     Juan Fernandez Island              11/15/45

NO      CE9       Antarctica                         11/15/45

NO      CN        Morocco                            11/15/45

YES     CN2       Tangier                            11/15/45    07/01/60

NO      CO        Cuba                               11/15/45

NO      CP        Bolivia                            11/15/45

YES     CR8/D     Damao, Diu                         11/15/45    01/01/62

YES     CR8/G     Goa                                11/15/45    01/01/62

-A2-

LOGPlus! Version 3.0 - Appendix A

YES     CR8/T     Portuguese Timor                   11/15/45    09/15/76

NO      CT        Portugal                           11/15/45

NO      CT3       Madeira Island                     11/15/45

NO      CU        Azores                             11/15/45

NO      CX        Uruguay                            11/15/45

NO      CY0       Sable Island                       11/15/45

NO      CY9       St. Paul Island                    11/15/45

NO      D2        Angola                             11/15/45

NO      D4        Cape Verde Island                  11/15/45

NO      D6        Comoros                            07/15/75

NO      DL        Federal Republic of Germany        09/17/73

YES     DL/       Germany                            11/15/45    09/17/73

NO      DU        Philippines                        11/15/45

NO      E3        Eritrea                            11/15/45

NO      EA        Spain                              11/15/45

NO      EA6       Balearic Island                    11/15/45

NO      EA8       Canary Island                      11/15/45

NO      EA9       Ceuta & Melilla                    11/15/45

YES     EA9/I     Ifni                               11/15/45    05/14/69

NO      EI        Ireland                            11/15/45

NO      EK        Armenia                            11/15/45

NO      EL        Liberia                            11/15/45

NO      EP        Iran                               11/15/45

NO      ER        Moldavia                           11/15/45

NO      ES        Estonia                            11/15/45

NO      ET        Ethiopia                           11/15/45

NO      EU        Belarus                            11/15/45

NO      EX        Kirghizia                          11/15/45

NO      EY        Tadzhikistan                       11/15/45

NO      EZ        Turkmenistan                       11/15/45

NO      F         France                             11/15/45

YES     FF        French West Africa                 11/15/45    08/07/60

NO      FG        Guadeloupe                         11/15/45

NO      FH        Mayotte Island                     07/15/75

YES     FH/C      Comoros                            11/15/45    07/06/75

YES     FI8       French Indo-China                  11/15/45    12/21/50

NO      FK        New Caledonia                      11/15/45

NO      FM        Martinique                         11/15/45

YES     FN8       French India                       11/15/45    11/01/54

NO      FO        French Polynesia                   11/15/45

NO      FO/X      Clipperton Island                  11/15/45

NO      FP        St. Pierre & Miquelon              11/15/45

YES     FQ8       French Equitorial Africa           11/15/45    08/17/60

NO      FR        Reunion Island                     11/15/45

NO      FR/G      Glorioso Island                    06/25/60

NO      FR/J      Juan de Nova, Europa               06/25/60

NO      FR/T      Tromelin Island                    11/15/45

NO      FS        Saint Martin Island                11/15/45

NO      FT8/W     Crozet Island                      11/15/45

-A3-

LOGPlus! Version 3.0 - Appendix A

NO      FT8/X     Kerguelen Island                   11/15/45

NO      FT8/Z     Amsterdam & St. Paul Island        11/15/45

NO      FW        Wallis & Futuna Islands            11/15/45

NO      FY        French Guiana                      11/15/45

NO      G         England                            11/15/45

NO      GD        Isle of Man                        11/15/45

NO      GI        Northern Ireland                   11/15/45

NO      GJ        Jersey                             11/15/45

NO      GM        Scotland                           11/15/45

NO      GU        Guernsey                           11/15/45

NO      GW        Wales                              11/15/45

NO      H4        Solomon Islands                    11/15/45

NO      HA        Hungary                            11/15/45

NO      HB        Switzerland                        11/15/45

NO      HB0       Liechtenstein                      11/15/45

NO      HC        Ecuador                            11/15/45

NO      HC8       Galapagos Islands                  11/15/45

NO      HH        Haiti                              11/15/45

NO      HI        Dominican Republic                 11/15/45

NO      HK        Colombia                           11/15/45

NO      HK0/A     San Andreas & Providencia          11/15/45

YES     HK0/B     Bajo Nuevo                         11/15/45    09/17/81

NO      HK0/M     Malpelo Island                     11/15/45

YES     HK0/S     Seranna Bank                       11/15/45    09/17/81

NO      HL        South Korea                        11/15/45

NO      HP        Panama                             11/15/45

NO      HR        Honduras                           11/15/45

NO      HS        Thailand                           11/15/45

NO      HV        Vatican City                       11/15/45

NO      HZ        Saudi Arabia                       11/15/45

NO      I         Italy                              11/15/45

YES     I1        Trieste                            11/15/45    04/01/57

YES     I5        Italian Somaliland                 11/15/45    07/01/60

NO      IS        Sardinia                           11/15/45

NO      J2        Djibouti                           11/15/45

NO      J3        Grenada                            11/15/45

NO      J5        Guinea-Bissau                      11/15/45

NO      J6        St. Lucia                          11/15/45

NO      J7        Dominica                           11/15/45

NO      J8        St. Vincent                        11/15/45

NO      JA        Japan                              11/15/45

NO      JD1       Minami Torishima                   11/15/45

NO      JD1/O     Ogasawara                          11/15/45

YES     JD1/T     Okino Tori-shima                   05/30/76    12/01/80

NO      JT        Mongolia                           11/15/45

NO      JW        Svalbard                           11/15/45

NO      JX        Jan Mayen                          11/15/45

NO      JY        Jordan                             11/15/45

YES     JZ0       Netherlands New Guinea             11/15/45    05/01/63

-A4-

LOGPlus! Version 3.0 - Appendix A

NO      K         United States of America           11/15/45

NO      KC6       Belau (W. Caroline Islands)        11/15/45

NO      KG4       Guantanamo Bay                     11/15/45

NO      KH0       Mariana Islands                    11/15/45

NO      KH1       Baker, Howland Islands             11/15/45

NO      KH2       Guam                               11/15/45

NO      KH3       Johnston Island                    11/15/45

NO      KH4       Midway Island                      11/15/45

NO      KH5       Palmyra, Jarvis Islands            11/15/45

NO      KH5/K     Kingman Reef                       11/15/45

NO      KH6       Hawaii                             11/15/45

NO      KH7       Kure Island                        11/15/45

NO      KH8       American Samoa                     11/15/45

NO      KH9       Wake Island                        11/15/45

NO      KL7       Alaska                             11/15/45

NO      KP1       Navassa Island                     11/15/45

NO      KP2       U.S. Virgin Islands                11/15/45

NO      KP4       Puerto Rico                        11/15/45

NO      KP5       Desecheo Island                    03/01/79

YES     KR6       Okinawa (Ryukyu Islands)           11/15/45    05/15/72

YES     KS4       Swan Island                        11/15/45    09/01/72

YES     KZ5       Canal Zone                         11/15/45    10/01/79

NO      LA        Norway                             11/15/45

NO      LU        Argentina                          11/15/45

NO      LX        Luxembourg                         11/15/45

NO      LY        Lithuania                          11/15/45

NO      LZ        Bulgaria                           11/15/45

NO      OA        Peru                               11/15/45

NO      OD        Lebanon                            11/15/45

NO      OE        Austria                            11/15/45

NO      OH        Finland                            11/15/45

NO      OH0       Aland Island                       11/15/45

NO      OJ0       Market Reef                        11/15/45

NO      OK        Czech Republic                     01/01/93

YES     OK/       Czechoslovakia                     11/15/45    01/01/93

NO      OM        Slovakia                           01/01/93

NO      ON        Belgium                            11/15/45

NO      OX        Greenland                          11/15/45

NO      OY        Faroe Islands                      11/15/45

NO      OZ        Denmark                            11/15/45

NO      P2        Papua New Guinea                   09/16/75

YES     P2/NG     Territory New Guinea               11/15/45    09/16/75

YES     P2/P      Papua Territory                    11/15/45    09/16/75

NO      P4        Aruba                              01/01/86

NO      PA        Netherlands                        11/15/45

NO      PJ2       Neth Antilles (Bonaire)            11/15/45

NO      PJ7       St. Maarten, St.Eustatius          11/15/45

YES     PK1       Java                               11/15/45    05/01/63

YES     PK4       Sumatra                            11/15/45    05/01/63

-A5-

LOGPlus! Version 3.0 - Appendix A

YES     PK5       Netherlands Borneo                 11/15/45    05/01/63

YES     PK6       Celebe & Molucca Islands           11/15/45    05/01/63

NO      PY        Brazil                             11/15/45

NO      PY0/F     Fernando de Noronha                11/15/45

NO      PY0/S     St.Peter & St.Paul Rocks           11/15/45

NO      PY0/T     Trinidade & Martin Vaz             11/15/45

NO      PZ        Suriname                           11/15/45

NO      R1/FJ     Franz Josef Land                   11/15/45

NO      R1/MV     Malyj Vysotskij Island             11/15/45

NO      S0        Western Sahara                     11/15/45

NO      S2        Bangladesh                         11/15/45

NO      S5        Slovenia                           06/26/91

NO      S7        Seychelles Islands                 11/15/45

NO      S9        Sao Tome & Principe                11/15/45

NO      SM        Sweden                             11/15/45

NO      SP        Poland                             11/15/45

NO      ST        Sudan                              11/15/45

NO      ST0       Southern Sudan                     11/15/45

NO      SU        Egypt                              11/15/45

NO      SV        Greece                             11/15/45

NO      SV/A      Mount Athos                        11/15/45

NO      SV5       Dodecanese                         11/15/45

NO      SV9       Crete                              11/15/45

NO      T2        Tuvalu                             01/01/76

NO      T30       W. Kiribati (Gilbert Islands)      11/15/45

NO      T31       C. Kiribati (Br Phoenix)           11/15/45

NO      T32       E. Kiribati (Line Islands)         11/15/45

NO      T33       Banaba (Ocean Island)              11/15/45

NO      T5        Somalia                            11/15/45

NO      T7        San Marino                         11/15/45

NO      T9        Bosnia-Hercegovina                 10/15/91

NO      TA        Turkey                             11/15/45

NO      TF        Iceland                            11/15/45

NO      TG        Guatemala                          11/15/45

NO      TI        Costa Rica                         11/15/45

NO      TI9       Cocos Island                       11/15/45

NO      TJ        Cameroon                           11/15/45

NO      TK        Corsica                            11/15/45

NO      TL        Central African Republic           08/13/60

NO      TN        Congo                              08/15/60

NO      TR        Gabon                              08/17/60

NO      TT        Chad                               08/11/60

NO      TU        Ivory Coast                        08/07/60

NO      TY        Benin                              08/01/60

NO      TZ        Mali                               06/20/60

NO      UA        European Russia                    11/15/45

NO      UA0       Asiatic Russia                     11/15/45

NO      UA2       Kaliningrad                        11/15/45

NO      UJ        Uzbekistan                         11/15/45

-A6-

LOGPlus! Version 3.0 - Appendix A

NO      UN        Kazakhstan                         11/15/45

YES     UN1       Karelo-Finnish Republic            11/15/45    07/01/60

NO      UR        Ukraine                            11/15/45

NO      V2        Antigua, Barbuda                   11/15/45

NO      V3        Belize                             11/15/45

NO      V4        St. Christopher & Nevis            11/15/45

NO      V5        Namibia (SW Africa)                11/15/45

NO      V6        Micronesia (E.Caroline Is.)        11/15/45

NO      V7        Marshall Islands                   11/15/45

NO      V8        Brunei                             11/15/45

NO      VE        Canada                             11/15/45

NO      VK        Australia                          11/15/45

NO      VK0       Macquarie Island                   11/15/45

NO      VK0/H     Heard Island                       11/15/45

NO      VK9/C     Cocos-Keeling Island               11/15/45

NO      VK9/L     Lord Howe Island                   11/15/45

NO      VK9/M     Mellish Reef                       11/15/45

NO      VK9/N     Norfolk Island                     11/15/45

NO      VK9/W     Willis Island                      11/15/45

NO      VK9/X     Christmas Island                   11/15/45

YES     VO        Newfoundland, Labrador             11/15/45    04/01/49

NO      VP2/E     Anguilla                           11/15/45

NO      VP2/M     Montserrat                         11/15/45

NO      VP2/V     British Virgin Islands             11/15/45

NO      VP5       Turks & Caicos Islands             11/15/45

NO      VP8/F     Falkland Islands                   11/15/45

NO      VP8/G     South Georgia Islands              11/15/45

NO      VP8/H     South Shetland Islands             11/15/45

NO      VP8/O     South Orkney Islands               11/15/45

NO      VP8/S     South Sandwich Islands             11/15/45

NO      VP9       Bermuda                            11/15/45

YES     VQ1       Zanzibar                           11/15/45    06/01/74

YES     VQ6       British Somaliland                 11/15/45    07/01/60

NO      VQ9       Chagos Island                      11/15/45

YES     VQ9/A     Aldabra                            11/15/45    09/15/76

YES     VQ9/D     Desroches                          11/15/45    06/29/76

YES     VQ9/F     Farquhar                           11/15/45    06/29/76

NO      VR6       Pitcairn Island                    11/15/45

YES     VS2       Malaya                             11/15/45    09/16/63

YES     VS4       Sarawak                            11/15/45    09/16/63

NO      VS6       Hong Kong                          11/15/45

YES     VS9/H     Kuria Muria Island                 11/15/45    11/30/67

NO      VU        India                              11/15/45

NO      VU4       Andaman & Nicobar Islands          11/15/45

NO      VU7       Laccadive Islands                  11/15/45

NO      XE        Mexico                             11/15/45

NO      XF4       Revilla Gigedo                     11/15/45

NO      XT        Burkina Faso (Upper Volta)         08/16/60

NO      XU        Kampuchea (Cambodia)               11/15/45

-A7-

LOGPlus! Version 3.0 - Appendix A

NO      XV        Vietnam                            11/15/45

NO      XW        Laos                               11/15/45

NO      XX9       Macao                              11/15/45

NO      XZ        Myanmar (Burma)                    11/15/45

YES     Y2        German Democratic Republic         09/17/73    10/03/90

NO      YA        Afghanistan                        11/15/45

NO      YB        Indonesia                          05/01/63

NO      YI        Iraq                               11/15/45

NO      YJ        Vanuatu (New Hebrides)             11/15/45

NO      YK        Syria                              11/15/45

NO      YL        Latvia                             11/15/45

NO      YN        Nicaragua                          11/15/45

NO      YO        Romania                            11/15/45

NO      YS        El Salvador                        11/15/45

NO      YU        Yugoslavia                         11/15/45

NO      YV        Venezuela                          11/15/45

NO      YV0       Aves Island                        11/15/45

NO      Z2        Zimbabwe (Rhodesia)                11/15/45

NO      Z3        Macedonia                          09/08/91

NO      ZA        Albania                            11/15/45

NO      ZB        Gibraltar                          11/15/45

NO      ZC4       Cyprus, British Bases              08/16/60

YES     ZC5       British North Borneo               11/15/45    09/16/63

YES     ZC6       Palestine                          11/15/45    07/01/68

YES     ZD4       Gold Coast, Togoland               11/15/45    03/06/57

NO      ZD7       St. Helena                         11/15/45

NO      ZD8       Ascension Island                   11/15/45

NO      ZD9       Tristan da Cunha & Gough           11/15/45

NO      ZF        Cayman Island                      11/15/45

NO      ZK1/N     North Cook Islands                 11/15/45

NO      ZK1/S     South Cook Islands                 11/15/45

NO      ZK2       Niue                               11/15/45

NO      ZK3       Tokelau Islands                    11/15/45

NO      ZL        New Zealand                        11/15/45

NO      ZL7       Chatham Island                     11/15/45

NO      ZL8       Kermadec Island                    11/15/45

NO      ZL9       Auckland & Campbell Islands        11/15/45

NO      ZP        Paraguay                           11/15/45

NO      ZS        Republic of South Africa           11/15/45

YES     ZS0       Penguin Islands                    11/15/45    03/01/94

NO      ZS8       Prince Edward & Marion Islands     11/15/45

YES     ZS9       Walvis Bay                         09/01/77    03/01/94

-A8-

LOGPlus! Version 3.0 - Appendix D

LOGPlus! v3 Order Form

======================

Where did you hear about LOGPlus!: ______________________

Your CallSign: ____________________

Name: _______________________________________________________

Address: ____________________________________________________

Address: ____________________________________________________

City/State/Country: _________________________________________

ZipCode: _______________________

Disk format:  5.25 (1.2M): _______

3.5  (720K): _______       3.5 (1.44M): _______

U.S.   $45.00        DX   $45.00       IOTA:  $3.00

Washington residence:  Add 8.2% state sales tax.

Total funds enclosed: __________

Computer system:  286 ____   386 ____   486 ____  Pentium ____

Conversion program(s) available.  Mark the one you need.  See

documentation for additional information on conversion programs.

DXLog: _____    EasyDX: _____   WB2DND: _____      G3ZCZ: _____

EQF-Log: _____  SwissLog: _____ HyperLog: ____  TurboLog: _____

2ndOP: _____    DXBase: _____   GEMRadio: _____ Other: _____

-D1-