������������������������������������������������������������ͻ

�                                                            �

� Sound System Source for your own productions (games&demos) �

�                                                            �

� coded and provided in 1995 by the Frontman of Crew242 !!!  �

�                                                            �

������������������������������������������������������������ͼ

RELEASE 3

Enhanced SB/SBPro player with volume amplifying and interpolation.

Added SB16 and PAS+/16 player with maximum quality and speed (hope so).

Added C interface for Borland C 3.1

Added auto-detection routines coded by Chicken of S!P.

Included TheCrab.MOD from the great game 'Future Dimension'.

RELEASE 2

Minor changes because of wrong config filename (sorry)!

Took out the specific file-volume routines of Future Dimension.

Added one of my favourite modules that always reminds me on good ol' C-64.

This is Thalamus.MOD by good ol' Rob Hubbard and converted by M. Simmonds.

RELEASE 1

First release.

������������������������������������������������������������������������������

This Sound System Source package includes:

SOUNDSYS.DOC  This file

DETSOUND.ASM  Auto-detection for all cards provided

SBMOD.COM     SoundBlaster (Pro) module player, eg. SBMOD THECRAB.MOD

SBMOD.ASM     SoundBlaster (Pro) player source

SB16MOD.COM   SoundBlaster 16 module player, eg. SB16MOD COMPLICA.MOD

SB16MOD.ASM   SoundBlaster 16 player source

PASMOD.COM    Pro Audio Spectrum +/16 module player, eg. PASMOD THALAMUS.MOD

PASMOD.ASM    Pro Audio Spectrum player source

USMOD.COM     Gravis UltraSound  module player, eg. USMOD CYBRNOID.MOD

USMOD.ASM     Gravis UltraSound  player source

CPLAY.C       Sample of a player in C

CINTSS3.C     C interface for all players.

CPLAY.PRJ     Project file to compile & link with Borland's C 3.1 IDE

SS3.CFG       Configuration file

and some object files...

Agreement:

This programs are free software only and may not be sold other than the cost

of the disk or physical handling. You can redistribute it only in the same

form as you have received it.

This program is distributed WITHOUT ANY WARRANTY nor is it responsible for

any damage of computer software and/or hardware.

������������������������������������������������������������������������������

What does this Sound System ?

For SoundBlaster (Pro/16):

- 4/8 channels Protracker/Fasttracker module playing

- 2/4 channels for FX

- Full Protracker 2.1A effects support

- 4 times amplified and linear interpolated mixing

- Mixing rates 10000 - 22222 Hz

- SBPro/SB16 stereo support

- 656 KB EMS support for FX

- 100% Assembler for high perfomance -> takes only 15% cpu time

- 12kb code, 8kb mixbuf, 16kb volumetable, 0-64kb trackbuf, 0-???kb samples

For Soundblaster 16 and Pro Audio Spectrum +/16:

- 4/8 8/16bit channels Protracker/Fasttracker module playing

- 8 8/16bit channels with panning for stereo FX

- Full Protracker 2.1A effects support

- Extended MOD effects support (Panning Effect E8x)

- 4 times amplified and 16bit interpolated mixing

- Mixing rates 10000 - 44100 Hz

- Always stereo

- 592 KB EMS support for FX

- 11kb code, 32kb mixbuf, 32kb volumetable, 0-???kb samples

For Gravis UltraSound:

- 4/6/8 channels Protracker/Fasttracker module playing

- 16 channels with panning for stereo FX

- Extended MOD effects support (Panning Effect E8x)

- Mixing rate always 44100 Hz

- Always stereo

- Full 8/16-bit DMA support for sample upload (optional without)

- Max 1 MB GUS memory for samples

- 100% Assembler for high perfomance -> takes almost no cpu time

- 13kb code, 0-64kb trackbuf

������������������������������������������������������������������������������

Where from, why, what for ?

This Sound System has actually been developed for FUTURE DIMENSION of Crew242,

a new, commercial, neckbreaking shootem up game of the new generation.

I decided to spread the Sound System as freeware (yes! the source is freeware)

because I dont like adlib neither midi sounds very much. Modules for everyone.

Anyway, modules produce the best sound and UltraSound is the best card for it.

Use it in any manner for your productions. For major assimilations it is

necessary to change the source, eg. you need an object instead of a com-file,

you dont want to have a configuration file, etc.

The sources are written for Borland C, Tasm 3.1 and its Tlink /t option

for COMs. Some people experienced that Tasm 4.0 doesn't work correctly,

so don't waste time and get a copy of Tasm 3.1.

����������������������������������������������������������������������������ͻ

�                                                                            �

� All I want is a short credit in your production for the Sound System like: �

�                                                                            �

� "Sound System by Frontman of Crew242" or "Sound System by Silvio Turello"  �

�                                                                            �

� I will also do that for the pioneers of module sound. Thanks goes to:      �

�                                                                            �

� Adrian Studer       for running the kewlest Swiss coder mailbox            �

� (Chicken/S!P)                                                              �

� Lars "Zap" Hamre    for inventing the protracker de facto standard         �

� (Amiga Freelancers)                                                        �

� Joshua C. Jensen    for porting it to pc                                   �

� (Form. Renaissance)                                                        �

� Serge Huber         for giving me a hint, how to make a SB-player          �

� (Numerus/Imphobia)                                                         �

� Robert Adolfsson    for showing me, how to make a GUSPLAY (-er)            �

� (Robban/Cascada)                                                           �

�                                                                            �

� Give always the complete package away !                                    �

�                                                                            �

� If you want to be nice and you are allowed to give away a copy of your     �

� production, I would really appreciate to get a copy of it just because     �

� I am always interested in good sounding soft. I will of course respect     �

� any requests not to spread the copy you send me. It will be exclusively    �

� for the Frontman of Crew242. Refer to the address below:                   �

�                                                                            �

� Silvio Turello, Giebeleichstr. 58, 8152 Glattbrugg, Switzerland            �

�                                                                            �

� Tel.: ++/1/810 70 96    Fax: ++/1/810 85 63                                �

�                                                                            �

� Email address until 1999: sturello@iiic.ethz.ch                            �

�                                                                            �

����������������������������������������������������������������������������ͼ

������������������������������������������������������������������������������

How ?

Now it is time to give some technical details about the routines.

The COM-format is quite unusual, but it has some important advantages,

eg. you always now, what is going on, because you can load it in your own

segment (at offset 100h) without relocating. the start address is 100h with

a jump table for its functions. the functions can be reached with a far call

to seg:104h, seg:108h, etc.

The following code is a prototyp of a possible implementation in your program:

;=============================================================================

INIT_MUSIC PROC NEAR

CALL	ALLOC_MUSIC		;ALLOCATE MEMORY FOR PLAYER

CALL	LOAD_PLAYER		;LOAD THE RIGHT PLAYER

MOV	MUSIC_FILE,0

RET

INIT_MUSIC ENDP

;=============================================================================

START_MUSIC PROC NEAR

CMP	PLAYER,0		;IS A PLAYER IN THE MEMORY?

JE	SHORT GMUA1

MOV	MUSIC_FILE,AX		;LOAD MUSIC

MOV	DX,OFFSET MOD_NAME

MOV	BP,2

CALL	PLAYER_FUNCTION

JC	SHORT GMUA2

CALL	MUSIC_SETTINGS		;SET PLAYING PARMS

CALL	LOAD_SAMPLES		;LOAD SAMPLES

MOV	BP,3			;START MUSIC

CALL	PLAYER_FUNCTION

JNC	SHORT GMUA1

GMUA2:	CALL	STOP_MUSIC		;DISABLE ALL MUSIC AFTER ERROR

MOV	PLAYER,0

CALL	FREE_MUSIC

GMUA1:	RET

START_MUSIC ENDP

;=============================================================================

STOP_MUSIC PROC NEAR

CMP	MUSIC_FILE,0

JE	SHORT GMUC1

MOV	BP,4			;STOP MUSIC

CALL	PLAYER_FUNCTION

MOV	BP,8			;REMOVE SAMPLES

CALL	PLAYER_FUNCTION

MOV	BP,5			;REMOVE MUSIC

CALL	PLAYER_FUNCTION

GMUC1:	MOV	MUSIC_FILE,0

RET

STOP_MUSIC ENDP

;=============================================================================

CLOSE_MUSIC PROC NEAR

CALL	DEALLOC_MUSIC		;REMOVE PLAYER

RET

CLOSE_MUSIC ENDP

;=============================================================================

PLAYER_FUNCTION PROC NEAR

PUSHA

PUSH	DS

SHL	BP,2

ADD	BP,0100H

MOV	MUSIC_CALL,BP

CALL	DWORD PTR DS:[MUSIC_CALL]	;MUSIC_CALL IS A FAR POINTER

POP	DS

POPA

RET

PLAYER_FUNCTION

;=============================================================================

The functions of the players are translated to a number in BP:

BP =  1	CONFIG_INIT               IF CARRY RETURNED IS 1 THEN ERROR

2	LOAD_MOD                  DS:DX IS POINTER TO FILENAME

3	PLAY_MUSIC                IF CARRY RETURNED IS 1 THEN ERROR

4	STOP_MUSIC

5	END_MUSIC

6	LOAD_SAMPLE               DS:DX IS POINTER TO FILENAME

CL=0/80H AMIGA/PC SAMPLE

RETURNED AX IS HANDLE OF THE SAMPLE

7	PLAY_SAMPLE               BX= HANDLE, CX= FREQUENCY

8	END_SAMPLE

9	SET_SAMPLERATE            AX=RATE 10000-44100

10	GET_VOLUME              \ AL=MASTER_VOLUME

11	SET_VOLUME              / BL=MUSIC_VOL, BH=FX_VOL

12	SET_SONGLOOP              AL=PATTERN ORDER, >127 IS NO LOOP

13	GET_SONGPOSITION        \ AL=0..127 POSITION IN PATTERN ORDER

SET_SONGPOSITION        /

14	GET_SONGMOD             \ AL=0 MUSIC & FX, =1 MUSIC ONLY

15	SET_SONGMOD             /   =2 FX ONLY, =3 NO SOUND

Each .ASM module is actually a full player for itself. The default of the

assembler variable "COM_OR_OBJ" is 0 that the .COM output is a player that

can be used in DOS, eg. USMOD TheCrab.MOD. The .COM files provided in

SoundSS3 have a DOS-interface and are ready for use.

If "COM_OR_OBJ" is <> 0, the output is the .OBJ file and the .COM cannot be

used as player because the DOS-interface is replaced by the C-interface.

Remember that for use in C you must set "COM_OR_OBJ=1" and recompile. After

that you can use the .OBJ for linking with C modules. The .OBJ files

provided in SoundSS3 are already like this and ready for linking with

C modules, eg. CPLAY. For implementations in C refer to CPLAY.

������������������������������������������������������������������������������

Some hints...

The players use a 1024 Hz interrupt generated by the timer or by the realtime

clock. If it is the timer, the old irq is chained at the normal frequency.

Most options can be set in the SS3.CFG configuration file.

The SoundBlaster player uses in this version 656/592kb EMS for FX and only

conventional memory to hold the whole module. It should be no problem to

use EMS for the module data too.

All players are very stable. That means, that they wont crash because of

a wrong baseport, irq, dma, etc. It may happen that DMA stops after use

of Playfile or some other players. In this case start ultrinit before

using usmod. If you encounter problems, remember that the interface does

never recover any registers especially es, ds segment registers because

it is speed and memory optimized code.

The players dont use the MOD patterns as internal format because of too many

repetitions in channels. The loader splits up the patterns into tracks for

each channel and generates instead of the pattern order table a track order

table for each channel. That means, if a channel plays always the same like

a drum channel, it needs just the memory of one track. It is similar to the

MTM format. The differences between MOD and MTM's memory requirements for the

same song can range from 20 to over 100kb.

The SB/SBPro/SB16/PAS players precalculate most periods to notes.

Refer to the actual loader for more details about the internal format.

If you encounter speed problems with SB16 or PAS players, you should use

a lower samplerate. In case of SB16 you can also use the SBPro player that

is also stereo on SB16 (fixed and working now) but much faster.

The big speed difference between SBPro and SB16 player is caused by the

16bit calculating and the panning (each channel is mixed twice!)

FAQ (frequently asked questions):

1. Q: Can you help me implementing your SoundSystem in my demo/game?

A: Sorry, I have my own projects and even for them not enough spare time.

I have already spent a lot of time providing this SoundSystem...

2. Q: Can you help me making a interface for a higher programming

language because I don't know Assembler very well?

A: With the C-interface provided in this version you have the base

to make another interface, eg. for TP. But in general I strongly

recommend to know Assembler.

3. Q: Where can I get docs/infos about Assembler? How can I learn it?

A: I have learnt Assembler with books. There are a lot of ASM beginner

books around, but you need such a book only if you are an absolut

beginner and don't even know what a binary/hex number is. After a

while you just want to have a reference like the Tasm reference book.

This book describes all CPU instructions very detailed but doesn't

feed up with unnecessary comment.

4. Q: I cannot make it running with a higher prog. language?

A: Note that in Assembler everything is very clear but in a higher prog.

language you don't know how the compiler does the code. In most

cases the compiler does something you don't expect and causes the

problem, eg. compiler uses some registers for internal variables you

don't see in your program but when you call an ASM procedure you

should recover these registers. Critical registers are DS, BP, SI, DI,

ES, etc.

5. Q: When and how can I get an update?

A: You will see new versions on some very well known ftp servers first.

6. Q: When will you support other music formats?

A: The SoundSystem is designed for non-specific music programs.

This means, you should not plan to make a player like Iplay or DMP

but for games and demos you can choose your own music format and this

should be MOD (maybe it is not the best but it is still powerful enough

to realize your project)

������������������������������������������������������������������������������

Personal greetings...

Maxwood of Majic12,

how about your game? Is it out now? As I told you, it does pay for you to

update the SoundSystem...

(compare the quality of SB16 sound to the one before!!!)

Andr� Baresel & Craig Jackson,

great and complete docs about SoundBlaster, unfortunately I already had

the SoundBlaster Profibuch from Addison-Wesley...

Stuart C. (The Dooze) from Australia,

here you are. Look at the C-interface and maybe you'll find a way to

implement SoundSS3 in TP7...

Guiseppe Alleva from Milano/Italy,

this is now the new version. It is a major update so I think it does pay

when you also update your graphics library...

PCheng from Vancouver/Canada,

sorry, but I'm still avoiding TP...

Steven J Streeting from the UK,

have you got now a copy of TASM3.1?

(Even better a copy of the whole Borland C++ 3.1 with the IDE)...

Roger Wong from the US,

how about POWball? Unfortunately at the time I'm writing our game is not

out yet but it will be in 2-3 weeks...

Btw there is a book out now with a lot of programming tricks called

"PC underground", maybe there you find something you were looking for...

Alec Lazarescu,

I think the C-interface is what you are looking for...

Guy English from the US,

hope you got a copy of TASM3.1...

and all that appreciate SoundSystemSource3...

������������������������������������������������������������������������������

Happy coding...

Frontman of Crew242

������������������������������������������������������������������������������