SB1.0-2.0/SBPro/Voicecard1000 with other OBJ-file

IF YOU UPLOAD THIS FILE, DONT RENAME OR/AND MADE CHANCES

Code by Jari Kaija (1991) all rights reserved

v1.2

SB - Playerdemo for PRE-Calculated sounddata.

This file shows, how to play 4-channel voices

together via SB.

Questions about all my codes, via SB-Connection BBSs

------------

Stereomode supported.

!        WORKS ONLY WITH 286 or HIGHER PROCESSORS       !

!        REMEMBER: USE 286-Processor COMPILING          !

!        (*.OBJ, is compiled with 286 extensions)       !

!       WHEN USING STEREO MODE, YOU MUST WAIT, UNTIL    !

!       DMA-TRANSFER ENDS BEFORE PLAY AGAIN...          !

!       USE "WAITDMA" PROCEDURE FOR THIS !              !

!       OTHERWISE; CHANNELS JUMPS LEFT TO RIGHT, LIKE   !

!       A HABITUAL DRUNKARD...  (test it, and you will  !

!       know what I mean... :)                          !

!       INCLUDED MIXER SETUP FOR STEREO/MONO/VOLUMES    !

****************************************************************************

BITTI = Wait until SB is ready to read data

Procedure Bitti;External;

****************************************************************************

RESETSB = Resets SB. (No effects to mixer in SB-PRO)

Segment    4000Hex - F000Hex   Soundbuffer

Addres     220Hex / 240Hex     SB's address

Interrupt  2,3,5,7,10          SB's IRQ

Procedure Resetsb(Segment,Addres,Interrupt:Word);External;

****************************************************************************

RESETDMA = You don't need this, if DMA-transfer works correctly.

It may halts your DMA-chip.

Procedure ResetDma;External;

****************************************************************************

SETVOICE = Soundlength and tell to object file, where

it finds sounddata.

Procedure Setvoice1(seg1,ofs1,len1:Word);External;

Procedure Setvoice2(seg2,ofs2,len2:Word);External;

Procedure Setvoice3(seg3,ofs3,len3:Word);External;

Procedure Setvoice4(seg4,ofs4,len4:Word);External;

****************************************************************************

CALCDATA = Calcs sounddatas

Totlen    0 - 65535 in mono mode

0 - 32760 in stereo mode

Speed     0 - 255   Playpitch

Stereo    0 / 1     Modeswitch 0=Mono, 1=Stereo

Procedure Calcdata(Totlen,Speed,Stereo:Word);External;

****************************************************************************

RESETMIXER = Reset your SBPRO mixer chip...

Procedure Resetmixer;External;

****************************************************************************

SETMIXER = Set your SBPRO mixer to Stereo/Mono mode, and volumesets...

0 - Mono

1 - Stereo

Both volumes->  Bits:   Left 7 6 5 4 Right 3 2 1 0

x x x x       x x x x  =  0-255

Procedure Setmixer(Mastervol,Vocvol,Mode:Word);External;

****************************************************************************

WAITDMA = Wait, until DMA-transfers ends

This procedure uses pregiven IRQ-value...

It halts, until IRQ line is high...

Waitdma needed, when processing DMA-transfer

in STEREO-mode.

Btw, if you play just one time your sounds or your

program has delay longer than DMA-transfer you don't

need this.

Procedure Waitdma;External;

****************************************************************************

PLAYDATA = Play sounds

Procedure Playdata;External;

****************************************************************************

Ett tmminen tapaus tll kertaa, kaikki muut proceduurit niss faileissa

pelaavatkin, PAITSI "WAITDMA"-prosetuuri. Laittelen sen kuntoon heti, kun

aika vain antaa vhnkin periksi. (Menee meinaan ensin emolta piirin vaihdok-

si).

Ongelmia tulee ilman Dmawaittia silloin, kun soitetaan stereomoodissa, ja

sample jatkuisi viel seuraavan PLAYDATA komennonkin jlkeen. Tuo ni

kun rupeaa hyppelehtimn milloin vasemmalle ja milloin oikealle...

Tuo SB/DMA siirto kun on sen verran idioottimainen sen 'epkytnnllisyy-

den' vuoksi stereomoodissa. Joka toinen databyte kun menee leftille ja

joka toinen rightille, ja muiden kuin minun (Morsen emon DMA-piiri) DMA

kun ei tunnu kertovan, ett miss mennn BYTEN tarkkuudella, saati sitten

ett teidn epstandardit koneenne osaisivat edes ilmoittaa DMA:n status-

portissa siirron tilan bitill kutakin kanavaa kohti, niin vaikeapa

siit on laskeskella, ett pitikhn se seuraavaksi laittaa parilliseen

vaiko parittomaan osoitteeseen tm uusi laskettu arvo...

Kunhan tuo DMAwaittikin rupeaa jekuttamaan, niin DMA:han psee uudestaan

vasta ksiksi, kun se on saanut vanhan siirtonsa loppuun asti ja aiheuttaa

Irkku-keskeytyksen. Jolloin DMA piirin rekisterit ovat kiltisti nollautuneet,

ja osoite on tietty automaattisesti parillisesta alkava, johon seuraavat

databytet laskeskellaan.

Monomoodissa tuolla ei ole pirunkaan merkityst, ett heitellnk DMA:lle

vaikka sadasosasekunnin vlein uudet soittoarvot...

(Ps, ostakaa tekin morsen emoja, niin nist temppuilusta psee eroon...)

Ryh, nyt te tietty kuolaatte viel tuota xxx.OBJ failia, joka tuolla

vinsulla jossain on, ett saisitte laskea nuoteille tarvittavat datat +

vibratot sun muut trackerimisyydet.

Sanoisinko, ett jos te ette jo tst pse omin avuin eteenpin, niin...

Maalaisjrki se on joka jyll :) (Yllttvn hyvin Assemblerin suhteen

itseasiassa)

- M maalaispoika oon......

TV: Jari Kaija alias BEAN

HAAGA

HELSINKI

FINLAND

EARTH

SOME UNLEVIABLE SOLARSYSTEM

UNIVERSUM

PR-PRODUCTS