32DE, 32-bit DOS-Extender, v0.15, Copyright (C) 1995 Sami Kantoluoto

������������������������������������������������������������������������������

32DE consists of two main parts:  the DOS extender and the DPMI host.  The

DOS extender takes care about extending the DOS  (INT21) API functions and the

DPMI host provides the client program with a subset of DPMI  (INT31)  v0.9 API

functionality.  You can use 32DE freely in public domain productions,  but any

commercial/shareware use and/or distribution without permission is prohibited.

If you use 32DE, it would be nice if you gave me the credits for it... ;)

1. Initialization and termination

���������������������������������

32DE has two directly callable FAR functions  to facilitate easy switching

to protected mode,   these being  _pm_info and _pm_init.   _pm_info gives info

about the machine  that the program currently runs on and  _pm_init swithes to

protected mode.  After a call  to _pm_init,  if the carry flag  is clear,  the

system will be in 32-bit protected mode, running in a 16-bit code segment.  If

the carry flag is set, then some error occurred and the mode switch failed.

Terminating the program is easy:  you do it just as you would in plain old

DOS  -  by issuing  INT 21h with AH=4C.  The header  then frees  all allocated

resources and returns to DOS...

_pm_info:

---------

parameters: -

returns:

CF = set on error

AX = return code:

0000h = OK... ;)

0001h - No 80386+ detected

0002h - system already in protected mode and no VCPI or DPMI present

0003h - DPMI - host is not 32bit

CF = clear if successful:

BX = paragraphs needed to protected mode data

CL = CPU type (5 = Pentium, 4 = 486 etc.)

CH = pmode type (3=DPMI, 2=VCPI, 1=XMS, 0=RAW)

_pm_init:

---------

parameters:

ES = segment of protected mode data

returns:

CF = set on error

AX = return code:

0000h = OK... ;)

0001h - No 80386+ detected

0002h - system already in protected mode and no VCPI or DPMI present

0003h - DPMI - host is not 32bit

0004h - Could not enable A20 gate

0005h - DPMI - could not enter 32bit protected mode

0006h - DPMI - could not allocate needed selectors

0007h - DPMI not present (only on DPMI-only extender)

0008h - XMS - Too old version (v3.0 needed)

CF - set on error, if no error:

CS - 16bit selector for real mode CS with limit of 64k

SS - 32bit selector for real mode SS with limit of 64k

DS - 32bit selector for real mode DS with limit of 64k

ES - 32bit selector for PSP with limit of 100h

FS - 0 (NULL selector)

GS - 0 (NULL selector)

2. The DOS extender

�������������������

The extended DOS function calls  are listed below  in a numerical order by

the value of the AH register.  All far pointers are in a selector:offset form,

as it is to be expected from a protected mode program...

AH=09h, Print string to standard output

---------------------------------------

Parameters:

DS:EDX -> the string (DOS '$'-terminated string)

Returns: -

AH=1Ah, Set DTA (Disk Transfer buffer Area) address

---------------------------------------------------

Parameters:

DS:EDX -> buffer area (43 bytes)

Returns: -

AH=1Bh, Get allocation information for current drive

----------------------------------------------------

Parameters: -

Returns:

AL = sectors per cluster (allocation unit)

CX = bytes per sector

DX = total number of clusters

DS:EBX -> media ID byte

AH=1Ch, Get allocation information for an arbitrary drive

---------------------------------------------------------

Parameters:

DL = drive number

Returns:

AL = sectors per cluster (allocation unit)

CX = bytes per sector

DX = total number of clusters

DS:EBX -> media ID byte

AH=1Fh, Get drive parameter block for current drive

---------------------------------------------------

Parameters: -

Returns:

AL = Status

00:

DS:EBX -> drive parameter block

FF:

invalid drive

AH=25h, Set interrupt vector

----------------------------

Parameters:

AL = vector number

DS:EDX -> interrupt handler routine

Returns: -

AH=2Fh, Get DTA (Disk Transfer buffer Area) address

---------------------------------------------------

Parameters: -

Returns:

ES:EBX -> DTA

AH=32h, Get drive parameter block for an arbitrary drive

--------------------------------------------------------

Parameters:

DL = drive number

Returns:

AL = status

00:

DS:EBX -> drive parameter block

FF:

invalid drive

AH=34h, Get address of InDOS flag

---------------------------------

Parameters: -

Returns:

ES:EBX -> one-byte InDOS flag

AH=35h, Get interrupt vector

----------------------------

Parameters: -

AL = interrupt vector number

Returns:

ES:EBX -> interrupt handler routine

AH=39h, Create subdirectory

---------------------------

Parameters:

DS:EDX -> directory name (ASCIIZ)

Returns:

CF = clear if successful

AX = unpredictable juttuj

CF = set on error

AX = error code

AH=3Ah, Remove subdirectory

---------------------------

Parameters:

DS:EDX -> directory name (ASCIIZ)

Returns:

CF = clear if successful

AX = unpredictable juttuj

CF = set on error

AX = error code

AH=3Bh, Change directory

------------------------

Parameters:

DS:EDX -> directory name (ASCIIZ)

Returns:

CF = clear if successful

AX = unpredictable juttuj

CF = set on error

AX = error code

AH=3Ch, Create or truncate file

-------------------------------

Parameters:

CX = file attributes

DS:EDX -> filename (ASCIIZ)

Returns:

CF = clear if successful

AX = file handle

CF = set on error

AX = error code

AH=3Dh, Open file

-----------------

Parameters:

AL = access and sharing modes

DS:EDX -> filename (ASCIIZ)

Returns:

CF = clear if successful

AX = file handle

CF = set on error

AX = error code

AH=3Fh, Read file

-----------------

Parameters:

BX = file handle

ECX = bytes to read

DS:EDX -> destination

Returns:

CF = clear if successful

EAX = bytes read

CF = set on error

AX = error code

AH=40h, Write file

------------------

Parameters:

BX = file handle

ECX = bytes to write

DS:EDX -> source

Returns:

CF = clear if successful

EAX = bytes written

CF = set on error

AX = error code

AH=41h, Delete file

-------------------

Parameters:

DS:EDX -> filemask (filename)

Returns:

CF = clear if successful

AX = unpredictable

CF = set on error

AX = error code

AX=4300h, Get file attributes

-----------------------------

Parameters:

DS:EDX -> filename (ASCIIZ)

Returns:

CF = clear if successful

CX = file attributes

AX = ???

CF = set on error

AX = error code

AX=4301h, Set file attributes

-----------------------------

Parameters:

CX = file attributes

DS:EDX -> filename (ASCIIZ)

Returns:

CF = clear if successful

AX = ???

CF = set on error

AX = error code

AH=47h, Get current directory

-----------------------------

Parameters:

DL = drive number (00 = current, 01 = A:, etc.)

DS:ESI -> 64-byte buffer for ASCIIZ directoryname

Returns:

CF = clear if successful

AX = ???

CF = set on error

AX = error code

AH=48h, Allocate memory

-----------------------

Parameters:

BX = paragraphs to allocate (lowmemory)

Returns:

CF = clear if successful

AX = selector for allocated block

CF = set on error

AX = error code

BX = size of largest available memory block

AH=49h, Free memory

-------------------

Parameters:

ES = selector of the memory block

Returns:

CF = clear if successful

CF = set on error

AX = error code

AH=4Ah, Resize memory block

---------------------------

Parameters:

ES = selector of the memory block

BX = new size

Returns:

CF = clear if successful

CF = set on error

AX = error code

BX = size of largest available memory block

AH=4Eh, Find first matching file

--------------------------------

Parameters:

CX = file attribute mask

DS:EDX -> filemask (filename)

Returns:

CF = clear if successful

DTA = findfile data block

CF = set on error

AX = error code

AH=4Fh, Find next file

----------------------

Parameters:

DTA = data block from previous findfirst/findnext call

Returns:

CF = clear if successful

DTA = findfile data block

CF = set on error

AX = error code

AH=56h, Rename file

-------------------

Parameters:

DS:EDX -> file to rename (ASCIIZ)

ES:EDI -> new filename (ASCIIZ)

Returns:

CF = clear if successful

CF = set on error

AX = error code

3. The DPMI host

����������������

The DPMI host functions are described in a future version of this file. If

you have questions, then get a DPMI 0.9 spec... ;)

4. Contact info

���������������

If you have something to ask/suggest/tell, contact me:

) E-mail: tstrike@kontu.bbs.fi

) Snail mail:

Sami Kantoluoto

Kukonm�entie 1

16600 J�rvel�

FiNLAND

) Voice:

+358-18-1333039 (home)

) BBS:

Eternal Fire, SysOp: Beaver

+358-18-7530120