STDIO.DOC -- Description of the I/O procedures in STDIO.ASM

===========================================================

From `BLUEBOOK of ASSEMBLY ROUTINES for the IBM PC & XT'

by Christopher L. Morgan

Copyright (C) 1984 by The Waite Group, Inc.

Purpose: These routines perform fundamental input/output functions for the

keyboard, screen, and communication lines.  The standard I/O routines

can be redirected.  The communications routines alleviate some of the

problems encountered in DOS and BIOS serial communications handling.

Contains:

---------

COM_INCK	--  Communications line check

COM_INIT	--  Initialize communications line

COM_OFF	--  Communications line off

COM_ON	--  Communications line on

COM_OUT	--  Communications line output

STD_CRLF	--  Standard output with carriage return & linefeed

STD_IN	--  Standard input with echo

STD_INCK	--  Standard input check

STD_INNE	--  Standard input with no echo

STD_MSGOUT	--  Standard output of a message

STD_OUT	--  Standard output

STD_OUTDR	--  Direct standard output

STD_SPACE	--  Standard output of a space

_______________________________________________________________________________

STD_IN -- Standard Input with Echo

---------------------------------

Function: This routine accepts input from the standard I/O device.

Input: This routine waits for one character code from the std input device.

Output: ASCII codes are returned in the AL register.  Each character is

echoed out the standard output device as it is input through the standard

input device.

Registers used: AH is modified; AL is used for output.

Routines called: DOS call #1 (Keyboard input) is used.

_______________________________________________________________________________

STD_INNE -- Standard Input with No Echo

--------------------------------------

Function: This routine accepts input from the standard I/O device. No echo

is generated.

Input: This routine waits for one character code from the std input device.

Output: ASCII codes are returned in the AL register.

Registers used: AH is modified; AL is used for output

Routines called: DOS call #8 (Console output without echo) is used.

_______________________________________________________________________________

STD_INCK -- Standard Input Check

-------------------------------

Function: This routine checks, but does not wait for input from the standard

input device.

Input: Input is from the standard I/O device.  No check is made for ^-Break.

Output: If a character is available, the routine returns with the flag

condition NZ and a character in AL.  If no character is available, then

the routine returns with the flag condition Z, and AL is garbage. No echo

is generated for the input characters.

Registers used: AH is modified; AL is used for output.

Routines called: DOS call #6 (Direct console I/O) is used.

_______________________________________________________________________________

STD_OUT -- Standard Output

-------------------------

Function: This routine sends individual characters out the std output device.

Input: Upon entry, an ASCII code is in AL.

Output: A single character is output through the standard output device.

Registers used: AH is modified; AL is used for input.

Routines called: DOS call #2 (Display output) is used.

_______________________________________________________________________________

STD_OUTDR -- Direct Standard Output

----------------------------------

Function: This routine sends individual characters out the standard output

device, using direct output.

Input: Upon entry, an ASCII code is in AL.

Output: A single character is output through the direct standard output call.

Registers used: AH is modified; AL is used for input.

Routines called: DOS call #6 (Direct console I/O) is used.

_______________________________________________________________________________

STD_CRLF -- Standard Output of a Carriage Return/Linefeed Sequence

-----------------------------------------------------------------

Function: This routine sends a <CR> and then a <LF> to the std output device.

Input: None

Output: ASCII code 13 for <CR> and ASCII code 10 for <LF> are sent to the

standard output device.

Registers used: AX is first saved and then restored.

Routines called: STD_OUT

_______________________________________________________________________________

SDTSPACE -- Standard Output of a Space

--------------------------------------

Function: This routine sends a space out through the standard output device.

Input: None

Output: ASCII code 32 for a space is sent to the standard output device.

Registers used: AX is first saved and then restored.

Routines called: STD_OUT

_______________________________________________________________________________

STD_MESSOUT -- Standard Output of a Message

------------------------------------------

Function: This routine sends a message out through the std output device.

Input: Upon input, DS:SI points to the message.  The message must terminate

with an ASCII code 0.

Output: The individual characters of the message are output through the

standard output device.

Registers used: AX is first saved and then restored; SI is used to point to

the input.

Segments referenced: The data segment must contain the message.

Routines called: STD_OUT

_______________________________________________________________________________

COM_INIT -- Initialize Communications Line

-----------------------------------------

Function: This routine initializes one of the two communications lines.

Input: Upon entry, AL contains the initialization byte as follows:

bit ---- 7  6  5 ---------- 4  3 ------------ 2 -------------- 1     0 ---- bit

---------          ------          --------          -----------

baud rate          parity          stop bit          word length

==========        =========        ========          ===========

000 =  110   	  00 = none   	    0 = 1            10 = 7 bits

001 =  150        01 = odd          1 = 2            11 = 8 bits

010 =  300        10 = none

011 =  600        11 = even

100 = 1200

101 = 2400

110 = 4800

111 = 9600

DX contains the unit number (0=COM_1:, 1=COM_2:).

Output: Output is sent to the hardware controlling the specified com line.

Registers used: None

Routines called: BIOS interrupt 14H (RS-232 I/O) is used.

_______________________________________________________________________________

COM_INCK -- Communications Line Input Check

------------------------------------------

Function: This routine checks, but does not wait for input from one of the

two serial communications lines.

Input: Upon entry, DX contains the unit number (0=COM_1:, 1=COM_2:).  During

the routine, input is from the specified communications line.

Output: If a byte is available, the routine returns with the flag condition

NZ and the byte in AL.  If no byte is available, the routine returns with

the flag condition Z, and AL is garbage.

Registers used: AH is modified; DX is used for input & AL is used for output.

Segments referenced: The system data segment is referenced.

_______________________________________________________________________________

COM_OUT -- Communications Line Output

------------------------------------

Function: This routine sends a byte out one of the two communications lines.

Input: Upon entry, AL contains the byte to be sent out and DX contains the

specified unit number (0=COM_1:, 1=COM_2:).

Output: Output is sent to the specified communications line.

Registers used: None.

Routines called: BIOS interrupt 14H (RS-232 I/O) is used.

_______________________________________________________________________________

COM_ON -- Communications Line On

-------------------------------

Function: This routine turns on the handshaking signal, DTR(line 20), and

RTS (line 4) on the specified communications line

Input: Upon entry, DX contains the unit number (0=COM_1:,1=COM_2:).

Output: Just to the communications line

Registers used: DX is used for input; no registers are modified.

Segments referenced: The system data segment is referenced.

_______________________________________________________________________________

COM_OFF -- Communications Line Off

---------------------------------

Function: This routine turns off the handshaking signal, DTR(line 20), on

the specified communications line.  RTS(line 4) is kept on as it was.

Input: Upon entry, DX contains the unit number (0=COM_1:,1=COM_2:).

Output: Just to the communications line

Registers used: DX is used for input; no registers are modified.

Segments referenced: The system data segment is referenced.

_______________________________________________________________________________

>>>>> Physical EOF STDIO.DOC <<<<<

###########################################################################################################################