[ Article crossposted from comp.lang.asm.x86 ]

[ Author was Jerzy Tarasiuk ]

[ Posted on 25 May 1995 17:37:03 +0200 ]

Real and Protected Modes.

Beginning from 80286, Intel CPUs have ability to work in Protected Mode

(older CPUs have Real Mode only). For compatibility reasons, all CPUs

start in Real Mode after reset. Below are presented main differences

between Real Mode and Protected modes for Intel CPUs. Note there are:

Real Mode, Protected Mode, Virtual 8086 Mode (they will be frequently

called RM, PM, VM86, respectively; also 286+(386+) will mean Intel

80286(80386) or better).

There are some differences between these modes in memory addressing

(PM can address all memory, while RM can't unless it is set in PM on

386+, and VM86 cannot unless using PM supporting it to remap memory

- this way EMM386 works); instruction set (some instruction are not

allowed in RM), privileges (something can be forbidden in PM for less

privileged code, many operations are forbidden in VM86), interrupt

handling. PM supports multitasking, PM can run tasks in VM86 (the

VM86 cannot function alone, must have PM code supporting it; it works

similarly 8086 CPU with few enhancements except interrupt servicing

which goes through PM). PM cannot store data to code segment (unless

by aliasing; MOV CS:[BX],AX is illegal in PM). VM86 and PM on 386+ can

have selective I/O port access restrictions (some ports can be accessed

without causing exception and other can't).

Memory addressing and Paging.

In any mode, opcode defines some offset and segment of referenced memory

address, e.g. mov ax,es:[bx+si+1] - segment es, offset bx+si+1, push si

- segment ss, offset sp-2, opcode itself is referenced by segment cs and

offset ip; the address is translated to Linear Address by adding the

offset to base of the segment and the Linear Address is then translated

to Physical Address which is outputed by CPU on its address pins.

In RM or VM86, the base is segment*10h; in PM the base is taken from

descriptor table (LDT or GDT) and can have any value.

The value in segment register is called "selector" and its bits 15-3

specify offset in LDT or GDT (the offset is multiply of 8), bit 2 is 0

for GDT, 1 for LDT, bits 1-0 specify RPL (Requested Privilege Level).

Unless Paging (possible in PM and VM86, on 386+ only) is enabled,

Physical Address = Linear. With Paging, low 12 bits of Linear Address

go to Physical, other are used as index to two-level page tables

(first bits 31-22 select page directory, then bits 21-12 select page).

Paging can also restrict access to some pages (in a way non-privileged

code can read it only or has no access at all), or define non-present

pages which have assigned physical addresses and put in memory in a way

transparent to program when access to their Linear Address is attempted.

Note Linear Address space is 4GB on 386+, and probably no system has so

much physical memory: Paging makes system able to simulate it has.

Segment has also limit. Initially, the limit is 0FFFFh for all segment

registers and cannot be changed in RM or VM86. In PM it is loaded from

LDT or GDT when segment register is loaded. On 286 in PM the limit can

be up to 0FFFFh, on 386+ in PM it can be up to 0FFFFFFFFh.

Also, PM allows "expand down" segments which allow access from address

limit+1 to maximum possible value of limit (depend on segment type).

Privilege Levels and Rules.

In RM, CPU has full privileges. In PM and VM86, they can be restricted.

This reduces possibility of making disasters by bad code.

Base rules: cannot access more privileged data or call less privileged

code than own privilege (although can return to less privileged code).

Additional: call to more privileged code cannot use any target address

caller wants, it can use addresses specified by system only; call to

more privileged code must change stack to make sure enough stack space

is available for called code (so caller cannot cause crash in it).

There are 4 levels: level 0 is full privilege (except Debug Registers,

which can be protected from access even from level 0; some instructions

are reserved for level 0 only), the bigger level the less privileges

are. Few terms used for Privilege Levels: CPL - Current PL, DPL -

Descriptor PL, RPL - Requested PL (in selector), IOPL (in flags) -

max CPL allowing I/O sensitive opcodes (CLI, STI, PUSHF, POPF,...).

Unless accessing Conforming Code segment, privilege rules require

max(CPL,RPL)<=DPL. To execute code (by FAR CALL or JMP) need DPL<=CPL

(note unless it is Conforming, must be DPL=CPL and RPL<=CPL) - cannot

call less privileged procedure, for example. To transfer control to

code with less PL (more privileged), must CALL via call gate (in such

a case, need max(CPL,RPL)<=gate_DPL, but for code the gate refers to

may be code_DPL<gate_DPL; the gate is entry in GDT or LDT; privilege

rules require also target_code_DPL <= CPL for CALL, = for JMP), this

also requires TR to point to valid TSS because it switches stack: old

SS:[E]SP are pushed on new stack, then parameters (as defined in call

gate) are pushed, finally CS:[E]IP are pushed. On return from the call

CPU detects RPL of CS on stack > CPL and switches stack back (if =, no

stack switch, < inhibited by privilege rules), for proper functioning

parameter counts on RET and in call gate must match. For stack segment

DPL must be equal CPL (so in more privileged mode no crash is possible

due to incorrect stack setting in less privileged, and in the less

privileged there is no access to more privileged mode stack).

The RPL is for system to block possibility to pass a pointer from user

code which is invalid in user mode and valid in system: system uses RPL

as for user code and gets access violation error in such a case.

It can be done using ARPL opcode which adjusts RPL for a selector, and

sets ZF if changed (to inform OS invalid access might be attempted).

OS uses it to set RPL of the pointer to CPL of the application code.

It is possible to check what access having to a segment by opcodes like

VERR, VERW, LAR, LSL. They all set ZF if having access, clear if not.

First two simply verify R/W access, LAR gets bits defining access right

for a segment, LSL gives the segment limit value. These opcodes allow

checking what would cause access violation, instead getting the error.

Some instructions are allowed at CPL=0 only. They are:

Clear Task�Switched Flag (CLTS), Halt Processor (HLT), loading some

system registers (GDTR,IDTR,LDTR,MSW,TR), any access to CRx,DRx,TRx.

Some other require CPL<=IOPL. They are: IN, INS, OUT, OUTS, CLI, STI.

Also, POPF behavior depends on CPL: if CPL>0, IOPL and VM aren't

changed by POPF, if CPL>IOPL, IF (interrupt enable) isn't changed.

Interrupts.

In every mode, there is an array containing information what action is

to be taken in case of interrupt. Its first entry corresponds to INT 0,

next to INT 1, and so on. It is called IDT(Interrupt Descriptor Table).

In RM, each entry in the IDT is simply far address of interrupt service

routine. Initially IDT is located at address 0 and has 100h entries

(400h bytes; some CPU-s have its limit 0FFFFh but the remainder isn't

accessible in RM); on pre-80286 CPUs the IDT address and size cannot be

changed, on 286+ can load and store them using LIDT and SIDT opcodes.

In PM the IDT has 8-byte entries which can be interrupt, trap or task

gates. Trap differs from interrupt by leaving interrupt flag same as

in interrupted code. Task gate causes calling another task. They all

have DPLs and interrupt instruction causes General Protection error

if CPL > interrupt or trap gate DPL. However, other interrupt sources

have "CPL 0" - they can access any gate needed.

Some conditions can cause an Exception. They are (for 80386): divide

error (0), debug exceptions (1), non-maskable interrupt (2), breakpoint

(3), overflow (4, on into opcode), bounds check (5, on bound opcode),

invalid opcode (6), coprocessor not available (7), double fault (8,E),

coprocessor segment overrun (9,P), invalid TSS (10,PE), segment not

present (11,PE), stack error (12,E), general protection error (13,E),

page fault (14,PE), coprocessor error (16); marked by P can occur in

PM and VM86 only, marked by E push error code on stack if they occur

in PM or VM86 (so stack is: error, IP, CS, flags; the error code is

usually either 0 or selector causing the exception (in case selector is

invalid or non-accessible), with flags on low order bits: bit 0 means

external source, bit 1 IDT selector, bit 2 LDT; for page fault it is

set of flags (bits 3-31 undefined): bit 0 set if page protection

violation, 1 if writing, 2 if user mode), most of them push IP of

opcode causing them, except 3,4,9 which push IP of next opcode.

Note: interrupt cannot be serviced at PL>CPL (unless via task switch),

attempt to do it causes General Protection error.

Interrupt processing in PM is more complicated when interrupt handler

has Privilege Level other than current code. It is handled similarly

CALL via gate: stack is switched, new SS:SP are taken from TSS, old

SS:SP are pushed on the new stack, then flags, CS, IP and eventually

error code (for some exceptions) are pushed.

In VM86 interrupt pushes GS,FS,DS,ES,SS,ESP,EFLAGS,CS,EIP (exception

also error code) onto PL 0 stack. There is VM bit in EFLAGS set to tell

interrupt occured in VM86. Note IDT must contain task gates and 80386

trap or interrupt gates pointing to a non-conforming code segment with

DPL=0 only - interrupt service must come through PL 0 or task switch.

The VM86 itself has CPL 3 and is allowed in 386 task only.

Descriptor Tables (PM only).

Global Descriptor Table(GDT) can contain descriptors of any type except

interrupt and trap gates. It is necessary for PM. First entry in GDT

isn't used - it corresponds to null selector which can be loaded into

segment register but causes exception if used for memory addressing.

Local Descriptor Table(LDT) can contain "normal" segment descriptors

(not e.g. TSS) and call or task gates only. Usually every task has its

own LDT (changed on task switch). The LDT must have descriptor in GDT.

Interrupt Descriptor Table(IDT) was discussed in "Interrupts" section.

"Normal" segment descriptors are referenced when a segment register is

loaded and they describe a memory area and give some access to it.

Bit 2 of selector used selects table: 0 means GDT, 1 means LDT.

Other descriptors can be Task State Segment(TSS), and gates. They can

be referenced "as a code segment", e.g. by far jump or call and they

cause transferring control to task or code segment referenced by them.

It is kind of indirect jump or call (they contain target selector).

TSS or gate pointing to TSS cause task switch. Gate can be used to

transfer control to more privileged code not accessible directly.

TSS can be also referenced by LTR (Load Task Register) opcode and it

is done once during PM initialization. LDT descriptor can be loaded

into LDTR(register) by LLDT opcode and usually it is done once.

Segment and System Descriptors.

The following segment types (in byte [descriptor+5]) are supported

(for all bit 7 means present in memory, bits 5-6 keep DPL which says

what is maximum CPL which can access the descriptor, the restriction is

for all descriptors, not segments only, except conforming segments):

10h+flags	- data: bit 1 - writable, bit 2 - expand down

18h+flags	- code: bit 1 - readable, bit 2 - conforming

for both, bit 0 is set by any access. The descriptor also contains

limit in word [0] (in 386 segments extended to bits 0-3 of byte [6])

and base in bytes [2..4] (in 386 segments extended to byte [7]).

Byte [6] keeps few additional flags: bit 7 - granularity (limit is in

4kB pages; e.g. limit 0 means 0..0FFFh accessible), bit 6 - 32-bit

addressing (applies to code and stack - use EIP, ESP, makes expand down

segment upper limit 4GB), bit 5 must be 0, bit 4 is for programmer.

01h+flags	- TSS: bit 1 - busy, bit 3 - 386 TSS

02h		- LDT

04h+flags	- call gate

05h		- task gate

06h+flags	- interrupt gate: bit 0 - trap, bit 3 - 386.

for all gates, word[2] keeps selector, word[0] and word[3] keep offset

of called code (ignored for task gate), byte[4] keeps word count (0-31)

for copying in case of inter-level call (call gate only, else ignored);

TSS and LDT have base and limit in same form as code and data segments

have, they can have bit 7 set in byte [6] to specify limit in pages.

Word [6] should be 0 for the descriptor to mean the same on 286/386.

LDT is similar GDT, except not all descriptor types are allowed.

TSS holds entire task state (all registers: general, segment, flags,

ip, ldtr); it also keeps link to caller TSS (valid if the task was

activated by INT or CALL) and stacks (SS and [E]SP) for PL 0,1,2

(they are used when more privileged code is invoked via gate from less

privileged). 386 TSS has also debug trap bit (if set, causes INT 1 on

task switch to the TSS), I/O bit map (saying which I/O addresses can

be accessed when CPL>IOPL without General Protection exception), and

CR3 value for the task (can remap memory on task switch).

Page tables:

both page directory and page table entries keep referenced address in

bits 31-12, have bits 11-9 reserved for programmer, must have bits 8,7,

4,3 set to 0; bit 5 is called A (accessed), it is set by CPU on access

to the entry, bit 6 is called D (dirty), it is set if referenced memory

is written; bit 0 is called P (present), all other are ignored if it is

not set; bit 2 allows user (CPL=3) access if set, bit 1 allows user to

write (together with bit 2 only), for CPL<3 read/write is allowed for

any setting of bits 1 and 2 (no protection against system this way).

Note page table entries used are usually cached by CPU: modifying them

in memory may cause no mapping change until the cache is reloaded. The

cache is flushed every time CR3 (which points to first page directory

entry) is loaded. Bits 0-11 of CR3 must be 0 (directory page-aligned).

Addressing through page tables: CR3+(Linear_Address SHR 20) AND 0FFCh

is address in Page Directory, the entry at the address contains Page

Table address; Page Table address + (Linear_Address SHR 10) AND 0FFCh

is address in Page Table and the entry at the address contains base

address of the page, combine it with bits 11-0 of Linear_Address and

the result is Physical Address. In case of any error, CR2 is set to the

Linear Address causing the error and error code explains what error.

Note: if Paging is enabled, CR3 must keep Physical Address of Page

Directory and all other addresses are Linear Addresses.

Switching to Protected Mode or back to Real Mode:

First: to get control in case of crash, need store in dword [0467h]

address where control is to be passed, and put 0Ah in CMOS register 0Fh

(by CLI; MOV AL,8Fh; OUT 70h,AL; (1us delay) MOV AL,0Ah; OUT 71h,AL;).

Also: normally, some circuitry in PC compatibles disables address line

A20; must enable it. If you use HIMEM, it can be enabled by a request

to HIMEM. If you also have DOS=HIGH, it is usually enabled, as it is

enabled by any DOS call. In other cases, you must send output port

value to keyboard controller to enable it before switching to PM.

Switch to PM: required is loading GDTR, then can enable protection by

setting CR0/MSW bit 0 (MOV EAX,CR0; OR AL,1; MOV CR0,EAX; or SMSW AX;

OR AL,1; LMSW AX; first on 386+, second on 286+); it is recommended

to load IDTR immediately before or after mode switch (same IDT can't be

valid in both modes); immediately after mode change should execute JMP

to flush prefetch queue which may be partially decoded (the decoding

may be mode dependent); need load CS and SS - they contain invalid

selectors and e.g. interrupt causes them to be put on stack and crash

on IRET; it is also recommended to load all segment registers (they can

be loaded with 0 to contain invalid selector and cause exception if any

of them is used to address memory) and LDTR; before first task switch

must load TR (selector of valid free TSS descriptor; the TSS will be

used to store state on task switch).

These is also a BIOS call which switches to PM and changes external

interrupt vector mapping (normally 1st controller has 08h..0Fh, 2nd

70h..77h, the 1st conflicts with some CPU exceptions; however it is

easy to distinguish external interrupt from an exception), it also

enables address line A20. See INT 15h, AH=89h description.

Returning to RM: it can be done by clearing bit 0 in CR0 but it needs

some preparation: must disable paging (go to code/stack which has

linear addresses same as physical, clear PG bit in CR0, clear CR3), go

to code segment with limit=64k and load all segment registers except

CS with valid descriptor of 64kB read/write expand-up byte-granular

present segment (attribute byte=93h, extended attribute=0) - otherwise

you can get RM with e.g. read-only or 32kB ES, which will soon cause

crash. After clearing the bit 0 of CR0 execute far jump to load CS and

flush prefetch queue and load segment registers for RM.

This is not available on 80286 which has no CR0 register (the Protect

Enable bit cannot be cleared by LMSW). The only way to get to RM again

is resetting the CPU: it can be done by the following code: CLI;

XOR CX,CX; wait_kbd_ctrlr_input_empty: IN AL,64h; TEST AL,2; LOOPNZ

wait_kbd_ctrlr_input_empty; MOV AL,0FEh; OUT 64h,AL; HLT; or by CPU

shutdown (resulting in case of exception while servicing double fault).

Note most programs running system in VM86 provide interface to switch

to PM and back to VM86, it is called VCPI (Virtual Control Program

Interface), can be tested for presence and invoked by INT 67h,AH=0DEh.

It requires 3 entries in GDT to be reserved for VCPI provider.