TWEAK 1.6� - Mold your own VGA modes

by Robert Schmidt of Ztiff Zox Softwear, 1992-93

This program and the accompanying source files

are hereby donated to the public domain.

(for whatever that's worth...)

(Sorry for not page-formatting this doc.  The headers can be found by

doing a text search with your favourite editor.)

Contents:

WHAT IS TWEAK, ANYWAY?

SUGGESTION BOX

DISCLAIMER

A QUICK PRIMER

WHAT HAS TWEAK GOT TO DO WITH THIS?

A COUPLE OF RULES

A TUTORIAL

EDITOR REFERENCE

FREQUENTLY ASKED QUESTIONS

THE TWEAK FILES

USING TWEAK MODES ON YOUR OWN

THE INCLUDED FILES

THE 'MISC' DIRECTORY

CREDITS

BIBLIOGRAPHY

HOW TO REACH ME

WHAT IS TWEAK, ANYWAY?

TWEAK is an utility to ease the work of twiddling with the registers on

a standard VGA compatible video card to produce and explore new, previously

undocumented screen modes with weird resolutions.  You will want to

purchase a technical VGA reference to get the full potential out of

TWEAK, but I included some files to get you started (see the section

on the 'MISC' directory).

TWEAK.EXE is the main executable.  This version is not compatible with

any versions prior to 1.0, so if you want to use files created by

version 0.9x, convert them with 09TO10.EXE.

Stop press!  In the subdirectory XINTRO is an article which serves as

an introduction to programming in Mode X (320x240 with 256 colors).

Make sure you read this if you're new to tweaking and Mode X!

MODES.DOC describes some of the more exciting 256x256, 400x300

and 400x600 256-color modes!

SUGGESTION BOX

All suggestions to enhance or modify TWEAK are welcome, especially bug

reports/fixes.  So are donations, for that matter.  See e-mail and

snail-mail addresses at the end of this file.

Please send me any changes you make to enhance TWEAK or to make it

compatible with other compilers or video cards, and I will start

including conditional sections for each supported compiler.

Do *not* re-release the changed source without my permission.

Well, how can I stop ya...?

Any suggestions and contributions will be credited in subsequent

versions, although I can't guarantee that your ideas will actually be

used.

DISCLAIMER

I don't think this is neccessary in PD stuff, but everybody else does

it:

The author author of this archive has used his best efforts in

preparing it.  However, the author makes no warranty of any kind,

expressed or implied, with regard to the programs, data or

documentation included with the archive.  The author shall not be

liable in any event for incidental or consequential damages in

connection with, or arising out of, the furnishing, performance,

quality, or use of the programs, data or documentation included in this

archive.

Phew.

Some time ago, putting illegal or unsupported values or combinations

of such into the video card registers might prove hazardous to both

your monitor and your health.  I have *never* claimed that bad things

can't happen if you use TWEAK, although I'm pretty sure it never will.

I've never heard of any damage arising from trying out TWEAK, or from

general VGA tweaking in any case.  I did receive a mail from a person

whose monitor failed producing the correct colors, after using mode

X.

A QUICK PRIMER

You never heard about neither documented nor undocumented modes, you

say?  Well, to begin with:  Your VGA card has a number of registers

that control the way the card works.  That is, how it is going to

translate the data that programs put into the video memory, to the

signals that produce text and/or graphics on your monitor.

The standard interface to a video card on all IBM compatible PCs is the

BIOS, which consists of several device independant routines for setting

screen modes, moving the cursor, writing text, scrolling blocks of text

etc.  The BIOS also takes care of setting the appropriate registers at

appropriate times, for example when changing screen modes.  That way,

programmers have a consistent interface to the VGA, and usually won't

need to tamper with the registers directly.

A screen mode specifies the resolution of the image you see on the

screen, i.e. the number of pixels (dots) horizontally and vertically,

the number of colors, and wether the screen should be capable of

showing just text, or if graphics are allowed.  These things are

controlled by the VGA registers, and the BIOS contains a number of

predefined tables of register values for the standard VGA modes we've

all come to love and honour.  Mode number 3, for example, is the 80

characters times 25 lines (80x25) text screen that most people use

daily.  Mode number 19 (or 13h in hexadecimal) is the 256-color 320x200

mode used in most popular games supporting the VGA.

WHAT HAS TWEAK GOT TO DO WITH THIS?

Well, some people, me included, are of the 'power-hungry' breed.  We want

to exploit the full potential of everything we get between our hands.

We think that the one 320x200 256-color mode supported by all standard VGA

BIOS'es is for wimps.  Some guy came up with the idea of modifying the

register configuration, to achieve greater resolution and a different

video memory layout.

TWEAK grants you direct access to all the most significant registers

that control such things as resolution and colors.  You may have thought

that your VGA was limited to 320x200 when it was displaying 256 colors?

Well, all VGAs are able to support no less than 400x600 in 256 colors.

(The problem is with the monitors, which might not be sophisticated

enough to support the relatively much higher clock frequency needed to

output that many pixels.  My monitor flickers and rolls occasionaly.)

Most newer VGA cards' BIOSes add several enhanced modes not supported

by the standard VGA defined by IBM, for example text modes with 132

characters on each line, or graphics resolutions of 800x600, 1280x1024

and more.  Such cards are usually referred to as Super VGAs.  To make it

possible to produce such resolutions, video card developers has had to

add new registers to the set of registers defined by the VGA.  TWEAK is

generally not able to support such extended registers, meaning that you

will not be able to tweak a Super VGA any more than is possible on a

standard VGA.  Extensions that are provided by utilizing undefined bits

in the standard VGA registers are supported however.  The included

Chips & Technologies specific 132x*.twk files are examples of such.

The main reason for not including Super VGA (SVGA) support is that

there are so many different SVGA standards.  Nearly every developer has

their own standard.  Also, I can see no real reason for wanting to tweak

any SVGAs, as the resolution dimensions they offer usually are far

superior to the VGA's resolutions.  With tweaked VGA modes there's

a much higher propability that they will work on most VGAs and SVGA,

than tweaked SVGA modes.

If this is the first you have read about VGA registers and the BIOS,

TWEAK.EXE and this documentation alone is not enough to get you going.

You will need, and probably want, a somewhat technical reference to the

VGA, which explains all standard VGA registers and the meaning of every

contained bit.  Although TWEAK doesn't support it, you'll probably want

a reference which also contains some discussion on Super VGAs, just for

your convenience when you want to program serious applications etc.  If

you're of the adventurous breed, check out the VGA.TXT and VGABIOS.TXT

files included in the .\MISC directory.

You see, tweaking the VGA is not as simple as putting the horizontal

resolution in one register, the vertical in another and the number of

colours in yet another.  Several registers has to be set to cooperating

values that work together to acheive what you want.  Single bits has to

be toggled into the correct states, even before you can see anything on

screen.  TWEAK simplifies this process by letting you do this

interactively, and by letting you test your register set at the touch

of one key (ENTER, actually).  The pre-TWEAK process was to make a small

program that set up the registers in the way the programmer thought

would work.  If it didn't he had to edit the program source file,

recompile, and re-run.  Not seldom, the PC crashed because of some

*really bad* register values, and a full reset was required.

TWEAK let you save and load register sets to and from files, and let

you select any video mode supported by your BIOS to work out from.

It supports text modes and graphics, 16-color and 256-color.  Monochrome

modes and CGA modes (4- and 2-color), however, are not supported,

because they use another set of I/O ports, and because few people are

really interested in them.

As of version 1.6, TWEAK further simplifies the VGA tweaking process by

providing instant feedback on the register values, in the form of a

live mode heuristic, i.e. a detection scheme tried to figure out what

type of mode your making, the number of colors, its physical and

virtual resolution, the number of pages (both horizontally and

vertically).

A COUPLE OF RULES

I'll be the first to admit:  TWEAK isn't much of an editor.  The screen

is boring, there is just one simple help window, no undo command, no

file pick lists etc.  In short, TWEAK is not meant for people not

knowing what they are doing.  I strongly beleive that nothing can be

damaged from the use of TWEAK, but for starters, I'll recommend the

following precautions:

*  When loading/saving, take care to check that you selected the

function (load or save) that you want, before typing a filename

and pressing Enter.  Also verify that the filename you type when

saving is the filename that you want, because TWEAK overwrites

any existing files with the same name without asking for permission.

*  If, when you press the Enter screen to test the screen mode,

nothing appears after 5-10 seconds, press Enter again.  If you're

not immediately returned to the editing screen, reboot your

computer by pressing the reset button.  Keep in mind that some of

the test patterns, especially the 256-color 4-planar one, can be

slow to put into the video memory buffer, so be a *little*

patient & tolerant.

That last rule is present to prevent that the monitor is exposed to crazy

sync timings for too long a time.  And I repeat, there usually never is

any reason to panic, even if your monitor makes strange sounds

('tweeeeee' for example, or 'flick-flick-flick').  Press ENTER, wait for

a couple of seconds more, then reset your computer if the edit screen

doesn't reappear, and try a different approach.

A TUTORIAL

To start TWEAK, change to the directory where you have put the files,

and type TWEAK at the DOS prompt.  Well, now you're in the editor, and

you can see the following things (you might want to print this tutorial

so you have it on paper):

*  One (possibly two) column(s) of VGA registers on the form:

ppp (ii)         Register Name : vv

where

-  ppp is the port number

-  ii is the index into that port, if applicable

-  vv is the selected value for this register

All numbers are hexadecimal.  The current register is marked by one

arrowhead on each side of the line.  When you first start TWEAK,

the current register is the top left.

*  A bit pattern display showing the bit pattern of the 8-bit value

contained in the currently selected register.

*  The bottom line tells you which test pattern is currently active.

*  You will also see the mode heuristic which usually makes a good

guess at what mode you have at the moment.

When executed, TWEAK starts up by reading the registers of your current

BIOS mode, so that you have something to work with.  This is usually a

text mode, 80x25 for example, so you'll probably want to set a graphics

mode for more interesting results.

Now try the following simple tutorial:

1. Press H, and a red window should appear, describing all keys

available for use in TWEAK.  These will also be described below.

2. Press M, then type the number 03.  Notice that you don't have to

press ENTER.  You have now selected BIOS mode 3 as the basis for

you explorations.  Notice also the information provided by the

mode heuristic on the right.

3. Press TAB until 'Text screen, 16 point' appears at the bottom line,

if it doesn't already.  TAB selects which test pattern to use, and

you have now readied the 16-point font version of the text screen

test.

4. Press ENTER.  You should see a screen with numbers along the top

row, and various characters and colors down the rest of the screen.

This is the text test pattern, as it looks when viewed in mode 3.

Press ENTER to return to editing mode.

5. Now press F10, and type '40x12' and press ENTER.  This loads the

file '40x12' and uses its contents as the current register set.

Notice how the mode heuristic immediately changes.

6. Press ENTER again.  A screen similar to the previous test pattern

should appear, but the characters should be twice as wide and twice

as high.  This is a tweaked VGA mode, which is not supported by the

BIOS.  It looks a little odd, as the bottom line is cut in half,

making this a 40x12.5 text mode, in fact.  Press ENTER to return.

7. Press F10, type '360x480.256' and ENTER.  Take a look at the

mode heuristic again.  Press TAB until the text 'Graphics

autodetect' appear in the bottom line.

8. Press ENTER.  This is a well known tweaked VGA mode that has even

been used in commercial games for the PC, but it remains

unsupported by the BIOS (and it probably always will).  If the

screen rolls, try adjusting your monitor knobs.  If you can't get a

steady picture, it probably means your monitor isn't capable of

handling the horizontal resolution of 360 pixels.

9. Try using the arrows to scroll the virtual screen.

9. Press ENTER to return to editing, then press ESC and 'Y' to quit

TWEAK.

The last graphics display might not have been especially pleasing to the

eye.  First, if the picture was rolling uncontrollably, you might think

that TWEAK is of no use for you, as your monitor isn't good enough.

However there are still interesting things to try out.  There are

tweaked modes 'available' with a little lower resolution that are almost

guaranteed to work on your monitor, and which are of great interest to

games programmers for example.  The file 320x240.256 contains the

register set for the infamous Mode X, which was 'discovered' and

documented by Michael Abrash in his monthly columns in Doctor Dobbs

Journal.

Following points 7 onward above, see if you can load Mode X into the

editor and get the test screen going!

Mode X has the following interesting properties:

*  The pixels are 'perfectly' square.  That is, if you try to draw a

circle with aspect ratio 1:1, it will look like a circle (unless

your monitor is adjusted to some extreme).  The BIOS mode 13h

(320x200x256) has pixels that are a little higher than they are

wide, making 1:1 circles look stretched vertically.

*  The video memory is divided into 4 planes, each of which contains

64 Kb.  By setting the Write Plane Enable register to the actual

plane, you can address the full 256 Kb of video memory through the

'tiny' address space of 64 Kb from 0xA000:0 to 0xA000:0xFFFF.

Thus, you can have more than three full screens in video memory at

any one time, making you able to perform animation tricks as

page flipping, and to do fast 32-bit video to video transfers.

This was just to get you going, and others have written tons of text on

drawing stuff (lines, circles, images) in Mode X.  The rest of this

part will deal with the commands available in TWEAK.  To get more

information about VGA programming in general and registers and Mode X

in particular, check out the reference list somewhere at the end.

Stop press!  Read the article in the XINTRO subdirectory for an

introduction to Mode X.

EDITOR REFERENCE

This reference applies only to version 1.0 of TWEAK or later, although

*most* keys are supported by the included version 0.95 too.

Select the register value you want to modify using the following keys:

Up         selects previous register

Down       selects next register

Home       selects first register

End        selects last register

Use these keys to modify a register value:

a 2-digit, hexadecimal number : sets the register to the given value

'-'        decreases the value of the register

'+'        increases the value of the register

F1 ... F8  toggle bits 7, 6, ..., 0 of the selected register,

respectively.

Use these keys for testing the set of register values:

TAB [->|]  cycle among the available test patterns, to find the

one you think will suit your custom register set.

Shift+TAB cycles in the opposite direction.

ENTER      sends your custom register set to the VGA registers, then

writes the selected test pattern to the video memory buffer.

Some other important or useful actions:

Backspace  toggles the active state of the current register.  If a

register is inactive, it color is grey instead of yellow.

When you test the mode (by pressing ENTER), inactive

registers will *not* be sent to the VGA card.  Also, they

will *not* be saved to the file when a Save command is

executed.  This feature is included to enable you to ignore

registers that doesn't affect your screen mode.  For

example, for most practical uses, the Color Compare register

can be deactivated.

'M'        prompts the user for a 2-digit, hexadecimal number

specifying a standard VGA BIOS screen mode number.  Note

that TWEAK does *not* support true monochrome modes.

The VGA is set to this mode using INT 10h with AX=mode

number.  Then all relevant registers are read from the

VGA into the editing set, and you're returned to the

editing screen.

F9 or 'S'  prompts the user for a file name, then saves the register

set to this file, overwriting any pre-existing

file by the same name.  Inactive registers are not saved,

thus there are no longer a fixed size to TWEAK's mode

files.

F10 or 'L' prompts the user for a file name, then loads the register

set from this file, if it exists.  Registers that are

supported by TWEAK, but that are not included in the file,

will be deactivated (greyed).

ESC        quits TWEAK, if you answer 'y' or 'Y' to the 'Really

quit?' question.  Make sure you have saved your work.

Pressing just ENTER at one of the two file name requests will cancel

the file operation.

There are currently 6 available test screens with TWEAK.  Numbers in

parentheses are all the standard VGA BIOS modes that display the test

patterns correctly:

o  An autodetected graphics screen test.  This will detect *most*

EGA and VGA graphics modes (of course no monochrome modes), and

display a pretty screen containing:

-  axes along the left and top borders visualizing the resolution.

-  text telling you both the physical and the virtual resolutions,

and the number of colors.

-  the entire palette available in this mode.  Note that the

palette is *not* set, so the colours are not guaranteed to be

the normal VGA palette, except in 16-color mode.

The following keys are available in the autodetect test screen:

-  arrows scroll the screen across the virtual area

-  ESC returns to the editor

If the mode being edited is not a graphics mode, a text screen

appears with some helping hints.

o  2 text test screens at 0B800h

-  one using the normal 8x16-point VGA font (0, 1, 2, 3)

-  one using the 8x8-point CGA font (normally used in 43/50-line

modes, but none of those are defined by the VGA BIOS.  Check

VGABIOS.DOC for the function to load a specific font in your

own programs.)

o  A screen for 4-planar, 16-color modes at 0A000h (0Dh, 0Eh, 10h,

12h)

o  A screen for 4-chained, 256-color modes at 0A000h (13h)

o  A screen for 4-planar, 256-color modes at 0A000h (None supported

by BIOS)

See the source (TESTPAT.CPP) for descriptions on how the test patterns are

supposed to look.  Generally, if it looks good, it should be correct.

FREQUENTLY ASKED QUESTIONS

Q:      I'd like to study the register configuration for a screen mode

supported by my (Super-)VGA.  How do I get to it?

A:      The 'M' key lets you select a BIOS mode to study.  Note that you

will need to know the mode number.  Note that TWEAK does not

support any SVGA-specific registers.

Q:      None of the tests seem to produce sane results.  The screen

a) goes black,

b) rolls or

c) the program crashes.

A:      a)  - Make sure you tried ALL 5 tests.

- Set the Color Plane Write Enable register to 0Fh.

- Note that Super VGA modes are generally not supported

(though they *might* work).

b)  - The timing/sync registers are not set correctly/in sync.

See your VGA reference for more information.

- Try adjusting the knobs on your monitor.

c)  - I know some (S)VGAs completely hang the system if

unsupported bit combinations are fed into the Misc.

Output Register (0x3c2).  This is one of the most useful

registers, however, so experiment with care, and take note

of what values causes the crash.

- You may have hit a major incompatibility/bug.  Send me a

mail, telling me what kind of hardware you're using.  I

will probably not be able to fix it, but an incompatibility

list will be emitted with the next release, if any.

Q:      How about a TSR to save the current register configuration from

any program?

A:      I've considered this, but haven't had the time.  A hint, though:

Get CBOOTxxx.ZIP from simtel or any mirror, in .../msdos/sysutl.

This program lets you break out from most applications,

*without* resetting the screen mode.  A typical session:

- Install CBOOT.

- Run FRACTINT (a fractal explorer package, supporting lots

of tweaked modes), and select the mode you want to 'grab'.

- Press ALT+SHIFT+B, and the CBOOT menu pops up.

- Press '7' to reset interrupt vectors.

- Press '6' to 'properly' exit from FRACTINT.

- You might not see it, but you should be at the DOS prompt.

- Run TWEAK directly, which will start up with the current

register configuration.  Now you're off!

From programs (games) which assume total keyboard domination, I

currently cannot help you grab modes.

Q:      I'd like to use tweaked modes in my own programs.

A:      Provided you have produced TWEAK files corresponding to your

modes, take a look at the example files for different approaches

to using the mode files.  The examples are in C, but are simple

enough, so translation to Pascal should be a breeze.

Q:      - What is the register which makes x do y/sets a to b/etc.?

- Is there a BIOS call to do x?

A:      First check the VGA.TXT and VGABIOS.TXT files in the MISC

directory.  If they don't help you, please consider buying a

technical reference to the VGA.  That would please you and me.

Q:      How do I make a mode with resolution x times y with z colors?

A:      - See the sample *.TWK, *.256 and *.16 files provided with

TWEAK.  Not a great lot, but you might be able to work out

something from one of those.

- Get FRACTINT or SVGABGI, and use the method mentioned above to

'grab' modes from these programs.

- Learn what each of the VGA timing registers means.

- Experiment!  That's what I had to do.

Q:      I can't find this MISC or XINTRO directory!

A:      Make sure you unzip the TWEAK archive with the -d option, which

is needed to extract subdirectories.  I.e:

PKUNZIP -D A:TWEAK10

If you didn't use -D, the files in the MISC directory are mixed

in with the rest of the TWEAK package files, all in the same

directory.

Q:      I have lots of files with modes that I saved with version 0.9 of

TWEAK...

A:      Use the 09TO10 utility to convert them.  Run 09TO10.EXE with no

parameters for a simple help screen.

Q:      What do I need to rebuild TWEAK and/or the utilities?

A:      You will need Borland C++.  I used version 3.1, but it might

work as far back as Turbo C++ 1.0.  I included a Makefile to

make rebuilding as painless as possible, provided you have

BCC.EXE in your path, and it knows where to find headers and

libraries (usually it does).  I know some of the sources

produce warnings, but feel free to ignore them.  I did.

Q:      What do I need to use the mode files produced by TWEAK in my own

programs?

A:      If you want to use the TwkUser module, you'll need a C compiler.

I don't think I used anything Borland specific here.

Otherwise the file format is pretty simple, so you should have

no problem making similar functions/procedures using any

language (assembler, Basic, Pascal, Prolog...).

Q:      Where can I find more information on tweaking the VGA?

A:      See the Bibliography section below.  Michael Abrash's articles

in Doctor Dobb's Journal from a year or so are probably the best

sources.  Join the rec.games.programmer newsgroup.  There has

been some discussion on tweaking there, especially mode X

(320x240x256) and how to optimize code for this mode.

Q:      How can I ever repay you for making such a great utility?

A:      Easy!  The cheapest way is to send me a cool postcard with some

(readable) words on it.  I will of course accept donations too,

even though TWEAK is public domain.  See the end of this file.

Q:      PKUNZIP refused to unzip the TWEAK archive!

A:      I guess you decoded this DOC file by hand, then...  well, make

sure you transfer in BINARY mode from the ftp site, and in

BINARY from your user account to your PC.

Q:      I can't find my question in the Frequently Asked Questions list!

Does this mean I'm stupid?

A:      It might.  :-)  However, I just thought up all these questions

myself, so if you have a suggestion for more FAQs, don't

hesitate to let me know!  I promise I won't laugh...

Q:	The mode heuristic tells me I have the mode I want, but none of

the test screens works, not even the autodetecting one!

A:	The heuristic is far from perfect.  It does NOT verify that the

timing values you are using are sane - it merely checks the

registers determining the logical resolutions.  The best advice

I can give is to get a VGA reference and try to learn what the

horizontal and vertical timing registers do.  TWEAK is less

intelligent than you (hopefully)!

THE TWEAK FILES

The file format used for saved files is pretty simple.  The files will

usually be bigger than the files saved with TWEAK 0.9, and version 0.9

files are *not* readable by version 1.0.  This is undetectable by TWEAK,

because of the simple nature of the files.  (No headers are present!)

If you try, there's a fat chance that your computer will hang.

Here is the format:

offset 0: WORD - port number of first register

offset 2: BYTE - index of first register

offset 3: BYTE - value of first register

offset 4: WORD - port number of second register

.

.etc.

.

Pretty simple, as you can see, but also flexible from TWEAK's point of

view.  This makes it easy to add new ports if neccessary.  The file size

is not constant, as registers that are disabled at save time are not

written to the file.  Divide the file size by 4 to determine the number

of registers in a file, or just read to EOF (as TWEAK does... :).

Also note that the VGA registers use varying methods for access.  For

some registers you just send the value directly to the port (ignoring the

index).  For some you send the index to the port, then the value to the

port+1.  For still some you have to ...  Rather, refer to the source

code for all the how-to's...  it's not difficult, just inconvenient, and

it hinders really general storage of register addresses and their values.

Use the 09TO10.EXE program to convert from version 0.9 files to version

1.0 files.  Run 09TO10.EXE with no parameters for information on how to

use the utility.

I have selected the following standard of file extensions for

TWEAK-files, but these are just suggestions for my own convenience:

o  *.TWK are text modes

o  *.16 are 16-color graphic modes

o  *.256 are 256-color graphic modes

Note that you'll always have to type an extension in TWEAK if you want

any, as TWEAK neither assumes anything nor provides default extensions.

I have also started addint a 'c' to the name (like in 256x256c.256) if

the mode is chained, i.e. uses linear addressing like mode 13h.  I add

an 's' if the mode is specific to my Chips & Technologies SVGA.

Make sure you read MODES.DOC!

USING TWEAK MODES ON YOUR OWN

The Register and RegisterTable classes used in TWEAK.CPP might be a

little huge and clumsy to use in your own programs, where you probably

just want to set the registers according to a linked-in array.

Therefore I have provided a simple C module for the following functions:

o  Reading a file saved from TWEAK into a dynamically allocated

array of registers.

o  Setting VGA registers according to the contents of an array of

registers.

o  Setting a single register.

The types and functions are declared in TWKUSER.H, which should be

included in every source file using these functions.  The definitions

are contained in TWKUSER.C, which should be compiled under the wanted

memory model and linked together with your program modules.  You are

free, in fact you're encouraged to modify the TWKUSER files to suit

your own needs.  In their present form, they just provide enough

functionality to get you started.

A couple of simple examples are provided - the second one is easiest to

follow, and I think you will prefer that one:

o  EXAMPLE1.C - demonstrates how to use a TWEAK-generated file

in your own program by loading the file at run-time, and setting

the VGA registers according to the file contents.  The mode file

has to be available at run-time.

o  EXAMPLE2.C - does the same, but now the TWEAK-file is converted to

a C-includable file by using the TWEAK2C utility.  Thus, the contents

of the TWEAK file is linked with the program as global data, and no

external file is needed in addition to the executable.

Both programs set the VGA to the famous Mode X, 320x240 in 256 colors,

then fill the screen with some colors.

Note that since the files produced by TWEAK does not keep any

information on what BIOS mode they are based on, you are responsible

for setting the palette used.  Keep in mind that you need to set both

the EGA and VGA palette for 256-color modes!  The easiest way to

accomplish this is to set mode 13h before outputting your register

settings.

I would very much like to provide similar Borland Pascal examples, but

as I don't have Turbo/Borland Pascal available at the moment, I'm just

going to skip it at this time.  If *you* feel like porting the TwkUser.C

and .H files to a Pascal unit, please do.  It should be a peice of cake.

If you mail the result to me, I'll probably include it in the next release

of TWEAK, and in any case you'll be credited for your contribution.

THE INCLUDED FILES

This is a dump of the 4DOS compatible DESCRIPT.ION file included in this

archive:

320x200.256 Planar 320x200x256

320x240.256 Planar 320x240x256 (Mode X)

360x480.256 Planar 360x480x256

400x300s.256 Tweaked C&T SVGA planar

400x600.256 Tweaked VGA, req. good monitor

400x600s.256 Tweaked C&T SVGA planar

432x600s.256 Tweaked C&T SVGA planar

800x600s.16 Standard C&T SVGA BIOS mode

40x12.twk Standard VGA BIOS mode 1, double scanned

80x43.twk Standard VGA mode, needs 8x8 font

80x50.twk Standard VGA mode, needs 8x8 font

09to10.cpp Version 0.9x to 1.0 conv. util. source

800x600.16 Tweaked VGA, req. good monitor

example1.c C source for EXAMPLE1.EXE

example2.c C source for EXAMPLE2.EXE

makefile Type MAKE ALL to update TWEAK project

namedreg.cpp C++ source defining NamedReg. members

register.hpp C++ header declaring Register & NamedReg

regtable.cpp C++ source defining RegisterTable

regtable.hpp C++ header declaring RegisterTable

screen.cpp C++ source defining screen functions

screen.hpp C++ header declaring screen functions

tweak.cpp C++ source defining TWEAK's main program

tweak.doc Documentation for the TWEAK archive

tweak2c.cpp C++ source for the TWEAK2C utility

twkuser.c C source defining som usable functions

twkuser.h C header with TwkUser.C prototypes

register.cpp C++ source defining Register members

320x240.c C file created by TWEAK2C

c&t.dat Text file: list of supported registers

256x256.256 Planar 256x256x256 (Mode Q)

256x256c.256 Chained 256x256x256 (Mode Q, chained)

testpat.cpp C++ source defining TestPatterns members

testpat.hpp C++ header declaring TestPatterns

256x240.256 Planar 256x240x256

360x270.256 Experimental mode

400x300.256 Planar 400x300x256 - great!

detect.cpp C++ source for mode detected module

detect.hpp C++ header for detect.cpp

misc.hpp Various common routines and macros

tweak095.cpp C++ source for TWEAK version 0.95

vgalib.cpp The mode-independant VGA library source

vgalib.hpp Header for VGALIB.HPP

regedit.cpp C++ source for the RegisterEditor class

regedit.hpp C++ header for REGEDIT.CPP

descript.ion Descriptions of all files

tweak.prj TWEAK archive BC++ 3.1 project file

320x400.256 Planar 320x400x256

360x360.256 Planar 360x360x256

360x400.256 Planar 360x400x256

376x564.256 Planar 376x564x256

tweakold.dat Data file: list of supported registers

132x25s.twk Standard C&T SVGA BIOS mode

132x43s.twk Standard C&T SVGA BIOS mode

132x50s.twk Standard C&T SVGA BIOS mode

132x60s.twk Ultravision C&T SVGA mode

modes.doc Document on Mode Q, 400x300 and 400x600

The only files required to run TWEAK are:

o  TWEAK.EXE

o  TWEAK.DAT

The register definitions have been moved out from the executable into

the external .DAT file, to increase flexibility.  You might edit the

TWEAK.DAT file as you like to include support for any registers you

might think of.  Remember to update the number in the first line to

reflect the number of defined registers.  Note that if your new registers

are not on one of the ports supported by TWEAK.EXE, you might need to

modify and recompile the TWEAK sources to accomodate the new port(s).

See REGISTER.CPP.

Some words on my convention of naming files made by TWEAK:

----------------------------------------------------------

o  The general name format is XXXxYYY.CCC, where

XXX is the horizontal resolution

YYY is the vertical resolution

CCC is the number of colors supported, except for text modes,

which are named *.TWK.

o  An 's' after YYY specifies a Super VGA specific mode which I grabbed

from the BIOS of my Chips & Technologies Super VGA.  These modes will

probably *not* work with your card unless it's C&T compatible!  I've

had reports that most of these modes even crash some sensitive

machines/VGA cards.

o  A 'c' signals a chained 256-color mode, much like mode 13h, as

opposed to unchained modes like mode X.

The Makefile & sources

----------------------

See the Makefile for all dependencies between the source files.  It's not

very complicated.

The following makes are defined in the Makefile:

make tweak: makes the TWEAK executable.

make oldtweak: makes the TWEAK095 executable.

make examples: makes EXAMPLE*.EXE.

make utilities: makes 09TO10.EXE and TWEAK2C.EXE.

make all: combines all the above makes.

The Makefile is Borland C++/MAKE specific, and uses bcc.exe for all the

work, with one reference to TWEAK2C.EXE.

When studying the sources, note that TWEAK was started as an experiment

in object oriented programming with C++.  Thus the entire project may look

a bit pompous in its use of classes, overloaded operators and such.

Bear with me.  At last I provided the TwkUser files to help you get

started with something down to earth.

In their current state, some of the source files produce a couple of

warnings.  These can be ignored.  I do.

Also note that TwkUser.* and the examples are C (but C++ compatible),

while TWEAK and its utilities are strictly C++.

The utilities TWEAK2C and 09TO10

--------------------------------

The following are dumps of the help screens from there programs:

"TWEAK2C version 1.0

by Robert Schmidt of Ztiff Zox Softwear 1993

Converts a TWEAK version 1.x file to an #include-able C file.

Syntax:  TWEAK2C <TWEAK-file> <C file to create> <array name>

All parameters are required."

"09TO10 version 1.0

by Robert Schmidt of Ztiff Zox Softwear 1993

Converts TWEAK version 0.9 files to TWEAK version 1.0 files.

Syntax:  09TO10 <oldfile> <newfile>"

For both programs, the following goes:  If the file to be created already

exists, the data contained in the file on disk will be overwritten with the

new data.

THE 'MISC' DIRECTORY

In this directory I have included files from other sources than myself.

READ.ME

VGA.TXT

VGABIOS.TXT

I found these files in an archive assembled by Finn Thoegersen

of Denmark.  I beleive VGABIOS.DOC was taken from Ralph Brown's

interrupt list.  VGA.TXT lists VGA registers and their purpose,

but as I never used either VGA.TXT or VGABIOS.TXT, I can't

guarantee their correctness.  The complete archive containing

similar info on most popular Super VGAs can be found on

garbo.uwasa.fi in /pc/doc-hard/vgadoc2.zip.

CGA160.TXT

A post grabbed from some newsgroup, discussing tweaking on the

ancient CGA adapter.  I think 16-color 'graphics' on a CGA

sounds pretty interesting, so I included it for your enjoyment.

SETMODEX.ASM

Michael Abrash's code to set the VGA in the infamous Mode X.

Provided as an example of how programmers were used to tweaking,

before TWEAK came along... :)

CREDITS

Alphabetically:

o  Michael Abrash for doing so much work on Mode X and PC graphics in

general.

o  Ralph Brown for the great work on the Interrupt List.

o  Peter McDermott for an improved 16-color test screen, and valuable

sugestions, he also inspired me to make the autodetecting test

screen.

o  Kai Rohrbacher for helpful bug reports and info on working modes.

o  Finn Thoegersen for the MISC\VGA*.TXT files.  See MISC\READ.ME

o  Yaniv Shaya for inspiring me to make finish version 1.0.  Good luck

with your project!

BIBLIOGRAPHY

o  George Sutty & Steve Blair : "Advanced Pogrammer's Guide to the

EGA/VGA" from Brady.  A bit old perhaps, but covers all *standard*

EGA/VGA registers, and discusses most BIOS functions and other

operations.  Contains disk with C/Pascal source code.

o  Michael Abrash : "Power Graphics Programming" from QUE/Programmer's

Journal.  Collections of (old) articles in Doctor Dobb's Journal on

EGA/VGA, read modes and write modes, animation, tweaking (320x240

and 360x480).  His newer ravings in DDJ covers fast 256-color

bitmaps, compiled bitmaps, 3D graphics, polygons, texture mapping

among other stuff.  Check out the XSHARP library available on all

simtel mirrors!

o  Ralph Brown's interrupt list is a must for every serious

programmer, containing, among 1 million other things, a VGA BIOS

interrupt reference.  Available for anonymous ftp from

oak.oakland.edu in directory /pub/msdos/info as inter*.zip (Usually

3 files, around 330 Kb each), and on most serious BBSes.

o  Richard F. Ferraro : "Programmer's Guide to the EGA and VGA video

cards including Super VGA".  I don't have this one, but heard it's

nice.  The Super VGA reference makes it attractive, though that is

no help with TWEAK.

o  Richard Wilton : "Programmer's Guide to PC & PS/2 Video Systems"

Less technical, more application/algorithm oriented.  Supposed to be

good.

HOW TO REACH ME

I welcome any suggestions for further improvement of TWEAK.  I also

accept donations if you think it's worth it, or if TWEAK has in any way

helped you out with a tricky problem and you'd like to show your

appreciation.  I will personally e-mail subsequent versions to people who

donate $5 or more.  Make checks payable to Robert Schmidt personally.

Postcards from all over the world are fun to get.  Please, if you

contact me by ordinary mail, use a postcard with some photos from the

place you live on the front!  I appreciate that a lot.

Internet e-mail: robert@stud.unit.no

(I guess this should be reachable from Compuserve and other networks

too.  I don't know how, though.)

If you ever join the IRC service on the Internet, direct a /msg Buuud

for a chat!  I'm on pretty often.

Ordinary (snail-)mail:

Ztiff Zox Softwear

c/o Robert Schmidt

Stud.post 170

Norwegian Institute of Technology

Trondheim

NORWAY

Good luck, and remember the most important bit is to have fun!