VGAKIT Version 5.2b

Here is my latest SVGA/VGA info pack. I've included the .OBJ versions in case

anyone who does not have an assembler wants to play with these routines.

5.0 Changes:

Support for VESA defined line widths, and correct support for bank sizes other

than 64k. Old versions of VGAKIT would adjust all bank boundries to 64k by

shifting up the bank numbers on cards that have 4k or 16k banks. To correct

this I've changed the code to use an address table for screen offsets and

bank numbers, this is very close to what I do in my own graphics code. To

simplify things I've made the address table a fixed array that supports up

to 1024 scanlines. In a real program I would probably allocate this array

at runtime using malloc or some similar memory allocation function.

5.2 Changes:

New VESA memory detect

VESA 1.2 support

Memory detect for Tseng3000

DAC Type call added

More accurate Tseng4000 detect

5.2b Changes:

DAC detect call for Tseng4000 boards would hang many other boards

now it is only made on Tseng4000 boards

MKADRTBL function would not handle bank sizes less than 64k correctly

VESASET function would not get the bank size correctly (it was using Winsize

instead of WinGranularity

John Bridges  10/22/1992

Compuserve ID: John Bridges [GRASP] 75300,2137

Internet: 75300.2137@compuserve.com (John Bridges)

I can be found regularly in the GRAPHVEN forum on Compuserve (where GRASP is

supported), as well as the GRAPHSUP forum (formally called PICS).

-------------------------------------------------------------------------------

The included modules are:

EXAMPLE.C	is a silly C program which shows that this code works,

it just plots a nice calculated pattern using the

point functions.

MKEXAMP.BAT	batch file to compile and link EXAMPLE.C into EXAMPLE.EXE

for Microsoft C 6.0

STICKS.C	is another silly C program which shows that this code works,

it uses the line function to draw a LOT of lines to make a

very nice complex weave type pattern. Amazing how complex the

image gets from such a trivial program.

MKSTICKS.BAT	batch file to compile and link STICKS.C into STICKS.EXE

for Microsoft C 6.0

BANKS.ASM	SVGA bank switch code, used to address the beyond 64k

addressing sceams of the various SVGA chipsets. Also

includes a global variable "curbk" which is the current

hardware bank number which can be checked before each

bank switch to prevent the extra overhead of switching

into a currently selected bank. An example of this is

in the point routine in POINT.ASM which only switches

memory banks when required.

BANKS.ASM includes the global variables ativga,tseng, tseng4

trident,video7,everex,paradise,chipstech, aheada, aheadb,

oaktech and vgamem. These variables are set by the WHICHVGA()

function which must be called at program startup time to

determine which SVGA (if any) you have installed. This

function returns a 1 (true) if you have a SVGA card installed.

MODE13X.ASM	Is the function to setup the non-standard (ie: not supported

by the BIOS) 360x480x256 mode, this mode works on virtually

VGA that is fully IBM compatable. It's a great mode for

those without SVGA capabilities.

SVGAMODE.ASM	Is a limited function which sets up 640x480x256 color

mode on a SVGA, it uses the global chipset flags to

determine the BIOS mode number.

POINT.ASM	Is a pair of simple point plotting functions,

point(xpos,ypos,color)

which plots a point at x,y coorinates xpos,ypos in SVGA

modes calling newbank() in BANKS.ASM to switch memory

banks when necessary.

point13x(xpos,ypos,color)

which plots a point at x,y coorinates xpos,ypos in 360x480

mode.  Both point functions will clip at screen boundries

correctly.

XPOINT.ASM	Is just like POINT.ASM except that it's point functions draw

the points XOR'd to the screen.

xpoint(xpos,ypos,color)

xpoint13x(xpos,ypos,color)

RDPOINT.ASM	Is just like POINT.ASM except that instead of plotting points

it reads the color of screen pixel.

color=rdpoint(xpos,ypos)

color=rdpoint13x(xpos,ypos)

LINE.ASM	Is a line plotting function callable from C, you pass it the

coordinates for the two end points of the line, the color to

draw the line in and the function to call to plot each point.

bline(x1,y1,x2,y2,color,xpoint);

here is a list of VGA Chipset producers and their BIOS mode numbers:

=======================================================

ACUMOS

------

width		height		colors

-----		------		------

0x59		  800		   600		     2

0x58		  800		   600		    16

0x5e		  640		   400		   256

0x5f		  640		   480		   256

0x5c		  800		   600		   256

0x5d		 1024		   768		    16

AX=MODE

=======================================================

AHEAD

-----

width		height		colors

-----		------		------

0x6A		  800		   600		    16

0x71		  800		   600		    16

0x74		 1024		   768		    16

0x60		  640		   400		   256

0x61		  640		   480		   256

0x62		  800		   600		   256

0x63		 1024		   768		   256

AX=MODE

=======================================================

ATI

---

width		height		colors

-----		------		------

0x54		  800		   600		    16

0x65		 1024		   768		    16

0x61		  640		   400		   256

0x62		  640		   480		   256

0x63		  800		   600		   256

0x64		 1024		   768		   256

AX=MODE

=======================================================

CHIPS & TECH

------------

width		height		colors

-----		------		------

0x70		  800		   600		    16

0x72		 1024		   768		    16

0x78		  640		   400		   256

0x79		  640		   480		   256

0x7b		  800		   600		   256

AX=MODE

=======================================================

EVEREX

------

width		height		colors

-----		------		------

0x01		  752		   410		    16

0x02		  800		   600		    16

0x11		 1280		   350		     4

0x12		 1280		   600		     4

0x13		  640		   350		   256

0x14		  640		   400		   256

0x15		  512		   480		   256

0x20		 1024		   768		    16

0x30		  640		   480		   256

0x31		  800		   600		   256

0x32		 1024		   768		   256

AX=0x70 BL=MODE

=======================================================

GENOA GVGA

----------

width		height		colors

-----		------		------

0x7c              512		   512		    16

0x59              720		   512		    16

0x79 or 0x6a      800		   600		    16

0x7d              512		   512		    256

0x5b              640		   350		    256

0x7e              640		   400		    256

0x5c              640		   480		    256

0x5d              720		   512		    256

0x5e or 0x6c      800		   600		    256

0x7f             1024		   768		    4

0x5f             1024		   768		    16

AX=MODE

=======================================================

NCR

---

width		height		colors

-----		------		------

0x59		  800		   600		     2

0x58		  800		   600		    16

0x5e		  640		   400		   256

0x5f		  640		   480		   256

0x5c		  800		   600		   256

0x5a		 1024		   768		     2

0x5d		 1024		   768		    16

AX=MODE

=======================================================

OAK TECH

--------

width		height		colors

-----		------		------

0x52		  800		   600		    16

0x53		  640		   480		   256

0x54		  800		   600		   256

0x56		 1024		   768		    16

0x59		 1024		   768		   256

0x58		 1280		  1024		    16

AX=MODE

=======================================================

PARADISE

--------

width		height		colors

-----		------		------

0x59		  800		   600		     2

0x58		  800		   600		    16

0x5e		  640		   400		   256

0x5f		  640		   480		   256

0x5d		 1024		   768		    16

0x5c		  800		   600		   256

AX=MODE

=======================================================

TRIDENT

-------

width		height		colors

-----		------		------

0x5b		  800		   600		    16

0x5f		 1024		   768		    16

0x5c		  640		   400		   256

0x5d		  640		   480		   256

0x5e		  800		   600		   256

AX=MODE

=======================================================

TRIDENT 8900

------------

width		height		colors

-----		------		------

0x5b		  800		   600		    16

0x5c		  640		   400		   256

0x5d		  640		   480		   256

0x5e		  800		   600		   256

0x60		 1024		   768		     4

0x5f		 1024		   768		    16

0x62		 1024		   768		   256

AX=MODE

=======================================================

TSENG  (Genoa, Orchid, Willow)

-----

width		height		colors

-----		------		------

0x29		  800		   600		    16

0x37		 1024		   768		    16

0x2d		  640		   350		   256

0x2e		  640		   480		   256

0x2f		  720		   512		   256

0x30		  800		   600		   256

AX=MODE

=======================================================

TSENG 4000

----------

width		height		colors

-----		------		------

0x29		  800		   600		    16

0x37		 1024		   768		    16

0x2d		  640		   350		   256

0x2f		  640		   400		   256

0x2e		  640		   480		   256

0x30		  800		   600		   256

0x38		 1024		   768		   256

AX=MODE

=======================================================

VIDEO7

------

width		height		colors

-----		------		------

0x60		  752		   410		    16

0x61		  720		   540		    16

0x62		  800		   600		    16

0x63		 1024		   768		     2

0x64		 1024		   768		     4

0x65		 1024		   768		    16

0x66		  640		   400		   256

0x67		  640		   480		   256

0x68		  720		   540		   256

0x69		  800		   600		   256

0x6A		 1024		   768		   256

AX=0x6f05 BL=MODE

=======================================================