1

From: PC-Share,	to CIS via Ward	Christensen 76703,302  03/10/85

<<< NOTE EGA-TECH - 597	LINES >>>

Technical Information on the Enhanced Graphics Adapter

EXT-Info-IBMPC-1VFRR 23-Feb-85 Info-IBMPC Digest V4 #17

From: Info-IBMPC Digest <Info-IBMPC@USC-ISIB.ARPA>DDN

To: Info-IBMPC: ;DDN

Identifier: EXT-Info-IBMPC-1VFRR

Length:	5 page(s)estimate

Posted:	23-Feb-85 13:10-PST  Received:	5-Mar-85 09:14-PST

Message:

Return-path: <INFO-IBMPC@USC-ISIB.ARPA>

Received: from USC-ISIB.ARPA by	OFFICE-2.ARPA; 23 Feb 85 15:44:23 PST

Info-IBMPC Digest	Saturday, 23 February 1985	Volume 4 Issue 17

This Week's Editor: Billy Brackenridge

Today's Topics:

Extended Graphics Adaptor Tech Info

How I Got my EGA Manual, a True	Story

Date: 23 Feb 1985 11:57:04 PST

Subject: Extended Graphics Adaptor Tech	Info

From: Billy <BRACKENRIDGE@USC-ISIB.ARPA>

Here is	a bit more technical information on the	extended graphics adaptor

card. It still isn't complete, but it is a start:

The adapter memory (base) is configured	a 4 16K	bit planes. The	graphics

memory expansion adds one bank of 16K to each of the 4 bit planes. Additional

memory may be added to increase	each bit plane by another 16K.

Modes of Operation

IBM Color Display

Alpha	Buffer	 Box   Max

Mode #	 Type	Colors	Format	Start	 Size  Pages   Resolution

------	 ----	------	------	------	 ----  -----   ----------

0	 A/N	  16	40x25	B8000	 8x8	8	320x200

1	 A/N	  16	40x25	B8000	 8x8	8	320x200

2	 A/N	  16	80x25	B8000	 8x8	8	640x200

3	 A/N	  16	80x25	B8000	 8x8	8	640x200

4	 APA	   4	40x25	B8000	 8x8	1	320x200

5	 APA	   4	40x25	B8000	 8x8	1	320x200

6	 APA	   2	80x25	B8000	 8x8	1	640x200

D	 APA	  16	40x25	A8000	 8x8	2/4/8	320x200

E	 APA	  16	80x25	A8000	 8x8	1/2/4	640x200

Modes 0-6 emulate CGA adapter support.

Modes 0,2,5 are	identical to 1,3,4 at the adapters direct drive	interface.

The maximum page fields	for modes D and	E indicate the number of pages for

64k,128k,256k bytes of graphics	memory.

IBM Monochrome Display

Alpha	Buffer	 Box   Max

Mode #	 Type	Colors	Format	Start	 Size  Pages   Resolution

------	 ----	------	------	------	 ----  -----   ----------

7	 A/N	   4	80x25	B0000	 9x14	8	720x350

F	 APA	   4	80x25	A0000	 8x14	1/2	640x350

Mode 7 provides	Mono adapter emulation support.

IBM Enhanced Color Display

Alpha	Buffer	 Box   Max

Mode #	 Type	Colors	Format	Start	 Size  Pages   Resolution

------	 ----	------	------	------	 ----  -----   ----------

0*	 A/N	16/64	40x25	B8000	 8x14	8	320x350

1*	 A/N	16/64	40x25	B8000	 8x14	8	320x350

2*	 A/N	16/64	80x25	B8000	 8x14	8	640x350

3*	 A/N	16/64	80x25	B8000	 8x14	8	640x350

10	 APA	 4/64	80x25	A8000	 8x14	1/2	640x350

16/64

* Note that modes 0,1,2,3 are also listed under	Color Display. BIOS provides

support	when enhanced display is attached.

The values in "Colors" indicates 16 colors out of a palette of 64 or 4 colors

out of 16.

Basic Operations

Alphanumeric	Modes

The data format for alpha	modes on the EGA is the	same as	the data

format on	the CGA	and Mono adapter cards.

As an added function, bit	three of the attribute byte may	be

re-defined by the	Character map select register to act as	a switch

between character	sets. This gives the programmer	access to 512

characters at one	time. This function is valid only when you have

128K or more of graphics memory.

When alpha mode is selected, BIOS	transfers character patters from

ROM into bit plane 2. The	processor stores character data	into

plane 0, and the attribute into plane 1. The programmer views

planes 0 and 1 as	a single buffer	in alpha modes.

The CRTC generates sequential addresses, and fetches one character

code byte	and one	attribute byte at one time. The	character code

and row scan count address bit plane 2, which contain the	character

generators. The appropriate dot patterns are then	sent to	the

palette in the attribute chip, where the color is	assigned

according	to the attribute data.

Graphics Modes

320x200 Two and Four Color Graphics (Modes 4 and 5)

--------------------------------------------------- Addressing,

mapping, and data	format are the same as the 320x200 pel mode of

the CGA. The display buffer is at	B8000. The bit image is	stored in

bit planes 0 and 1.

640x200 Two Color	Graphics (Mode 6)

-----------------------------------

Addressing, mapping and data formats are the same	as the 640x200

pel mode on the CGA. Buffer starts at B8000 and bit image	is in

plane 0.

640x350 Monochrome Graphics (Mode	F)

---------------------------

This supports the	Mono display with the following	attributes:

black, video, blinking, intensified. Maps	0,1 and	2,3 are	chained

together to form 2 32k bit planes. The first map is the video bit

plane, the second	is the intensity plane.	Both planes reside at

A0000.

Two bits,	one from each plane, define one	pel on the screen. The

bit definition for the pels are given in the following table. Video

plane is C0, intensity is	C2.

C2  C0  Pixel	Color  Valid Attributes

--  --  -----------  ----------------

0   0	  Black		0

0   1	  Video		3

1   0	  Blinking	C

1   1	  Intensified	F

The byte organization of the memory is sequential. The first 8 pels on

the screen are defined by the contents in A000:0H, the second 8	in

A000:1,	and so on. The first pel within	a byte is bit 7	in the byte, the

last is	bit 0.

Mono graphics works in odd/even	mode, which means that odd CPU address go

to odd planes, and even	addresses to even planes. Since	both planes

reside at A0000, the user must select the plane	or planes to update. This

is accomplished	by the map mask	register of the	sequencer.

16/64 Color Graphics Modes (Mode 10)

------------------------------------

These modes support graphics in	16 colors on either medium or hi

resolution monitor. This uses all four bit planes. The planes are denoted

as C0,C1,C2, and C3.

C0 =	Blue Pels

C1 =	Green Pels

C2 =	Red Pels

C3 =	Intensified Pels

Four bits (one from each plane)	define one pel on the screen. Color

combinations are as follows:

I  R	 G  B  Color

-  -	 -  -  ------------------

0  0	 0  0  Black

0  0	 0  1  Blue

0  0	 1  0  Green

0  0	 1  1  Cyan

0  1	 0  0  Red

0  1	 0  1  Magenta

0  1	 1  0  Brown

0  1	 1  1  White

1  0	 0  0  Dark Gray

1  0	 0  1  Light Blue

1  0	 1  0  Light Green

1  0	 1  1  Light Cyan

1  1	 0  0  Light Red

1  1	 0  1  Light Magenta

1  1	 1  0  Yellow

1  1	 1  1  Intensified White

The display buffer resides at A0000, The map mask register of the

sequencer is used to select any	or all of the bit planes to be updated

when a memory write to the display buffer is performed.

Color Mapping

Character		      Mode 10H	    Mode 10H

Attribute	  Monchrome   64KB	    > 64KB

---------	  ---------   ---------	   ----------

00H	    Black      Black	    Black

01H	    Video      Blue	    Blue

02H	    Black      Black	    Green

03H	    Video      Blue	    Cyan

04H	    Blink      Red	    Red

05H	    Inten      White	    Magenta

06H	    Blink      Red	    Brown

07H	    Inten      White	    White

08H	    Black      Black	    Dark Grey

09H	    Video      Blue	    Light Blue

0AH	    Black      Black	    Light Green

0BH	    Video      Blue	    Light Cyan

0CH	    Blink      Red	    Light Red

0DH	    Inten      White	    Light Magenta

0EH	    Blink      Red	    Yellow

0FH	    Inten      White	    Intens White

External Registers

------------------

This section describes registers of the	EGA card not contained in the LSI

device.

Name			Port  Index

-----------------------	----  -----

Misc Output Register	3C2    -

Feature	Control	Reg	3?A    -

Input Status Reg 0	3C2    -

Input Status Reg 1	3?2    -

? = B in Monchrome modes, D in color modes

Misc	Output Register

--------------------

Write only. Hardware	reset causes all bits to zero.

Bit 0 - 3BX/3DX CRTC	I/O Address - This bit maps the	CRTC I/O

addresses for the mono or CGA emulation. 0 sets the CRTC addresses to

3BX and Input status	reg 1 to 3BA for mono emulation. 1 sets	the CRTC

address to 3DX and status reg 1 to 3DA for CGA emulation.

Bit 1 - Enable RAM -	0 disables ram from the	processor, 1 enables ram

to respond at addr's designated by the Contol Data Select value in the

Graphics Controllers.

Bit 2,3 - Clock Select, according to	the following table:

Bit 2	  Bit 3

-----	  -----

0	   0	 14Mhz from Processor I/O channel

0	   1	 16Mhz On-board	clock

1	   0	 External clock	from feature connector

1	   1	 Not used

Bit 4 -	Disable	Video Drivers -	0 activates internal video drivers, 1

disables them. When disabled, the source of the	direct drive color output

becomes	the feature connector.

Bit 5 -	Page Bit for Odd/Even -	Selects	between	2 64K pages of memory

when in	Odd/Even modes (0,1,2,3,7). 0 selects low pase,	1 selects high

page of	memory.

Bit 6 -	Horiz Retrace Polarity - 0 selects positive, 1 selects negative.

Bit 7 -	Vert Retrace Polarity -	0 selects positive, 1 selects neagative.

Feature	Control	Register

------------------------

Write only. Ouput address is 3BA or 3DA.

Bits 0,1 - Feature Control Bits	- Output of these bits go to FEAT0

(Pin19)	and FEAT1 (Pin 17) of the feature connector.

Bits 2,3 - Reserved.

Bits 4-7 - Not Used.

Input Status Register Zero

--------------------------

Read Only. Input address is 3C2.

Bits 0-3 - Not Used.

Bit 4 -	Switch Sense - When set	to 1, this allows proc to read the 4

config switchs on the board. The setting of the	CLKSEL field determines

switch to be read. The switch setting can also be determined by	reading

40:88H in RAM. 0 indicates switch is closed

Bit 5,6	- Feature Code - Input from FEAT0 and FEAT1 on the feature

connector.

Bit 7 -	CRT Interrupt -	1 indicates video being	displayed, 0 indicates

vertical retrace is occurring.

Input Status Register One

-------------------------

Bit 0 -	Display	Enable - 0 indicates the CRT raster is in a Horizontal or

vertical retrace interval. This	bit is real time status	of display enable

signal.

Bit 1 -	Light Pen Strobe - 0 indicates light pen trigger has not been

set, 1 indicates set.

Bit 2 -	Light Pen Switch - 0 indicates switch closed, 1	is switch open.

Bit 3 -	Vertical Retrace - 0 indicates video information being displayed,

1 indicates CRT	is in vertical retrace.	This bit can be	programmed to

interrupt the proc at int level	2 at the start of retrace. This	is done

by bits	4,5 of the Vertical End	Register of the	CRTC.

Bit 4,5	- Diagnostic Usage.

Bit 6,7	- Not Used.

Sequencer Registers

-------------------

Name		   Port	Index

---------------- ---- -----

Address	   3C4	 -

Reset		   3C5	 00

Clocking Mode	   3C5	 01

Map Mask	   3C5	 02

Char Map Select  3C5	 03

Memory Mode	   3C5	 04

Sequencer Address Register

--------------------------

A pointer register located at output address 3C4H. Loaded with a value to

indicate where the sequencer data is to	be written

Bits 0-4 - Sequencer Address.

Bits 5-7 - Not Used.

Reset Register

--------------

Write only. Written to when address register is	00H.

Bit 0 -	Asynchronous Reset - 0 commands	the sequencer to asynchronous

clear and halt.	1 causes sequencer to run unless bit 1 is zero.	Reseting

sequencer can cause loss of data in display buffer.

Bit 1 -	Synchronous Reset - 0 commands sequencer to synchronous	clear and

halt. bits 0 and 1 both	must be	1 to allow sequencer to	operate. Reset

the sequencer with this	bit before changing clock mode to allow	memory

contents to be preserved.

Clocking Mode Register

----------------------

Write only. Written when address register is 01H.

Bit 0 -	8/9 Dot	Clocks - 0 directs sequencer to	generate character clocks

9 dots wide, 1 causes 8	dots wide. Mono	alpha (Mode 7) is the only mode

that uses 9 dot	clocks.

Bit 1 -	Bandwidth - 0 makes CRT	memory cycles occur 4 out of 5 available

memory cycles; 1 makes CRT memory cycles occur on 2 out	of 5 available

memory cycles. All hi-res modes	must use 4 out of 5 in order to	refresh

the display image.

Bit 2 -	Shift Load - When 0, video serializes are reloaded every

character clock, 1 causes load every other clock. The later mode is

useful when 16 bits are	fetched	per cycle and chained together in the

shift registers.

Bit 3 -	Dot Clock - 0 selects normal dot clocks	derived	from the

sequencer master input.	1 causes the clock to be divided by 2 to generate

the dot	clock. Dot clock divided by to is used for modes 0,1,4,5.

Bit 4-7	- Not Used.

Map Mask Register

-----------------

Write only. Written when address register is 02H.

Bit 0-3	- Map Mask - A 1 in bits 3 through 0 enables the proc to write to

the corresponding maps 3 through 0. If this register is	0FH, the CPU can

perform	a 32 bit write on a single memory cycle. Data scrolling	is

enhanced when this register is 0FH. When odd/even modes	are selected,

maps 0,1 and 2,3 should	have the same mask value.

Character Map Select Register

-----------------------------

Write only. Written when the address register is 03H.

Bit 0,1	- Character map	select B - Selects the map used	to generate alpha

characters when	attribute bit 3	is 0, according	to the following table:

Bit1	 Bit0  Map Selected   Table Location

----	 ----  ------------   ------------------------

0	  0	    0	      1st 8K of	Plane 2	Bank 0

0	  1	    1	      2nd 8K of	Plane 2	Bank 1

1	  0	    2	      3rd 8K of	Plane 2	Bank 2

1	  1	    3	      4th 8K of	Plane 2	Bank 3

Bit 2,3	- Character map	select A - Selects the map used	to generate alpha

characters when	attribute bit 3	is 1, according	to the following table:

Bit3	 Bit2  Map Selected   Table Location

----	 ----  ------------   ------------------------

0	  0	    0	      1st 8K of	Plane 2	Bank 0

0	  1	    1	      2nd 8K of	Plane 2	Bank 1

1	  0	    2	      3rd 8K of	Plane 2	Bank 2

1	  1	    3	      4th 8K of	Plane 2	Bank 3

Bits 4-7 - Not used.

In alpha modes,	bit 3 of the attribute byte normally has the function of

turning	the forground intensity	on or off.  This bit may be redefined as

a switch between character sets.This function is selected only when the

values in character map	select A and B are different.  When they are the

same, the function is disabled.	 Memory	mode register bit 1 must be a 1

in order to enable this	function; otherwise bank 0 is always selected.

128K of	memory is required for 2 character sets, 256K for 4 sets.

Memory Mode Register

--------------------

Write only. Written when address register is 04H.

Bit 0 -	Alpha -	0 indicates non-alpha mode active. 1 indicates alpha mode

is active and enables character	generator map select function.

Bit 1 -	Extended Memory	- 1 indicates memory expansion card installed. 0

indicates card not installed

Bit 2 -	Odd/Even - 0 directs even processor addresses to maps 0	and 2,

odd to maps 1,3. 1 causes processor addresses to sequentially address the

bit map. The maps are accessed according to the	value in the Map Mask

Register.

------------------------------

Date: Thu, 21 Feb 85 22:35:27 est

From: "John Levine, P.O.Box 349, Cambridge MA 02238-0349 (617-494-1400)"

<ima!johnl@cca-unix> Subject: How I Got	my EGA Manual, a True Story

To: brackenridge@USC-ISIB.ARPA

I was in the same boat as you -- I had the Seminar Proceedings issue on

the EGA, but couldn't find the manual pages.  Then out of the blue, a

local guy called me up and offered to swap a copy of the seminar

proceedings for	a copy of the EGA manual.  Needless to say, I took him

up on it.

It turns out that he took the totally straightforward approach,	entirely

by accident, and it worked.  He	bought a copy of the "Options and

Adapters Technical Reference," which costs $125, a total ripoff,

particularly since almost everything in	it is in the XT	technical

reference, too.	 In the	front of this manual is	a postcard you send in

for the	update service.	 Wait a	month, and a mountain of updates arrives

in the mail, including the EGA manual.	It is over 100 pages including a

lot of diagrams.  About	half of	it is the assembler listing of the BIOS

code in	the ROM.

John Levine, ima!johnl or Levine@YALE.ARPA

------------------------------

End of Info-IBMPC Digest

*** CREATED 03/07/85 22:56:13 BY BILL.FRANTZ/BILL ***

+---------------> Bulletiinit <---------------+

:                                             :

:        1. EGA-kortin ohjelmointiin          :

:        2. MIDI-standardi                    :

:        3. Star Wars, Episode III            :

:        4. Purkkihullun pivkirja           :

:        5. U.S. Army - tilastotietoja        :

:        6. Lista Lontoon videoliikkeist     :

:                                             :

:  L = logintiedotteet,  Q = jatka eteenpin  :

:                                             :

+---------------------------------------------+

Enter desired bulletin number: