-----------------------------

The BGI Driver Toolkit

Creating Device Drivers

for the Borland Graphics Interface

--------------------------------------

Copyright (c) 1988,89 Borland International

Revision 1

May 15, 1989

Introduction

=======================

The Borland Graphics Interface (BGI) is a fast, compact, and

device-independent software package for graphics development built into the

Turbo Pascal, Turbo C, and Turbo Prolog language products. Device

independence is achieved via loadable device-specific drivers called from a

common Kernel. This document describes basic BGI functionality, as well as

the steps necessary to create new device drivers. Accompanying this

document are files containing sample code and other pertinent information.

File Name	      File Description

BH.C		      BGI loader header building program source

BH.EXE		      BGI loader header building program

DEVICE.INC	      Structure and macro definition file

DEBVECT.ASM	      Vector table for sample (DEBUG) driver

DEBUG.C		      Main module for sample driver

MAKEFILE	      Build file

BUILD.BAT	      A batch file for MAKE-phobics

BGI Run-time Architecture

=============================

Programs produced by Borland languages create graphics via two entities

acting in concert: the generic BGI Kernel and a device-specific driver.

Typically, an application built with a Borland compiler will include

several device driver files on the distribution disk (extension .BGI) so

that the program can run on various types of screens and printers.

Graphics requests (for example, draw line, draw bar, etc.) are sent by the

application to the BGI Kernel, which in turn makes requests of the device

driver to actually manipulate the hardware.

A BGI device driver is a binary image; that is, a sequence of bytes without

symbols or other linking information. The driver begins with a short

header, followed by a vector table containing the entry points to the

functions inside. The balance of the driver comprises the code and data

required to manipulate the target graphics hardware.

All code and data references in the driver must be near (i.e., small model,

offset only), and the entire driver, both code and data, must fit within

64K. In use, the device driver can count on its being loaded on a paragraph

boundary. The BGI Kernel uses a register-based calling convention to

communicate with the device driver (described in detail below).

BGI Graphics Model

=======================

When considering the functions listed here, keep in mind that BGI performs

most drawing operations using an implicit drawing or tracing color (COLOR),

fill color (FILLCOLOR), and pattern (FILLPATTERN).  For example, the

PIESLICE call accepts no pattern or color information, but instead uses the

previously set COLOR value to trace the edge of the slice, and the

previously set FILLCOLOR and FILLPATTERN values for the interior.

For efficiency, many operations take place at the position of the current

pointer, or CP. For example, the LINE routine accepts only a single (x,y)

coordinate pair, using the CP as the starting point of the line and the

passed coordinate pair as the ending point. Many functions (LINE, to name

one) affect CP, and the MOVE function can be used to explicitly adjust CP.

The BGI coordinate system places the origin (pixel 0,0) at the upper

left-hand corner of the screen.

Header Section

=======================

The device header section, which must be at the beginning of the device

driver, is built using macro BGI defined in file DEVICE.INC. The BGI macro

takes the name of the device driver to be built as an argument. For

example, a driver named DEBUG would begin as shown here:

CSEG	  SEGMENT PARA PUBLIC 'CODE'	; any segment naming may be used

ASSUME  DS:CSEG, CS:CSEG	; cs=ds

CODESEG

INCLUDE DEVICE.INC		; include the device.inc file

BGI	  DEBUG			; declare the device header section

The device header section declares a special entry point known as EMULATE.

If the action of a device driver vector is not supported by the hardware of

a device, the vector entry should contain the entry EMULATE. This will be

patched at load time to contain a jump to the Kernel's emulation routine.

These routines will emulate the action of the vector by breaking down the

request into simpler primitives. For example, if the hardware has the

functionality to draw arc, the arc vector will contain the address

of the routine to dispatch the arc data to the hardware and would

appear as follows:

dw	offset	ARC		; Vector to the arc routine

If, as is often the case, the hardware doesn't have the functionality to

display arcs, the vector would instead contain the EMULATE vector:

dw	EMULATE

The Kernel has emulation support for the following vectors:

BAR		Filling 3D rectangles

ARC		Elliptical arc rendering

PIESLICE	Elliptical pie slices

FILLED_ELLIPSE	Filled Ellipses

The Driver Status Table

=======================

BGI requires that each driver contain a Driver Status Table (DST) to

determine the basic characteristics of the device that the driver

addresses. As an example, the DST for a CGA display is shown here:

STATUS STRUC

STAT	DB	0		; Current Device Status (0 = No Errors)

DEVTYP	DB	0		; Device Type Identifier (must be 0)

XRES	DW	639		; Device Full Resolution in X Direction

YRES	DW	199		; Device Full Resolution in Y Direction

XEFRES	DW	639		; Device Effective X Resolution

YEFRES	DW	199		; Device Effective Y Resolution

XINCH	DW	9000		; Device X Size in inches*1000

YINCH	DW	7000		; Device Y Size in inches*1000

ASPEC	DW	4500		; Aspect Ratio = (y_size/x_size) * 10000

DB	8h

DB	8h		; for compatibility, use these values

DB	90h

DB	90h

STATUS	ENDS

The BGI interface provides a system for reporting errors to the BGI Kernel

and to the higher level code developed using Borland's language packages.

This is done using the STAT field of the Driver Status Table. This field

should be filled in by the driver code if an error is detected during the

execution of the device installation (INSTALL). The following error codes

are predefined in include file GRAPHICS.H for Turbo C and in the Graphics

unit for Turbo Pascal.

grOk		      =   0	Normal Operation, No errors

grNoInitGraph      =  -1

grNotDetected      =  -2

grFileNotFound     =  -3

grInvalidDriver    =  -4

grNoLoadMem	      =  -5

grNoScanMem	      =  -6

grNoFloodMem       =  -7

grFontNotFound     =  -8

grNoFontMem	      =  -9

grInvalidMode      = -10

grError	      = -11	Generic Driver Error

grIOerror	      = -12

grInvalidFont      = -13

grInvalidFontNum   = -14

grInvalidDeviceNum = -15

The next field in the Device Status Table, DEVTYP, describes the class of

the device that the driver controls; for screen devices, this value is

always 0.

The next four fields, XRES, YRES, XEFRES, and YEFRES contain the number of

pixels available to BGI on this device in the horizontal and vertical

dimensions, minus one. For screen devices, XRES=XEFRES and YRES=YEFRES.

The XINCH and YINCH fields are the number of inches horizontally and

vertically into which the device's pixels are mapped, times 1000. These

fields in conjunction with XRES and YRES permit device resolution (DPI, or

dots per inch) calculation.

Horizontal resolution (DPI) = (XRES+1) / (XINCH/1000)

Vertical resolution (DPI)  = (YRES+1) / (YINCH/1000)

The ASPEC (aspect ratio) field is effectively a multiplier/divisor pair

(the divisor is always 10000) that is applied to Y coordinate values to

produce aspect-ratio adjusted images (e.g., round circles). For example,

an ASPEC field of 4500 implies that the application will have to transform

Y coordinates by the ratio 4500/10000 when drawing circles to that device

if it expects them to be round. Individual monitor variations may require

an additional adjustment by the application.

The Device Driver Vector Table

==============================

The routines in the device driver are accessed via a vector table.

This table is at the beginning of the driver and contains 16-bit

offsets to subroutines and configuration tables within the driver.

The format of the vector table is shown below.

VECTOR_TABLE:

DW	INSTALL		; Driver initialization and installation

DW	INIT		; Initialize device for output

DW	CLEAR		; Clear graphics device; get fresh screen

DW	POST		; Exit from graphics mode, unload plotter, etc

DW	MOVE		; Move Current Pointer (CP) to (X,Y)

DW	DRAW		; Draw Line from (CP) to (X,Y)

DW	VECT		; Draw line from (X0,Y0) to (X1,Y1)

DW	EMULATE		; Reserved, must contain Emulate vector

DW	BAR		; Filled 3D bar from (CP) to (X,Y)

DW	PATBAR		; Patterned rectangle from (X,Y) to (X1,Y1)

DW	ARC		; Define ARC

DW	PIESLICE	; Define an elliptical pie slice

DW	FILLED_ELLIPSE	; Draw a filled ellipse

DW	PALETTE		; Load a palette entry

DW	ALLPALETTE	; Load the full palette

DW	COLOR		; Set current drawing color/background

DW	FILLSTYLE	; Filling control and style

DW	LINESTYLE	; Line drawing style control

DW	TEXTSTYLE	; Hardware Font control

DW	TEXT		; Hardware Draw text at (CP)

DW	TEXTSIZ		; Hardware Font size query

DW	RESERVED	; Reserved

DW	FLOODFILL	; Fill a bounded region

DW	GETPIX		; Read a pixel from (X,Y)

DW	PUTPIX		; Write a pixel to (X,Y)

DW	BITMAPUTIL	; Bitmap Size query function

DW	SAVEBITMAP	; BITBLT from screen to system memory

DW	RESTOREBITMAP	; BITBLT from system memory to screen

DW	SETCLIP		; Define a clipping rectangle

DW	COLOR_QUERY	; Color Table Information Query

;

;	35 additional vectors are reserved for Borland's future use.

;

DW	RESERVED	; Reserved for Borland's use (1)

DW	RESERVED	; Reserved for Borland's use (2)

DW	RESERVED	; Reserved for Borland's use (3)

.

.

.

DW	RESERVED	; Reserved for Borland's use (33)

DW	RESERVED	; Reserved for Borland's use (34)

DW	RESERVED	; Reserved for Borland's use (35)

;

;	Any vectors following this block may be used by

;	independent device driver developers as they see fit.

;

Vector Descriptions

===================

The following information describes the input, output, and function

of each of the functions accessed through the device vector table.

DW	offset INSTALL		; device driver installation

The Kernel calls the INSTALL vector to prepare the device driver for use. A

function code is passed in AL. The following function codes are defined:

>>> Install Device:   AL = 00

Input:

CL = Mode Number for device

Return:

ES:BX --> Device Status Table (see STATUS structure, above)

The INSTALL function is intended to inform the driver of the operating

parameters that will be used. The device should not be switched to

graphics mode (see INIT). On input, CL contains the mode in which the

device will operate. (refer to BGI setgraphmode statement)

The return value from the Install Device function is a pointer to a Device

Status Table (described earlier).

>>> Mode Query:   AL = 001h

Input:

Nothing

Return:

CX  The number of modes supported by this device.

The MODE QUERY function is used to inquire about the maximum number of

modes supported by this device driver.

>>> Mode Names:   AL = 002h

Input:

CX  The mode number for the query.

Return:

ES:BX --> a Pascal string containing the name

The MODE NAMES function is used to inquire about the ASCII form of the mode

number present in CX. The return value in ES:BX points to a Pascal string

describing the given mode. (Note: A Pascal, or _length_, string is a string

in which the first byte of data is the number of characters in the string,

followed by the string data itself.) To ease access to these strings from

C, the strings should be followed by a zero byte, although this zero byte

should not be included in the string length. The following is an example

of this format:

NAME:	db	16, '1280 x 1024 Mode', 0

==================================================================

DW   offset   INIT		; Initialize device for output

Input:

ES:BX  --> Device Information Table

Return:

Nothing

This vector is used to change an already INSTALLed device from text mode to

graphics mode. This vector should also initialize any default palettes and

drawing mode information as required. The input to this vector is a

device information table (DIT). The format of the DIT is shown below and

contains the background color and an initialization flag. If the device

requires additional information at INIT time, these values can be appended

to the DIT. There in no return value for this function. If an

error occurs during device initialization, the STAT field of the Device

Status Table should be loaded with the appropriate error value.

; ************** Device Information Table Definition **************

struct	DIT

DB	0		; Background color for initializing screen

DB	0		; Init flag; 0A5h = don't init; anything

;   else = init

DB	64 dup 0	; Reserved for Borland's future use

; additional user information here

DIT	ends

==================================================================

DW   offset   CLEAR	; Clear the graphics device

Input:

Nothing

Return:

Nothing

This vector is used to clear the graphics device to a known state. In the

case of a CRT device, the screen is cleared. In the case of a printer

or plotter, the paper is advanced, and pens are returned to the station.

==================================================================

DW   offset   POST		; Exit from graphics mode

Input:

Nothing

Return:

Nothing

This routine is used to close the graphics system. In the case of graphics

screens or printers, the mode should be returned to text mode. For

plotters, the paper should be unloaded and the pens should be returned to

station.

==================================================================

DW   offset   MOVE		; Move the current drawing pointer

Input:

AX		the new CP x coordinate

BX		the new CP y coordinate

Return:

Nothing

Sets the Driver's current pointer (CP) to (AX,BX). This function is

used prior to any of the TEXT, ARC, SYMBOL, DRAW, FLOODFILL, BAR, or

PIESLICE routines to set the position where drawing is to take place.

==================================================================

DW   offset   DRAW		; Draw a line from the (CP) to (X,Y)

Input:

AX		The ending x coordinate for the line

BX		The ending y coordinate for the line

Return:

Nothing

Draw a line from the CP to (X,Y). The current LINESTYLE setting is used.

The current pointer (CP) is updated to the line's endpoint.

==================================================================

DW	VECT		; Draw line from (X1,Y1) to (X2,Y2)

Input:

AX		X1; The beginning X coordinate for the line

BX		Y1; The beginning Y coordinate for the line

CX		X2; The ending X coordinate for the line

DX		Y2; The ending Y coordinate for the line

Return:

Nothing

Draws a line from the (X1,Y1) to (X2,Y2). The current LINESTYLE setting is

used to draw the line. Note: CP is NOT changed by this vector.

==================================================================

DW	BAR		; fill and outline rectangle (CP),(X,Y)

Input:

AX		X--right edge of rectangle

BX		Y--bottom edge of rectangle

CX		3D = width of 3D bar (ht := .75 * wdt); 0 = no 3D effect

DX		3D bar top flag; if CX <> 0, and DX = 0, draw a top

Return:

Nothing

Fills and outlines a bar (rectangle) using the current COLOR, FILLCOLOR,

and FILLPATERN. The current pointer defines the upper left corner of the

rectangle and (X,Y) is lower right. An optional 3D shadow effect

(intended for business graphics programs) is obtained by making CX nonzero.

DX then serves as a flag indicating whether a top should be drawn on the

bar.

==================================================================

DW	PATBAR		; fill rectangle (X1,Y1), (X2,Y2)

Input:

AX		X1--the rectangle's left coordinate

BX		Y1--the rectangle's top coordinate

CX		X2--the rectangle's right coordinate

DX		Y2--the rectangle's bottom coordinate

Return:

Nothing

Fill (but don't outline) the indicated rectangle with the current fill

pattern and fill color.

==================================================================

DW	ARC		; Draw an elliptical arc

Input:

AX		The starting angle of the arc in degrees (0-360)

BX		The ending angle of the arc in degrees (0-360)

CX		X radius of the elliptical arc

DX		Y radius of the elliptical arc

Return:

Nothing

ARC draws an elliptical arc using the (CP) as the center point of the

arc, from the given start angle to the given end angle. To get circular

arcs the application (not the driver) must adjust the Y radius as follows:

YRAD := XRAD * (ASPEC / 10000)

where ASPEC is the aspect value stored in the DST.

==================================================================

DW	PIESLICE	; Draw an elliptical pie slice

Input:

AX		The starting angle of the slice in degrees (0-360)

BX		The ending angle of the slice in degrees (0-360)

CX		X radius of the elliptical slice

DX		Y radius of the elliptical slice

Return:

Nothing

PIESLICE draws a filled elliptical pie slice (or wedge) using CP as the

center of the slice, from the given start angle to the given end angle.

The current FILLPATTERN and FILLCOLOR is used to fill the slice and it is

outlined in the current COLOR. To get circular pie slices, the application

(not the driver) must adjust the Y radius as follows:

YRAD := XRAD * ASPEC / 10000

where ASPEC is the aspect value stored in the driver's DST.

==================================================================

DW	FILLED_ELLIPSE	; Draw a filled ellipse at (CP)

Input:

AX		X Radius of the ellipse

BX		Y Radius of the ellipse

Return:

Nothing

This vector is used to draw a filled ellipse. The center point of the

ellipse is assumed to be at the current pointer (CP). The AX Register

contains the X Radius of the ellipse, and the BX Register contains the Y

Radius of the ellipse.

==================================================================

DW	PALETTE		; Load a color entry into the Palette

Input:

AX		The index number and function code for load

BX		The color value to load into the palette

Return:

Nothing

The PALETTE vector is used to load single entries into the palette. The

register AX contains the function code for the load action and the index

of the color table entry to be loaded. The upper two bits of AX determine

the action to be taken. The table below tabulates the actions. If the

control bits are 00, the color table index in (AX AND 03FFFh) is loaded

with the value in BX. If the control bits are 10, the color table index in

(AX AND 03FFFh) is loaded with the RGB value in (Red=BX, Green=CX, and

Blue=DX). If the control bits are 11, the color table entry for the

background is loaded with the value in BX.

Control Bits		Color Value and Index

00		Register BX contains color, AX is index

01		not used

10		Red=BX	Green=CX  Blue=DX, AX is index

11		Register BX contains color for background

==================================================================

DW	ALLPALETTE	; Load the full palette

Input:

ES:BX --> array of palette entries

Return:

Nothing

The ALLPALETTE routine loads the entire palette in one driver

call. The register pair ES:BX points to the table of values to be loaded

into the palette. The number of entries is determined by the color entries

in the Driver Status Table. The background color is not explicitly loaded

with this command.

==================================================================

DW	COLOR		; Load the current drawing color.

Input:

AL		The index number of the current drawing color

AH		The index number of the fill color

Return:

Nothing

The COLOR vector is used to determine the current drawing color. The value

in AL is the index into the palette of the new current drawing color. The

value in the AH register is the color index of the new fill color.

All primitives are drawn with the current drawing color until the color

is changed.

The fill color is used for the interior color for the bar, polygons, pie

slice, and floodfill primitives.

==================================================================

DW	FILLSTYLE	; Set the filling pattern

Input:

AL		Primary fill pattern number

ES:BX	If the pattern number is 0FFh, this points to user define

pattern mask.

Return:

Nothing

Sets the fill pattern for drawing. The fill pattern is used to fill all

bounded regions (BAR, POLY, and PIESLICE). The numbers for the

predefined fill patterns are as follows:

Code	Description			8 Byte fill pattern

0	  No Fill	   000h, 000h, 000h, 000h, 000h, 000h, 000h, 000h

1	  Solid Fill	   0FFh, 0FFh, 0FFh, 0FFh, 0FFh, 0FFh, 0FFh, 0FFh

2	  Line Fill	   0FFh, 0FFh, 000h, 000h, 0FFh, 0FFh, 000h, 000h

3	  Lt Slash Fill    001h, 002h, 004h, 008h, 010h, 020h, 040h, 080h

4	  Slash Fill	   0E0h, 0C1h, 083h, 007h, 00Eh, 01Ch, 038h, 070h

5	  Backslash Fill   0F0h, 078h, 03Ch, 01Eh, 00Fh, 087h, 0C3h, 0E1h

6	  Lt Bkslash Fill  0A5h, 0D2h, 069h, 0B4h, 05Ah, 02Dh, 096h, 04Bh

7	  Hatch Fill	   0FFh, 088h, 088h, 088h, 0FFh, 088h, 088h, 088h

8	  XHatch Fill	   081h, 042h, 024h, 018h, 018h, 024h, 042h, 081h

9	  Interleave Fill  0CCh, 033h, 0CCh, 033h, 0CCh, 033h, 0CCh, 033h

10	  Wide Dot Fill    080h, 000h, 008h, 000h, 080h, 000h, 008h, 000h

11	  Close Dot Fill   088h, 000h, 022h, 000h, 088h, 000h, 022h, 000h

0FFh	  User is defining the pattern of the fill.

In the case of a user-defined fill pattern, the register pair ES:BX point

to 8 bytes of data arranged as a 8x8 bit pattern to be used for the fill

pattern.

==================================================================

DW	LINESTYLE	; Set the line drawing pattern

Input:

AL		Line pattern number

BX		User-defined line drawing pattern

CX		Line width for drawing

Return:

Nothing

Sets the current line-drawing style and the width of the line. The line

width is either one pixel or three pixels in width. The following table

defines the default line styles:

Code		Description			 16 Bit Pattern

AL = 0      Solid Line Style			1111111111111111B

AL = 1      Dotted Line			1100110011001100B

AL = 2      Center Line			1111110001111000B

AL = 3      Dashed line			1111100011111000B

AL = 4      User-defined line style

If the value in AL is four, the user is defining a line style in the BX

register. If the value in AL is not four, then the value in register BX is

ignored.

==================================================================

DW	TEXTSTYLE	; Hardware text style control

Input:

AL		Hardware font number

AH		Hardware font orientation

0 = Normal,   1 = 90 Degree,   2 = Down

BX		Desired X Character (size in graphics units)

CX		Desired Y Character (size in graphics units)

Return:

BX		Closest X Character size available (in graphics units)

CX		Closest Y Character size available (in graphics units)

The TEXTSTYLE vector is used to define the attributes of the hardware font

for output. The parameters affected are the hardware font to be used, the

orientation of the font for output, the desired height and width of the

font output. All subsequent text will be drawn using these attributes.

If the desired size is not supported by the current device, the closest

available match to the desired size should be used. The return value from

this function gives the dimensions of the font (in pixels) that will

actually be used.

For example, if the desired font is 8x10 pixels and the device supports

8x8 and 16x16 fonts, the closest match will be the 8x8. The output of the

function will be BX = 8, and CX = 8.

==================================================================

DW	TEXT		; Hardware text output at (CP)

Input:

ES:BX	--> ASCII text of the string

CX		The length (in characters) of the string.

This function is used to send hardware text to the output device. The text

is output to the device beginning at the (CP). The (CP) is assumed to be

at the upper left of the string.

==================================================================

DW	TEXTSIZ		; Determine the height and width of text

; strings in graphics units.

Input:

ES:BX	--> ASCII text of the string

CX		The length (in characters) of the string.

Return:

BX		The width of the string in graphics units.

CX		The height of the string in graphics units.

This function is used to determine the actual physical length and width of

a text string. The current text attributes (set by TEXTSTYLE) are used to

determine the actual dimensions of a string without displaying it. The

application can thereby determine how a specific string will fit and reduce

or increase the font size as required. There is NO graphics output for

this vector. If an error occurs during length calculation, the STAT field

of the Device Status Record should be marked with the device error code.

==================================================================

DW	FLOODFILL	; Fill a bounded region using a flood fill

Input:

AX		The x coordinate for the seed point

BX		The y coordinate for the seed point

CL		The boundary color for the Flood Fill

Return:

Nothing	  (Errors are returned in Device Status STAT field).

This function is called to fill a bounded region on bitmap devices. The

(X,Y) input coordinate is used as the seed point for the flood fill. (CP)

becomes the seed point. The current FILLPATTERN is used to flood the

region.

==================================================================

DW	GETPIXEL	; Read a pixel from the graphics screen

Input:

AX		The x coordinate for the seed point

BX		The y coordinate for the seed point

Return:

DL		The color index of the pixel read from the screen.

GETPIXEL reads the color index value of a single pixel from the graphics

screen. The color index value is returned in the DL register.

==================================================================

DW	PUTPIXEL	; Write a pixel to the graphics screen

Input:

AX		The x coordinate for the seed point

BX		The y coordinate for the seed point

DL		The color index of the pixel read from the screen.

Return:

Nothing

PUTPIXEL writes a single pixel with the the color index value contained in

the DL register.

==================================================================

DW	BITMAPUTIL	; Bitmap Utilities Function Table

Input:

Nothing

Return:

ES:BX	--> BitMap Utility Table.

The BITMAPUTIL vector loads a pointer into ES:BX, which is the base of a

table defining special case-entry points used for pixel manipulation.

These functions are currently only called by the ellipse emulation routines

that are in the BGI Kernel. If the device driver does not use emulation

for ellipses, this entry does not need to be implemented. This entry was

provided because some hardware requires additional commands to enter and

exit pixel mode, thus adding overhead to the GETPIXEL and SETPIXEL vectors.

This overhead affected the drawing speed of the ellipse emulation routines.

These entry points are provided so that the ellipse emulation routines can

enter pixel mode, and remain in pixel mode for the duration of the ellipse-

rendering process.

The format of the BITMAPUTIL table is as follows:

DW	offset	GOTOGRAPHIC	; Enter pixel mode on the graphics hardware

DW	offset	EXITGRAPHIC	; Leave pixel mode on the graphics hardware

DW	offset	PUTPIXEL	; Write a pixel to the graphics hardware

DW	offset	GETPIXEL	; Read a pixel from the graphics hardware

DW	offset	GETPIXBYTE	; Return a word containing the pixel depth

DW	offset	SET_DRAW_PAGE	; Select page in which to draw primitives

DW	offset	SET_VISUAL_PAGE ; Set the page to be displayed

DW	offset	SET_WRITE_MODE	; XOR Line Drawing Control

The parameters of these functions are as follows:

GOTOGRAPHIC	; Enter pixel mode on the graphics hardware

This function is used to enter the special Pixel Graphics mode.

EXITGRAPHIC	; Leave pixel mode on the graphics hardware

This function is used to leave the special Pixel Graphics mode.

PUTPIXEL	; Write a pixel to the graphics hardware

This function has the same format as the PUTPIXEL entry described

above.

GETPIXEL	; Read a pixel from the graphics hardware

This function has the same format as the GETPIXEL entry described

above.

GETPIXBYTE	; Return a word containing the pixel depth

This function returns the number of bits per pixel (color depth) of

the graphics hardware in the AX register.

SET_DRAW_PAGE	; Select alternate output graphics pages (if any)

This function take the desired page number in the AL register and

selects alternate graphics pages for output of graphics primitives.

SET_VISUAL_PAGE ; Select the visible alternate graphics pages (if any)

This function take the desired page number in the AL register and

selects alternate graphics for displaying on the screen.

SET_WRITE_MODE	; XOR Line drawing mode control. XOR Mode is selected

if the value in AX is one, and disabled if the value in AX is zero.

==================================================================

DW	SAVEBITMAP	; Write from screen memory to system memory

Input:

ES:BX	Points to the buffer in system memory

to be written. ES:[BX] contains the width of the

rectangle -1. ES:[BX+2] contains the heigth of

the rectangle -1.

CX		The upper left X coordinate of the rectangle.

DX		The upper left Y coordinate of the rectangle.

Return:

Nothing

The SAVEBITMAP routine is a block copy routine that copies screen pixels

from a defined rectangle as specified by (SI,DI) - (CX,DX) to the system

memory.

==================================================================

DW	RESTOREBITMAP	; Write screen memory to the screen.

Input:

ES:BX	Points to the buffer in system memory

to be read. ES:[BX] contains the width of the

rectangle -1. ES:[BX+2] contains the heigth of

the rectangle -1.

CX		The upper left X coordinate of the rectangle.

DX		The upper left Y coordinate of the rectangle.

AL		The pixel operation to use when transferring

the image into graphics memory. Write mode for

block writing.

0: Overwrite mode

1: XOR mode

2: OR mode

3: AND mode

4: Complement mode

Return:

Nothing

The RESTOREBITMAP vector is used to load screen pixels from the system

memory. The routine reads a stream of bytes from the system memory into the

rectangle defined by (SI,DI) - (CX,DX). The value in the AL register

defines the mode that is used for the write. The following table defines

the values of the available write modes:

Pixel Operation			Code

Overwrite mode			 0

Logical XOR			 1

Logical OR			 2

Logical AND			 3

Complement			 4

==================================================================

DW	SETCLIP		; Define a clipping rectangle

Input:

AX		Upper Left X coordinate of clipping rectangle

BX		Upper Left Y coordinate of clipping rectangle

CX		Lower Right X coordinate of clipping rectangle

DX		Lower Right Y coordinate of clipping rectangle

Return:

Nothing

The SETCLIP vector defines a rectangular clipping region on the screen. The

registers (AX,BX) - (CX,DX) define the clipping region.

==================================================================

DW	offset COLOR_QUERY	; Device Color Information Query

This vector is used to inquire about the color capabilities of a given

piece of hardware. A function code is passed into the driver in AL. The

following function codes are defined:

>>> Color Table Size	AL = 000h

Input:

None:

Return:

BX    The size of the color lookup table.

CX    The maximum color number allowed.

The COLOR TABLE SIZE query is used to determine the maximum number of

colors supported by the hardware. The value returned in the BX register is

the number of color entries in the color lookup table. The value returned

in the CX register is the highest number for a color value. This value is

usually the value in BX minus one; however, there can be exceptions.

>>> Default Color Table    AL = 001h

Input:

Nothing

Return:

ES:BX   --> default color table for the device

The DEFAULT COLOR TABLE function is used to determine the color table

values for the default (power-up) color table. The format of this table is

a byte containing the number of valid entries, followed by the given number

of bytes of color information.

Device Driver Construction Particulars

======================================

The source code for a sample, albeit unusual, BGI device driver is included

with this Toolkit to assist developers in creating their own. The

demonstration driver is provided in two files, DEBVECT.ASM and DEBUG.C.

This "Debug" driver doesn't actually draw graphics, but instead simply

sends descriptive messages to the console screen (via DOS function call 9)

upon receiving commands. Instead of simply playing back commands, your own

driver would be structured similarly, but would access control ports and

screen memory to perform each function.

Cookbook

========

1. Compile or assemble the files required.

2. Link the files together, making sure that the device vector

table is the first module within the link.

3. Run EXETOBIN on the resulting .EXE or .COM file to produce a .BIN

file. There should be no relocation fixups required.

4. Run program BH (provided with the toolkit) on the .BIN

file to produce the .BGI file.

The resulting driver is now ready for testing. Examine the file TEST.C for

an example of installing, loading, and calling a newly-created device

driver.

Examples

; To call any BGI function from assembly language, include the

; structure below and use the CALLBGI macro.

CALLBGI MACRO	P

MOV	SI,$&P			; PUT OPCODE IN (SI)

CALL	CS:DWORD PTR BGI_ADD	; BGI_ADD POINTS TO DRIVER

ENDM

; e.g., to draw a line from (10,15) to (200,300):

MOV	AX, 10

MOV	BX, 15

MOV	CX, 200

MOV	DX, 300

CALLBGI VECT

; To index any item in the status table, include the status table

; structures below and use the BGISTAT macro.

BGISTAT MACRO	P			 ; GET ES:<SI> --> BGI STATUS

LES	SI, CS:DWORD PTR STABLE  ; GET LOCATION OF STATUS TO SI

ADD	SI, $&P			 ; OFFSET TO CORRECT LOCATION

ENDM

; e.g., to obtain the aspect ratio of a device:

BGISTAT ASPEC

MOV	AX, ES:[SI]		 ; (AX)= Y/X *10000