The solution to running Windows 3.0 in Enhanced mode with an Core CNT-MCK and

and Aox MicroMASTER 386 board is as follows:

1> Set the Core CNT-MCK controller to 4 Word Burst using the referance disk.

2> Set the Aox MicroMASTER to 'Start with Installable Device Driver = YES' and

'Reported Memory = MM Memory Only' through the use of the referance disk.

3> Make sure the Config.sys file reads as follows:

Device = Bpparms.sys /m /r	(Relocates MCK Memory)

Device = Mmidd.sys		(Initalize Aox Card)

Files = nn

Buffers = nn

Device = Himem.sys		(Windows Memory Manager)

Device = Smartdrv.sys		(Windows Disk Cache Utility)

If problems persist please call Core Technical Support at 407-997-6033 or Aox

Techinical Support at 617-890-4402.