If cleaning the rails on your disk drive does not solve

the problem(s) you are experiencing, your next step should be to

clean the read/write heads. Don't try this procedure

Supplies:

10 good firm swabs.  Cotton swabs are preferred

over foam swabs.  (the foam sheds particles)  A Q-tip swab

will work, but then you must pick out the fibers as they are

very loosely wound to the stick.

"Rice paper", (equivalent to

a white colored LENS PAPER for cleaning optical lenses such

as objectives, eyepieces, etc., and which is suitable for plastic

eye-glass lenses, anything else is too abrasive) "Rice paper"

should only be used on an already clean head, as it will not

allow you to 'scrub' the surface.

Also get 2 to 6 nickels.

Cleaning agents:

You have many choices of cleaners, but be safe by only using

isopropyl alcohol.  Normal flammability precautions apply.

Cleaning Procedure:

1) Remove the cover of the PCjr so you can see what you are doing.

2) Thoroughly soak your swab in the cleaning agent (Alcohol),

but not so that it is dripping.

3) Insert the swab where you would normally insert a diskette.

4) Don't get the swab on the rails that the Head/carriage asm

moves along.  If you do, get another clean swab and start again.

(Heads don't like yukky stuff on them)

5) Locate the swab between the heads and gently swab (scrub, etc)

the Air Bearing Surfaces (ABS - the part that contacts the diskette).

Use utmost care in doing this, as there are delicate flexures

that provide the proper gram load (and provide the alignment

of the read/write gap to the data tracks).  Make sure you do not

damage these!!!!!!  Without measuring of the gram load,

it will be somewhere between 10 and 30 grams.  For reference,

a nickel weighs exactly 5 grams.  Do not push against the

ABS with more than 30 grams pressure, or damage may result!  It may

be helpful to gently lift the top read/head SLIGHTLY to give you

additional working room.  Again, remember that the flexure

is very delicate, and do not pull up on it with more than

30 grams.

6) Retract your now dirty swab and throw it away, and start

with a fresh clean swab.  Follow the above steps until you are

satisfied the Air Bearing Surface is clean.

7) Get another clean swab, but this time only lightly wet it.

Swab the ABS again.  etc, etc

8) Get another clean swab, but this time DRY. Wipe the ABS.

9) Replace the cover, and clean up your mess.

THE FOLLOWING IS OPTIONAL:

9) If you have access to "rice paper", now you can use it.

Wet the paper. Position it between the ABS. Load the

heads onto the rice paper.  (You will have to fool the PCjr into

thinking it has a diskette inserted at this point).

Finally, clean the ABS with a back and forth motion for a few seconds.

Unload the heads (fool PCjr into thinking you have now removed

a diskette) and retract the rice paper.

10)Replace the cover, and clean up your mess.

Contaminated Diskettes

You might also look for any "contaminated" diskettes that will

make the read/write heads dirty.  The following TRUE story may

make you chuckle and, provide some insight!

The problem started when a four-year-old complained that his

computer game wouldn't work.  Knowing that sometimes children

aren't as careful in handling diskettes as they should be, the

parents proceeded to spin the diskette in its jacket.  Lo and

behold they found a spot where the 4 yr old had touched the

diskette with dirty fingers.  (Probably after finishing his peanut

butter and jelly sandwich).  With nothing to lose they used a swab

only damp with alcohol, and gently wiped the dirty area. The swab

was dry enough so that the alcohol wouldn't soak under the

protective jacket.  The diskette soon looked "good as new". They

booted up the computer with the washed diskette and it worked fine.

They're convinced that the read/write head picked up some  "yukkies"

from the contaminated diskette.  Prior to cleaning the read/write

heads, only about half of the diskettes they tried would work.

I also have young children who were introduced to the PCjr at the

age of 3.  One of the reasons I installed the IBM 3.5 inch drive

as the "A" drive was to prevent bent diskettes and the peanut-butter

and jelly syndrome.  We certainly enjoy the PCjr more since there

are fewer computer rules with the sturdier 3.5 inch diskettes.

Last words of caution:

NEVER, NEVER, NEVER

USE AN ABRASIVE MEDIA DISKETTE!  OR A CLEANING DISKETTE

OR ANY THING THAT EVEN REMOTELY SOUNDS LIKE IT!

THESE ITEMS ARE ABRASIVE, AND WILL DRAMATICALLY SHORTEN THE LIFE

OF A GOOD WORKING DRIVE.

However, If the drive is dead, even after you have cleaned it, maybe the

ferrite (What the Read/Write part of the head is made of) has

grown a magnetically dead layer.  At this point, then it would be

OK to use the abrasive cleaning diskette.  Use it for only a few

seconds, and certainly for not more than 30 seconds.

The surface of the head that contacts the media (head people

call this the ABS (short for Air Bearing Surface) is very

sensitive to abrasion, scratches and contamination.

Cleaning as described above with cotton swabs and alcohol will

keep scratching to a minimum, if not nonexistent.

If the drive is old, the media may have worn through

what is called the apex of the read write gap degrading it's

read and write performance until it fails completely.  When the

gap becomes wide (worn through the apex) The magnetic flux

transitions get weaker and weaker until the head write signal

can not magnetize the oxide elements on the diskette.  Likewise,

it takes a strong flux transition on the diskette to cause a

read signal in a wide gap head.