Add-A-Drive v2.0  Released December 1992

������͵  Add-A-Drive v2.0  �

�                                  ������������������Ŀ  ������������������Ŀ

� Micro House International, Inc.  �      42  MB      �  ������0   MB��������

�        Boulder, Colorado         �  �������������  �  ��������������������

�   From the makers of DrivePro    ��������������������  ��������������������

� - Professional Hard Drive Setup   E:     Installed      4_Drives v2.0 only

�

����������������������������������������������������������������������������Ŀ

�      Copyright 1992 by Dustbowl Designs, Inc. All rights reserved.         �

�  DrivePro Package version.  This program is not shareware or freeware.     �

�           Unauthorized redistribution is strictly prohibited.              �

������������������������������������������������������������������������������

=====Introduction=============================

Add-A-Drive is a CONFIG.SYS block device driver that allows the

utilization of a single hard disk drive on the standard Secondary Port

Address using the DOS operating system.  This permits two incompatible

controllers and/or host adapters to coexist in the same system.

*  With Add-A-Drive you can combine MFM, RLL, ESDI and SCSI

hard disk drives with the new low-cost AT/IDE interface

drives.

*  This means you DO NOT have to retire your older MFM drives

if you purchase a new AT/IDE hard disk drive!!!

=====System Requirements======================

A few simple conditions must be met in order to utilize the

Add-A-Drive device driver.

They are:

1. System must be 286 CPU or higher.

2. DOS level must be v3.0 or higher.

3. A second Controller or Host Adapter which must have the

option to set a Secondary Port Address to 170h and to

disable IRQ14 monitoring. (Seagate ST-07A and ST-08A AT/IDE

Host Adapters can do this.)

=====Preparing "Secondary Port" Disk Drives====

The disk drive that will be attached to the Secondary Port Address

needs to be partitioned and formatted on the Primary Port Address

prior to using the Add-A-Drive device driver.  Use DrivePro to

accomplish this task.  This will probably require temporarily removing

the existing hard disk drive controller or preparing the "Secondary"

drive in a different system.

BEFORE disengaging the "Primary" drives, BE SURE to write down the

CMOS drive type(s). If 'User Defined' or 'Custom' drive types are

being used, write down the associated cylinder, head, write precomp,

lzone and sector per track values.

The installation steps to take for an unprepared drive without data

are roughly as follows:

1.  Finish reading the rest of this document.

2.  Satisfy yourself that BACKUPs of your data are current and in

good condition.

3.  Install (on the default Primary Address) the drive you

eventually intend to use on the Secondary and set the

appropriate drive type number in the system CMOS setup.

4.  Boot to the floppy diskette.

5.  Use DrivePro to setup the hard drive.

6.  Confirm that the hard drive boots without the floppy in A:

and can perform reads and writes, and is good working

condition.

7.  Restore the CMOS drive types to the ones used on the original

1st (and 2nd) drives. Power down the system and reinstate the

controller for the original drives.

8.  Set the second Host Adapter for the "Secondary" drive to

Secondary Port Address 170h and disable IRQ14 monitoring.

(see below for Seagate ST-07A/ST-08A Host Adapter jumper

settings).

9.  Reboot. The system should start in the same way it always

has.

10.  Copy the ADDADRV.HDD program into the root directory of the

C: drive and Edit the CONFIG.SYS file to include the line:

DEVICE=C:\ADDADRV.HDD

This line should precede any other block device drivers (such

a RAMDRIVE.SYS). If a Disk Manager (tm Ontrack Computer

Systems) partition is installed on the original "Primary"

drives, the DEVICE=C:\ADDADRV.HDD line should follow after

DMDRVR.BIN.

11.  Reboot. An Add-A-Drive installation screen (shown above) will

indicate the loading status of the device driver. The drive

letters assigned to the "Secondary" drive(s) will depend on

the number of partitions on your C: and D: drives.

=====Background===============================

In the past, three major factors have stood in the way of achieving

the elusive goal of supporting more than two hard disk drives or even

two disk drives of dissimilar interfaces.  First, DOS versions prior

to the v5.0 would support a maximum of two physical hard disk drives.

The v5.0 DOS FDISK supports up to eight physical hard disk drives if

the Host Adapter is designed to follow the FDISK conventions of DOS

v5.0. (Some SCSI interface Host Adapters are now out in the

marketplace which have this support.)

Second, standard interface hard disk drive controllers (except for

SCSI) are limited to two physical drives.  In the case of MFM, RLL and

ESDI, the drives usually have a Drive Select 3 and 4 jumper (DS3 &

DS4), but the controllers for these interfaces use the industry

standard "Daisychain" configuration which allows only two drives to be

connected. (The rare "Radial" configuration uses the DS3 and DS4.) In

the case of the new AT interface for hard disk drives, also known as

IDE, the controller is built onto the drive itself and is limited by

design to two physical drives per Host Adapter - Master and Slave.

Third, controllers for these interfaces usually require a drive type

number to be set in CMOS and only two slots are ever given by the

system BIOS.

Until Add-A-Drive, two AT controllers have generally been unable to

coexist because of a condition known as a "port address conflict."  In

effect, both controllers canceled each other out because they needed

to use the same Hard Drive Primary Port Address, 1F0h.  In its default

configuration, DOS only recognizes hard disk drives that are attached

to this Primary Port Address and communicates with them through the

system bios and INT13.  A block device driver is required to service

drives that are attached to the common Secondary Port Address, 170h;

the Add-A-Drive device driver serves this function.

=====Outline==================================

An brief outline of how Add-A-Drive works is as follows:

After partitioning and formatting in the DOS environment, every hard

disk drive, regardless of interface, controller, or CPU has a very

special sector of information called the Master Boot Record (MBR). The

MBR is always the very first physical sector on the disk (cyl 0, hd 0,

sector 1), the contents of which 1) preserve the defined partition

structure, 2) contain the first executable instruction to begin

loading the active operating system, and 3) a final word signature in

the sector, AA55h, indicating a valid Master Boot Record.

Once a valid MBR is identified, the values stored in the partition

record of the MBR indicate the location of the DOS Boot Record (DBR)

which is generally located at logical cyl 0, hd 1, sector 1.  This DOS

boot sector contains two vital components - the BIOS Parameter Block

(BPB) structure and a loader routine for the operating system .  The

BPB provides a logical description of the hard disk drive's geometry

(logical blocks, heads, sectors per track, bytes per sector, etc.),

while the operating system loader establishes the names of the system

files IO.SYS and MSDOS.SYS, their root directory locations and then

loads them into memory .

Next, the IO.SYS file is read into memory, followed by the MSDOS.SYS.

These are the names of the two hidden files which accompany

COMMAND.COM after formatting a bootable drive. These programs begin an

initialization routine which establishes the resident character and

block devices found during the Power-On Self Test (POST) and

motherboard ROM BIOS initialization.  Once the basic hardware has been

identified and the DOS kernel has been positioned into low memory, the

next stage is to run a system module within IO.SYS called SYSINIT.  It

is at this time that the first of the Primary Port Address drive

letter designations are assigned (A:, B:, C:, etc.). These block

devices are given special handling at this point.  For each, a pointer

to a BPB array is established in memory.

Next, begins the process we are most interested in, namely the

processing of the CONFIG.SYS file.  The entire file is read into

memory and analyzed for block and character type device drivers.  When

block device drivers are identified, like Add-A-Drive, their entries

are added to the drive translation table and given the next available

drive letter designation.

Device drivers, like regular programs (but not quite like regular

programs), have initialization code and resident code.  The

initialization code in Add-A-Drive is the display banner and secondary

drive identification, while the resident portion is a collection of

routines handling very specific AT register set commands. The AT

register commands "talk" directly to the Secondary Port Address, 170h,

and therefore to the drive itself, and cover drive specific operations

like Read, Write, Recal, and Reset. It should be noted that the Format

command operation is not supported by Add-A-Drive, which is the reason

why the drive must be partitioned and prepared in a normal C:/D:

environment.

After all components of the CONFIG.SYS file are processed, SYSINIT's

final function is to load and execute the MS-DOS shell program,

COMMAND.COM.  How DOS manipulates the resident (critical error

handling, for example) and transient portions (batch file processes,

COPY, and RENAME, etc.) of the shell are beyond the scope of this

overview.

Thereafter, during the normal course of operation of the computer

system, DOS commands which are directed towards the drive letter(s)

assigned to the Add-A-Drive block device(s) are executed through the

memory resident portion of Add-A-Drive.

Add-A-Drive will occupy a minimum of 2,720 bytes of memory under DOS

5.0. The amount of memory required for the device driver will climb

slightly (�48 bytes) depending on the number of logical drives in

extended partitions, if any.

=====Error Messages===========================

The device driver may display a small assortment of error messages

during the loading process of the CONFIG.SYS file. These usually

relate to unformatted status, jumper, cabling and power issues.

If the device driver is activated in the CONFIG.SYS when there is no

controller or host adapter set to the secondary port address, the

system may "hang" while trying to boot.  Since the problem is part of

the CONFIG.SYS file, you will need to boot to the A: drive first, in

order to correct the C:\CONFIG.SYS file.

=====App Notes================================

Some Application Notes:

1. The effect of the program on the performance of the system is

negligible.

2. The use of Add-A-Drive on XT's is DISALLOWED, since it relies

exclusively on AT register set commands which are only

available on 286's or higher.

3. If power supply leads are in short supply, a simple "Y"

adapter that splits two leads off of a single, are available

at most computer supply stores.

4. If both controllers have support for floppy drives, disable

the floppy support on the second.

5. Add-A-Drive can be moved into upper memory under DOS 5 with

DEVICEHIGH=C:\your...path\ADDADRV.HDD

6. Add-A-Drive shows all signs of complete stability with

Microsoft Windows (tm).  In fact, our entire Windows

subdirectory structure is located on the "Secondary" drive!

We have thrown in everything we can think of to test Windows

compatibility, and all tests have proven successful. If you

intend to use the drive supported by the Add-A-Drive for the

Swap File, then in must be of the Temporary Swap File Type. (A

Permanent Swap File requires Int13 support.)

7. We've run three of the most popular file defragmenters without

problems.  In any case, ALWAYS follow the software

manufacturer's instructions and BACKUP you data first.

8. DOS 2.x and above will run with the same partition and

capacity limits allowed by that version of DOS, however only

DOS 3.0 and above will show the proper drive letters on the

Add-A-Drive initialization screen.

9. 10 partitions per physical drive are allowed under Add-A-Drive

support.

10. It is possible to install a single MFM or RLL drive on the

secondary port address and utilize Add-A-Drive.  This process,

however, is not as straight forward as putting the AT/IDE on

the secondary because typical ST412 interface controllers (MFM

and RLL) usually do not have the built-in ability to disable

the IRQ14.  If no jumper on the MFM controller exists to

disable IRQ14, then the line can usually be disabled manually

where it plugs into the motherboard.  On a 16-bit controller,

the IRQ14 line is found on the back side card edge, on the

"short" 16-bit extension.  It is the 7th contact from the left

or 12th from the right (D7).  Usually, placing a piece of simple

transparent tape over the contact will cause it to be disabled

when the card is plugged into the motherboard.  It is

advisable that you refrain from attempting this configuration

unless you have prior experience with modifications of this

sort. See below:

�

�  � Metal Clamp

��������Ŀ

�  �  0  �    Perspective: Looking at the backside of the card

����������

�  �            8-bit long slot             16-bit short slot

�  �_____      31 total contacts        ___ 18 total contacts______

�        �������������������������������   ������������������

�                                                �

�                                                D7 (IRQ14)

count 7 spaces from left or

12 spaces from right

Note: Many manufacturers only plate the contacts that will be

needed on the card.  Be sure to count spaces, not

actual plated contacts.

11. As noted earlier, some programs will not operate through

Add-A-Drive (i.e. FDISK, FORMAT).  This is because these

programs bypass DOS and do not update the partition's FAT

(File Allocation Table).  Other examples of this are usually

drive diagnostics utilities.  If programs of this sort are

deemed necessary to run, the drive will need to be reinstated

on the primary port address in order to be "seen" by the

program.

=====Seagate ST-07A and ST-08A================

===== AT IDE Host Adapters====================

The ST-08A is a combination floppy drive and hard drive host adapter,

while the ST-07A is for hard drives only.  The floppy support on the

ST-08A can be disabled making it functionally equivalent to the

ST-07A.

J2                    �� Drive-on-a-Card Mounting Point

Diskette                �

Connector     J3 LED    �      J1 AT Interface

(ST08A only)  �     �����      Hard Disc Connector

�  ������������������������������������������������������Ŀ �����

�  �   1      1     �             �                    ������

�  �   ::     ����             ::::::::::::::::::::    ������

�  �   ::                      1                          � �

������ ::                           �������������������������� JP2

�   ::                           1 JP2                 � �  Disc/Diskette

�   ::                          ��                     � �  Address

�   ::         (ST08A only)                            � �  Select

�   ::  �1���� JP4 Diskette Controller Enable          � �

�       :                                   ��������������� JP6 I/O

�       ����}�� J4 Aux DC Power to          1       ������  Channel

�            1�    Hard Disc                ��      ������  Ready

�          ������Ŀ                                    � �

� � � � � � � �����Ŀ � � � � � � � � � � � � � ������� �

���������������� �  ������������������������������      �

�� JP5 Disable Interrupt or            �

Interrupt While Busy IRQ14

If set up as the 2nd Host Adapter for use with Add-A-Drive:

1.  Add a jumper to JP2 (Disc/Diskette Address Select) which sets the

port to the secondary address at 170h.

2.  Remove jumper from JP5 (Interrupt While Busy, IRQ14). The jumper

can be conveniently stored on a single pin.

3.  If ST-08A, then remove jumper from JP4 (Diskette Controller

Enable) to disable the Floppy Controller).

=====Legal Disclaimer=========================

LICENSE AGREEMENT

Add-A-Drive v2.0  Released December 1992

Dustbowl Designs, Inc. provides  the accompanying object code software

("Software")  and nonexclusively  licenses  its  use on  the following

terms and conditions. The Software is copyrighted by Dustbowl Designs,

Inc. with all rights reserved.  YOU ASSUME FULL RESPONSIBILITY FOR THE

SELECTION OF THE  SOFTWARE TO ACHIEVE YOUR INTENDED  PURPOSES, FOR THE

PROPER INSTALLATION  AND USE. Dustbowl Designs,  Inc. DOES NOT WARRANT

THAT THE  SOFTWARE WILL MEET  YOUR REQUIREMENTS, THAT  THE SOFTWARE IS

FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE SOFTWARE WILL BE

ERROR FREE. Dustbowl Designs, Inc. EXPRESSLY DISCLAIMS ALL WARRANTIES,

WHETHER  ORAL  OR  WRITTEN,  EXPRESSED  OR  IMPLIED, INCLUDING WITHOUT

LIMITATION WARRANTIES  OF MERCHANTABILITY OR FITNESS  FOR A PARTICULAR

PURPOSE. IN  NO EVENT WILL  Dustbowl Designs, Inc.  BE LIABLE TO  YOU,

YOUR  CUSTOMERS   OR  OTHER  USERS   FOR  ANY  INDIRECT,   INCIDENTAL,

CONSEQUENTIAL,  SPECIAL  OR  EXEMPLARY  DAMAGES  ARISING  OUT OF OR IN

CONNECTION WITH THE USE OR INABILITY TO USE THE SOFTWARE.

Add-A-Drive is  a trademark of Dustbowl  Designs, Inc. The Add-A-Drive

software and documentation are Copyright 1992 by Dustbowl Designs with

all rights reserved. The Add-A-Drive software program is not shareware

or freeware and unauthorized redistribution is strictly prohibited.

This special Seagate Technology version is Licensed through:

Micro House International,  Inc.

4900 Pearl East Circle, Suite 101

Boulder, Colorado  80301

ph: 800-926-8299   Sales (USA and Canada)

303-443-3389   Sales (international callers)

303-443-3389   Technical Support

303-443-9957   BBS

303-443-3323   FAX

The authors of:

"The Encyclopedia of Hard Drives"

"The Hard Disk Technical Guide"

"The Encyclopedia of Main Boards"

The makers of:

"DrivePro - Professional Hard Drive Setup"

"EZ-Drive - Quick IDE Installation Software"

End of License agreement.

*****************************************************************

Contents of the Add-A-Drive Distribution Set.

ADDADRV.HDD    Device driver file

ADDADRV.DOC    Documentation file with installation instructions.

*****************************************************************

=====Upgrade Offer============================

As noted in the screen representation and at the beginning of this

document, Add-A-Drive supports a SINGLE hard disk drive on the

Secondary Port Address.  In the event that you may want to add a

SECOND drive to the Secondary Port Address, we offer the program

4_Drives.  Information and pricing for 4_Drives can be obtained from

Micro House International.

#