Congratulations on your purchase of the

USB-USB Network Bridge cable. This unique cable allows you to easily construct a network via USB port and share all the resource such as files, printers peripheral equipment, and more.

Package Checklist

The product package of the USB-USB Network Bridge Cable includes the following items:

- USB-USB Network Bridge cable

- Driver Installation Diskette

- This user’s manual

Overview

The USB-USB Network Bridge Cable provides a quick and easy solution to network up to 17 USB computers via hot Plug & Play technology without adding any Network Interface Cards. It is suitable for small offices, home offices, mobile users, and network gamers. With the USB-USB Network Bridge cable, users can play multi-player games, share files, printers, peripheral equipment, and more.

Features

The USB-USB Network Bridge Cable provides the following features:

- USB host to host communication

- Single cable solution for network communication

- Standard Windows Explorer user interface

- Maximum Data transfer rate: Over 5 Mbps

- Construct USB Network up to 17 PCs

- Power management: current reduces to 450A at suspend mode

- Sharing resources, such as Printers, CD-ROMs, Modems, Scanners, etc.

- Support TCP/IP, NetBEUI, IPX/SPX protocols

- Full NDIS driver implementation

- No external power needed

- Compliant with USB specification version 1.1

- USB full speed connection

- Support Windows 98

- Hot PnP for easy use

- Suitable for SOHO and mobile network environment

- Patent pending

System Requirements

Before you use the USB-USB Network Bridge Cable, make sure your computer is an IBM PC-compatible PC with the following minimum system requirements:

- Intel-compatible 486DX4-100 MHz CPU or higher

- One standard USB port (4-pin) and has the symbol

- Windows 98

Driver Installation

Before you start installing the driver, please don’t plug your USB-USB Network Bridge Cable into the USB port of your computer.  Follow the steps below to install the USB-USB Network Bridge driver:

- Power on the computers where you will connect USB-USB Network Bridge Cable and make sure that the USB port is enabled and working properly.

- Insert the USB-USB Network Bridge driver diskette into the floppy disk drive and run the Setup program inside the dirver diskette.

- The InstallShield Wizard will guide you step-by-step to complete the installation process.

- In Choose Destination Location Window, you can either install USB-USB Network Bridge driver to C:\program files\ USB-USB Network Bridge Driver V1.1 or you can install it to the specified directory you want.  Click NEXT.

- In Enter Information Windows, enter the name of the computer and Workgroup.  Please ensure that each computer has been given a unique name, not to conflict with each other.  Make sure the Workgroup name is the same for all the computers in the network.

- Click Finish to complete the setup process.

- During the installations, the system may ask you to insert the Windows 98 CD-ROM, please insert your Windows 98 CD-ROM into your CD-ROM drive then click OK.

- If the system cannot find any files, please direct to your CD-ROM drive and click OK.

- The system will ask you to reboot the computer when the installation is complete.  Please remove the diskette from your floppy disk drive, connect the USB-USB Network Bridge cable and restart Windows system.

Attention: 	If the following screen appears, do not bypass it otherwise you will not be able to access your network.

- Since USB-USB Network Bridge Driver V1.1 has finished installing the network driver during the installation, your USB network environment is set up automatically when the installation is complete.  Now you can see all the computers in your network in Network Neighborhood and you can share programs, files and other peripheral devices through the standard Windows interfaces, e.g. Windows Explorer and Network Neighborhood.

- To start sharing the resources, please follow the instructions below.  Click the Start menu, point to Programs\Windows Explorer.

- Expand the desktop and select the drive, folder, file or printer you want to share.

- Right click on the resource.

- Select Sharing.

- Select Shared As and enter the Shared Name to be recognized by other computers.

- Select an access type. For example, choose Full to give the complete access right to this resource or Read-Only to give others the read-only right to the resource.

- Click OK.

Note:	When the installation is complete, system will assign IP address and 	Submask automatically. Don’t try to change any settings unless you are an

expert network user. Any improper settings would cause system fail on some network functions.

Driver Uninstallation

Follow the steps below to remove the the USB-USB Network Bridge driver from Windows system:

- Click the Start menu, point to Settings\Control Panel. Double click 	Add/Remove Programs icon.

- In Install/Uninstall tab, select USB-USB Network Bridge Driver v1.1.

- Click Add/Remove button and then click OK.

- Restart Windows system.

Troubleshooting

If you have successfully installed the driver, but can’t see either your or the other computers’ names in Window’s Network Neighborhood, please check the following points.

- Make sure the USB-USB Network Bridges Cables are firmly connected to the USB ports of your computers or USB hubs.

- Click the Start menu, point to Settings\Control Panel. Double click Network icon. In Identification tab, ensure each computer has been given a unique name, not to conflict with each other.  Make sure the Workgroup name is the same for all the computers in the network.

- In Configuration tab, check the USB-USB Network Bridge Adapter is in The following network compenents are installed list.

- In Configuration tab, click File and Print Sharing button.  Make sure both I want to be able to give others access to my files and I want to be able to allow others to print to my printer(s) functions are selected.  It would cause the File and printer sharing for microsoft networks service be installed and enable this computer to share its files and printers to the others.

After checking all these points, if the network is still not working properly, please contact your supplier instead of revising the settings by yourself unless you are an expert network user.

Disclaimer

Information in this document is subject to change without notice. The manufacturer does not make any representations or warranties (implied or otherwise) regarding the accuracy and completeness of this document and shall in no event be liable for any loss of profit or any other commercial damage, including but not limited to special, incidental, consequential, or other damages.

No part of this document may be reproduced or transmitted in any form by any means, electronic or mechanical, including photocopying, recording or information recording and retrieval systems without the express written permission of the manufacturer.

All brand names and product names used in this document are trademarks, or registered trademarks of their respective holders.

FCC Statement

This device generates and uses radio frequency and may cause interference to radio and television reception if not installed and used properly. This has been tested and found to comply with the limits of a Class B computing device in accordance with the specifications in Part 15 of FCC Rules. These specifications are designed to provide reasonable protection against such interference in a residential installation. However, there is no guarantee that interference will not occur in a particular installation. If this device does cause harmful interference to radio or television reception, which can be determined by plugging the device in and out, the user can try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the device and receiver.

- Connect the computer into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Copyright © 1998 Prolific Technology Inc. All rights reserved.