01

RX TEMP - High Reactor Temperature

This alarm indicates high reactor core temperature.   Imme-

diate action should be taken to reduce core temperature  by

using Emergency SCRAM 'ESC', SCRAM 'F1', or Rod Height 'R0'.

Also consider using Emergency Cooling 'E100'.

Warning  (Yellow)  Point:   Reactor core temperature  above

750F.

Danger (Red) Point:  Reactor core temperature above 800F.

02

RX PRESS - Low Reactor Pressurization

This alarm indicates low reactor core pressurization, which

could result in localized boiling and fuel element failure.

Use Emergency SCRAM 'ESC'.

Warning (Yellow) Point:  Reactor pressure below 2 X Reactor

temperature.

Danger (Red) Point:   Reactor pressure below 1.8 X  Reactor

temperature.

03

RX Level - Low Reactor Level

This  alarm  indicates a loss of reactor coolant such  that

the reactor core is in danger of becoming  uncovered.   Use

Emergency Cooling 'E100',  or Make-up Feed to Primary 'M100

DP'.

Warning (Yellow) Point:  None.

Danger (Red) Point:  Reactor coolant level below 100%.

04

Rod Bottom - Rod Bottom Indication

This indication indicates when the control rods are comple-

tely inserted.

Warning (Yellow) Point:  Control Rods on bottom, completely

inserted.

Danger (Red) Point:  None

05

RAD HIGH - High Radiation Level

This  alarm indicates above normal radiation levels in  the

containment building.  This may be due to normal continuous

high  power operation or to fuel element failure.   No  im-

mediate action necessary.

Warning  (Yellow) Point:   Radiation level above 200 Milli-

rem/hour.

Danger  (Red)  Point:   Radiation level  above  500  Milli-

rem/hour.

06

Press Lvl - Pressurizer Level

This  alarm indicates that the pressurizer level is out  of

specification.   A  level  too high will result in a  pres-

surizer failure and primary coolant  leakage.   Pressurizer

level too low will result in pressurizer heater failure and

loss of pressurization to the reactor.   Control level with

Primary Drain Valve 'F2' and Make-up Feed 'DP'.

Warning  (Yellow)  Point:   Pressurizer level above 85%  or

below 50%.

Danger (Red) Point:   Pressurizer level above 90% or  below

45%.

07

Pri Flow - Primary Coolant Flow Rate

This  alarm indicates that the primary coolant flow rate is

out  of range.   If the flow is too low,  the reactor  core

receives inadequate cooling.  If too high, the primary pump

is overstressed and may fail prematurely.

Warning (Yellow) Point:   Primary flow above 90%,  or below

.12 X Reactor temperature.

Danger (Red) Point:  Primary flow 0.

08

Emerg Low - Low Emergency Coolant Level

This alarm indicates that the Emergency Coolant  reservoir

is below safe operating levels.

Warning (Yellow) Point:  Emergency Coolant below 25%.

Danger (Red) Point:  Emergency Coolant level below 10%.

09

Xchg Temp - High Heat Exchanger Temperature

This alarm indicates that the Heat Exchanger temperature is

exceeding design limits.   Action should be taken to reduce

Primary  temperature and to increase Secondary Coolant Flow

'S100'.

Warning (Yellow) Point:   Heat Exchanger temperature  above

495F.

Danger (Red) Point:  Heat Exchanger temperature above 500F.

10

Sec Level - Secondary Coolant Level

This alarm indicates low Secondary Coolant level.   Make-up

Feed 'DS' should be used to bring the level back to 100%.

Warning (Yellow) Point:  Secondary Coolant level below 90%.

Danger (Red) Point:  Secondary Coolant level below 50%

11

Sec Flow - Secondary Coolant Flow

This  alarm indicates that the Secondary Coolant flow  rate

is out of range.  Low Secondary Coolant flow can cause Heat

Exchanger  failure or Secondary Coolant pump damage.   High

flow can result in overstressing the pump.

Warning (Yellow) Point:   Secondary Coolant Flow rate above

90%.

Danger (Red) Point:  Secondary Coolant Flow rate 0.

12

Cnd In Temp - High Condenser Inlet Temperature

This  alarm  indicates high  condenser  inlet  temperature.

This can result in damage to the turbine, Secondary Coolant

pump and to the Condenser circulating pump.

Warning (Yellow) Point:  Condenser inlet temperature  above

275F.

Danger  (Red)  Point:   Condenser inlet  temperature  above

300F.

13

Cnd Hi Press - High Condenser Pressure

This  alarm  indicates high Condenser Pressure due to  high

condensate temperature or low Secondary Coolant flow.  High

Condensate  pressure  damages the Secondary  Coolant  pump.

Increase Secondary Coolant Flow 'S100'.

Warning (Yellow) Point:  Condenser Pressure above 4 Psi.

Danger (Red) Point:  Condenser Pressure above 10 Psi.

14

Cnd Out Tmp - High Condenser Outlet Temperature

This  alarm  indicates high Condenser  outlet  temperature.

High condensate temperature causes damage to the  Secondary

Coolant pump.   Increase Secondary Coolant flow 'S100',  or

decrease Heat Exchanger Temperature.

Warning (Yellow) Point:  Condenser Outlet Temperature above

195F.

Danger  (Red)  Point:   Condenser Outlet Temperature  above

212F.

15

Turb Speed - Turbo-Generator Speed

This alarm indicates that the Turbo-Generator system is not

on  design speed of 3,600 RPM.   Turbo-Generator  overspeed

damages  the turbine and generator.   Generator Trip  valve

'F3'  and Generator Field control 'F4' effect Turbo-Genera-

tor speed.

Warning (Yellow) Point:   Turbo-Generator speed other  than

3,600 RPM.

Danger (Red) Point:  Turbo-Generator speed above 3,636 RPM.

16

LOil Temp - High Lubricating Oil Temperature

This  alarm  indicates High  Lubricating  Oil  temperature.

This  can  damage the Lubricating Oil pump and  the  Turbo-

Generator.

Warning (Yellow) Point:   Lubricating Oil Temperature above

200F.

Danger  (Red)  Point:   Lubricating Oil  Temperature  above

212F.

17

LOil Level - Lubricating Oil Level

This alarm indicates low Lubricating Oil level.   Low level

can damage the Lubricating Oil Pump and Turbo-Generator.

Warning (Yellow) Point:  Lube Oil Level below 35%.

Danger (Red) Point:  Lube Oil Level below 15%.

18

LOil Press - Lubricating Oil Pressure

This alarm indicates unsafe Lubricating Oil Pressure.   Low

Lube Oil pressure can damage the Turbo-Generator.  Lube Oil

Pump control 'F8'.

Warning  (Yellow)  Point:   Lube Oil pressure above 90  and

below 79.

Danger (Red) Point:   Lube Oil pressure above 95 and  below

72.

19

MkFeed Level - Low Make-up Feed Level

This  alarm indicates when Make-up Feed level is below safe

operating levels.   Make-up feed replenishment will  slowly

take place during normal operations; system refilled during

maintenance shutdowns.

Warning (Yellow) Point:  Make-up Feed level below 40%.

Danger (Red) Point:  Make-up Feed level below 5%.

20

Pump Temp - High Pump Temperature

This  alarm indicates that one or more of the six pumps  is

overheating.   The  pumps  are Primary  Coolant,  Secondary

Coolant,  Condenser Circulation,  Lube Oil, Emergency Cool-

ing, and Make-up Feed.

Warning (Yellow) Point:  Pump temperature above 175F.

Danger (Red) Point:  Pump temperature above 212F.

21

Cntrl Warn - Control/Auxiliary Power Warning

This  alarm indicates loss of auxiliary power or  an  error

detected  by  the automatic control system.   Loss of  aux-

iliary  power results in failure of all pumps and  controls

if the generator is not producing power.

Warning (Yellow) Point:   Automatic Control off above  600F

reactor temperature.

Danger  (Red) Point:   Automatic Set-point off and  Control

set point above 494F.  Loss of Auxiliary Power.

22

Turb Trip - Turbo-Generator Tripped

This alarm indicates that the Turbo-Generator trip valve is

open,  and  that  the  secondary coolant  is  being  dumped

directly into the condenser.

Warning (Yellow) Point:  Turbo-Generator Trip valve open.

Danger  (Red) Point:   Turbo-Generator not tripped and Heat

exchanger temperature below 212F.

23

Pri Drain - Primary Drain Valve Open

This  alarm indicates that the Primary Drain Valve is  open

and allowing Primary Coolant to drain from the pressurizer.

Warning (Yellow) Point:  Primary Drain Valve open.

Danger (Red) Point:  None.

24

Pr Htr OFF - Pressurizer Heaters Off

This alarm indicates that the Pressurizer Heaters are  off.

The  Reactor  temperature should not exceed 212F  when  the

Pressurizer Heaters are off.

Warning (Yellow) Point:  None

Danger (Red) Point:  Pressurizer Heaters Off

99

################################################################################################################################