If you are using DOS 3.3 or higher and have COM3 and/or COM4, DOS will

then recognize these ports and you will be able to use the DOS MODE

command.

DOS assigns COM and LPT numbers, from LOW to HIGH, to all responding I/O

ports in order as they are found and non-responding I/O ports addresses

are skipped.

MACHINE         COMx             I/O            INT.        MEM

ALL             COM1            3F8 HEX         IRQ4       40:00

ALL             COM2            2F8 HEX         IRQ3       40:02

PC/XT/AT        COM3            3E8 HEX         IRQ4       40:04

PC/XT/AT        COM4            2E8 HEX         IRQ3       40:06

PC/XT/AT        COM5            2E0 HEX         IRQ4       NONE

PC/XT/AT        COM6            3E0 HEX         IRQ3       NONE

PS/2            COM3           3220 HEX         IRQ3       40:04

PS/2            COM4           3228 HEX         IRQ3       40:06

PS/2            COM5           4220 HEX         IRQ3       NONE

PS/2            COM6           4228 HEX         IRQ3       NONE

PS/2            COM7           5220 HEX         IRQ3       NONE

PS/2            COM8           5228 HEX         IRQ3       NONE

SERIAL PORT COUNT IS AT MEM 40:11 BITS 2, 3, AND 4.