APPENDIX F

SPEECH SYNTHESIZERS

Vocal-Eyes is a very easy to get along with screen reader.  By that we mean,

it works with many voice synthesizers.  Vocal-Eyes was not written for a

specific synthesizer however, it takes advantage of each synthesizers unique

features.  Some screen readers only work with one synthesizer and others

require you to get a completely different version for each synthesizer you

wish to use.  In our minds, this is not acceptable.  Here are the

synthesizers Vocal-Eyes directly supports:

- Sounding Board by GW Micro

- All Accent boards by Aicom Corporation

- All artic boards compatible with SONIX.COM and TTS.COM by Artic

Technologies

- Apollo by Dolphin Systems

- Braille 'n Speak by Blazie Engineering

- DECtalk by Digital Equipment Corporation

- Portable DECtalk by The Childrens Hospital

- Echo PC and GP by Street Electronics

- Internal ECHO PC+ and MC and 1000 by Street Electronics

- DoubleTalk PC by RC Systems

- Audapter by Personal Data Systems

- ASP by Automated Speech Functions

- Personal Speech System versions A and B by Votrax

- Prose 4000 by Speech Plus Incorporated

- Votalker by Votrax

Did you see your synthesizer in this list?  If not, all is not lost.  There

is a Generic option which will probably work.  Many of you may say, "Sure it

will work but I will not be able to use any of its features such as rate,

pitch, volume and most of all the silence command."  Fear not, you can setup

what Vocal-Eyes should send for each of the 10 possible rates, 10 volume

levels, 10 pitch levels, 26 tones, silence string and even an initialization

string.  Chances are, we already fully support the synthesizer you are using.

But if not, as you can see, you can customize the generic option to your

individual synthesizer.  If your synthesizer supports all of the above

commands, Vocal-Eyes will use them.

When you first startup Vocal-Eyes, it scans memory for your synthesizer

driver.  First, Vocal-Eyes will search for the Sounding Board software.  If

it does not find it, it will look for one of the supplied synthesizer

drivers.  Each of these drivers will be discussed in detail later in this

appendix.

The Sounding Board is the only synthesizer which does not require one of the

supplied drivers.  If you are using a synthesizer other than the Sounding

Board, the corresponding driver must be loaded prior to loading Vocal-Eyes.

If no synthesizer was found, you will get the message "Synthesizer driver not

loaded" and Vocal-Eyes will not be loaded.  If you are using the Sounding

Board, issue the commands to get it going.  If you are using another

synthesizer load the proper driver.  Now rerun Vocal-Eyes and this time it

will work.  Notice you do not have to tell Vocal-Eyes what port your

synthesizer is located in.  Vocal-Eyes gets all the information it needs

directly from the synthesizer driver automatically.

Below is a list of each synthesizer and what is required in order to use it

with Vocal-Eyes.  Also, any unique features relating to each synthesizer will

be discussed.  If you require any further information relating to your

synthesizer, please refer to your synthesizer's owners manual.

SOUNDING BOARD

The Sounding Board is by far the best choice if you require an extremely

responsive synthesizer.  Every feature Vocal-Eyes supports, the Sounding

Board handles.  In fact, the Sounding Board hot keys can be used to enhance

Vocal-Eyes performance.  Fastforward and Rewind work great.  If you press

either the increase or decrease hot keys for the rate or volume, Vocal-Eyes

will also, automatically adjust the screen voice setting accordingly.  The

only hot key which will not work with Vocal-Eyes is the punctuation hot key.

This is because Vocal-Eyes uses a much more sophisticated punctuation scheme.

Vocal-Eyes therefore automatically disables this hot key.  You may wish to

use the Sounding Board's hot key redefinition feature to undefine the

punctuation hot key since it will not work with Vocal-Eyes anyway.  Feel free

to setup and use all of the hot keys as you see fit.

The Rate, Volume, Pitch and Tone commands all map exactly into the Sounding

Board commands.  For example a Vocal-Eyes rate of 7 matches exactly rate 7 of

the Sounding Board.

In order to use the Sounding Board, you must load the programs SBLOAD.COM and

SB.COM.  These programs are supplied with the disk you received with your

Sounding Board.  However, the very latest version is also on your Vocal-Eyes

disk.  Vocal-Eyes requires version 2.0 or greater.  If you try and use

Vocal-Eyes with an earlier version of the Sounding Board software, you will

get an error message and Vocal-Eyes will not be loaded.  Simply replace the

version you were using with the version supplied on your Vocal-Eyes disk.

Therefore, assuming you don't require any Sounding Board command line

parameters, you would have to type the following at the DOS prompt:

SBLOAD

SB

Now simply start Vocal-Eyes in the usual manner.  Vocal-Eyes will take over

from here.

If you are using macros and have the MACRO.COM program loaded you will notice

when you enter Review Mode by pressing the fast review key, not only do the

macro keys no longer work, the Sounding Board hot keys also no longer work.

If you enter via the Review hot key, both the macro and the Sounding Board

hot keys work as always.  This is done on purpose for those using macros.

NON SOUNDING BOARD USERS

As we said earlier, all synthesizers except for the Sounding Board, you must

load the supplied driver.  Although we are referring to these as drivers, you

do not load them in the CONFIG.SYS.  These are actually .COM files which are

loaded directly from the DOS prompt.  The following lists all device drivers

and what synthesizers they are to be used with.

All ACCENT synthesizers                           ACCENT.COM

Apollo (external)                                 APOLLO.COM

Apollo (internal)                                 INAPOLLO.COM

Artic boards which use SONIX.COM and TTS.COM      ARTIC.COM

ASP                                               VEASP.COM

Audapter                                          AUDAPTER.COM

Braille 'n Speak                                  BNSPEAK.COM

DECtalk                                           DECTALK.COM

DoubleTalk PC                                     DBLTALK.COM

Echo PC or Echo GP                                ECHOPC.COM

Internal Echo PC+, MC or 1000                     INTECHO.COM

Prose 4000                                        PROSE4.COM

Personal Speech System version A                  PSSA.COM

Personal Speech System version B                  PSSB.COM

Portable DECtalk                                  PDECTALK.COM

Votalker                                          ARTIC.COM

All other synthesizers                            GENERIC.COM

The appropriate device driver must be loaded before you load Vocal-Eyes.  The

following describes the available command line parameters which can be used

for any of the above drivers.

/!Cn or /!Ln  (Port assignment)

Certain synthesizers require you to specify the port which they are connected

to.  If this is the case, you must include this command on the command line.

If you do not specify this parameter and your particular synthesizer requires

it, you will get the following error message.

Speech port not specified!

Don't worry if you get this error message, simply issue the command again.

But this time include the port assignment command.

This parameter informs Vocal-Eyes to which port your synthesizer is

connected.  Enter this parameter by typing /! and then following it with

either a C or an L (C standing for COM port and L for LPT port) and a number

designating the number of the port.  /!C1, for instance, informs Vocal-Eyes

that your synthesizer is connected to COM1.  /!L3, on the other hand,

designates LPT3 as the appropriate port.  You can specify any of the three

LPT ports 1-3 or any of the four COM ports 1-4.

Drivers which require a port assignment are:

All Accent synthesizers, external Apollo, ASP, Audapter, Braille 'N Speak,

DECtalk, Portable DECtalk, Echo PC, Personal Speech System and Generic.

Drivers which do not require a port assignment are:

Internal Apollo, Artic, Votalker, DoubleTalk PC, Internal Echo and Prose

4000.  If you specify a port on any of these drivers, it will be ignored.

/!Bn  (Baud rate assignment)

This parameter informs Vocal-Eyes the baud rate of your synthesizer.  If you

are using a parallel synthesizer, or one which did not require a port

assignment, this command will be ignored.  If you do not specify this

parameter, the device driver will default to 9600 BAUD.  If you wish to use

the command, use one of the following forms:

/!B1 = 1200 baud

/!B2 = 2400 baud

/!B3 = 300 baud

/!B4 = 4800 baud

/!B9 = 9600 baud

/!Pn  (Parity assignment)

This parameter informs Vocal-Eyes the parity required by your speech

synthesizer.  If you are using a parallel synthesizer, or one which did not

require a port assignment, this command will be ignored.  Enter N for No

parity, O for Odd parity or E for Even parity after the /!P.  For example,

/!PO would signal odd parity and /!PN would signal no parity.  If you do not

use this parameter, the device driver will default to no parity.

/!Dn  (Data bits assignment)

This parameter informs Vocal-Eyes the number of data bits required by your

speech synthesizer.  If you are using a parallel synthesizer, or one which

did not require a port assignment, this command will be ignored.  Enter /!D7

for 7 data bits or /!D8 for 8 data bits.  If you do not use this parameter,

the device driver will default to 8 data bits.

/!Sn  (Stop bits assignment)

This parameter informs the number of stop bits required by your speech

synthesizer.  If you are using a parallel synthesizer, or one which did not

require a port assignment, this command will be ignored.  Enter /!S1 for 1

stop bit or /!S2 for 2 stop bits.  If you do not use this parameter, the

device driver will default to 1 stop bit.

EXCEPTION DICTIONARY

With Vocal-Eyes even if your synthesizer does not directly support an

exception dictionary, you can still set one up.  Each driver supports a user

exception dictionary.  Those of you familiar with the Sounding Board will

feel at home with this feature as it was modeled after it.

Of course no matter which voice synthesizer you will be using, there will

always be unusual words, specialized terms, proper names, and symbols that it

will not pronounce exactly as you might like.  For that reason, we've given

you an ability to add your own custom dictionaries of exceptional words.  You

must first create a standard ASCII file containing the exceptional word(s)

and a mis-spelling of the word(s) that sounds proper.  For example, if you

would like to have the abbreviation "lb" to be spoken as "pounds" and the

roman numeral VI to be spoken as "6", enter these four lines in an ASCII

file:

lb

pounds

vi

6

Note:  Upper or lower case is not important.

Enter the correct spelling that is mis-pronounced on the first line, and the

mis-spelling that sounds better on the following line.  Give this file a

meaningful name, and save it on disk.  How about "VE.DIC."  You'll probably

find the exception dictionary feature quite useful for abbreviations, roman

numerals, acronyms, and proper names.

As was said earlier, this file must be in a standard ASCII format.  If you

are using a word processor, make sure you pick this option when saving.  If

you are using Noteworthy, simply save it as you would save any file.

Noteworthy is perfect for this type of editing as well as batch files.

There are a few rules of what can and can not be in this file.

1) Each entry must take two lines in the file.  The correctly spelled word on

line one and the mis-spelled word on line two.

2) The first line (correctly spelled word) can ONLY contain letters A-Z and

the apostrophe.  No other symbols are allowed.

3) The second line (mis-spelled word) can contain any standard printable

characters including spaces.

Let us see how to use the feature.

When you first startup your specific synthesizer driver for your current

session, it will attempt to load the dictionary file called "VE.DIC" by

default.  Assuming the file exists, it will automatically be loaded and used.

Nothing is required other than loading the specified driver as you have

always done.

However, if you wish to load a file with a different name you will need to

use the following command line parameter:

Parameter:  /Efilename

The synthesizer driver will load an ASCII file containing new rules for

text-to-speech translation by using the "E" parameter.  "filename" can be any

valid DOS file specification, including drive and directory information.  For

example, if you are using an ECHO PC synthesizer and you wanted to load a

dictionary file called 'LOTUS.DIC' on the C: drive under the directory called

'LOTUS', you could issue the command:

ECHOPC /EC:\LOTUS\LOTUS.DIC

After the dictionary is loaded successfully, the driver will confirm its

acceptance of the new rules with a message.  Remember, by default, the first

time the driver is issued it will look for a file called "VE.DIC" on the DOS

default log path, and load it, if it's located there.  The number of new

rules that can be added depends on a buffer setting.  The default buffer size

will accommodate approximately thirty to forty entries.

If there was a problem while trying to load the dictionary, you could receive

one of the two error messages:

* Dictionary load error *

* Dictionary format error *

The Dictionary load error means the driver could not find the specified file

name or the file was too large for the supplied buffer.  Verify the file

exists and your buffer is large enough to hold the supplied dictionary.

The format error indicates that you did not follow the above rules while

creating the dictionary file.  Verify that the file is an ASCII format and

the two entries contain valid characters.

Previously, we said the default buffer size would allow around thirty to

forty entries.  This number of course depends on the size of the words you

are defining but it also depends on the buffer size.  The drivers allocate

512 characters of storage for dictionary entries by default.  Depending on

the size of each entry, this allocation will accommodate up to approximately

40 entries.  The arithmetic here is 512/12=42.  We are using six characters

as the average word size, and we are doubling that to encompass the

mis-pronounced word and the properly pronounced substitute.  Should you find

this allocation to be too small, use the exception dictionary buffer

parameter to increase the size of the buffer as needed.

To adjust the buffer size, use the following command:

Parameter:  /!Ex

Notice the ! (exclamation point).  As always, since this command contains the

exclamation point it can only be issued the first time you startup your

driver each session.  The x is a number between 0 and 9.  The number

determines how many 512 character chunks to allocate as part of the buffer.

The default is 1 or 512 characters.

For example, if you wanted to triple the number of possible entries, the

command would read like this:

DBLTALK /!E3

The "E" in this parameter stands or "exception", and the number corresponds

to some multiple of 512 characters.  Notice that the dictionary buffer can be

set to 0 characters.  This is because if you do not wish to use an exception

dictionary file, you should not be penalized by taking valuable memory.

Therefore if you are not using an exception dictionary, use the command:

DBLTALK /!E0

This will set the buffer size to 0 freeing up a valuable 512 characters.  Of

course, you will not be able to load any exception dictionary though.

You can if you wish combine both commands to increase the buffer size and

load the actual file on one command line.  Because you will be issuing the

/!Ex parameter, this must be done the first time the driver is loaded each

session.  For example:

DBLTALK /!E5/EWP

This will load the DoubleTalk PC driver and increase the exception dictionary

buffer to 5*512 bytes or 4 1/2K.  It will then load the file WP.DIC.

REFRESH COMMAND

The final command line parameter allows you to fine tune how the driver

communicates to your synthesizer.  The driver is constantly sending commands

to your synthesizer.  However, with interruptability enabled with Vocal-Eyes,

those commands may not be noticed by the synthesizer.  Therefore, the driver

will occasionally send the entire command sequence every so often to ensure

the synthesizer is in the correct mode.  Actually, after every ten silence

commands this happens.  If you are using a synthesizer which takes some time

to digest the new commands, you may notice an annoying pause at certain

times.  Therefore, the following command can be used to disable this feature.

Be aware however, it may be possible for your synthesizer to get stuck in a

wrong rate or pitch for instance.  Normally, this is not a problem unless you

have your keyboard and screen voices different from each other.  If they are

the same, your synthesizer should always be in the correct mode.

Parameter /Rx (x=0 or 1)  0=disable, 1=enable.  The default is enable.  This

means Vocal-Eyes will occasionally send out the entire command sequence to

the synthesizer to guarantee it is set properly.

To cause the driver to disable this feature, issue the following command line

parameter when the driver is first loaded each session:

AUDAPTER /!C1 /!R0

The 'R' stands for refresh and the 0 disables the feature.  If your

synthesizer issues a slight pause about every ten keystrokes, you might want

to give this command a try.

When you have typed in the name of your driver followed by whichever switches

are appropriate--don't forget to press ENTER after typing in the complete

command-- a message should be printed to the screen and also spoken through

your synthesizer.  The message contains the name of the synthesizer and the

version number of the device driver.  If you do not wish to hear this

message, add a ; (semicolon) to the end of the command line parameter.  This

signals the driver not to speak the initial message.  For example:

ECHOPC /!C1/!B4/!E5/!R0/EBADWORDS;

This will load the device driver for the external echo synthesizer using COM1

at 4800 baud, 8 data bits, 1 stop bit and no parity.  The exception

dictionary buffer is increased to 4 1/2K and the exception dictionary

BADWORD.DIC is loaded.  Finally the refresh command has been disabled.  The

welcome message will be printed to the screen but will not be spoken since

the command line ended with the ;.

PSSB /!L1

This will load the device driver for the Personal Speech System version B

synthesizer using LPT1.  Since this is being used on a parallel synthesizer,

there is no need to specify the baud, data bits, stop bits or parity.  The

initial message will be printed to the screen and spoken.

PROSE4

This simply starts up the driver for the Prose 4000 synthesizer.  This is a

synthesizer which does not require a port assignment.

Now that you understand all of the commands available for each of the

synthesizer drivers, lets talk specifically about each of the drivers in

turn.

ACCENT.COM

As we said, this should be used if you plan on using any of the ACCENT

synthesizers.  The accent boards support all ten rates, pitch values and

volumes but it does not support all 26 possible tone settings.  They only

support 10 possible tones.  Therefore, Vocal-Eyes tones A-D are mapped into

the accents tone 0.  Vocal-Eyes tones E-L are mapped into the accent's tones

1-8 respectively.  Finally Vocal-Eyes tones M-Z are mapped into the accent

tone 9.  This means all tones between M and Z will sound the same.  The same

is true for tones A-D.  All other features work as expected.

Before, you can use the internal accent, you have to load their device

driver.  Please refer to your accent user's manual for a complete description

of this process.  All necessary accent files should be loaded before you

attempt to load the ACCENT.COM driver.  By default, the internal Accent

synthesizer intercepts LPT3.  This means you should supply /!L3 on the ACCENT

command line.

APOLLO.COM

This driver will work with the external Apollo synthesizers.  The synthesizer

itself does not require any setup.  Simply execute the APOLLO driver.  The

Apollo supports the 10 rate levels and 10 pitch settings but does not support

the volume or tone settings.

The Apollo synthesizers do not support any handshaking.  This means when the

8K buffer in the synthesizer fills, any new data will be lost.  It is

suggested that you not try and send more than 8,000 characters to the

synthesizer at one time.  With true handshaking, the synthesizer will signal

to the computer that its internal buffer is full so stop sending data.  As

the synthesizer speaks, the buffer slowly empties.  When there is enough room

for more data, the synthesizer will tell the computer to start sending data

again.  Without this ability, the computer has no idea when the synthesizer

buffer is full.  Again, this will only be a problem if you send more than 8K

of data to the synthesizer at once.

ARTIC.COM

This is to be used for all ARTIC synthesizers which support the SONIX.COM and

TTS.COM.  Newer ARTIC cards combine these two programs into one.  Every card

produced by ARTIC is capable of being used by Vocal-Eyes.  However, you may

have to purchase their interface which allows non-Artic products to function

with it.

Also, the Votalker board from Votrax should use this driver.  You will

recall, the Votalker also uses SONIX.COM and TTS.COM.

Before you can load the ARTIC.COM driver, you must first load the above two

programs.  Assuming you do not require any command line parameters the

following can be typed from the DOS prompt:

SONIX

TTS

If you try and load the ARTIC.COM driver before loading the above two files,

the driver will print "SONIX not loaded!"  Simply load SONIX and TTS and

reload ARTIC.COM.  Those of you familiar with PORTTALK.COM will be happy to

know it is not required with Vocal-Eyes.  This driver will support all of

Vocal-Eyes' features.

This driver does not require you to specify a speech port.

AUDAPTER.COM

This driver is to be used with the Audapter synthesizer.  The Audapter

supports all 10 rate, pitch and volumes commands as well as all 26 tone

settings.

BNSPEAK.COM

This driver is to be used with the Braille 'n Speak from Blazie Engineering.

We do not recommend using the Braille 'n Speak in place of a true voice

synthesizer.  However, it works great for temporary installations.  It does

not support any of the 10 rates, pitch settings, volumes or tone settings

directly from Vocal-Eyes.  It does support the silence command but does not

respond very quickly to it.

In order for the Braille 'n Speak to be used as an external synthesizer, you

first must enable the serial port and then place it into what is called

"Speech box mode."  The following can be used to enable the Braille 'n Speak:

First perform a warm reset to assure the default settings:

Dots 1, 2, 3, 4, 5, 6-chord

Dots 1, 2, 3, 4, 5, 6-chord

Next enable the speech port:

P-chord

Dots 2, 6

Y

Next enable the Speech Box Mode

Dots 3, 4, 5-chord

Dots 1, 2, 3, 4, 5, 6

E-chord

At this point, you can load the BNSPEAK.COM driver.  If you wish to adjust

the rate, pitch or volume you must issue the proper commands directly on the

Braille 'n Speak keyboard.  The Braille 'n Speak does not offer the ability

for Vocal-Eyes to handle these features directly.  The following is what is

required to adjust these three settings:

Dots 3, 4, 5-chord

At this point, the following keys adjust the following features:

Dot 1 Softer, Dot 4 Louder;  (volume)

Dot 2 Slower, Dot 5 Faster; (rate)

Dot 3 Lower, Dot 6 Higher; (pitch)

Once you have the desired settings, you must press:

E-chord

Now the Braille 'n Speak will use your new settings.  If you have any further

questions relating to the Braille 'n Speak, please refer to your Users Guide.

DECTALK.COM

This will allow you to use the well known DECtalk voice synthesizer.  This

synthesizer probably has the most pleasing voice quality but is by no means

a very responsive synthesizer.  It also costs much more the all the others.

However, Vocal-Eyes uses it to its full potential.  We are sure you will be

pleased with the results.

Since the DECtalk is a completely self-contained external synthesizer, you

don't have to load any additional software other than the DECTALK.COM driver.

It supports all ten rates and pitch settings but does not support any of the

volume settings.  Changing the volume from within Vocal-Eyes will have no

effect at all.  To adjust the volume, you must use the volume control on the

DECtalk itself.  Also, we decided to map in the 7 different voices the

DECtalk offers into the tone settings.  Therefore, Vocal-Eyes tones A-H will

use Huge Harry.  Tone I will use Perfect Paul.  Tone J will use Frail Frank.

Tone K will use Rough Rita.  Tone L will use Beautiful Betty.  Finally tones

M-Z will use Kit the Kid.  Also notice when Vocal-Eyes increases the pitch,

it increases the pitch for the current voice.  Some screen readers will

switch you to a different voice.  We feel increasing the Pitch is a more

subtle way of accomplishing the same task.

The DECtalk is probably the most complicated speech synthesizer available.

Because of this, it seems to respond somewhat slower than other synthesizers.

For example, you may silence the speech but the DECtalk may keep talking for

a few seconds before it decides to stop speaking.  Rest assured, Vocal-Eyes

told the DECtalk to stop speaking the moment you requested it to.

DBLTALK.COM

This is used for the internal DoubleTalk PC synthesizer.  The DoubleTalk

supports all ten speech rates, pitch and volume settings.  However, it only

supports 2 tone settings.  Vocal-Eyes tones A-M are mapped into the first

tone setting and Vocal-Eyes tones N-Z are mapped into the second.

You will notice using a rate of 9 will cause the DoubleTalk PC to talk much

faster but use a much higher pitch.  This is sort of a hyper-rate.  If you do

not feel comfortable with this rate, feel free to slow it down to a standard

rate.  This is only true for rate 9.

No other software is needed to setup the DoubleTalk PC.  Simply execute the

DBLTALK driver.  If the driver could not find the DoubleTalk PC, you will get

an error message.

Because the driver is capable of locating the DoubleTalk PC, no speech port

is required.

ECHOPC.COM

Although this is called ECHOPC.COM it can be used with the Echo GP as well.

This synthesizer like the DECtalk is a completely self-contained external

synthesizer, you don't have to load any additional software other then the

ECHOPC.COM driver.

The Echos support all ten pitch settings and volume settings.  However, it

only offers two speech rates.  Therefore, Vocal-Eyes speech rates of 0-4 will

issue the slower of the two speeds and rates 5-9 will issue the faster of the

two.  Also, the Echos do not support any of the tone settings at all.

Changing the tone value in Vocal-Eyes will have no effect at all on the

speech.

Unlike the DECtalk, the external Echos will stop speaking immediately when

requested to.  However, they have a side effect of squeaking or hiccuping.

This can get annoying if you are using interruptability.  You may wish to

turn interruptability off when working with Echo.

INAPOLLO.COM

This driver will work with the internal Apollo synthesizers.  The synthesizer

itself does not require any setup.  Simply execute the INAPOLLO driver.  The

Apollo supports the 10 rate levels and 10 pitch settings but does not support

the volume or tone settings.

This driver will work for the default settings of the standard desktop

internal.  However, you can adjust the port address on this board.  Also, the

Toshiba model has a different address still.  Therefore, you may have to

include the following command line parameter to specify the correct speech

base address.

/!Bx x=0-2  (0=308HEX, 1=318HEX, 2=Toshiba model)

By default, Vocal-Eyes will assume you are using the desktop with the

standard address of 308HEX.  If you have changed the address to 318H, you

must supply:

INAPOLLO /!B1

Or if you are using the internal Toshiba version, you must supply:

INAPOLLO /!B2

The Apollo synthesizers do not support any handshaking.  This means when the

8K buffer in the synthesizer fills, any new data will be lost.  It is

suggested that you not try and send more than 8,000 characters to the

synthesizer at one time.  With true handshaking, the synthesizer will signal

to the computer that its internal buffer is full so stop sending data.  As

the synthesizer speaks, the buffer slowly empties.  When there is enough room

for more data, the synthesizer will tell the computer to start sending data

again.  Without this ability, the computer has no idea when the synthesizer

buffer is full.  Again, this will only be a problem if you send more than 8K

of data to the synthesizer at once.

You do not have to specify a speech port.

INTECHO.COM

This driver will work with either the Echo PC+, MC or 1000.  All three of

these are treated the same.  The only difference is the machines they work

on.

In order to use these synthesizers, you need to load the program TALK.EXE.

This is supplied with your synthesizer.  Please refer to your synthesizer

manual if you have any questions on how to load this program.  If you try and

load the INTECHO.COM driver prior to loading the TALK.EXE program, the driver

will print "The ECHO software was not found."  Simply load TALK.EXE and

reload INTECHO.COM.

One of the options the TALK program offers is what key do you wish to silence

the speech with.  Do not assign a key for this feature.  Allow Vocal-Eyes to

handle this.

These synthesizers support all ten rates, pitch settings and volume levels.

However, it does not support any of the tone settings.  Vocal-Eyes' tone

options will have no effect at all.

These synthesizers require the complete attention of the machine while it is

speaking text.  Therefore, You may notice some sluggishness while using any

of these synthesizers.

These synthesizer do not require a port assignment.  Vocal-Eyes will

automatically find the synthesizer.

PDECTALK.COM

This will allow you to use the Portable DECtalk voice synthesizer.  This

synthesizer uses the same voice as the original DECtalk and has the same

characteristics.

Since the Portable DECtalk is a completely self-contained external

synthesizer, you don't have to load any additional software other than the

PDECTALK.COM driver.

It supports all ten rates and pitch settings but does not support any of the

volume settings.  Changing the volume from within Vocal-Eyes will have no

effect at all.  Also, we decided to map in the 7 different voices the DECtalk

offers into the tone settings.  Therefore, Vocal-Eyes tones A-H will use Huge

Harry.  Tone I will use Perfect Paul.  Tone J will use Frail Frank.  Tone K

will use Rough Rita.  Tone L will use Beautiful Betty.  Finally tones M-Z

will use Kit the Kid.  Also notice when Vocal-Eyes increases the pitch, it

increases the pitch for the current voice.  Some screen readers will switch

you to a different voice.  We feel increasing the Pitch is a more subtle way

of accomplishing the same task.

The Portable DECtalk is slightly different than the original DECtalk.  First,

it is capable of speaking at a much faster rate.  You will notice rate 9 for

the original DECtalk does not speak nearly as fast as the Portable DECtalk

rate 9.

The other difference is a flaw in the Portable DECtalk itself.  It has a

feature which will automatically powerdown the DECtalk if no character has

been received through the serial port within a configurable time.  This is to

conserve battery power.  Once a character is received, it was suppose to

power up and continue on normally.

However, if the Portable DECtalk is in the process of speaking something but

nothing has been sent to it through the serial port for some time, it would

shut down and not finish speaking the remainder of the text.  The PDECTALK

driver will disable this feature when it is first loaded.  However, if you

power down the Dec and power it back up again, the auto-powerdown feature may

be re-enabled.  Therefore, you should never power down the Portable DECtalk

unless you are finished with it.

The DECtalk is probably the most complicated speech synthesizer available.

Because of this, it seems to respond somewhat slower than other synthesizers.

For example, you may silence the speech but the DECtalk may keep talking for

a few seconds before it decides to stop speaking.  Rest assured, Vocal-Eyes

told the DECtalk to stop speaking the moment you requested it to.

PROSE4.COM

This should be used if you wish to use Vocal-Eyes with the PROSE-4000

synthesizer.  Even though this is an internal synthesizer, nothing else is

required to be loaded.  The PROSE4.COM contains all necessary information.

If you have the PROSE.COM utility, you should not use it.  This would

interfere with PROSE4.COM.

All ten rates, pitch settings and volume levels are supported.  However, the

PROSE does not support the tone settings.  Therefore, changing Vocal-Eyes'

tone settings will have no effect at all.  Like the DECtalk, if you silence

the speech it may take a moment to silence the speech.  Rest assured

Vocal-Eyes sent the proper commands to the PROSE.  Sometimes, it takes it a

moment or so to actually silence the speech however.

A speech port assignment is not required for this synthesizer.  However, if

you have changed the onboard IRQ from the default of 2 or have changed the

speech port address from the default of 300 HEX, you must supply a command to

the driver indicating the change.

/!Ix - x = 2-7.  This parameter will supply to the PROSE4 driver which IRQ

the Prose 4000 has been setup for.  By default the driver will assume IRQ 2.

This is the default for the Prose 4000 board.  If you have changed the IRQ

from 2, you must supply this parameter.  For example, if the IRQ has been

changed to 7, you must supply:

PROSE4 /!I7

/!Bx - x = 0-4.  (0=300HEX, 1=308HEX, 2=310HEX, 3=318HEX, 4=320HEX) This

parameter will supply to the PROSE4 driver the speech base address.  By

default the driver will assume 300 HEX.  This is the default for the Prose

4000 board.  If you have changed the base address from 300 HEX, you must

supply this parameter.  For example, if the speech address has been changed

to 318 HEX, you must supply:

PROSE4 /!B3

If necessary, you can combine both the IRQ command and the speech base

address.  This driver does not require a port address.

PSSA.COM

This is a self contained external synthesizer so nothing other then PSSA.COM

needs to be loaded.

This should be used for the Personal Speech System version A only.  This is

a relatively old synthesizer.  It really was not designed for the blind

community.  You will notice as you speed the voice up, the voice becomes very

choppy and difficult to understand.  It supports all ten rates, pitch

settings and volume levels.  However, it does not support any tone settings.

Changing Vocal-Eyes' tone option will have no effect whatsoever.

PSSB.COM

This is just like the PSSA.COM except it is for Personal Speech System

version B. It has the same features and limitations as version A.

VEASP.COM

This driver is to be used with the ASP synthesizer.  It supports all of the

Vocal-Eyes commands.

GENERIC.COM

This can be used if the synthesizer you are using is not directly supported

with its own driver.

You will need to setup your synthesizer as required in its users manual.

Once all its required software is loaded, simply load the GENERIC.COM driver.

At this point, you should be able to use your synthesizer with Vocal-Eyes.

However, you will not have any control over the rate, pitch, volume, tone and

worst of all, you will not be able to silence the voice from Vocal-Eyes.

If your synthesizer supports any of these features, you can tell the generic

driver how to use it.  When you first load GENERIC.COM, it looks for a file

called "GENERIC.DAT."  If this file does not exist, it will default to not

supporting any features at all.  However, if it does exist, the information

will be used giving Vocal-Eyes control it normally would not have.

The format of the GENERIC.DAT file is very important.  There is not much room

for error.  First of all, the file must be in a standard ASCII format.  It

must contain exactly 59 lines.  The first 10 lines relate to the 10 rate

commands, followed by the 10 pitches, 10 volumes, 26 tones, silence string,

initialization string and finally a list of punctuation characters that can

be sent to the synthesizer.  This all adds up to 59 lines.  Even if your

synthesizer does not support all features, you must make an entry for all 59

lines.  If there is a problem in the format of the file, the GENERIC driver

will print "Format error in GENERIC.DAT" and abort the load process.  You

should correct the problem and attempt to load the driver again.

In the following example, we will create a GENERIC.DAT driver for the

Sounding Board.  Of course, this is not needed for the Sounding Board but it

will make a good demonstration.

The first 10 lines of the file relate to rates 0 through 9.  Each rate

command must be on a line by itself.  First you need to figure out what

command your synthesizer needs for the 10 rates.  The Sounding Board uses

CTRL-ER0-9.  Therefore, the first 10 lines of your file would look like this:

/5/R0

/5/R1

/5/R2

/5/R3

/5/R4

/5/R5

/5/R6

/5/R7

/5/R8

/5/R9

You are probably wondering what the /5/ is used for at the beginning of each

line.  Many word processors or text editors will not let you place ESCAPE or

control characters directly in a file.  Therefore, whenever you wish to enter

one of these codes, use the format:

/n/

Where n is the ASCII value of the character you wish entered.  For example,

to enter an 'ESCAPE Control-Z A' as one of your entries, the following line

could be used:

/27//26/A

The /27/ is the ASCII value of the ESCAPE.  /26/ is the ASCII value of

Control-Z and of course the A can be enter directly.  What if you really

wanted to enter a '/' in your string?  Simply place two slashes together.

For example:

/5////6/B

This line would place a Control-E (/5/) followed by a / (//) followed by a

Control-F (/6/) and finally a B (B).

Therefore in our Sounding Board example, the /5/ is simply the required

Control-E character.

Now that we have defined the first 10 lines as the rate, the next 10 lines

need to define pitches 0-9.  The Sounding Board requires CTRL-EP0-9.

Therefore, the following would be lines 11-20.

/5/P0

/5/P1

/5/P2

/5/P3

/5/P4

/5/P5

/5/P6

/5/P7

/5/P8

/5/P9

Now, we need to define the 10 possible volumes 0-9.  The Sounding Board

requires CTRL-EV0-9.  Therefore, the following would be lines 21-30:

/5/V0

/5/V1

/5/V2

/5/V3

/5/V4

/5/V5

/5/V6

/5/V7

/5/V8

/5/V9

The next 26 lines need to define the 26 possible tones A-Z.  The Sounding

Board requires CTRL-EA-Z.  Therefore, the following would be lines 31-56:

/5/TA

/5/TB

/5/TC

.

.

.

/5/TX

/5/TY

/5/TZ

The next line in the GENERIC.DAT file should be the string which when sent to

the synthesizer will silence the speech.  On the Sounding Board this is

CTRL-X.  Therefore, the following would be line 57 in our GENERIC.DAT file.

/24/

Remember, /24/ simply means ASCII 24 which is a CTRL-X.

The next line contains the initialization string.  This string will only be

sent to your synthesizer one time.  You can use this feature to make sure

certain settings are always set the way they should be.  The Sounding Board

really does not require such a string.  Therefore, leave this line completely

blank.  Line 58 in our GENERIC.DAT file will simply be a blank line.

The final line holds all the punctuation characters which can be sent to the

synthesizer.  On the Sounding Board this is every character.  However, for

example, the Personal Speech System uses both the ! (exclamation point) and

the @ (at sign) for command strings.  If you send these characters, the

synthesizer will give an error.  So for the final line 59, you would type the

following if you are using the Personal Speech System:

~`()_^-{}|:[]"?\;,.'

Notice the leading space.  Make sure you include this character.  You should

try and send as many punctuation characters as possible so your synthesizer

can use them for intonation.  Notice the @ and ! have been left out.  There

are a few others which have been left out since they also can be used as

command characters.  Sounding Board users could have included each and every

punctuation character.

Now that you have all 59 lines typed in your file, save the file in a

standard ASCII format.  Assuming you have no errors, each time you load

GENERIC.COM, it will load the GENERIC.DAT file and use the information

specified.

It may take a bit to get this file setup but once it is done, you no longer

have to worry about it but your synthesizer will work forever.