*            New SS.Control Structures For CoCo Ver 3 L-II* New SS.CnSet setstt (similar to SS.WnSet) sets up and displays various* mouse-driven fancy controls on a window. SS.Mouse packet now returns CONTROL* status byte also if mouse is clicked within a control area. User then does* the normal SS.MnSel call, which will recognize being over a control, and* will allow manipulation of that control. SS.MnSel will return the control* ID number in (A), and it's new position/selection in (B).* Since they don't clear the screen anywhere, doing the call again allows* changing a displayed default value realtime, plus using these as indicators!SS.CnSet equ $C0 temporary valuecndata   equ $2E pointer at end of windentry*TYPESCT.Buttn equ 0 Radio/toggle/invisible/any size, any number.CT.Slide equ 1 Used to simulate volume/position sliders. May be vert/horz.*COMMON PARMS (Control descriptor)         org 0CN.TYPE  rmb 1 Control typeCN.ID    rmb 1 ID number of control (50-100)CN.DIR   rmb 1 Direction of layout (0=vert, 1=horz)CN.X     rmb 1 Upper left col,rowCN.Y     rmb 1CN.SZX   rmb 1 Size of control areaCN.SZY   rmb 1CN.AUTO  rmb 1 (0=not automatic)CN.ENBL  rmb 1CN.INFO  equ . Rest of controlCN.RES   rmb $20-.CN.SIZ   equ .*Slides:         org CN.INFOCS.BACK  rmb 2 Background gfx buffer and logicCS.SLID  rmb 2 Slider gfx buffer and logicCS.MIN   rmb 2 Minimum value (gotten from memory each time)CS.MAX   rmb 2 Maximum value   "CS.DEF   rmb 2 Default value   "CS.INC   rmb 2 Increment amtCS.RES   rmb CN.SIZ-.*Buttons:         org CN.INFOCB.RADIO rmb 1 Action flag: radio/toggleCB.XSIZ  rmb 2 Size of button(s)CB.YSIZ  rmb 2CB.NITS  rmb 1 Item countCB.ITEMS rmb 2 Pointer to button(s) info:CB.RES   rmb CN.SIZ-.*Each Button:         org 0BN.TTL  rmb 20 Title (00=none, spaces=for buff)BN.ENBL  rmb 1 Enable flagBN.VIS   rmb 1 Visible flagBN.DIS   rmb 2 Gfx grp/bfn for DISABLEDBN.ON    rmb 2 Gfx grp/bfn for ON  (Put on before title)BN.OFF   rmb 2 Gfx grp/bfn for OFF (Put on before title)BN.RES   rmb 32-.BN.SIZ   equ .******************************************************************************                            EXAMPLE BUTTON*****************************************************************************      ._______________ CN.X=03      |           .___ CN.SZX=06      |           |0 1 2 3 4 5 6 7 8 9123     __  __  __  __  <--- CN.Y=03      XX  ||  ||  ||       CN.SZY=014      |   |      |   |_______________ BN.OFF Buffer      |      |___________________ BN.ON  Buffer    ******************************************************************************                       EXAMPLE SLIDE/INDICATOR*****************************************************************************          .__________ CN.X=05          |0 1 2 3 4 5 6 7 8 9 12          __    <--- CN.Y=02     Buffer CS.BACK put at row/col 5,2 (40,16)           || 3          ||           ||4          ||           ||5         ====   <--- CS.DEF=03   Buffer CS.SLID put at row/col 5,5 (40,40)          ====6          ||           ||7          ||           ||8          --    <--- CN.SZY=06          |__|_______ CN.SZX=02