Enter/Exit Protected Mode Example

This assembler program was written to provide an example of how to

enter and exit Protected Mode (PM) under various starting points one

might encounter in DOS:

* Real Mode (RM);

* Virtual Mode (VM) using VCPI;

* Virtual Mode (VM) using DPMI.

VCPI:

To simplify stack references, we set the B-bit in the stack selector

so we can use ESP for base addressing as necessary.

DPMI:

We enter PM as a 32-bit client.  This has several implications among

which are

* Interrupts are passed to us with a 32-bit stack frame, and we

terminate using IRETD or RETFD as appropriate.

* DPMI calls made by us must use 32-bit register pointers (ES:EDI

instead of ES:DI) where appropriate.

* The B-bit in the stack selector so we can use ESP for base

addressing.

Please feel free to use this code to learn more about this process.

If you have any questions about how this program works, please contact

Bob Smith at Internet address "bsmith@sudleyplace.com".