LPT I/O V1.4

by John Augustine

LPT (parallel port) input/output (digital data acquisition, etc.) was

written in Assembly for MS Dos systems.  Requires Color Graphics (CGA) or

better. Runs on 8086 to Pentium computers.

This program is NOT for those who know little or nothing about the parallel

port.  Some applications for using LPT will probably require some type of

interface, the simplest of which can be an isolation or load resistor such as

a 10k resistor to allow you to view the patterns on a scope (example).  A

little knowledge about electronics would be very helpful.

PLEASE NOTE:

This is NOT a "Free" program.  A small Donation will be Greatly Appreciated.

For a minimum donation of $5.00 (U.S.A.), as an added BONUS, pick one or more

programs that you would like to receive from the "OTH-PGMS.DOC" (List of MS

Dos based programs).  Also, as a "Donator", I will be glad to help you with

a project involving the use of LPT, but know this, I am not a hardware

"expert".

TIP:

If you have a second (older) PC, use it with this program to experiment.

LPT USES:

There are many possible uses for LPT which I have not thought about at this

time.  LPT allows you to capture (Record) into memory, digital data (TTL level

(0 or +5V) input) via pin 12 (parallel port), normally used for out of paper

detection (printer).  Afterwards, you can view it in binary format (two modes)

or save it to a file as reference, or for "playback" whenever you need to

simulate a digital source.  The digital data can be transmitted ("playback")

via all the data pins (pins 2-9) in the parallel port, in order to simulate

the digital data source.  Note: Only one of those data pins is normally

needed, unless you have a special use which requires more than one line/pin

(up to 8).  Tip: You can create your own digital data to transmit out of the

parallel port by creating a binary file with specific patterns.  Note:

"TOOLBOX" (see "OTH-PGMS.DOC") makes this task easy.

You can simulate a digital data sequence for testing purposes, etc.

Another simple example for using LPT is to Record the patterns of a remote

control using a simple inexpensive IR receiver, which you can buy at Radio

Shack.

You can use LPT to check your parallel port for defects (example if a

data pin is "stuck" at one level).

GETTING STARTED:

Simply type (without quotes) "LPT" and press Enter

OR

If you want to view a digital data file or simulate a digital data source

(from saved data):

Type (without quotes) "LPT FILENAME" and press Enter.  "FILENAME" represents

the name of the data file.  Doing this presets the filename for Loading.

VIEWING DIGITAL DATA:

You can view data in two binary modes.  This allows you to see the bits

sequence (digital data) which starts at the top (left) row (moving to right).

One mode displays in "1" and "0" format.  If you press the "S" (switch) key,

another format is displayed, which is better suited for seeing patterns.

Pressing the "U" or "D" key moves the binary reference bar/scale up and down

so you can pin point bit/s.  Other commands allow you to move throughout the

data buffer.  Note: If you did not record or load a data file, what you see is

"residue" from a previous application, etc.

LPT NOTES:

The Menu has different options.  Within the "Record, Playback..." function,

is the option to "Set Delay.....".  The effect of this delay will vary with

PCs and their CPU speed.  The delay setting determines how long to wait before

taking a sample (bit) in "Record" mode or how long to wait before transmitting

the next bit during "playback".  Note: This delay number should be the same

for Playback as it was for recording, in order to simulate the original

digital data.  On the other hand, you can slow down or speed up the digital

data if you prefer.

There is information about the storage capacity in bits and bytes displayed

on the main menu.  Use the bytes information, if you are creating a binary

data file (experiment), so you don't create a file that is larger than the

memory buffer.

The length in time, digital data can be recorded, depends on the buffer

size, which will vary some from PC to PC, the CPU speed, and the Delay

setting.  Note: Only "conventional" memory is used for storage due to time

balancing in the Record or Playback procedures.  Note: Pressing Esc during

recording or playback will abort that function.

SQUARE WAVE GENERATOR:

An adjustable square wave generator is one of the features in LPT.  You can

use a scope to view it, but I suggest you put a 10K resistor between one of

the data pins (2 - 9) and ground.  Put your scope (or other) across that

resistor.

"TEST PIN 12 (INPUT) FOR ACTIVITY":

This was a feature I added for certain tests I was making.  It will count

the number of transitions from one state to the other that occurs within

1/18th of a second.

PEEK OR POKE:

This function allows you to "peek" or "poke" the Data and Status ports for

the parallel port.  This is useful for testing purposes.

CONCLUSION:

This is NOT a "Free" program.  A small Donation will be Greatly Appreciated.

For a minimum donation of $5.00 (U.S.A.), as an added BONUS, pick one or more

programs that you would like to receive from the "OTH-PGMS.DOC" (List of MS

Dos based programs).  I will send it/them to you on a 3.5" disk, or attach

it/them to e-mail if you prefer.  Please include your e-mail address with your

donation and the name/s of the other program/s you want (if you want any), to

receive via e-mail.  You will be entitled to receive all improved and new

programs (you select) developed in the future (will be sent to you by e-mail).

You will NOT be expected to pay for any other program now or in the future.

In effect, with just one small donation, you paid for one or more extra (your

choice) programs instead of one.  Remember to include a note listing which

program/s you would like to receive (if you want any), and if you want to be

notified of future developments.  The donation will motivate me to develop

more programs and make improvements (add new features).

Comments or complaints are always welcomed (even if you did not give a

Donation).  If you use "Snail Mail" and expect a reply, please include a

S.A.S.E.

E-Mail:  jaugust@bellatlantic.net

John Augustine N3AOF

3129 Earl St.

Laureldale, Pa 19605

(610) 929-8850