SIDED(TM) Version 1.03 Freeware release

1.0 Introduction

2.0 System requirements

2.1 OS independence

2.2 Running SIDED from multitasking operating systems

3.0 Getting started

3.1 Command line options

4.0 Explanation of log messages

4.1 List of all error and warning codes

4.2 ATAPI Units and SIDED

4.3 What to do with this information

1.0 Introduction:

This file explains the error codes generated by SIDED.

It also explains the command line options etc.

Make sure you have also read the file terms.doc before using SIDED.

2.0 System requirements:

* DOS version 3.3 or higher

* A 286+ processor

* An ATA/IDE drive + controller

* 200 KB of free memory

It's not necessary to install the drive in the CMOS, in some cases the system

won't even boot when a malfunctioning drive has been entered in CMOS setup.

However do not forget to properly jumper the drive.

SIDED will also test EIDE drives connected to a IDE controller.

2.1 OS independence:

SIDED does not perform any tests to detect software crashes therefore it can be used

on every  harddisk regardless of the installed operating system (OS).

We recommend that you put a copy of SIDED on a bootable DOS disk.

2.2 Running SIDED from multitasking operating systems:

SIDED runs under DOS so theoretically it can be used under every multitasking

OS's that support a DOS box. Since SIDED accesses the drive using register

I/O it's not recommended to do this, since it may result in corrupted data.

SIDED has a build in detection which has been tested under Windows 3.11,

Windows 95, Windows NT version 3.51 and OS/2 which prevents running it under

these multitasking environments. This detection can be overruled by using

the command line parameter "nowindows", as stated earlier this is not

recommended.

3.0 Getting started:

SIDED is an easy to use program, it supports the keyboard and the mouse

With a mouse just click the option which you want to select. Selected

options will show the character 'V'. With a keyboard you can select options

by using the distinguishing color (red on VGA color, highlighted on Hercules).

Overview of all keys:

* 0:  Select controller 0 (1F0)

* 1:  Select controller 1 (170)

* X: Select controller X (...) You can select your own controller here,

use this option with care since writing to non IDE units can harm your

system.

* M: Select the master drive on the currently selected controller.

* S: Select the slave drive on the currently selected controller.

* C: Create a log file of this session.

* E: Beep when an error is detected.

* B: Begin diagnostics.

* Q: Quit to DOS

3.1 Command line options

Overview of command line options:

timeout:xxxx     Set time-out to xxxx ms, the default is 30s (30000ms)

nowindows        Override automatic detection of multitasking environments

herc             Override video card detection to hercules/MDA mode

noherc           Override video card detection to non-hercules/MDA mode

reset            Reset drive after init, this will wake up sleeping drives

register         Register this copy (needs key-code)

Use the option nowindows only if you are sure the detection failed.

The options herc and noherc can be used if the screen doesn't appear to be

right. The option reset is useful if the drive does not respond and the "init

state" (The first diagnostics test) reports the drive not ready.

WARNING:

If you're using a memory manager and you have mapped B000:0000 - B7FF:FFFF

to be used as UMB then do NOT use the herc option.

4.0 Explanation of log messages

SIDED knows 4 types of responses:

* V:  The OK response reports a successful completion of the test.

* X:  The test FAILED, the value between the brackets ([]) specifies which error occurred (see 4.1).

* ?:  The WARNING response indicates that something happened that

isn't normal but isn't really a failure either. It also has a

error/warning value (see 4.1).

* >:  The COMMENT response indicates which test is performed etc.

The information reported by it can be ignored. Comments do

not have an error/warning value.

4.1 List of all error and warning codes

This is a list of all failure and warning codes. The codes exist out of

4 digits. The upper 2 digits indicate in which diagnostics block the

failure/error occurred.

For example error/warning code 0x0201 indicates the error occurred in the

"Checking I/O registers & Soft-reset" section (main code 02) and that

register I/O failed (sub code 01)

List of main-codes:

01 Checking initial state

This section checks for the current drive status. It does not make the

assumption that the drive is available. If not, the error 0x0102

"Init state: controller not ready [FF]" will be reported. Drives which are

in an undefined state often report this error with the value "D0" between

the brackets.

02 Checking I/O registers & Soft-reset

This section checks whether the drive register access is OK. If one of the

registers fails, diagnostics will be terminated. When the result values all

miss the same bit (or group of bits) its likely that there is a cable

failure. The soft-reset will return the drive to a pre defined state to

enable further diagnostics.

03 R/W- buffer (write)

04 R/W- buffer (read)

These sections verify access to the drive sector buffer.

Three patterns are written to the sector buffer and read back

to verify access and buffer integrity. Unfortunately this command is not

supported on all drives.

05 On-drive diagnostics

This section performs the diagnostic tests as implemented in the harddisk's

firmware. The manufacturer defined values are returned.

06 Verifying sector (partition sector)

This section reads the partition sector without actual data transfer.

It is used to check the data transfer logic.

Some drives always read a sector into cache before verifying. On these

drives the verify command is quite useless. It is even possible that

the drive will report "sector not found" or equivalent errors when

"only" the on-drive cache buffer is defect.

07 Reading ID sector

This section reads the ID sector. Except for the real old drives

the ID sector is stored in the drive's firmware. The firmware is

stored in a (E)PROM on the logic-board so it does not include HDA access.

08 Reading sector (partition sector)

The test performed in this section is almost equal to the one in section 6.

However this test also includes the actual data transfer.

If section 6 succeeds and this section fails the failure is probably on the

logic-board.

List of codes:

0x0101: The value in the alternative status register differs from the value

in the primary status register. Running this program with the 'nowindows'

option will result in this warning under some OSs.

0x0102: The on-drive controller reported not ready, probably because it is

busy. This warning will also occur when no drive was found on the selected

port.

0x0103: The HDA (the mechanical part of the harddisk) reported not ready.

The drive can be asleep (green-/powersaving mode). Another option is

that the 12V power (needed for the drive's engine) isn't reliable.

Check the power supply cable and verify the voltages from the power supply.

0x0104: The R/W data bit is still high. This warning should be cleared by the

soft-reset as performed in "Checking I/O registers & Soft-reset" section.

0x0201: The register I/O failed. Diagnostics will terminate after this error

because the drive can't be tested properly. All further tests assume a correct

working register I/O. Verify whether there is really a drive present on the

selected controller. Also check the cable and power.

0x0202: An ATAPI unit was detected. Diagnostics will be limited to

on-drive diagnostics and then stop.

0x02F0, 0x06F0, 0x07F0 and 0x08F0: The HDA (the mechanical part of the

harddisk) reported that it was not ready. Check the power supply and cables.

SIDED will perform a recovery drive reset to allow diagnostics to continue.

0x02F1, 0x03F1, 0x04F1, 0x5F1, 0x06F1, 0x07F1 and 0x08F1: The on-drive

controller (or advanced host adapter) reported it was not ready. The

on-drive controller is not ready to accept commands in this mode. SIDED

will reset the drive when this error occurs and skip this section.

0x0301: The R/W buffer command isn't implemented on this harddisk. This is a

warning since the manufacturer is free to implement this command.

0x030A, 0x040A, 0x70A and 0x080A: The drive reported ready while the

transfer wasn't completed yet. This can mean the host adapter is pre-fetching

or that a glitch occurred on the IOR channel which fakes a real read cycle.

Check the length of your IDE cable, it should be shorter then 18 inches

(45cm).

0x030B, 0x040B, 0x70B and 0x080B: After sending or receiving 256 words (1

sector) the drive's DRQ bit is still set. This is probably an IDE cable

error (see 0x030A).

0x03F2 and 0x04F2: The DRQ bit, a bit which indicates whether the drive is

ready to send or receive data isn't set. This error could also mean the

R/W-buffer command is not (properly) implemented.

0x03FF: The R/W buffer reported an error, but it wasn't the 0x0301 warning.

It's probably due to a malfunctioning logic board.

0x040C: This error indicates that the read data is not equal to the

previously written data. This can mean the drive's buffer is defect,

however more common it means a cable failure.

0x0501, 0x502 0x503 0x504 0x505 0x506: The errors reported by SIDED are the

error-codes as assigned to the ST506. ATA drives aren't bound to these

error-codes. Contact your harddisk vendor if you have a question about

these values. The error-code is the FIRST value after the bracket '['.

0x0507: Some harddisks can only report an error occurred, but can not specify

which error when they are jumpered as a slave unit. In these cases you'll get this

error. Retest the drive jumpered as a master to find the specific error.

0x0507 0x0681,0x0781 and 0x0881: An error was detected. The specific error is

found in following log entries.

0x0682,0x0782 and 0x0882: A bad partition sector was detected. Bad sectors

are marked during low-level format (at the factory). Drives with a

partition sector marked as bad do not leave the factory, so this error

shouldn't occur.

0x0683,0x0783 and 0x0883: An unrecoverable ECC error, the partition sector has

some errors and the error correction system (ECC) wasn't able to correct

them.

0x0684,0x0784 and 0x0884: Media Changed, normally this only occurs with

removable disk systems. It means that a new disk was inserted and the drive

has to determine the type. SIDED does not determine the type so both verify

and read will fail when this error occurs.

0x0685,0x0785 and 0x0885: Sector ID or Target sector not found, the drive

could not find the specified sector. Since the partition sector is tested,

this error means that the drive is unusable in a default setup.

0x0686,0x0786 and 0x0886: Media change request, the eject button was

pressed. This only occurs with removable disk systems. SIDED will not

handle this error.

0x0687,0x0787 and 0x0887: Command aborted; this error means that a command

was aborted, either because the command wasn't supported (0x0787) or because

a command couldn't be completed.

0x0688,0x0788 and 0x0888: Track 0 not found. This error should no occur

since SIDED does not recalibrate the drive.

0x0689,0x0789 and 0x0889: Address mark not found, during the read or verify

command the proper sector ID field was found but no address mark was detected.

The sector likely has a hard error, since the partition sector is tested this

is a serious error.

0x070C: Suspicious data was found in the ID sector, this may indicate a data

transfer error.

0x070D: All the high bytes contain the same value. This indicates an on-drive

buffer, cable, host adapter or main board failure.

0x080D: All the high bytes contain the same value this can mean an error (see

0x070D) or the sector is contains equal data. Since the partition sector is

tested this means the drive is probably not partitioned.

0x070E, 0x080E: All low bytes contain the same value (see 0x70D,0x080D).

0xFFFF: This error code indicates that the drive recovery reset failed to

reset the drive, it will terminate diagnostics.

4.2 ATAPI Units and SIDED:

SIDED is not intended for use with ATAPI units, when you select such a

unit it will run limited diagnostics.

4.3 What to do with the information generated by SIDED?

SIDED performs several tests, but it doesn't really combine test results.

To help you pinpoint the exact location of the failure it is recommended to

retest the drive in another computer. If this is not possible try to replace

the power cable, the IDE cable and the host adapter to exclude errors in

these components.

Remember that in most cases only ONE of the components is defect.