TIDES.DOC

DOCUMENTATION FOR TIDES - VERSION 3.08 - 1995 FEB 03

Edward P. Wallner

32 Barney Hill Road

Wayland, MA 01778-3602

(508)-358-7938

epwallnr@world.std.com

This is a program for predicting the height of the tide or the

strength of the tidal current at various stations. It runs on IBM

compatible machines under MS-DOS or IBM-DOS. The method and format

followed in the predictions are generally those used by the National

Ocean Service (NOS) in the standard tide and current tables.

This program may be freely copied and used for non-commercial

purposes. Feel free to try the program and pass it on without

alteration and without charge. If you use the program a $15

registration fee must be paid.

The author assumes no responsiblity for any errors in the program or

the predictions.

Detailed predictions are made for high and low water (or maximum ebb

and flood) at reference stations for which the tidal constants have

been derived from observations made over a long period of time. The

constants for subordinate stations are found by making corrections to

the constants at the reference stations. The predictions for

subordinate stations are less accurate than for reference stations,

particularly for the lower high and higher low water where tides are

nearly diurnal or when the time differences for high and low water

differ greatly. Current floods/ebbs at subordinate stations may also

be less accurate. For stations where the current is rotary the current

along the principal axis is predicted. The times and speeds of flood

and ebb in these cases are approximate.

The TIDES program predicts the height of the tide or strength of the

current at any time. The values at reference stations are of the same

order of accuracy at any time as for the high/low or flood/ebb values.

The predictions for subordinate stations may be less accurate at

intermediate times, the more so the larger the difference between the

time corrections for high and low water.

PROGRAM OPERATION

Just enter TIDES and follow the prompts. When a date is to be input a

default date is displayed. It may be modified using the numerical keys

and Home, End, left and right arrows and backspace. In general

entering "Q" or "Esc" will stop any operation and return you to the

menu.

A station filename may be entered on the command line after the

"TIDES" command. Any filename can include a valid PATH prefix.

Initially the start date is today and the end date is a week from

today. To run the next week's tides for the default station just enter

"A".

2

If the start date is earlier than 1582/10/15 the Julian calendar is

assumed, otherwise the Gregorian calendar is assumed.

The main menu gives options for:

A - Computing and printing highs and lows

B - Computing and printing hourly heights

C - Plotting heights at a given interval (the default of 0.5 hours

generally works well)

D - Computing the height at specified dates and times or intervals

E, F & G - Changing the time interval, station, and default station

H - Creating additional data files

I - Changing from standard to daylight time or vice-versa

(In the U.S. Daylight Saving Time extends from 2 a.m. on the first

Sunday in April to 2 a.m. on the last Sunday in October)

J - Changing from metric to English units or vice-versa

K - For redirecting the printed output to a file or another printer

(The output will be an ASCII file which can be edited using any text

editor)

L & M - Printing the astronomical and constituent factors used in the

predictions (used mainly for verifying the program)

N - Shell to DOS without exiting TIDES

O - Finding the extreme high, low and range in the time interval

Q - Quitting TIDES

The equivalent quantities are computed for tidal currents.

When the "CHANGE STATION" or "CHANGE DEFAULT STATION" option is

executed a list of stations for which tidal constants have already

been stored in the TIDES format is shown, including the file names

under which they are kept. This is generally a selected set of

stations for a specific area.

When the desired station is on the screen press S to stop paging. You

will then be prompted to enter the station file name which is shown in

parentheses after the station coordinates.

The list can be edited using EDLIN or any other text editing program.

For example undesired stations can be eliminated and other station

titles moved to the desired line.

The station names and locations stored in file STALIST.T&C are

displayed by the program as a reminder, but the name of any file of

data in the TIDES format can be used whether it is on the screen or

in the STALIST.T&C list or not. PATH prefixes can be used in the

filename. New stations for which no data file exists can be created

using menu option H.

CREATING ADDITIONAL DATA FILES

Option H is used to generate data files for the TIDES program both for

stations for which harmonic constants are available and for

subordinate stations, for which only differences from a reference

station are available.

3

First a title and file name for the station are entered. The prompt

for the title has places for the latitude and longitude of the station

which are optional inputs for better identification and do not affect

the computation. For stations for which harmonic constants are

available you are then prompted to enter the type of station (tide,

current, or hydraulic current) and the amplitude and epoch of each of

37 tidal constituents. The constants are entered in sequence from the

NOS form 444 of "Standard Harmonic Constants for Prediction", columns

B (?.??xH) and D (k'). If there is no data in column  B, use column H

(Amplitude). If the amplitude of a constituent is zero entering "0" or

just a carriage return will enter 0 for both the amplitude and epoch.

The modified epoch D (k') should be entered as a number from 0 to 360;

ignore any minus sign in column D. In some data sources the modified

epoch is designated by "g" instead of "k'".

Several minor constituents can optionally be inferred from the major

constituents if they are not entered directly. This generally gives a

slight improvement in accuracy at the cost of a slight increase in

running time. In places near an amphidromic point, such as New

Zealand, inferred constants are not helpful.

For subordinate stations the corresponding reference station data must

exist in a data file. The list of stations for which data is already

stored is shown and the file name of the reference station entered.

The time differences between the reference and subordinate stations at

high and low (or maximum flood and ebb) are entered, then the height

differences. In some cases the range of the tide at both stations is

requested, as is the maximum flood and ebb for current stations. This

data can be taken from Table 2 of the NOS tide or current tables.

When Option H is run the title of the new station is added to the end

of the STALIST.T&C file. This file must be edited if it is desired to

place the station title in a different line.

REQUIREMENTS AND CHARACTERISTICS

TIDES should run on any IBM machine or compatible with 128K of free

memory and the text output should be OK on any printer. The plot of

tide heights on the monitor works on the Color Graphics Adapter (CGA)

board and in printing uses codes for EPSON or IBM printers (IBM

extended ASCII.) Special codes to set up NEC pinwriter printers are

also available. For HP Laserjet printers the menu should be set to

"PC-8 Symbol Set."

TIDES.EXE, the executable version of the prediction program, runs with

or without an 80x87 coprocessor. TIDES computes the highs and lows for

Boston for a month in about 1 minutes on an IBM-XT with an 8087

coprocessor, and in about 38 minutes without an 8087. (An 8087

coprocessor can be bought for less than $100 and is a good investment

if you do much number crunching, such as tidal predictions.) On a

486DX2-66 a years highs and lows for boston took 22 seconds.

Predictions for Boston and New York for 1986 agreed with the standard

tables to the minute and tenth of a foot for most cases with some

times off by 2 or 3 minutes and an occasional difference of .1 or .2

foot. Currents in Cape Cod Canal were generally within 0.1 knot. (If

the tide level remains nearly constant for an extended period the

predicted time of high or low can vary considerably but the height

will still be accurate.)

4

Predictions should be accurate for any era within a few millenia of

the present unless the physical conditions at the station change.

The source program TIDES.BAS is written in QuickBASIC. The source

code is available to registered users on request.

DATA SOURCES

A partial list of places for which I currently have harmonic constants

is given at the end of this document.

The harmonic constants for various North American reference stations

are published by:

National Oceanographic and Atmospheric Administration

National Ocean Service N/OMA132

Rockville, Maryland 20852

I have a complete set of this data for all U.S. reference stations

including Alaska, about 76 other U.S. stations and all tide and many

current stations in British Columbia and all tide stations in New

Zealand and Australia. I will send a copy of a limited number of

station constants and subordinate station corrections to registered

users who let me know the area of interest. Some stations for which

constants are available are listed below.

Differences for tides and currents for subordinate U.S. locations are

given in the annual tables published by the National Ocean Service. In

1989 the datum to which height of tide is referred was changed from

mean low water to mean lower low water for all U.S. stations. Though

the difference is generally only a couple of tenths of a foot or less,

data should be taken from 1989 or later tables. The constants

themselves do not change appreciably over many years so any later

table can be used.

The International Hydrographic Organization maintains a data bank of

tidal constants for  4208 stations worldwide. If you wish constants

for foreign locations send me the latitudes and longitudes (to the

nearest minute) and I will eventually send the constants for the

closest station in the IHO data set if available. (The IHO data uses

the symbols H for amplitude and G for phase in place of k'.)

The tide tables published by the British Admiralty give data for use

in their Simplified Harmonic Method of Tidal Prediction. This data is

given for about 4400 tide and 148 current stations world-wide. A PC

program implementing this method is also distributed. Harmonic

constants are given for M2, S2, K1 and O1 as well as data from which

M4, M6, S4, S6, MS4, MSf, Sa and SSa can be computed. Predictions of

about the same accuracy can be made with TIDES if inferred constants

are included in the data file.

The French "Table des mares des grands ports du monde" gives Z0,

Sa, Q1, O1, K1, N2, M2, S2, MN4, M4 and MS4 for 901 ports world-wide.

This data is from IHO sources and may not be up-to-date.

5

TIDAL THEORY

The theory used in the TIDES program is given in:

Manual of Harmonic Analysis and Prediction of Tides

Paul Schureman

Coast and Geodetic Survey Special Publication No.98

U.S. Gov't. Printing Office, Washington D.C. 1941

This book has been reprinted recently and is available from NOS for

$3.50 postpaid. I have a few copies and can send one for my cost of

$3.50 + postage for 1 pounds ($1.48 in the U.S.)

A supplement to this document, "Computer Applications to Tides in the

National Ocean Survey" is also available from NOS at no cost.

MISCELLANEOUS

The present version of the program does not give predictions of slack

water at current stations and for rotary currents the north and east

components of velocity must be predicted separately at hourly

intervals and combined vectorially by the user. If there is a demand

for these features I may get around to adding them sometime.

Starting in the 1989 tables the datum was changed to mean lower low

water for all U. S. ports, which meant a change of a few tenths of a

foot in Z0 (item 1, line 2 of -.TID data files) for ports on the

Atlantic coast. The new values (or close approximations) have been

used in the data on this disk.

If you have any suggestions, comments or problems please let me know.

6

HARMONIC CONSTANTS ARE AVAILABLE FOR THE STATIONS LISTED BELOW

------------- and many other stations ----------------------

REFERENCE STATIONS COVERED BY NATIONAL OCEAN SURVEY

TIDES - EAST COAST

Albany, New York                     Punta Palmas, Venezuela

Amuay, Venezuela                     Philadelphia, Pennsylvania

Baltimore, Maryland                  Portland, Maine

Boston, Massachusetts                Reedy Point, Delaware

Breakwater Harbor, Delaware          St. Marks River Entrance, Florida

Bridgeport, Connecticut              St. Petersburg, Florida

Caleta Percy, Magellan Strait, Chile***

Charleston, South Carolina           Sandy Hook, New Jersey

Cristobal, Panama                    San Juan, Puerto Rico

Eastport, Maine                      Savannah, Georgia

Galveston, Texas                     Savannah River Entrance, Georgia

Hampton Roads, Virginia              Suriname Rivier, Surinam

Isla Zapara, Venezuela               Tampico Harbor, Mexico

Key West, Florida                    Washington, D.C.

Mayport, Florida                     Willets Point, New York

Maimi Harbor Entrance, Florida       Wilmington, North Carolina

Mobile, Alabama

New London, Connecticut

Newport, Rhode Island

New York, New York

Pensacola, Florida

Punta Gorda, Venezuela

TIDAL CURRENTS - EAST COAST

Baltimore Harbor Approach, MD        Mobile Bay Entrance, FL

Boston Harbor, MA                    Pollock Rip Channel, MA

Cape Cod Canal, MA                   Portsmouth Harbor Entrance, NH

Charleston Harbor, SC                St. Johns River Entrance, FL

Chesapeake & Delaware Canal          Savannah River Entrance, GA

Chesapeake Bay Entrance              Tampa Bay Entrance, FL

Delaware Bay Entrance                The Narrows, New York Harbor, NY

Galveston Bay Entrance, TX           The Race, Long Island Sound

Hell Gate, East River, NY            Throgs Neck, Long Island Sound

Key West, FL                         Vieques Passage, Puerto Rico

Miami Harbor Entrance, FL

TIDES - WEST COAST

Aberdeen, Washington                 Matarani, Peru

Anchorage, Alaska ***

Astoria, Oregon

Balboa, Panama                       Nikishka, Alaska

Buenaventura, Columbia               Nushagak Bay, Alaska

Callao, Peru                         Port Townsend, Washington

7

Cordova, Alaska

Dutch Harbor, Alaska                 Puntarenas, Costa Rica

Guayaquil, Ecuador                   St. Michael, Alaska

Guyamas, Mexico                      Salina Cruz, Mexico

Honolulu, Hawaii                     San Diego, California

Humboldt Bay, California             San Francisco, California

Juneau, Alaska                       Seattle, Washington

Ketchikan, Alaska                    Seldovia, Alaska

Kodiak, Alaska                       Sitka, Alaska

Kwajelein Atoll, Marshall Is.        Sweeper Cove, Alaska

La Union, El Salvador

Los Angeles, California              Talara, Peru

Massacre Bay, Alaska                 Valdez, Alaska

TIDAL CURRENTS - PACIFIC COAST

Admiralty Inlet, WA                  Rosario Strait, WA

Akutan Pass, Aleutian Islands        San Diego Bay Entrance, CA

Ch'ang Chiang Entrance, China        San Francisco Bay Entrance, CA

Deception Pass, WA                   San Juan Channel (S. Entrance), WA

Grays Harbor Entrance, WA            Sergius Narrows, AK

Huang P'u Chiang, China              Strait of Juan de Fuca Entrance

Isanotski Strait, AK                 The Narrows, Puget Sound, WA

Kvichak Bay, AK                      Unimak Pass, Aleutian Islands

North Inian Pass, AK                 Wrangell Narrows, AK

*** Complete prediction requires more than 37 constituents.

8

OTHER TIDE STATIONS

Alameda, CA                          Apalachicola, FL

Apia, Samoa Islands

Aquia Creek, Potomac River, VA       Aransas Channel, TX

Arthur Harbor, Palmer Station, Antarctica

Atlantic City (Steel Pier), NJ       Avon, NC

Bar Harbor, ME                       Bayou Rigaud (Grand Isle), LA

Bayshore, L.I., NY                   Bay St. Louis, MS

Bears Bluff, N. Edisto R., SC        Block Island (Old Harbor), RI

Boca Congrejos, Isla Verde, PR       Buzzards Bay Entr, Cape Cod Canal, MA

Burntcoat Head, Nova Scotia          Canton Island, Phoenix Is., S. Pac.

Cape Cod Canal, E. Entrance, MA      Cape Hatteras (Fishing Pier), NC

Cape May (ferry terminal), NJ        Cape Steenboom, New Guinea

Carlos Point, Estero Bay, FL         Charleston, OR

Chesapeake Beach, MD                 Coconut Point, FL

Colonial Beach, VA                   Constantine Harbor, AK

Crescent City, CA                    Daytona Beach, Fl

Daytona Beach Shores (Sunglow Pier), FL

Elkhorn Slough, CA                   Esbjerg, Denmark

Friday Harbor, San Juan Island, WA

Galveston, Pleasure Pier, TX         Garibaldi, OR

Guam (Apra Harbor), Marianas         Hirtshals, Denmark

Indian River Inlet, DE

Lewisetta, Potomac River, VA         Turkey Point, FL

La Jolla, CA                         Eureka, CA

Point of Pines, SC                   Port San Luis, CA.

Neah Bay, WA                         Cross Island, AK

Nantucket, MA                        Malakal Harbor, Palau Is. Caroline Is.

Matanzas Pass, Estero Island, FL     McMurdo Sound, Antarctica

Mokuoloe Island, HI                  Montauk, Long Island, NY

Monterey Harbor, CA                  Morehead City, NC

Moss Landing, CA                     Myrtle Beach (Springmaid Pier), SC

Naples, FL                           New Bedford, MA

Newport Bay, CA                      Nikiska, Cook Inlet, AK

Old Saybrook Point, CT               Padre Island (South End), TX

Ponape Harbor, Caroline Islands      Port Aransas, TX

President Channel, Orcas Is., WA     Providence, RI

Puerto Soberania, Palmer Peninsula

Punta Tunas, PR                      Reykjavik, Iceland

Rincon Island, CA                    Rockland, ME

Sabine Pass, TX                      Saint John, New Brunswick

Seavey Island, ME                    Shell Point, FL

Southbeach, OR                       Stamford, CT

Tarawa, Gilbert Islands              Toke Point, WA

Virginia Beach, VA                   Wachapreague, VA

Wallops Island, VA                   Yakutat, AK

9

INTERNATIONAL HYDROGRAPHIC ORGANISATION DATA

Banyuls, France                      Socoa, France

Toulon, France                       Marseille, France

Saint Servan, France                 Cherbourg, France

Nice, France                         Venezia, Italy

Grado, Italy                         Esbjerg, Denmark

Aarhus, Denmark                      Hirtshals, Denmark

Saint John, New Brunswick, Canada    Burntcoat Head, Canada

Grand Turk, UK

Cabo San Lucas, B.C.S., Mexico       La Paz, B.C.S., Mexico

Magdalena Bay, Mexico                San Carlos, B.C.S., Mexico

Topolobampo, Sin., Mexico            San Juanico Bay, Mexico

Yavaros, Son., Mexico                Ballenas Bay, Mexico

Guaymas, Som., Mexico                Isla Guadalupe, B.C., Mexico

Bahia de los Angeles, B.C., Mexico   Puerto Penasco, Son., Mexico

Ensenada, B.C., Mexico               Salina Cruz, Oax., Mexico

Matavia, Tahiti, France              Motuoini, Tahiti, France

Nuku'alofa, Tonga                    Urangan Jetty, Australia

Brisbane Bar, Australia              Caloundra Heads, Australia

Brisbane (W. Inner Bar), Australia   Snapper Rocks(Danger Point), Australia

Ballina, Australia                   Iluka, Australia

Victoria Dock, Singapore             Horsburgh Lighthouse, Singapore

Tg. Segenting, Malaysia              Colombo, Sri Lanka

Hong Kong, UK                        Kao-Hsiung, China

Fujairah, Oman (U.A.E.)              Naha Ko, Japan

Kobe, Japan                          Nagoya, Japan

Sinko, Yokohama, Japan

CANADIAN DATA

Constants for all reference tide stations in the NOS tables for the

East and West coasts are available as well as most stations in British

Columbia. Tidal current constants for Grand Manan Channel (Bay of

Fundy Entrance) are also on hand.