TASM SUMMARY

--------------------------------------------------------------------------------------------------------------------------

COMMAND LINE:                                               | DIRECTIVES:

tasm -vv [-<option>] src_file [obj_file [list_file        |   #INCLUDE <filename> Read <filename> as source

[export_file [symbol_file]]]] |   #DEFINE  <label> [<macro_def>] Define a macro

-<vv>   Select version (-48,-51,-65,-85,-80,-68,-05,-32)  |   #IFDEF   <label>    Assemble following if <label> DEFINEd

-o<nn>  Specify number of obj bytes per record (hex)      |   #IFNDEF  <label>    Assemble following if <label> not DEFd

-c      object file written as a contiguous block         |   #IF      <expr>     Assemble following if <expr> nonzero

-d<mac> define a macro                                    |   #ELSE               Alternate block to assemble

-f<xx>  Fill entire memory space with <xx> (hex)          |   #ENDIF              End of conditional block

-g<m>   Obj format(0=Intel,1=MOSTech,2=Motorola,3=bin)    |

-h      Produce hex table of the assembled code           |   .ORG     <expr>     Set Instruction Pointer

-l      Produce a label table in the listing              |   .BYTE    <expr>     Define a byte

-m      Produce object in MOS Technology format           |   .WORD    <expr>     Define a word (lsb, msb)

-b      Produce object in binary (.COM) format            |   .EQU     <expr>     Assign a value to label

-e      Expand the source lines in listing file           |   .DB      <expr>     Same as BYTE

-t<tab> Select version (if non numeric)                   |   .DW      <expr>     Same as WORD

-p      Page the listing file                             |   .TEXT    <string>   Define bytes as ASCII characters

-x<m>   Enable extended instruction set (m = class mask)  |   .BLOCK   <expr>     Block space reserve

-q      Quite, disable listing file                       |   .TITLE   <string>   Set a title to appear on listing

-r<kb>  Set read buffer size (Kbytes, default 2K)         |   .CODES/NOCODES      Enable/disable opcodes in list file

-s      Generate symbol table file                        |   .SET     <expr>     Set value of label

------------------------------------------------------------|   .EJECT              Do Top of Form on listing

OPERATORS:                                                  |   .LIST/NOLIST        Enable/disable listing

|   .SYM/AVSYM <file>   Generate symbol table file

Operator  Type          Description                       |   .PAGE/NOPAGE        Enable/disable paging of list file

__________________________________________                |   .LSFIRST            Set least significant byte first

+        Additive       addition                          |   .MSFIRST            Set most significant byte first

-                       subtraction                       |   .ADDINSTR <instdef> Add instruction

*        Multiplicative multiplication                    |   .END                End of source code

/                       division                          |   =        <expr>     Alternate form of EQU

%                       modulo                            |   *=       <expr>     Alternate form of ORG

<<                      logical shift left                |------------------------------------------------------

>>                      logical shift right               | CONSTANTS:

~        Unary          bit inversion (one's complement)  |   Numeric constants: <digits><suffix> or <prefix><digits>

-                       unary negation                    |     <digits> = digit string taken from (0-9,a-f,A-F)

=        Relational     equal                             |     <suffix> = radix indicator defined as follows:

==                      equal                             |

!=                      not equal                         |     Prefix   Suffix              Radix

<                       less than                         |     -------------------------------------

>                       greater than                      |     %        b or B               2

<=                      less than or equal                |     @        o or O               8

>=                      greater than or equal             |              d or D (or nothing)  10

&       Binary          binary 'and'                      |     $        h or H               16

|                       binary 'or'                       |

^                       binary 'exclusive or'             |   Character constants: '<char>' (e.g. 'a')

Note: 16 bit values used.  Relational Ops yield 1 or 0.     |   String    constants: "<chars>" (.e.g "abc")

--------------------------------------------------------------------------------------------------------------------------

NOTES:

1. Statement format: <label> <operation> <operand> <comment>

2. The backslash can be used to separate multiple statements on a line (e.g. lda byte1 \ sta byte2).

3. '$' or '*' represent the current Instruction Counter.

4. Comments preceded by semicolon (';').

5. TASM is case insensitive for mnemonics, but not labels.

--------------------------------------------------------------------------------------------------------------------------