RESET  --  Utility to reset the Tandy 640x200x16 (TGA) video mode to CGA

Version 1.0

By Christopher J. McConnell

Email: cjm@naucse.cse.nau.edu

The purpose of RESET is to provide a way to reset the Tandy high

resolution video mode (TGA) to the lower modes without having to enter and exit

Deskmate.

The inspiration for this program came from a using Fractint and Cshow

on my Tandy 1000 TL and I couldn't switch from TGA to CGA without exiting the

program, entering Deskmate, exiting Deskmate, and re-entering the program.

So I wrote this program so I can reset my video mode without Deskmate.  In

fact because of this program I have deleted Deskmate entirely from my system.

For those interested, here is more detail on the problem.  When a third

party vendor of software figures out how to get into and use the TGA mode,

they figure that Tandy would keep their video modes standard enough for the

standard exit.  The standard exit procedure is to leave the video mode in it's

current settings, because a new video mode will reset the registers to what

is needed.  This is not the case with TGA and CGA.  To change to CGA from TGA,

one has to manually reset certain registers that CGA can't touch because these

registers are non-IBM compatible.  Thus the registers remain set for TGA and

one get's garbage on the screen when you try to use CGA after using TGA.  Tandy

did know about this side effect, and took the initiative to reset the video

upon exit of Deskmate.  This way Tandy could claim full IBM compatibility on

CGA and still have their TGA mode.  RESET restores the registers used for TGA

to settings that are compatible for CGA, so now one can see both kinds of

graphics without too much hassle.

I use RESET in two ways, the first is to have RESET in a batch file that

runs Fractint (for example) like this:

FRACT.BAT

c:

cd \games\fractint

fractint

reset

This will reset your video mode automatically every time you exit Fractint,

so when you go to play another game, your video will look the way it's supposed

to.

The second way I use RESET is after viewing a fractal or gif in TGA, I'll

select CGA mode, shell out to DOS, run RESET, go back into the program, and

see my fractal or gif in CGA.  Some programs like Fractint have a shell to DOS

function built in, some like CSHOW do not, so you may have to pick up a hot-key

program that will shell to DOS so you can shell out to do this.

Terms and Limitations:  This utility is only meant to be executed on the

Tandy 1000 series of computers.  I have not tested

it on any other series of Tandy, and since these

registers are Tandy specific then running RESET

on any other type of computer would be senseless.

Unpredictable results will happen on non-Tandy

computers.

If you find a bug in this program that I did not find,

tell me, I will fix it and re-post the program as

soon as possible.

RESET is shareware.  If you like the program then

send $5 for the use of it. There is a bit of time

and effort put into programs such as this, and this

should be recognized in some manner.

Send your registration to:

Christopher J. McConnell

492 W. Philomena Dr.

Flagstaff, Az 86001