ERRORS/PROBLEMS AND HOW TO SOLVE THEM

- First try this -

If you are using QEMM386, 386MAX or a similar memory management program and

Terminate hangs when exiting to DOS or shelling or if you have any other

problem, then try renaming your autoexec.bat and config.sys first and then

reboot the computer.

REN AUTOEXEC.BAT A.BAT <�

REN CONFIG.SYS C.SYS <�

After this, your computer is totally clean of any device drivers and other

things that might mess things up. Now run Terminate and see if it still

hangs when exiting or shelling.

When using MS-DOS 6.0 or above, you can also press the F5 key on your

keyboard to bypass AUTOEXEC.BAT and CONFIG.SYS. Refer to your DOS manual

for more detailed information.

If it works perfectly, there is a conflict with your DOS configuration. Try

installing one device-driver at a time, reboot the machine and see if you

get the error message again. If it turns out to be QEMM or other memory

manager try excluding X=B000-FFFF. This worked on a PS/2 33MHz. If this

works, try X=B000-EFFF and proceed until Terminate hangs again. Other

programs which use XMS/EMS or high memory could also have problems, if you

did not exclude the correct area. You may want to say "but all my other

programs work", but the reason for your problems could be that as Terminate

is a very advanced program, it uses all the resources it can get to

increase the performance.

- Messy screen -

If your screen messes up when moving the mouse, try installing a newer

mouse driver. Download LMOUSE61.ARJ from DAN BBS: it also works with 100

and 132 column screen mode.

- XMS problems on old 286-machines -

If you have a 286 with an old BIOS and use HIMEM.SYS, there could be some

problems. Try getting the newest HIMEM.SYS from your DOS manufacturer or

use the /NOXMS command on the Terminate command line.

- PS/2 problem -

The mouse driver MOUSE.COM will sometimes conflict with EMS when using

Terminate. The solution is not to load the mouse driver into high memory.

If you use MOUSE.EXE 9.0 from MicroSoft, use the /E parameter to force the

driver to use convential memory.

- Real time-clock problems on XT-machines -

If Terminate detects that you are using an XT-machine (8088, 80186, 8086,

NEC V20, NEC V30), the soundcard check will be turned off by default. The

reason for this is that on some machines with a special realtime card

installed, the soundcard check sets off the date/time. If you want to try

to use the sound card anyway, start Terminate with a /YESCARD parameter on

the commandline.

- Terminate stops and nothing happens after a BBS login using IEMSI -

This is a common problem which appears when using IEMSI and enabling Avatar

codes at the same time. Avatar has the problem that it uses codes which are

identical to Xon/Xoff, which is something used for software flowcontrol.

The solution is easy: Make sure that your modem only uses hardware

flowcontrol (CTS/RTS) and turn off software flowcontrol. On some modems

this is done by typing:

AT&H3  Enter

AT&W   Enter       (Save setup in modem)

Please look up these parameters in your modem manual, if Terminate should

not support your modem directly.