GETTING STARTED QUICKLY

For people who don't know much about serial communications and modems, we

provide this small document, so you don't have to worry about all the

options.

Remember always: If you don't understand an option/feature, do not change

its setting! All options have been set to sensible defaults for normal

use, and there is no immediate need to change anything - although you

will need to set up the following things:

At startup you will be asked about which colour palette you want to use.

Terminate will check if you have a colour screen and place the cursor on

"colour" if you do. Just press enter if you are not sure.

IMPORTANT: If you don't understand a toggle or an option, do not change it,

just leave as it was. All options have already been set to give

best performance for most machines.

Terminal settings: You can either let Terminate auto-configure itself for

your modem or set your modem up manually.

- If you have a modem faste than 14400 bps, you should set your bps rate to

19200 or 38400. Fast modems can use data compression and if you only use

9600, you will not be able to do fast transfers. If you are using faster

speeds like 28800 set the speed to 57600. With ISDN you will need a

fossil driver like cFos.

- For normal BBS calls you should always use 8 databits, no parity, 1

stopbit.

- If you use 2400, you should disable Hardware RTS/CTS flowcontrol in the

Terminal options menu. We always recommend that 9600 and faster modems

should use Hardware RTS/CTS flowcontrol, which is the safest way of

sending data when using a speed to the comport at 19200 or above.

- Don't ever run 57600 bps on external modems, unless you are sure that

your cable is protected from noise - try 38400 first. Remember that you

must also set RTS/CTS flowcontrol in your internal modem configuration -

check in your modem manual.

- RTS/CTS flowcontrol requires an internal modem OR an external modem AND a

proper modem cable which has the RTS and CTS (Request to send/Clear to

send) pins connected.

When you are finished with the basic configuration, you can move to the

Modem & Dialing Menu and select a modem from Quick Install. If your modem

is not featured in the list, you must set the following things manually -

or at least check if they work (read your modem manual to check for further

details. Remember that the factory defaults on your modem may be set in

such a way that you won't have to change anything...):

Carrier Detect (CD) follows line (usually &C1):

- Type AT&C1 and press enter

When DTR is low, the line must be dropped, (Usually &D2 or &D3):

- Type AT&D2 and press enter

Not all modems like this, if you can't hangup your modem, move to the

Modem & dialing menu and Enter a 0 at the hang-up options which will

allow Terminate to send a hangup string to your modem instead.

Type ATS0=0

Then your modem will not pick up the phone - so Terminate can make the

decision.

If you can save your setup in your modem, type AT&W, otherwise you should

use this initstring: AT&C1&D2S0=0^M

If your modem string is not sent because the carrier is high when you start

up Terminate, you should enable the FORCE INITSTRING switch in the Options

menu under "Modem And Dialing".

If your modem is standard Hayes compatible you don't need to set up

anything else, everything has been set up already.

You could now press Alt-D for the phonebook and import or enter some

entries - then you are ready to enter the world of CyberSpace!