Chapter 1. Introduction

These release notes provide you with information about the 1.42 VESA TSR

(Terminate and Stay Resident) software. The VESA TSR, an extension to the

Power 9100 BIOS software, is a DOS TSR which supports some VESA extended

modes that are not supported by the basic VGA BIOS: specifically, extended

modes 110h to 11Bh. It is necessary to install the TSR only if you plan

to use these extended modes via a DOS application.

Chapter 2 describes the VESA TSR software.

Chapter 3 describes known bugs, miscellaneous annotations, and  product

limitations.

1.1. Release Contents

_________________________________________________________________________

Item							Release Version

_________________________________________________________________________

One 31/2-inch high-density diskette			1.42 Production

containing the Power 9100 VESA TSR software

_________________________________________________________________________

Figure 1. Release contents

1.2. Enhancements and Changes Since the Last Release

The following enhancements and changes have been made since the last release

(release 1.00):

1.  Support for Bt489, ATT20C511, and RGB514 DACs has been added.

2.  Release notes have been added to the product package.

3.  The release number has been stepped up so that it matches its companion

Power 9100 BIOS software release number.

1.3. Bugs and Problems Corrected Since the Last Release

The following bugs and problems (specified as originally reported) in the

1.00 VESA TSR release have been fixed or closed in this release:

1.  Fixed: If EMM386 is loaded, the system hangs during loading of the

VESA TSR.

2.  Fixed: SetGetVideoMemoryWindowControl in ModeInfoBlock must be a FAR

call.

3.  Fixed: The higher-order word of 32-bit registers was not

preserved during set mode.

4.  Fixed: Systems with the RGB514 DAC may hang during loading of the VESA

TSR.

5.  Fixed: The get/set window control function (05h) does not return the

correct window number in register DX when the get function is called; applied

to modes 110h and 11Bh.

Chapter 2. Power 9100 VESA TSR Software

2.1. VESA TSR Software Diskette Contents

The distribution diskette contains the files listed in figure 2.

____________________________________________________

File		Description

P91VESA.COM	VESA TSR executable file

RELNOTES.DOC	These release notes in ASCII format

____________________________________________________

Figure 2. BIOS diskette files

2.2. Supported RAMDACs, VESA Functions, and Extended SVGA Modes

2.2.1. Supported RAMDACs

The TSR supports the following RAMDACs or compatibles:

Brooktree Bt485 or Bt489

IBM RGB514 or RGB525

AT&T ATT20C511

2.2.2. Supported VESA Functions

The TSR supports all VESA functions except those listed in figure 3.

_______________________________________________________

Function	Description

_______________________________________________________

6		Set/get logical scan line length

7		Set/get display start

8		Set/get DAC palette format

_______________________________________________________

Figure 3. VESA functions not supported by the VESA TSR

2.2.3. Supported Extended SVGA Modes

Figure 4 summarizes the extended SVGA modes supported by the TSR.

_______________________________________________________________________________

OEM      VESA     Mode        Reso-     Colors   Memory   Pixel  Hor.   Vert.

Mode     Mode     Type        lution             Address  Clock  Freq.  Freq.

(7-bit)  (15-bit)                                         (MHz)  (KHz)  (Hz)

_______________________________________________________________________________

6C      110      Graphics    640x480   15-bit   A000     25.17  31.5    60

(32K)

6E      111      Graphics    640x480   16-bit   A000     25.17  31.5    60

(64K)

69      112      Graphics    640x480   24-bit   A000     25.17  31.5    60

(16M)

6D      113      Graphics    800x600   15-bit   A000     36     37.8    60

(32K)

6F      114      Graphics    800x600   16-bit   A000     36     37.8    60

(64K)

70      115      Graphics    800x600   24-bit   A000     36     37.8    60

(16M)

71      116      Graphics   1024x768   15-bit   A000     65     48.3    60

(32K)

72      117      Graphics   1024x768   16-bit   A000     65     48.3    60

(64K)

73      118      Graphics   1024x768   24-bit   A000     65     48.3    60

(16M)

74      119      Graphics   1280x1024  15-bit   A000    115     65.3    60

(32K)

75      11A      Graphics   1280x1024  16-bit   A000    115     65.3    60

(64K)

76      11B      Graphics   1280x1024  24-bit   A000    115     65.3    60

(16M)

_______________________________________________________________________________

Figure 4. Extended SVGA modes supported by the TSR

2.3. Installing the TSR

To install the TSR:

1.  Insert the VESA TSR distribution diskette into either diskette drive A:

or diskette drive B:.

2.  At the DOS prompt, enter the command:

A:\P91VESA.COM

or

B:\P91VESA.COM

When installation is complete, the program displays a message verifying

that the TSR is installed. If an attempt is made to install the TSR when

it has already been installed, the program displays a message saying that

the TSR is already installed.

Chapter 3. Known Bugs, Notes, and Limitations

3.1. Power 9100 VESA TSR Bugs

1.  Mouse Systems Mouse Driver Ver. 7.01 causes an incompatibility with

VESA modes 110h and higher.

3.2. Notes

See the Power 9100 BIOS Software Release Notes for more information about

the BIOS and other VESA mode support.

The Bt485 does not support the 24-bit modes (112h, 115h, 118h, and

11Bh).